3GPP TSG RAN WG1 #104-e																		R1-2101469

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.5.2

Source: 	Qualcomm Incorporated

Title: 	Potential Enhancements on UL-AOA positioning

Document for:		Discussion and Decision

# Introduction

At RAN #90-e meeting, a new WID on NR Positioning Ennhancements was approved [1] with the following RAN1 centric objectives:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1] - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1] - UL AoA for network-based positioning solutions. - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions. Note: RAN1 will discuss the candidate solutions and provide updates for this objective, with status to be reviewed in RAN#91e.

In this paper, we present our views on potential enhancements related to enhancements on UL AoA positioning.

# UL AoA Enhancements

NR Positioning Protocol A standard (TS 38.455) in 5G specifies information element formats for a gNB to report positioning related measurements to a location server. Specifically, the ‘measurement report’ information element includes options to report the SRS RSRP and gNB-side Rx-Tx time difference.

TRPMeasurementQuantities-Item ::= ENUMERATED { gNB-RxTxTimeDiff, uL-SRS-RSRP, uL-AoA, uL-RTOA, ... }

We observe that a gNB may only report a single AoA, RSRP and timing measurement to the LMF. However, in several scenarios, such a report may appear to be limited, and further enhancements should be considered to improve performance.

Specifically, for the scenario of UMI 4 GHz case (with absolute time of arrival modelled), significant gains are expected when a TRP can report multiple AoAs and, for each AOA, multiple Timing measurements and their corresponding relative powers, to the LMF.

Note that in the UMI with absolute time of arrival modelled, for a NLOS link, neither the angles nor the earliest time of arrivals are physically consistent , whereas for the LOS links, there are always NLOS components which can make the estimation of the AoA a rather difficult job. For example, in the figure shown below, we plot a Power Delay/Angle Heatmap of one of the channels. From such a heatmap it is clear that there is a lot of information that can be extracted and Reporting to the location entity multiple AoA and timing measurements would be helpful in advanced fusing algorithms.

Proposal 1: Support a gNB to report multiple tuples (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) within a single report, such that

- The UL-RSRP corresponds to a relative RSRP associated to the reported path in the angle/delay domain

- The RTOA/gNB-Rx-Tx corresponds to the delay of the associated reported path in the angle/delay domain

- The UL-AoA corresponds to the received angle (potentially 2-dimensional) of the associated reported path in the angle/delay domain

- FFS: Max number of  (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) tuples that can be sent in a single report

# Conclusions

Overall, we make the following proposals on beneficial enhancements to be specified during NR Rel-17 WI phase:

Proposal 1: Support agNB to report multiple tuples (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) within a single report, such that

- The UL-RSRP corresponds to a relative RSRP associated to the reported path in the angle/delay domain

- The RTOA/gNB-Rx-Tx corresponds to the delay of the associated reported path in the angle/delay domain

- The UL-AoA corresponds to the received angle (potentially 2-dimensional) of the associated reported path in the angle/delay domain

- FFS: Max number of  (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) tuples that can be sent in a single report

# References

- RP-202900, “New WID on NR Positioning Enhancements”, CATT, Intel Corporation, Ericsson; 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 - 11, 2020.