3GPP TSG RAN WG1 #104-e	                         			 R1-2101503

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.7.1.1

Source:	InterDigital, Inc.

Title:	Paging enhancements for UE power saving

Document for:	Discussion

# Introduction

In RAN1 #103-e, evaluation of several paging enhancement techniques for idle/inactive UEs was performed and it was agreed to support paging early indication:

Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

- FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS.

- Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact.

In this contribution, we discuss and compare DCI based and sequence based paging early indication.

# Discussion

## PDCCH-based paging indication:

The PDCCH based paging indication can use a new DCI format or repurpose an existing format such as DCI format 2_6. Each UE may monitor a bit field within the DCI where the bit field indicates to the UE whether to monitor an associated paging occasion (PO) or not. The index of the bitfield can be determined by the UE using the UE ID.

Supporting a separate paging indication for distinct UE groups can be easily achieved with the PDCCH based paging indication by adding bitfields to the DCI, potentially at the expense of some degradation in BLER performance. In addition, if supported, the bitfield can contain separate indications for multiple POs.

.

## Sequence-based indication

The paging indication can also be based on a sequence instead of a PDCCH. For example, an SSS based paging indication signal may be written as

wn=1-2x0n+c0modN[1-2x1n+c1modN]

where n is the sequence sample index, N is the sequence length;  x0(n) and x1(n)  characterize the generator polynomials and c0, c1 are the cyclic shift parameters. The UE may compare the received signal to the indication sequence, e.g., by correlating the received signal with the sequence, and determine to monitor an associated PO if the correlation value is above a threshold. The indication sequence may be generated by setting one or both cyclic shift parameters as functions of the UE ID.

## Evaluation results

The misdetection performance of PDCCH-based and sequence-based paging indication has been compared using link level simulations. For the PDCCH-based indication, a payload of 12 bits is assumed, and aggregation levels 8 and 16 are simulated. For the sequence-based indication, the 127-length SSS sequence is used and the false alarm rate is set to 1%. Note that the number of REs used for the sequence is 127 while it is 576 and 1152 for ALs 8 and 16, respectively.

The misdetection performance is provided in Figure 2-1. As we can see from the figure, both the PDCCH-based and the sequence-based indication has acceptable misdetection performance.

Figure 2-1 Misdetection performance of PDCCH-based and sequence-based paging indication

## Discussion

One aspect that needs to be considered is the capacity of the paging indication. The overhead of the PDCCH-based indication is dictated by the AL and stays the same regardless of the number of UE groups supported (given that the DCI size is not increased beyond a point as to degrade the misdetection performance). Since the paging indication needs to be detectable by the lowest SNR UEs in a cell, with the lowest SNR, an AL of at least 8 may be necessary.

The overhead of the sequence-based indication, however, depends on the number of UE groups. For example, for 1 UE group, only one sequence is needed, resulting in an overhead of 127 subcarriers. On the other hand, assuming an overhead of 576 REs similar to the PDCCH with AL8, at most 4 UE groups can be supported, which is less than the capacity of the PDCCH-based indication.

Another aspect to consider is the detection complexity: the complexity of the sequence-based indication is less than that of PDCCH-based indication. In terms of specification effort, there does not seem to be much difference between the two techniques: The search space for the indication channel/signal should be specified regardless of the method adopted. In addition, the DCI format for the PDCCH-based indication needs to be specified while for the sequence-based method, the details of the sequence design need to be addressed. However, since it has been agreed to use either SSS or TRS sequences, this work is not expected to demand a big effort.

Based on the above discussion, we propose the following:

Proposal 1: Sequence-based paging early indication is adopted for Rel-17.

# Summary

In this contribution, we have discussed paging early indication and have made the following proposal:

Proposal 1: Sequence-based paging early indication is adopted for Rel-17.

# References

Chairman’s Notes, 3GPP WG1 #103-e

# Appendix

Table 1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C 300ns |
| Antenna number at BS | 1 |
| Antenna number at UE | 1 |
| Subcarrier spacing | 30 kHz |
| Sequence | SSS |
| PDCCH | Aggregation level: 8 or 16 DCI size: 12 bits, 24-bit CRC |