3GPP TSG RAN WG1 #104-e	R1-2101530

e-Meeting, Janurary 25th – Feburary 5th, 2021

Source:	Sharp

Title:	Correction to FDRA field description

Agenda Item:	7.2.2

Document for:	Discussion and Decision

Correction to FDRA size in DCI format 0_0 CRC scrambled by C-RNTI or TC-RNTI

The latest specification TS38.212V16.4.0 [1] specifies size determination of frequency domain resource assignment (FDRA) field in DCI format 0_0 with CRC scrambled with C-RNTI as below. The current procedure produces 0 bits for FDRA field unless neither of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured. This is because the second “if” statement highlighted as below doesn’t work since the first “if” statement is at higher indent level than the second one. The same issue exists in DCI format 0_0 with CRC scrambled by TC-RNTI. In the next section, we propose a correction for FDRA size in DCI format 0_0 with CRC scrambled by C-RNTI and TC-RNTI.

The following is a copy of FDRA field description for a DCI format with CRC scrambled by C-RNTI in TS38.212V16.4.0.

- Frequency domain resource assignment – bits if neither of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - if any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5+Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. - 6+Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. If the DCI format 0_0 is monitored in a UE-specific search space, the value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12 where NRB-set,ULBWP is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214]. If the DCI 0_0 is monitored in a common search space Y = 0.

The following is a copy of FDRA field description for a DCI format with CRC scrambled by TC-RNTI in TS38.212V16.4.0.

- Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz

  - Text proposal

Text proposal#1 --------- beginning of text proposal for TS 38.212 ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if neither of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5+Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. - 6+Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. If the DCI format 0_0 is monitored in a UE-specific search space, the value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12 where NRB-set,ULBWP is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214]. If the DCI 0_0 is monitored in a common search space Y = 0. - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] -------- Unchanged contents are omitted The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] -------- Unchanged contents are omitted --------- end of text proposal

Correction to applicability of the procedure regarding resource allocation ‘dynamicSwitch’

In the latest specification TS38.212V16.4.0 [1] for DCI format 0_1 there is a procedure on how the UE should interpret the FDRA field if the "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part for the case when the parameter resourceAllocation is configured as 'dynamicSwitch'. In the current spec this paragraph appears as a sub-bullet under the condition that interlacing is configured which is not correct; 'dynamicSwitch' is only relevant when interlacing is not configured. In RAN1 there was no agreement on dynamic switching between Type 2 and the other resource allocation types (Type 0 and Type 1). To fix this, the highlighted paragraph below can be moved such that it is a sub-bullet under the condition that interlacing is NOT configured.

- Frequency domain resource assignment – number of bits determined by the following, where is the size of the active UL bandwidth part: - If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured - bits if only resource allocation type 0 is configured, where is defined in Clause 6.1.2.2.1 of [6, TS 38.214], - bits if only resource allocation type 1 is configured, or bits if resourceAllocation is configured as 'dynamicSwitch'. - If resourceAllocation is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1. - For resource allocation type 0, the LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214]. - For resource allocation type 1, the LSBs provide the resource allocation as follows: - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. - 6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12 where NRB-set,ULBWP is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214]. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocation is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

  - Text proposal

Text proposal#2 --------- beginning of text proposal for TS 38.212 ##### 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213]. - DFI flag – 0 or 1 bit - 1 bit if the UE is configured to monitor DCI format 0_1 with CRC scrambled by CS-RNTI and for operation in a cell with shared spectrum channel access. For a DCI format 0_1 with CRC scrambled by CS-RNTI, the bit value of 0 indicates activating type 2 CG transmission and the bit value of 1 indicates CG-DFI. For a DCI format 0_1 with CRC scrambled by C-RNTI/SP-CSI-RNTI/MCS-C-RNTI and for operation in a cell with shared spectrum channel access, the bit is reserved. - 0 bit otherwise; If DCI format 0_1 is used for indicating CG-DFI, all the remaining fields are set as follows: - HARQ-ACK bitmap – 16 bits, where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 indicates NACK. - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213] - All the remaining bits in format 0_1 are set to zero. Otherwise, all the remaining fields are set as follows: - UL/SUL indicator – 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1. - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where - if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id; - otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1; If a UE does not support active BWP change via DCI, the UE ignores this bit field. - Frequency domain resource assignment – number of bits determined by the following, where is the size of the active UL bandwidth part: - If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured - bits if only resource allocation type 0 is configured, where is defined in Clause 6.1.2.2.1 of [6, TS 38.214], - bits if only resource allocation type 1 is configured, or bits if resourceAllocation is configured as 'dynamicSwitch'. - If resourceAllocation is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1. - For resource allocation type 0, the LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214]. - For resource allocation type 1, the LSBs provide the resource allocation as follows: - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocation is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part. - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured - 5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. - 6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation. The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12 where NRB-set,ULBWP is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214]. - Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Adopt Text Proposal#1

Proposal 2: Adopt Text Proposal#2

References

- 3GPP TS38.212V16.4.0, December 2020