3GPP TSG-RAN WG1 Meeting #104-e                         R1-2101534

e-Meeting, January 25th – February 5th, 2021

Source:	Sharp

Title:	Remaining issues on synchronization mechanism for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views on remaining issues on synchronization mechanism for NR sidelink.

Discussion

Indication of the non-TDD case in sl-TDD-Config

In the latest version of TS 38.213 indication of non-TDD case in sl-TDD-Config is captured in the following yellow highlighted text.

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots. For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1'; else - a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 […]

In case of shared SL carrier and SUL carrier, tdd-UL-DL-ConfigurationCommon is provided in the cell, but it is for the non-SUL carrier, and cannot be used to derive the 12 bits in sl-TDD-Config. Therefore, this should be added as another “non-TDD” case.

A TP is proposed as follows,

-------------------------------------------- Start of TP -------------------------------------------

< Unchanged parts are omitted >

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots.

For paired spectrum or supplementary uplink band, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1],

-	a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1';

else

-	a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1

< Unchanged parts are omitted >

-------------------------------------------- End of TP --------------------------------------------

Conclusion

In this contribution, we discuss remaining issues on synchronization for NR sidelink, and propose a TP for TS 38.213 to add the case of shared SL carrier and SUL carrier as another “non-TDD” case in deriving sl-TDD-Config.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.