3GPP TSG RAN WG1 #104-e	R1-2101537

e-Meeting, January 25th – February 5th, 2021

Source:	Sharp

Title:	Enhancements on Multi-TRP PUSCH

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

# Background

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed [1] and the revised WID was agreed at RAN#89-e [2]. In the WID, the following objectives were agreed for multi-TRP enhancement.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1#103-e meeting, some agreements were made for PUSCH enhancement. In this contribution, we share our views on PUSCH enhancement for MTRP operation.

# Discussions

In the last meeting, the following agreements were made.

Agreement For single DCI based M-TRP PUSCH repetition schemes, support codebook based PUSCH transmission with following enhancements. - Support the indication of two SRIs. - Alt1: Bit field of SRI shall be enhanced. - Alt2: No changes on SRI field - Support the indication of two TPMIs. - The same number of layers are applied for both TPMIs if two TPMIs are indicated - The number of SRS ports between two TRPs should be same. - FFS: Details on indicating two TPMIs (e.g, one TPMI field or two TPMI fields) - Increase the maximum number of SRS resource sets to two - FFS: configuration details of each SRS resource set (e.g., number of SRS resources in a resource set) Agreement For single DCI based M-TRP PUSCH repetition schemes, support non-codebook based PUSCH transmission with following considerations. - Increase the maximum number of SRS resource sets to two, and associated CSI-RS resource can be configured per SRS resource set. - FFS: Enhancements on SRI field in DCI to indicate the two beams for repetitions Agreement For single DCI based M-TRP PUSCH repetition Type B, at least nominal repetitions are used to map beams - Further study details and applicability of each mapping method - Further study the slot based beam mapping in the cases of nominal repetition across slot boundaries Agreement For PUSCH multi-TRP enhancements, - For per TRP closed-loop power control for PUSCH, further study the following alternatives when the “closedLoopIndex” values are different. - Option.1: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for both PUSCH beams - Option.2: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for one of two PUSCH beams at a slot. - Option 3: A second TPC field is added in DCI formats 0_1 / 0_2. - Option 4: A single TPC field is used in DCI formats 0_1 / 0_2, and indicates two TPC values applied to two PUSCH beams, respectively. - FFS: Transition period for beam / power / frequency change. Agreement Support both type 1 and type 2 CG PUSCH transmission towards MTRP. Further study the following alternatives, - Alt.1 : single CG configuration - Repetitions of a TB transmitted towards MTPR on multiple PUSCH transmission occasions of single CG configuration. - At least for codebook-based CG PUSCH, support configuring 2 SRIs/TPMIs. - Alt.2 : multiple CG configurations - Repetitions of a TB transmitted towards MTRP on more than one PUSCH transmission occasions, where one or more transmission occasions are from one CG configuration and another one or more PUSCH transmission occasions are from another CG configuration. - 1 SRI/TPMI is configured/indicated for each CG configuration. - Further study required beam mapping principals, low overhead mechanisms for beam selection, and other enhancements for Alt.1 and Alt.2. Agreement For single DCI based PUSCH multi-TRP enhancements, support the following RV mapping for PUSCH repetition Type A, - DCI indicates the first RV for the first PUSCH repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the second TRP (The same method as PDSCH scheme 4) - FFS: Reuse of the same method for PUSCH repetition Type B. Agreement For single DCI based M-TRP PUSCH repetition Type A and B, further study required enhancements on PTRS-DMRS association. Working Assumption For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) - Companies are encouraged to provide further simulation results to decide details.

For the indication of the two SRIs, the following two alternatives has been agreed

- Alt1: Bit field of SRI shall be enhanced.

- Alt2: No changes on SRI field

In our view, each Tx beam can be flexibly indicated, so Alt. 1 is preferred. In addition, it is unclear what is not changed in Alt. 2, e.g. bit size of SRI field is not changed, ….

Proposal 1: For the indication of the two SRIs, Alt1 (Bit field of SRI shall be enhanced) is preferred.

For per TRP closed loop power control, the following four options are listed:

- Option.1: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for both PUSCH beams

- Option.2: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for one of two PUSCH beams at a slot.

- Option 3: A second TPC field is added in DCI formats 0_1 / 0_2.

- Option 4: A single TPC field is used in DCI formats 0_1 / 0_2, and indicates two TPC values applied to two PUSCH beams, respectively.

In our view, option 3 and option 4 are preferred because there is restriction about the closed loop TPC index applied to each TPC loop when we take option 1 and option 2. We slightly prefer option 3 to have full flexibility.

Proposal 2: For per TRP closed loop power control, Option 3 (A second TPC field is added in DCI formats 0_1 / 0_2) is preferred.

# Conclusion

In this contribution, we have the following observation and proposals:

Proposal 1: For the indication of the two SRIs, Alt1 (Bit field of SRI shall be enhanced) is preferred.

Proposal 2: For per TRP closed loop power control, Option 3 (A second TPC field is added in DCI formats 0_1 / 0_2) is preferred.

# References

- RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN#86, December 2019

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, RAN#89-e, September 2020.