3GPP TSG RAN WG1#104-e	 	 	R1-2101633

e-Meeting, January 25th – February 5th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on multi-cell PDSCH scheduling via a single DCI for NR DSS

Agenda Item:	8.13.2

Document for: 	Discussion and Decision

- Introduction

In this contribution, we present our view on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI for Rel-17 NR DSS.

- Single DCI scheduling PDSCH on multiple cells

In Rel-15, dynamic spectrum sharing (DSS) was introduced to operate NR serving cell on overlapping frequency with LTE cell. DSS enables NW operators to provide NR service area together with current LTE area by reusing LTE frequency band. If the self-carrier scheduling is used for every serving cell in CA operation, the UE is required to monitor PDCCH on every serving cell. To reduce PDCCH monitoring effort and to increase available resources for PDSCH/PUSCH on some of serving cells, cross-carrier scheduling is supported in NR similar to LTE.

In general, the number of available PDCCH resources for NR UE on serving cell operating DSS is limited since NR resources have to avoid LTE resources (e.g. PDCCH, CRS) on the serving cell. In particular, only 1 or 2 symbols per slot may be available for NR PDCCH on the serving cell operating DSS. The current scheduling mechanism supports a single DCI scheduling PDSCH on a single cell. If a single DCI scheduling PDSCH on multiple cells is supported, PDCCH resource efficiency can be improved if the DCI size is sufficiently less than the size of two separate DCIs scheduling PDSCH on a single cell respectively. Hence, we are slightly supportive for the enhancement (i.e. a single DCI scheduling PDSCH on multiple cells).

Observation 1:

- PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI can improve PDCCH resource efficiency.

- Considerable DCI fields for assumed scenarios

At the previous meeting, whether single DCI scheduling PDSCH on multiple cells enhancement provides performance gain or not was discussed. Prior to the discussion based on simulation results, targeting scenarios of single DCI scheduling PDSCH on multiple cells were agreed as follows [1]:

Agreements:

- For the study on single DCI scheduling PDSCH on two cells

  - Consider the following scenarios as baseline for evaluation

    - UE configured with Inter-band CA with PCell and an SCell

      - PCell for the UE is operated on a DSS carrier (i.e.,  same carrier is also used for serving LTE users)

      - Case 1: Different SCS for PCell and SCell

      - Case 2: Same SCS for PCell and Scell

  - Additional scenarios can also be evaluated, e.g. as below

  - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers)

  - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers)

  - Note: other combinations not precluded

- Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

Here, we discuss the considerable fields in single DCI scheduling PDSCH on multiple cells for the agreed scenarios. In single DCI scheduling PDSCH on multiple cells scenario, we think CRC field attached to DCI (i.e. 24-bit) can be shared between the scheduled multiple cells (i.e. two CRC fields are not needed) and the CRC is scrambled by UE unique (common between multiple scheduled cells) RNTI.

Observation 2:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), CRC field attached to DCI (i.e. 24-bit) can be shared between the scheduled multiple cells.

In case a UE is configured with Inter-band CA with PCell (DSS carrier) and an SCell, PDCCH can be allocated to either PCell or SCell in order to schedule PDSCH on PCell and/or SCell. It means the UE needs to assume not only cross-carrier scheduling but also self-carrier scheduling. If single DCI scheduling PDSCH on multiple cells has 3-bit carrier indicator field (CIF) for each scheduled cells, total 6-bit is required and it’s not necessary at least for the agreed scenario since there are only two possible scheduled cells in the scenario. Therefore, 3-bit CIF in total may be reasonable assumption for single DCI scheduling PDSCH on multiple cells even in scenarios with more SCells.

Observation 3:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), 3-bit CIF may be reasonable assumption for single DCI scheduling PDSCH on multiple cells.

In case of inter-band CA, the channel conditions of PCell and SCell may be different. Considering the principle of HARQ that TB mapping is within a CC, we can consider the number of transport blocks (TB) scheduled by single DCI on multiple cells is two so that HARQ processes can be performed individually on CCs with potentially different channel conditions. On the other hand, if single TB allocation across multiple cells is allowed e.g., for the case that channel conditions of both cells are good, it may be able to optimize DCI size as well as feedback size.

Observation 4:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), at least the case where the number of PDSCH TBs on multiple cells scheduled by single DCI is two can be considered.

In the agreed scenario, we need to consider both different SCS case and same SCS for PCell and SCell. If Time domain resource assignment field (i.e. the maximum size is 4-bit) is shared between the scheduled cells, the time domain resource of PDSCH depends on scheduled cell’s SCS configuration. Hence, it may be better to separate Time domain resource assignment field for each scheduled cell.

Observation 5:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Time domain resource assignment field for each scheduled cell.

DSS frequency band (i.e. PCell in the assumed scenario) and NR frequency band can be operated on different bandwidth. Considering the different availability of frequency resource, we think Frequency domain resource assignment field should be separated for each scheduled cell.

Observation 6:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Frequency domain resource assignment field for each scheduled cell.

Moreover, we need to discuss whether/how to support some indications in DCI for multiple scheduled cells. For example, there are rate matching indicator, BWP indicator, CSI request and SRS request.

Observation 7:

- Whether/how to support some indications in DCI for multiple scheduled cells can be considered.

  - e.g. rate matching indicator, BWP indicator, CSI request and SRS request

Regarding DCI size, we need to discuss how to determine the size of DCI scheduling PDSCH on multiple cells. In the current specification, the DCI size is determined based on PDSCH-Config configured for the scheduled serving cell. To support a single DCI scheduling PDSCH on multiple cells, how to determine the DCI size needs to be clarified.

Observation 8:

- How to determine the size of DCI scheduling PDSCH on multiple cells can be considered.

- Other potential discussion points

If a single DCI scheduling PDSCH on multiple cells is introduced in Rel-17, we need to discuss and specify some points to determine UE behavior.

In the current specification, NW can indicate a scheduled cell by using carrier indicator field (CIF) in DCI to the UE. CIF has 0 or 3 bits to indicate a scheduled cell from up to 8 configured cells. However, the current CIF does not support indication of multiple scheduled cells. Therefore, we need to discuss how to indicate the scheduled cells by using a single DCI. In addition, we may need to discuss whether/how to support dynamic switching between scheduling a single cell and scheduling multiple cells.

Observation 9:

- How to indicate the scheduled cells by using a single DCI to the UE can be considered.

- Whether/how to support dynamic switching between scheduling a single cell and scheduling multiple cells can be considered.

Regarding scheduled TBs on multiple cells, we can consider three possible scenarios as shown in Fig.1. Depending on the scenario, the required information in DCI may be different. Therefore, we need to discuss whether the same TB and/or different TBs is/are scheduled on multiple cells by using a single DCI. If scenario(C) in Fig.1 is supported, we additionally need to discuss how to combine the scheduled PDSCH on multiple cells.

Fig.1: Potential scenarios for a single DCI scheduling PDSCH on multiple cells

Observation 10:

- Whether the same TB and/or different TBs is/are scheduled on multiple cells can be considered.

Regarding scheduled PDSCHs, when we consider single DCI scheduling PDSCH on multiple cells, there are two potential options as shown in Fig.2.

Fig.2: Potential scheduling options for a single DCI scheduling PDSCH on multiple cells

Option.1 schedules PDSCHs on self-carrier and one cross-carrier via single DCI. On the other hand, option.2 schedules PDSCHs on two cross-carriers via a single DCI. In order to achieve high scheduling flexibility, we prefer to support both options if multi-cell PDSCH scheduling via single DCI is supported.

Proposal 1:

- The following both scheduling options should be supported if multi-cell PDSCH scheduling via single DCI is supported.

  - Option.1: cross-carrier and self-carrier scheduling PDSCHs via a single DCI

  - Option.2: only cross-carrier scheduling PDSCHs via a single DCI

- Conclusion

In this contribution, we discussed on how to support multi-cell PDSCH scheduling via a single DCI for Rel-17 NR DSS. Based on the discussion, we made following observations and proposal.

Observation 1:

- PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI can improve PDCCH resource efficiency.

Observation 2:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), CRC field attached to DCI (i.e. 24-bit) can be shared between the scheduled multiple cells.

Observation 3:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), 3-bit CIF may be reasonable assumption for single DCI scheduling PDSCH on multiple cells.

Observation 4:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), at least the case where the number of PDSCH TBs on multiple cells scheduled by single DCI is two can be considered.

Observation 5:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Time domain resource assignment field for each scheduled cell.

Observation 6:

- In the assumed scenario (e.g. Inter-band CA with PCell (DSS carrier) and an SCell), it may be better to separate Frequency domain resource assignment field for each scheduled cell.

Observation 7:

- Whether/how to support some indications in DCI for multiple scheduled cells can be considered.

  - e.g. rate matching indicator, BWP indicator, CSI request and SRS request

Observation 8:

- How to determine the size of DCI scheduling PDSCH on multiple cells can be considered.

Observation 9:

- How to indicate the scheduled cells by using a single DCI to the UE can be considered.

- Whether/how to support dynamic switching between scheduling a single cell and scheduling multiple cells can be considered.

Observation 10:

- Whether the same TB and/or different TBs is/are scheduled on multiple cells can be considered.

Proposal 1:

- The following both scheduling options should be supported if multi-cell PDSCH scheduling via single DCI is supported.

  - Option.1: cross-carrier and self-carrier scheduling PDSCHs via a single DCI

  - Option.2: only cross-carrier scheduling PDSCHs via a single DCI

References

- 3GPP RAN1#102-e, Chairman’s Notes, Aug. 2020.

- 3GPP RAN1#103-e, Chairman’s Notes, Nov. 2020.