3GPP TSG RAN WG1 #104-e			R1-2101634

e-Meeting, January 25th – February 5th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on efficient activation/deactivation mechanism for SCells

Agenda Item:	8.13.3

Document for: 	Discussion and Decision

- Introduction

At the RAN1#103-e meeting, the techniques for efficient SCell activation/deactivation were discussed and RAN1 made following agreements [1].

Agreements: As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS - FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214 - “2-slot with four CSI-RSs resources (4 samples)” for FR1 - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2 - The working assumption can be confirmed after RAN4 check. (A LS for such request is planned). Agreements: For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e - Alt 1: the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell. - FFS detailed design of this integrated triggering signaling. - Potential examples of single triggering signaling for further discussions - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers - A DCI for both triggers - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS. - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well - Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation - FFS detailed design of separate triggering signaling. - Potential examples of separate triggering signaling for further discussions - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS - Note: temporary RS should be triggered by DCI or MAC-CE. - Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation. - FFS handling of SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered Draft LS (R1-2009786) to RAN4 is approved. Final LS in R1-2009798.

In this contribution, we discuss efficient activation/deactivation mechanism for SCells.

- Discussion

According to the WID, the objective of the WI in RAN1 is to support efficient SCell activation/deactivation based on RAN1 leading mechanism. In Rel-16, the dormant BWP was introduced, and L1-based indication for the switching to/from the dormant BWP was supported, i.e., the indication in DCI format 0_1/1_1/2_6. On the other hand, SCell activation/deactivation is still based on MAC CE indication or timer-based mechanism, and also SCell activation delay could be long due to e.g., long SSB periodicity. Thus, fast SCell activation should be considered. In addition, even if the dormant behaviour is introduced, SCell would enter deactive state after long duration with no traffic, and fast SCell activation from deactive state is still beneficial.

  - Fast SCell activation

According to TS38.133, upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as an example is shown in Fig.1. THARQ is the duration between DL data transmission and ACK. Tactivation_time is SCell activation delay for time/frequency tracking, AGC setting, MAC CE processing and so on, which depends on the SCell measurement cycle and whether the SCell is known or not. TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Figure 1: Example of SCell activation delay

- Known cell and unknown cell

At the previous meeting, it was taken as a working assumption that at least for the case of known cell, temporary RS which has the functionalities of AGC setting and time/frequency tracking is supported during the SCell activation procedure for efficient SCell activation for both FR1 and FR2. Based on this, for the case of known cell, Tactivation_time is enough to include only 3ms for MAC CE processing time and TTRS, i.e., the time to the arrival of temporary RS, instead of TFirstSSB. Thus, the SCell activation delay can be reduced as TTRS can be shorter than TFirstSSB. On the other hand, for the case of unknown cell, cell search including PSS/SSS and MIB acquisition is needed in addition to AGC setting and time/frequency tracking. In other words, according to the current working assumption, the SSB is still needed and the temporary RS cannot cover all functionalities of the SSB during SCell activation procedure. However, the temporary RS can provide at least the functionalities of AGC setting and time/frequency tracking, and hence the SCell activation delay can be reduced well even for the case of unknown cell. Specifically, four SMTC periodicity included in the SCell activation delay would be roughly reduced to two SMTC periodicity plus TTRS. Therefore, even for the case of unknown cell, the support of temporary RS is beneficial. For other functionalities, e.g., CSI measurement/acquisition and cell search, further study is needed.

Proposal 1: For both cases of known cell and unknown cell, the temporary RS should be supported as functionalities of at least AGC setting and time/frequency tracking for both FR1 and FR2.

- FFS potential functionalities of CSI measurement/acquisition and cell search

- Indication on SCell activation and triggering temporary RS

At the last meeting, it was agreed to support at least the integrated indication between triggering SCell activation and triggering temporary RS. Considering the existing indication, i.e., SCell activation via MAC-CE and triggering aperiodic RS via DCI, there are two alternatives of MAC-CE or DCI for the integrated indication. The integrated indication via DCI can additionally reduce the MAC CE processing delay, and existing indication of triggering aperiodic RS can be considered as a baseline.

Proposal 2: Triggering SCell activation and triggering the temporary RS can be integrated into a DCI.

Even if triggering SCell activation and triggering temporary RS are integrated, depending on RAN4 feedback, i.e., how many symbols of TRS are needed, additional separated indication of triggering temporary RS might be needed.

Observation 1: Additional separated indication of triggering temporary RS might be needed in addition to the integrated indication, depending on RAN4 feedback, i.e., how many symbols of TRS are needed.

Also, it should be clarified how UE behaves when the UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE. The following options can be considered, and down-selection from them is needed.

- Option 1: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE behaviour is same as Rel-16 UE.

- Option 2: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE tries to receive the indication of triggering temporary RS within a certain duration. If UE does not receive the indication of triggering temporary RS, UE behaviour is same as Rel-16 UE.

  - Note that when UE is not configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, the UE behaviour is same as Rel-16 UE.

Proposal 3: Down-select from following options:

- Option 1: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE behaviour is same as Rel-16 UE.

- Option 2: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE tries to receive the indication of triggering temporary RS within a certain duration. If UE does not receive the indication of triggering temporary RS, UE behaviour is same as Rel-16 UE.

  - Note that when UE is not configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, the UE behaviour is same as Rel-16 UE.

- DCI format for triggering SCell activation and triggering temporary RS

DCI format 0_1 and 1_1 can be considered as the basic DCI format for triggering SCell activation and triggering temporary RS. In addition, DCI format 2_6 is monitored outside active time, and can indicate wakeup indication and dormancy indication. For SCell activation, indication by DCI format 2_6 is beneficial since the opportunity to indicate wakeup indication would be similar as that to indicate SCell activation. Thus, DCI format 2_6 should be also considered for triggering SCell activation and triggering temporary RS.

Proposal 4: For triggering SCell activation and triggering temporary RS, at least DCI format 0_1/1_1/2_6 should be supported.

- The number of SCells for triggering SCell activation and temporary RS

In the case of MAC CE based SCell activation/deactivation, one octet MAC CE can indicate up to 7 SCells, and four octets MAC CE can indicate up to 31 SCells. Also, in the case of dormancy indication, DCI format 0_1/1_1/2_6 can indicate up to 5 SCell groups, and DCI format 1_1 without scheduling can indicate up to 15 SCells. Basically, the opportunity to indicate SCell activation for each SCell would be common for a UE, e.g., based on traffic amount for the UE. Thus, it is beneficial that triggering SCell activation and temporary RS can be indicated simultaneously for the sufficient number of SCells. Considering DCI size overhead, “SCell dormancy indication” field like method is good starting point, and reusing “SCell dormancy indication” field for triggering SCell activation and temporary RS should be considered.

Proposal 5: For indication of triggering SCell activation and temporary RS for multiple SCells, “SCell dormancy indication” field should be reused.

- BWP for the temporary RS

During SCell activation procedure, the BWP on the SCell is still inactive. Thus, in order to obtain the benefit of the temporary RS, the UE needs to measure the temporary RS on the inactive BWP during SCell activation procedure. Also, it was discussed which BWP the UE measures the temporary RS on. In terms of flexibility, it is beneficial that gNB can indicate which BWP the UE measures the temporary RS on along with the indication of triggering the temporary RS. If DCI-based triggering of the temporary RS is supported, bandwidth part indicator can be used for the purpose. If there is no indication on which BWP the UE measures the temporary RS on, the first Active Downlink BWP can be used.

Proposal 6: The UE measures the temporary RS regardless of the state of BWP during SCell activation procedure.

Proposal 7: gNB indicates which BWP the UE measures the temporary RS on along with the indication of triggering the temporary RS.

- If there is no indication on which BWP the UE measures the temporary RS on, the first Active Downlink BWP can be used.

- The case of co-located cells

Although SCell activation delay can be generally reduced based on the temporary RS, for some specific cases further fast SCell activation might be possible. For example, if time/frequency tracking for a deactive cell can be derived from another active cell since these two cells are co-located, the UE might be able to skip SSB reception and processing, and SCell activation delay can be reduced significantly. gNB can indicate such combination of cells which share some information, e.g., time/frequency tracking, for another cell.

Proposal 8: Study the cases where the UE can skip SSB reception and processing, e.g., the UE has the active cell which is co-located with the activating cell.

- Conclusion

In this contribution, efficient activation/deactivation mechanism for SCells was discussed. Based on the discussion, the following observation and proposals were made:

Proposal 1: For both cases of known cell and unknown cell, the temporary RS should be supported as functionalities of at least AGC setting and time/frequency tracking for both FR1 and FR2.

- FFS potential functionalities of CSI measurement/acquisition and cell search

Proposal 2: Triggering SCell activation and triggering the temporary RS can be integrated into a DCI.

Observation 1: Additional separated indication of triggering temporary RS might be needed in addition to the integrated indication, depending on RAN4 feedback, i.e., how many symbols of TRS are needed.

Proposal 3: Down-select from following options:

- Option 1: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE behaviour is same as Rel-16 UE.

- Option 2: When UE is configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, UE tries to receive the indication of triggering temporary RS within a certain duration. If UE does not receive the indication of triggering temporary RS, UE behaviour is same as Rel-16 UE.

  - Note that when UE is not configured with temporary RS via higher layer signalling and receives the existing SCell activation command in MAC-CE, the UE behaviour is same as Rel-16 UE.

Proposal 4: For triggering SCell activation and triggering temporary RS, at least DCI format 0_1/1_1/2_6 should be supported.

Proposal 5: For indication of triggering SCell activation and temporary RS for multiple SCells, “SCell dormancy indication” field should be reused.

Proposal 6: The UE measures the temporary RS regardless of the state of BWP during SCell activation procedure.

Proposal 7: gNB indicates which BWP the UE measures the temporary RS on along with the indication of triggering the temporary RS.

- If there is no indication on which BWP the UE measures the temporary RS on, the first Active Downlink BWP can be used.

Proposal 8: Study the cases where the UE can skip SSB reception and processing, e.g., the UE has the active cell which is co-located with the activating cell.

References

- 3GPP RAN1#103-e, Chairman’s note, November 2020.