3GPP TSG RAN WG1 #104e			R1-2101642

e-Meeting, January 25th – February 5th, 2021

Source:	NTT DOCOMO, INC.

Title:	TB processing over multi-slot PUSCH

Agenda Item:	8.8.1.2

Document for: 	Discussion

- Introduction

At the RAN1#90-e meeting, a new WID [1] on “NR coverage enhancements” was approved. The detailed objectives of PUSCH enhancements are as follows.

The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD. The detailed objectives of the work item are as follows: - Specification of PUSCH enhancements [RAN1, RAN4] - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1] - Increasing the maximum number of repetitions up to a number to be determined during the course of the work. - The number of repetitions counted on the basis of available UL slots. - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1] - TBS determined based on multiple slots and transmitted over multiple slots. - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4] - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4] - Potential optimization of DMRS location/granularity in time domain is not precluded - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

In this contribution, we discuss on the TB processing over multi-slot PUSCH for coverage enhancements.

- Discussion on TB processing over multi-slot PUSCH

TB processing over multi-slot PUSCH was captured as one of objectives for PUSCH enhancements in the WID.

Fig. 1 shows the simulation result where PUSCH is assigned on the special slot and uplink slot in DDDDDDDSUU  (S: 6D 4G 4U) and DDDSU (S: 10D 2G 2U) TDD pattern for FR1 and FR2, respectively. As shown in the figures, TB processing over multi-slot PUSCH improves the coverage performance of PUSCH in S+U slots. Therefore, S+U slots should be considered for TB processing over multi-slot PUSCH as they are widely used in typical TDD patterns.

Proposal 1: S+U slots in TDD configuration should be considered for TB processing over multi-slot PUSCH

Figure 1. Link level simulation results capturing the gain from TB over multi-slots for PUSCH.

- Signalling mechanism for TB processing over multi-slot

Since current mapping types for time domain resource allocation cannot allocate more than 14 symbols for one PUSCH, new signalling scheme needs to be introduced in order to realize one PUSCH over multiple slots. Candidate signalling mechanisms are listed as below.

Option 1: Introduce a new mapping type in TDRA which can allocate more than 14 symbols

Option 2: Integrate several PUSCH resources

2a: Multi-PUSCH assigned by DCI(s) (Fig.2)

2b: Several actual repetition (Fig.3)

Option 1 is to introduce a new mapping type in TDRA, which can allocate more than 14 symbols for one TB. Option 2 is to integrate several PUSCH resources. In this case, the several PUSCH resources are indicated to be configured for one PUSCH resource which constitutes of one TB. For example, multi-PUSCH indicated by one DCI could be integrated into one PUSCH resource based on specific indication for integration, as shown in Fig. 2. Fig. 3 shows the other example where several actual repetitions are integrated into one PUSCH based on specific indication for integration. Through these ways, it is possible to allocate more than 14 symbols for one PUSCH.

Figure 2. Integration of multi-PUSCH indicated by one DCI.

Figure 3. Integration of PUSCH repetition into one PUSCH resource.

- DM-RS position

In the current specification, DM-RS configuration types cover only up to 14 OFDM symbols in PUSCH allocations. When one PUSCH resource has more than 14 OFDM symbols, DM-RS configuration type should be extended to cover more than 14 symbols for uniform DM-RS symbol distribution. On top of that, the maximum number of additional DM-RS symbols should be increased, as one PUSCH has more OFDM symbols.

Proposal 2: DM-RS configuration should be extended so that one PUSCH can have more than 14 OFDM symbols with uniform DM-RS symbol distribution.

Proposal 3: It is better to support more than 3 additional DM-RS positions in case that one PUSCH has more than 14 OFDM symbols.

- Conclusion

In this contribution, we discussed on TB processing over multi-slot PUSCH for coverage enhancements. Based on the discussion we made following proposals.

Proposal 1: S+U slots in TDD configuration should be considered for TB processing over multi-slot PUSCH

Proposal 2: DM-RS configuration should be extended so that one PUSCH can have more than 14 OFDM symbols with uniform DM-RS symbol distribution.

Proposal 3: It is better to support more than 3 additional DM-RS positions in case that one PUSCH has more than 14 OFDM symbols.

References

- 3GPP, RP-202928, “New WID on NR coverage enhancements”. Dec. 2020.