3GPP TSG RAN WG1 #104-e		R1-2101644

e-Meeting, January 25th –February 5th, 2021

Source:	TCL

Title:	Enhancements on Multi-Beam Operation

Agenda Item:	8.1.1

Document for:	Discussion and Decision

- Introduction

The WI in Rel-17 NR for the objective of enhancement on multi-beam operations is presented in [1]. In RAN1#103-e meeting [2], we reached the following agreements:

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured Agreement On Rel-17 unified TCI framework, to accommodate the case of separate beam indication for UL and DL: - Utilize two separate TCI states, one for DL and one for UL. - FFS: Contents of separate UL TCI state - Note: For FR1, UE does not expect UL TCI to provide a reference for determining common UL TX spatial filter(s), if UL TCI is supported for FR1 - For the separate DL TCI: - The source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC - For the separate UL TCI: - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions - FFS: Whether the UL TCI state is taken from a common/same or separate TCI state pool from DL TCI state - Note that TCI state pool for joint DL and UL beam indication is still FFS - FFS: Whether Rel.17 supports TCI configured for single channel (e.g. PDSCH only, single CORESET) - Note: This does not preclude the type of UE supporting only 1 beam tracking loop, i.e. UE reports value of 1 in UE FG 2-62. Agreement In Rel-17 enhancement for facilitating fast uplink panel selection, the following use cases are assumed: - MPE mitigation - UE power saving - UL interference management - Support different configurations across panels - UL mTRP Agreement In Rel-17 enhancement on MP-UE to facilitate fast UL panel selection and MPE mitigation, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s) Agreement In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported: - FFS: Whether NW-initiated panel selection/activation is also supported - FFS: Whether specification support for this feature is necessary and if so the details of such spec support. Agreement On UE reporting for MPE mitigation for Rel-17, investigate and, if needed, specify the following: - Reporting of P-MPR report based on Rel.16 framework. - FFS: Whether panel/beam level based P-MPR report is supported - FFS: Maximum reported number of panels, e.g. single or multiple - Reporting SSBRI(s)/CRI(s) and/or indication of panel selection for the purpose of indicating: - Alt1: alternative UE panel(s) or TX beam(s) for UL transmission - Alt2: feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account - FFS: indication of panel selection details (e.g. explicit/implicit) - Any additional reporting content: down-select from the following in RAN1#104-e - Alt0: no additional reporting content - Alt1: Additional reporting content is included (for example P-MPR + L1-RSRP, virtual PHR + L1-RSRP, L1-RSRP/SINR with and without MPE effect, virtual PHR, P-MPR or virtual PHR + CRI/SSBRI, estimated max UL RSRP) - Note: Other options are not precluded - FFS: Whether the above reporting is triggered by UE or configured by NW Agreement On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs: - The above applies to intra-band CA - The above applies to joint DL/UL and separate DL/UL beam indications - Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS - The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs - FFS: The above also applies to inter-band CA - FFS: TCI state pool for CA - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state. - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs - Opt-2: configuring RRC TCI state pool per individual CC - FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels Agreement On Rel-17 unified TCI framework: - A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication). - FFS: The pool for separate DL and UL TCI state update (beam indication) - Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling - FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication,etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied Agreement In RAN1#104-e, on the Rel-17 L1-based TCI state update (beam indication) for the unified TCI framework, interested companies are to provide the following: - How to use DCI formats 1_1 and 1_2 for UL-only (in case of separate DL/UL) TCI state update (beam indication) - Note: The agreement implies that DCI formats 1_1 and 1_2 can be used for UL-only TCI state update beam indication). - FFS: Using DCI format 1_1 and 1_2 without DL assignment, and with a new acknowledgment mechanism directly in response to decoding DCI format 1_1 and 1_2, e.g., analogous to SPS PDSCH release - Whether/how to support at least one additional DCI format dedicated for UL-only beam indication (in case of separate DL/UL), including: - Whether the format can also be used for DL-only beam indication (in case of separate DL/UL) and joint DL/UL beam indication - Whether it is a “brand new” format or based on some extension of the existing DCI formats other than 1_1 and 1_2 (e.g. 1_0, 0_0, 0_1, or 0_2) - If UL-related DCI is used, whether it is accompanied with UL grant or not - Acknowledgment mechanism Agreement On Rel.17 DCI-based beam indication: - Regarding application time of the beam indication: if beam indication is received, down-select from the following: - Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication - Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y - FFS: When to apply the minimum indication delay (e.g., when the newly indicated beam is different with the previously indicated beam) Agreement On Rel.17 DCI-based beam indication, the beam application time is to be down-selected or modified from the following: - Alt1: The beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - Alt2: The beam application time is fixed and defined in specification - Alt3: The beam application time can be configured by the gNB where the minimum value of beam application time is fixed and defined in specification - Consider multi-panel UE, layer 1/2 inter-cell cases, carrier aggregation aspects

In this contribution, we present our discussion on some objectives of enhancement on multi-beam operation. In particular, we will present the views on unified TCI state framework and common beam and fast UE panel selection issues.

- Unified TCI Framework and Common Beam

Unified common beam indication

In Rel-15/Rel-16 separate signaling frameworks for DL TCI state and UL spatial relation are specified. However, for the separate signaling mechanism in Rel-15/Rel-16, some problems may exist such as latency and signaling overhead because two groups of RRC and MAC CE signaling are needed to configure and indicate beams for both DL and UL transmission. Thus, the unified beam indication design for UL and DL transmission is proposed in Rel-17. From the point of view of saving RRC signaling overhead, the common beam mechanism is also a solution of the unified TCI framework, and a common beam can be configured and shared by both DL and UL. There are two alternatives, i.e., joint beam indication for DL and UL transmission and separate beam indication for DL and UL transmission.

For the joint TCI design, the source reference RS(s) in the TCI state provide QCL assumption and spatial filter for both DL and UL transmission. Due to the UE mobility and dynamic blockage, the beam should be updated dynamically, The UE would make the CSI measurement when receives the beam sweeping from gNB, and then report the measurement results to gNB, the UE also performs UL beam sweeping, the uplink signal which gNB receives is SRS rather than the reported CSI. Thus, the source reference RS(s) can be the DL reference RS or UL reference RS, and DL or UL reference RS can provide both DL and UL beam indication, this case is general, e.g., beam correspondence. The source reference RSs configured in the TCI pool provide a common beam for DL and UL.

It is agreed that the separate beam indication mechanism is designed for DL and UL transmission in RAN1#103e meeting, and two different TCI states are utilized for DL and UL transmission, respectively. For the separate beam indication scheme, RRC configures at least two reference RSs for the separate beam indication scenario, one for DL and another one for UL. From our perspective, for the separate beam indication, there are two alternatives listed as follows:

- Alt 1: Both DL and UL states are taken from a common/same TCI state pool;

- Alt 2: UL TCI state is taken from a separate TCI state pool from DL TCI state.

For Alt 1, both DL and UL reference RSs are configured in the same TCI state, in which the DL reference RS provides DL common beam and UL reference RS provides UL common beam. For Alt 2, DL and UL reference RSs are configured in different TCI states to provide DL and UL common beam, respectively.

Proposal 1: For the separate beam indication, there are two alternatives listed as follows:

- Alt 1: Both DL and UL states are taken from a common/same TCI state pool;

- Alt 2: UL TCI state is taken from a separate TCI state pool from DL TCI state.

For unified common beam indication, whether joint or separate beam indication, the multiple reference RSs at least configured in one TCI state pool. However, due to multiple beams configured in the same TCI pool, how to distinguish the common beam from the multiple configured beams in the TCI pool. Thus, common beam identification designs are needed to propose to distinguish whether the indicated beam is a common beam or not.

Proposal 2: For unified common beam indication, common beam identification designs are needed to propose to distinguish whether the indicated beam is a common beam or not.

Carrier Aggregation

To reduce the signaling overhead for beam indication, MAC CE signaling is used to activate the same set of TCI-state ID(s) in Rel-16. As far as we know, the default beam is also a solution for signaling overhead, especially for the beam indication in multiple CCs, gNB can configure the same Tx beam for DL transmission in all the CCs.

In the RAN1#103e meeting, for intra-band CA, it is agreed that the common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs. However, the TCI state pool for CA is also an issue that needs to be further studied. For the unified TCI state pool for CA, there are two options list as following:

- Opt-1: sharing a single RRC TCI state pool for the set of configured CCs

- Opt-2: configuring RRC TCI state pool per individual CC.

For Opt-1, all the configured CCs sharing a common TCI state pool, and the unified common beam indication is applied to all configured CCs. Multiple reference RSs are configured in a single TCI state pool, each reference RS is associated with a CC.  However, this per channel/resource per CC beam indication will lead to a lot of QCL-type D collision case, and finally make the communication system more complicated. Besides, in different CCs, due to the large bandwidth, the frequency interval between different CCs is too large, not all CCs can use the same common beam.  Thus, we have

Proposal 3: For the unified common beam indication for carrier aggregation, multiple common beams are supported for multiple CCs.

Besides, due to multiple reference RSs configured in the same TCI state pool, and each RS is associated with a beam. Thus, for the unified common beam indication for carrier aggregation, multiple CCs share a common TCI state pool, and each beam configured in the TCI state pool is associated with one of the multiple CCs.

For the unified TCI frame design, as mentioned in Rel-17, the joint and separate common beam designs for carrier aggregation are considered. For the separate common beam indication, the DL and UL CCs may not apply to the same beam simultaneously. When carrier aggregation is only applied to DL CCs, how to determine the beams of  UL  CCs, and vice versa.

Proposal 4: For the unified common beam indication, when carrier aggregation is only applied to DL CCs, how to determine the beams of  UL CCs needs to be clarified, and vice versa.

For Opt-2, gNB configures TCI state pool per individual CC. For the beam management of CA, each CC performs independent beam management will increase resource overhead as the number of  CCs increases. Thus, we consider that all CCs jointly perform beam management. The UE would make the CSI measurement when receives the beam sweeping from gNB, and then report the measurement results to gNB, gNB select the best DL beam according to the reported CSI. the UE also performs UL beam sweeping, and gNB measures the SRS and selects the best UL transmit beam for UE. And when configuring RRC TCI state pool per individual CC, the gNB configures multiple TCIs for both DL and UL CCs, the relationship between the DL (UL) beam are configured TCI state pools need to be clarified.

Proposal 5: For the unified common beam indication, when configuring RRC TCI state pool per individual CC,

gNB configures multiple TCIs for both DL and UL CCs, how to determine the TCI state pool for each CC need to be clarified.

- Fast UL Panel Selection

In Rel-15/Rel-16, gNB configures SpatialrelationInfo for UE to indicate UL beam selection, the UE is provided with spatial relation info for a PUCCH and SRS resource, which contains a CSI-RS resource, SS/PBCH block or an SRS resource. For the UE with multi-panel, beam correspondence is not supported, gNB configures multiple SRS resources for different panels. gNB is configured with at least one SRS resource set for beam management. Usually, different SRS resource sets are associated with different antenna panels. Different SRS resource sets can be transmitted at a time, and the SRS resources in the same SRS resource set can only be transmitted by time-division multiplexing (TDM). When the UE switches from one panel to another, UE needs to inform gNB to make the beam alignment between the gNB and UE. The gNB will also switch the uplink Rx beam, and reschedules the downlink transmission. Therefore, the separate configuration of uplink and downlink beam indication requires gNB to configure uplink and downlink signaling, which will lead to delay and signaling overhead. The unified framework for DL and UL beam indication is proposed, and multiple DL and UL reference RSs are configured in a common TCI state pool.

To select the best UL beam, power consumption is one critical issue. In RAN1#103e meeting, it is agreed that

MPE mitigation and UE power saving should be assumed. For the UE with multi-panel, different configurations are supported across multiple panels, i.e., each UE panel can comprise an independent unit of uplink power and timing control. When gNB or UE detects the event that triggers panel switching, e.g., MPE issue, the current UE panel can not continue use for UL transmission. UE switches the current transmit panel to the candidate panel and then report the switching event to gNB. When the current transmit panel and the candidate panel are configured independently, gNB needs to reconfigure the candidate panel and indicate the UL transmission.

In RAN1 #103e meeting, to facilitate fast UL panel selection and MPE mitigation, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s). Assuming that panel1 is the current transmit panel for DL and UL, panel2 is the candidate panel, the same TCI state is shared by the transmit panel 1 and candidate panel 2.When the event that needs to perform panel switching is detected, the UE switches from the current panel to the candidate panel, and report the event to gNB, because of the candidate panel configured in the same TCI state with that of the current transmit panel, the reconfiguration of gNB is not required, and thus reducing the delay and signaling overhead.

Proposal 6: For UE with multi-panel, the transmit and candidate panels sharing a single TCI state is supported.

As mentioned above, to facilitate fast uplink panel selection, the DL and UL beams are measured and the related transmit and candidate panels are selected according to some criterion.

Proposal 7: The beams of the transmit and the candidate panels for DL and UL are configured in the same TCI state, and how to select the transmit and candidate panels should be studied and specified.

For UE with multi-panel, when each UE panel is configured individual TCI state. When the event that needs to perform panel switching is detected, the UE switches from the current panel to the candidate panel, in order to align the uplink and downlink beams of the gNB and UE, gNB needs to know which panel is the target panel. UL beam management is based on SRS resources, UE needs to transmit SRS resource to gNB for uplink beam sweeping and measuring.

Proposal 8: For UE with multi-panel, UE transmits SRS to gNB for further measuring. From the perspective of gNB, how to identify the target panel according to the received SRS should be clarified.

- Conclusion

In this contribution, we presented our views on enhancement for multi-beam operation. Based on the discussion, the following proposals are provided:

Proposal 1: For the separate beam indication, there are two alternatives listed as follows:

- Alt 1: Both DL and UL states are taken from a common/same TCI state pool;

- Alt 2: UL TCI state is taken from a separate TCI state pool from DL TCI state.

Proposal 2: For unified common beam indication, common beam identification designs are needed to propose to distinguish whether the indicated beam is a common beam or not.

Proposal 3: For the unified common beam indication for carrier aggregation, multiple common beams are supported for multiple CCs.

Proposal 4: For the unified common beam indication, when carrier aggregation is only applied to DL CCs, how to determine the beams of  UL CCs needs to be clarified, and vice versa.

Proposal 5: For the unified common beam indication, when configuring RRC TCI state pool per individual CC,

gNB configures multiple TCIs for both DL and UL CCs, how to determine the TCI state pool for each CC need to be clarified.

Proposal 6: For UE with multi-panel, the transmit and candidate panels sharing a single TCI state is supported.

Proposal 7: The beams of the transmit and the candidate panels for DL and UL are configured in the same TCI state, and how to select the transmit and candidate panels should be studied and specified.

Proposal 8: For UE with multi-panel, UE transmits SRS to gNB for further measuring. From the perspective of gNB, how to identify the target panel according to the received SRS should be clarified.

- Reference

RP-193133 New WID: Further ehancements on MIMO for NR

Chairman’s Notes RAN1#103-e