3GPP TSG RAN WG1 #104-e		                        R1-2101649

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.4

Source: 	ASUSTeK

Title:  	Remaining issues on type-1 HARQ-ACK codebook considering multiple resource pools

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about remaining issues on type-1 HARQ-ACK codebook considering multiple resource pools.

- Discussion

Justification

- As for sidelink resource allocation mode-1, UE can be configured with multiple resource pools by sl-PoolToAddModList-r16, and different resource pool can be configured with different PSFCH period.

- When UE is scheduled to perform sidelink transmission on different resource pool by different DCI format 3_0, type-1 HARQ-ACK codebook generation for multiple resource pools according to TS 38.213 is missing.

- A simple solution is to apply pseudo code in TS 38.213 separately for each resource pool, and concatenate the generated HARQ-ACK codebook for each resource pool.

- Type-1 HARQ-ACK codebook generation considering ackNackFeedbackMode = joint in Rel-16 eMIMO (relevant Text is cited in appendix) could be a starting point for type-1 HARQ-ACK codebook considering multiple resource pools.

Text Proposal

According text proposal for type-1 HARQ-ACK codebook is provided below.

<   Text Proposal for 38.213 [1]  >

#### 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel If a UE is provided a set of sidelink resource pools, where sidelink resource pools are placed in the set according to an ascending order of a sidelink resource pool index, the UE generates a Type-1 HARQ-ACK codebook for sidelink resource pool in the set of sidelink resource pools separately in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for each sidelink resource pool in the set of sidelink resource pools according to an ascending order of the sidelink resource pool index to obtain a total number of HARQ-ACK information bits. For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1 b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and sl-PSFCH-Period, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while

Proposal: 	Adopt the text proposal in updating of TS38.213 section 16.5.1.1, for type-1 HARQ-ACK codebook

- Conclusion

In this contribution, we have following proposals:

Proposal: 	Adopt the text proposal in updating of TS38.213 section 16.5.1.1, for type-1 HARQ-ACK codebook

<   Text Proposal for 38.213  >

#### 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel If a UE is provided a set of sidelink resource pools, where sidelink resource pools are placed in the set according to an ascending order of a sidelink resource pool index, the UE generates a Type-1 HARQ-ACK codebook for sidelink resource pool in the set of sidelink resource pools separately in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for each sidelink resource pool in the set of sidelink resource pools according to an ascending order of the sidelink resource pool index to obtain a total number of HARQ-ACK information bits. For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1 b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and sl-PSFCH-Period, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while

- Reference

- 3GPP TS 38.213 V16.4.0 (2020-12), “NR; Physical layer procedures for control (Release 16)”

- Appendix

Following text quoted from [1] specify type-1 HARQ-ACK codebook considering ackNackFeedbackMode = joint in Rel-16 eMIMO.

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel … If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ackNackFeedbackMode = joint where - a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and - a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of HARQ-ACK information bits.