3GPP TSG RAN WG1 #104-e		                        R1-2101650

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.4

Source: 	ASUSTeK

Title:  	Remaining issues on sidelink procedure

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about remaining issues on sidelink power control, including:

- Maintenance of HARQ-ACK determination of groupcast feedback option 1

- DL pathloss in case of Uu-interface L1 problem

- Discussion

2.1  Maintenance of HARQ-ACK determination of groupcast feedback option 1

Justification

1. For unicast (cast type indicator value of “10”) and groupcast feedback option 2 (cast type indicator value of “01”), UE reports to higher layer HARQ-ACK information if the UE receives a PSFCH.

For groupcast feedback option 1 (cast type indicator value of “11”), the condition “if the UE receives a PSFCH” is against sub-bullet condition “if the UE determines absence of PSFCH reception for the PSFCH reception occasion”.

2. In RAN1 #103-e, it is agreed that the UE is allowed to drop the PSCCH/PSSCH retransmission in some scenarios. Then, for groupcast feedback option 1, the condition “absence of PSFCH reception for the PSFCH reception occasion” will pass, and an ACK value will be reported to higher layers. However, the ACK value is unreasonable since the UE drops the corresponding PSCCH/PSSCH retransmission.

Agreements:

- If the time between PSFCH reception and next scheduled PSCCH/PSSCH retransmission is less than Tprep + delta, the UE is allowed to drop the PSCCH/PSSCH retransmission with SL HARQ feedback enabled.

  - Note: it is RAN1 understanding that the UE is allowed to drop the PSCCH/PSSCH retransmission only if the UE can not complete the PSFCH processing and the preparation of the next PSCCH/PSSCH retransmission in the time between PSFCH reception and the next scheduled PSCCH/PSSCH retransmission

Text Proposal 1

According text proposal for HARQ-ACK determination of groupcast feedback option 1 is provided below.

<   Text Proposal 1 for 38.213 >

### 16.3.1 UE procedure for receiving HARQ-ACK on sidelink A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in Clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [10, TS 38.133]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource. If the UE skips a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, the UE reports a NACK value to higher layers. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE determines absence of PSFCH reception for the PSFCH reception occasion associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value; otherwise, report a NACK value to higher layers

Proposal 1: 	Adopt the text proposal 1 in updating of TS38.213 section 16.3.1, for HARQ-ACK determination of groupcast feedback option 1.

2.2  DL pathloss in case of Uu-interface L1 problem

Agreements:

For open-loop power control based on DL pathloss, the UE calculates DL pathloss using

- The RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

- A RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

In RAN1 #101-E meeting [1], it is agreed that a UE can be configured to use DL pathloss for sidelink power control. When the UE is configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS for PUSCH power control scheduled by DCI format 0_0. When the UE is not configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS resource from the SS/PBCH block that the UE uses to obtain MIB.

RAN2 agrees that mode-1 UE is allowed to continue using the configured SL grant type 1 when beam failure occurs or T310 is running [2][3]. In case these Uu-interface L1 problems happens, the mode-1 UE may not detect DL RSs normally, especially the DL RS for PUSCH power control scheduled by DCI format 0_0. In other words, DL pathloss derived from DL RS for DCI format 0_0-based PUSCH may be not accurate and meaningless for sidelink power control. Thus, for a UE configured to monitor PDCCH for detection of DCI format 0_0, one simple way during Uu-interface L1 problems is change or fallback to use SS/PBCH block that the UE uses to obtain MIB.

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 2:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Justification

- When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Text Proposal 2

According text proposal for DL pathloss used for sidelink power control is provided below.

<   Text Proposal 2 for 38.213 >

### 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where <Omited> - if dl-P0-PSSCH-PSCCH-r16 is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] <Omited> where - PO,D is a value of dl-P0-PSSCH-PSCCH-r16 if provided - αD is a value of dl-Alpha-PSSCH-PSCCH-r16, if provided; else, αD=1 - PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c <Omited> ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as - if p0-DL-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of p0-DL-PSFCH - αPFSCH is a value of alpha-DL-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

- Conclusion

In this contribution, we have following proposals:

Proposal 1: 	Adopt the text proposal 1 in updating of TS38.213 section 16.3.1, for HARQ-ACK determination of groupcast feedback option 1.

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 2:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Proposal 3: 	Adopt the text proposal 2 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

- Reference

- 3GPP TSG RAN WG1 #101-E Chairman’s Notes

- Report of 3GPP TSG RAN2#107 meeting

- Report of 3GPP TSG RAN2#108 meeting

- 3GPP TS 38.213 V16.4.0 (2020-12), “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.331 V16.2.0 (2020-09), “NR; Radio Resource Control (RRC) protocol specification (Release 16)”