3GPP TSG-RAN WG1 Meeting #104-e	R1-2101690

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.6

Source:	Ericsson

Title:	Maintenance for multi-DCI based multi-TRP in Rel-16

Document for:	Discussion

# Introduction

In this paper we address an open issue related to multi-DCI based multi-TRP PDSCH transmission.

# Discussion

## PDCCH monitoring priority based on QCL-TypeD for multi-DCI based M-TRP

In RAN1#103-e, the issue of PDCCH monitoring priority based on QCL-TypeD for multi-DCI based M-TRP was discussed.  According to the current specification in TS 38.213 (see Section 10.1), simultaneous reception of PDCCHs using different QCL-TypeD is not possible.

Since simultaneous reception of PDSCH is supported for multi-DCI based multi-TRP as a UE capability, it is also beneficial if the multi-DCI UEs with this capability can also simultaneously receive PDCCH.  Hence, we make the following proposal:

For the PDCCH monitoring priority rule based QCL-TypeD in multi-DCI based M-TRP, apply the priority rule within CORESETs with same CORESETPoolIndex for a UE capable of two simultaneous QCL-TypeD.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the PDCCH monitoring priority rule based QCL-TypeD in multi-DCI based M-TRP, apply the priority rule within CORESETs with same CORESETPoolIndex for a UE capable of two simultaneous QCL-TypeD.