3GPP TSG RAN WG1 Meeting #104-e	R1-2101737

E-meeting, January 25th – February 5th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on UE capability xDD differentiation for SUL/SDL bands

Document for:	Discussion and Decision

# Introduction

During RAN2 meeting #110-e, an issue on how to differentiate the UE capabilities by FDD or TDD for the SUL/SDL bands in Rel-15 and Rel-16 has been raised. In the latest RAN meeting #90-e, this issue has been discussed for SUL but not for SDL. In this contribution, we will provide further analysis on this issue for SDL based on the following moderator’s conclusion in [1].

Moderator's conclusion: Per UE Capabilities that are FDD/TDD differentiated when applied to SUL carriers are always considered indicated by the FDD capability (i.e. in effect the capabilities are not FDD/TDD differentiated for this case). Per UE capabilities that are TDD only are not applicable to SUL. RAN2 is tasked to prepare Rel-15 and 16 CRs to capture this agreement.

# Discussion

As in our contribution [2] last RAN1 meeting, the specific per-UE capabilities with suffix of XDD-Diff and the reason for the UE reporting different values for TDD/FDD on those capabilities is provided as below:

Table 1. Phy-ParametersXDD-Diff field components and the reason for xDD differentiation respectively

| UE Capability | Reason for xDD differentiation |
| --- | --- |
| dynamicSFI | SFI for TDD/FDD is different |
| twoPUCCH-F0-2-ConsecSymbols | For IoDT consideration |
| twoDifferentTPC-Loop-PUSCH | For IoDT consideration |
| twoDifferentTPC-Loop-PUCCH | For IoDT consideration |
| dl-SchedulingOffset-PDSCH-TypeA | For IoDT consideration |
| dl-SchedulingOffset-PDSCH-TypeB | For IoDT consideration |
| ul-SchedulingOffset | For IoDT consideration |

Note that although there are two SDL bands overlapping with TDD bands, i.e., n75 SDL band overlapping with n50 TDD band and n76 SDL band overlapping with n51 TDD band, all the subframes on SDL are designated for downlink transmission, which is the same as an FDD downlink carrier. Thus from the IoDT perspective, the FDD IoDT results could be the same for SDL, and SDL has also followed the same SFI mechanism as FDD. In conclusion, we think the FDD capability indication for SUL when applying per UE capabilities can be reused for SDL.

Proposal: Regarding all the per-UE capabilities within Phy-ParametersXDD-Diff field differentiated by FDD and TDD both in Rel-15 and Rel-16, FDD capability indication is always considered for SDL bands.

# Conclusions

In this contribution, issue for UE capability xDD differentiation for SDL bands was discussed. The following proposal is given:

Proposal: Regarding all the per-UE capabilities within Phy-ParametersXDD-Diff field differentiated by FDD and TDD both in Rel-15 and Rel-16, FDD capability indication is always considered for SDL bands.

# References

RP-202911 “Report from Email Discussion [90E][36][SUL_UL-DL]”, Intel, 3GPP RAN#90 Electronic Meeting, December 7th – 11th, 2020.

R1-2008778 “Discussion on UE capability xDD differentiation for SUL SDL bands”, Huawei, HiSilicon, RAN1#103-e, October 26th – November 13th, 2020.