3GPP TSG-RAN WG1 Meeting #104-e	R1-2101763

e-Meeting, January 25th – February 5th, 2021

Agenda item:		7.2.5

Source:		Moderator (Apple Inc.)

Title:	Feature lead summary on PUSCH enhancements for NR eURLLC in AI 7.2.5

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, Sections 2 summarizes the issues raised in the contributions submitted for eURLLC PUSCH enhancements under AI 7.2.5. The agreed scope of the email discussions is provided in Section 3.

# 2	Issues

## Issue #1: New RRC parameter for TDRA indication to support up to 64 entries in a TDRA table for Type 1 configured grant with PUSCH repetition Type B

A draft CR was proposed in [1] to address the issue that timeDomainAllocation in configuredGrantConfig (applicable for Type 1 configured grant) has a value range of 0 to 15, while for PUSCH repetition Type B, there can be up to 64 entries in a TDRA table. It was proposed that a new RRC parameter is introduced, and TS 38.214 is modified accordingly.

The draft CR for TS 38.214 Clause 6.1.2.3 in [1]:

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant When PUSCH resource allocation is semi-statically configured by higher layer parameter configuredGrantConfig in BWP-UplinkDedicated information element, and the PUSCH transmission corresponding to a configured grant, the following higher layer parameters are applied in the transmission: - For Type 1 PUSCH transmissions with a configured grant, the following parameters are given in configuredGrantConfig unless mentioned otherwise: - For the determination of the PUSCH repetition type, if the higher layer parameter pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured and set to 'pusch-RepTypeB', PUSCH repetition type B is applied; otherwise, PUSCH repetition type A is applied; - For PUSCH repetition type A, the selection of the time domain resource allocation table follows the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1. - For PUSCH repetition type B, the selection of the time domain resource allocation table is as follows: - If pusch-RepTypeIndicatorDCI-0-1 in pusch-Config is configured and set to 'pusch-RepTypeB', pusch-TimeDomainResourceAllocationListDCI-0-1 in pusch-Config is used; - Otherwise, pusch-TimeDomainResourceAllocationListDCI-0-2 in pusch-Config is used. - It is not expected that pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured with 'pusch-RepTypeB' when none of pusch-RepTypeIndicatorDCI-0-1 and pusch-RepTypeIndicatorDCI-0-2 in pusch-Config is set to 'pusch-RepTypeB'. - The higher layer parameter timeDomainAllocation or timeDomainAllocation-r16 value m provides a row index m+1 pointing to the determined time domain resource allocation table, where the start symbol, length and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) are determined following the procedure defined in Clause 6.1.2.1; - Frequency domain resource allocation is determined by the N LSB bits in the higher layer parameter frequencyDomainAllocation, forming a bit sequence f17, …, f1, f0, where f0 is the LSB, according to the procedure in Clause 6.1.2.2 and N is determined as the size of frequency domain resource assignment field in DCI format 0_1 for a given resource allocation type indicated by resourceAllocation, except if useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, in which case uplink type 2 resource allocation is used wherein the UE interprets the LSB bits in the higher layer parameter frequencyDomainAllocation as for the frequency domain resource assignment field of DCI 0_1 according to the procedure in Clause 6.1.2.2.3; - The IMCS is provided by higher layer parameter mcsAndTBS; - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; - When frequency hopping is enabled, the frequency offset between two frequency hops can be configured by higher layer parameter frequencyHoppingOffset. - For Type 2 PUSCH transmissions with a configured grant: the resource allocation follows the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI. - The PUSCH repetition type and the time domain resource allocation table are determined by the PUSCH repetition type and the time domain resource allocation table associated with the UL grant received on the DCI, respectively, as defined in Clause 6.1.2.1.

## Issue #2: Part 2 CSI dropping for UCI multiplexing on PUSCH repetition Type B

It was pointed out in [2] that for UCI multiplexing on PUSCH repetition Type B, there is some text on Part 2 CSI dropping in TS 38.214 Clause 5.2.3, which needs to be modified and was missed at the time when the related agreements were made.

The following TP was proposed in [2] for TS 38.214 Clause 5.2.3 on Part 2 CSI dropping:

### 5.2.3 CSI reporting using PUSCH <unchanged part omitted> When the UE is scheduled to transmit a transport block on PUSCH not using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when is larger than α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , , , , , , QACK/CG-UCI' and are defined in Clause 6.3.2.4 of [5, TS 38.212]. Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the to be less than or equal to α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'. When the UE is scheduled to transmit a transport block on PUSCH using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr is larger than minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' , l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , Nsymb,nominalPUSCH, Nsymb,actualPUSCH, Msc,nominalUCIl, Msc,actualUCIl, , , QACK/CG-UCI', , and are defined in Clause 6.3.2.4 of [5, TS 38.212]. Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr to be less than or equal to minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACKCG-UCI'-QCSI-1' , l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1'. When part 2 CSI is transmitted on PUSCH with no transport block, lower priority bits are omitted until Part 2 CSI code rate, which is given by OCSI-2+LCSI-2/(NL⋅Q'CSI,2⋅Qm) where OCSI-2, LCSI-2, NL,Q'CSI,2,Qm are given in clause 6.3.2.4 of [5, 38.212] before HARQ-ACK puncturing part 2 CSI if any, is below a threshold code rate lower than one, where - is the CSI offset value from Table 9.3-2 of [6, TS 38.213] - R is signaled code rate in DCI <unchanged part omitted>

# 3	Scope for email discussions

Given that both issues #1 and #2 are essential, it is recommended that both issues are discussed in the email discussion. The following was agreed as the scope of the email discussion:

[104-e-NR-L1enh-URLLC-03] Email discussion/approval on remaining issues on PUSCH enhancements – Sigen (Apple) by Feb 3

- Issue 1: New RRC parameter for TDRA indication to support up to 64 entries in a TDRA table for Type 1 configured grant with PUSCH repetition Type B

- Issue 2: Part 2 CSI dropping for UCI multiplexing on PUSCH repetition Type B

# References

- R1-2100090, Draft CR on resource allocation for uplink transmission with configured grant, ZTE

- R1-2101347, Remaining issues on UCI and PUSCH enhancements for eURLLC, Apple