3GPP TSG RAN WG1 #104-e-	R1-2102084

e-Meeting, January 25th – February 5th, 2021

Source:	moderator (vivo)

Title:	Feature lead summary#3 on Enhancements on Multi-TRP inter-cell operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

Following agreements were reached in RAN1#103e

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

In section 2, issues raised in contributions are summarized and proposals are provided.

Outcome of GTW session (25th, 29th Jan)

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Conclusion:

Reuse Rel-15/16 mechanism of QCL chain rule between the source and target RS/channel for non-serving cell RS/channel

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP  in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

Updated FL proposals

Updated proposal 1-3:

For inter-cell MTRP operation,

- CORESETs in pdcch-config can be associated with a non-serving cell based on the activated TCI state for the CORESET

- Number for non-serving cell configuration,

  - Option1: gNB can configure one non-serving cell to be associated with the configured TCI states by RRC

  - Option2: gNB can configure more than one non-serving cells to be associated with the configured TCI states by RRC

Further discuss following options and down select in RAN1#104bis-e

Option1: Explicitly indicate non-serving cell PCI in the TCI state

- FFS other non-serving cell information

Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

- FFS: how the flag is linked to non-serving cell

Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

- Each group is associated with a CORESETPoolIndex value.

- FFS: how to link the group of TCI states to non-serving cell.

Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

- Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

- FFS: detailed re-indexing rule(s) of non-serving cell RSs

Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

- FFS: how the indicator is linked to non-serving cell

- Note: when there is only one non-serving cell, it means the same as Option2.

Updated Proposal 5: Further study following three possible schemes for rate matching

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB

- Scheme2: PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Scheme3: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB