3GPP TSG RAN WG1 Meeting #104-e                                          	                    R1-2102129

e-Meeting, January 25th – February 5th, 2021

Source:	Moderator (CATT)

Title:	              Feature lead summary #2 on AI 7.2.4 Sidelink synchronization mechanism

Agenda Item:	7.2.4

Document for:	Discussion and Decision

- Introduction

In this meeting, the agreements and endorsed TP and final CR information can be found in section 2 of discussion outcomes.

- Discussion outcomes

[104-e-NR-5G_V2X-02] Clarification on the S-SSB slot

- SY-4: Determination of slots including S-SSB transmission

- SY-6: Restriction of S-SSB slot

- Corrections for SY-1 Correction on SL-BCH, SY-2 Corrections/clarifications on S-SSB and SL-SSID can be discussed in the CR preparation.

Update on 1/28: to check on 1/29   to check on 2/1

Update on 2/1: latest two TPs are endorsed. To check the draft CRs 2/2

Update on 2/3:

- The four draft CRs (38.211, 38.212, 38.213 (two)) are approved.

Issue SY-4: Determination of slots including S-SSB transmission

Endorsed

- Adopting the following TP:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- 16.1 Synchronization procedures ------------------- < Unchanged parts are omitted > -------------------------- A UE is provided, by sl-NumSSB-WithinPeriod, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB+1⋅iS-SSB, where ------------------- < Unchanged parts are omitted > -------------------------- -------------------------- End of Text Proposal for TS 38.213 --------------------------

Issue SY-6: Restriction of S-SSB slot

Endorsed

- Adopt the following TP in TS 38.213

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- 16.1 Synchronization procedures --------------------------<Unchanged parts omitted>-------------------------- A UE is provided, by sl-NumSSB-WithinPeriod, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB⋅iS-SSB, where - index 0 corresponds to a first slot in a frame with SFN satisfying (SFN mod 16)=0 - iS-SSB is a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1 - NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by sl-TimeOffsetSSB - NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by sl-timeInterval For paired spectrum, an S-SS/PSBCH block can be transmitted/received only in a slot of an UL carrier. For unpaired spectrum, an S-SS/PSBCH block can be transmitted/received only in a slot of which all OFDM symbols are semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration-r16 if provided or sl-TDD-Config-r16 of the received PSBCH if provided. Or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], an S-SS/PSBCH block can be transmitted/received in any slot of the spectrum. --------------------------<Unchanged parts omitted>-------------------------- -------------------------- End of Text Proposal for TS 38.213 --------------------------

Final CRs

- R1-2102097  (38.211, CR0067)

- R1-2102098  (38.212, CR0062)

- R1-2102095  (38.213, CR0195)

- R1-2102096  (38.213, CR0196)