3GPP TSG RAN WG1 Meeting #104-e                                          	                    R1-2102130

e-Meeting, January 25th – February 5th, 2021

Source:	Moderator (CATT)

Title:	              FL summary #2 on other issues in NR Sidelink enhancement

Agenda Item:	8.11.2

Document for:	Discussion and Decision

- Introduction

The 1st FL summary of all issues and contribution proposals can be found as in R1-2101801.

In this meeting, the agreements on the remaining issues for sidelink evaluation methodology update for power saving are captured in section 2.

- Discussion outcomes

[104-e-NR-R17-SL-03] Email discussion on 8.11.2 (remaining issues for sidelink evaluation methodology update for power saving) – Teng (CATT)

Issue 1.1.1 Reference system deployments for Commercial use cases

Agreements:

- For commercial use case, at least following layout options are supported:

  - Option 3 of TR 36.843: Urban macro (500m ISD) (all UEs outdoor)

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

  - Option 1: Urban macro (500m ISD) + 1 RRH/Indoor Hotzone per cell for optional

    - UE dropping as in Table A.2.1.1-1

      - Mix of outdoor and indoor UEs

  - Option 5 of TR 36.843: Urban macro (1732m ISD) for optional

    - UE dropping as in Table A.2.1.1-1

      - All UEs are outdoors UEs

      - Mix of outdoor and indoor UEs

Issue 1.1.2 Traffic models for Public Safety use cases

Agreements:

- For public safety use case, at least the following options are supported for traffic model:

  - Option 2: VoIP model specified in TR 36.843

  - Option 4: FTP model 3 in TR 38.840 with packet size of 0.5Mbytes and mean inter-arrival time of 200ms

  - Option 7: Periodic traffic model 3 specified in TR 37.885

  - Option 9: VoIP model specified in TR36.843 with change of the value of outage definition into 0.01 and with packet delay budget of 75 ms

- Companies are encouraged to provide results for more than one traffic model including option 7

Issue 1.1.3 Traffic models for Commercial use cases

No agreement.

Issue 1.2.3 Definition of different profiles for P2V/V2P/V2V

Agreements:

- For V2P evaluation, the mixture of at least V2P traffic and V2V traffic is supported.

- Each Tx V-UE performs either only V2V traffic or only V2P traffic.

- NOTE: Companies are encouraged to report the ratio between V-UEs performing V2V traffic and V-UEs performing V2P traffic.