3GPP TSG-RAN WG1 Meeting #104e	R1-2102207

e-Meeting, January 25 – February 5, 2021

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR | 0170 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for the configuration of SRS for positioning in SRS carrier switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In RAN1#102, we made the following agreements. Agreement: Carrier switching for aperiodic SRS for positioning is not supported in Release 16 Agreement: Carrier switching for periodic and semi-persistent SRS for positioning is not supported in Release 16. To reflect that, the following paragraph was added to the specification The UE does not expect to be configured with SRS-PosResource on a BWP not configured with PUSCH/PUCCH transmission.. This was not aligned with the understanding of SRS carrier switching, as the condition of SRS carrier switching should be a carrier, rather than a BWP, that is not configured for PUSCH/PUCCH transmission. For example, positioning SRS can be configured within a BWP not configured for PUSCH/PUCCH transmission while at least another BWP on the same carrier is configured for PUSCH/PUCCH transmission. |
| Summary of change: |  | The following changes were agreed to clarify the configuration of SRS for positioning in SRS carrier switching, as per outcome of the email discussion [104-e-NR-Pos-02], summarized in R1- 2102026, section 2.4. 1. Replace the term "BWP" with "carrier of a serving cell with slot formats comprised of DL and UL symbols" in the paragraph describing SRS for positioning being not supported for SRS carrier switching. 2. A typo in the paragraph is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification description is not aligned with agreement and the generic assumption applied for SRS carrier switching. One typo in the paragraph remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: There is no isolated impact for the changes, as the changes are attempting to clarify and align with existing description for SRS carrier switching. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

#### 6.2.1.4	UE sounding procedure for positioning purposes

When the SRS is configured by the higher layer parameter SRS-PosResource-r16 and if the higher layer parameter spatialRelationInfoPos-r16 is configured, it contains the ID of the configuration fields of a reference RS according to Clause 6.3.2 of [TS 38.331]. The reference RS can be an SRS configured by the higher layer parameter SRS-Resource or SRS-PosResource-r16, CSI-RS, SS/PBCH block, or a DL PRS configured on a serving cell or a SS/PBCH block or a DL PRS configured on a non-serving cell.

The UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol.

If the UE is not configured with the higher layer parameter spatialRelationInfoPos-r16 the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource-r16 across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

The UE is only expected to transmit an SRS configured by the higher layer parameter SRS-PosResource-r16 within the active UL BWP of the UE.

When the configuration of SRS is done by the higher layer parameter SRS-PosResource-r16, the UE can only be provided with a single RS source in spatialRelationInfoPos-r16 per SRS resource for positioning.

For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource-r16 collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs.

The UE does not expect to be configured with SRS-PosResource-r16 on a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission.

< Unchanged parts are omitted >