3GPP TSG RAN WG1 Meeting #106-e	R1-2106446

E-meeting, August 16 –August 27, 2021

Agenda Item:	8.2.5

Source:	Huawei, HiSilicon

Title:	PDSCH/PUSCH enhancements for 52-71GHz spectrum

Document for:	Discussion and decision

Introduction

In RAN1#105-e, some progress was made on schemes for multi-slot scheduling and HARQ for PDSCH and PUSCH enhancement for 52-71GHz spectrum [1]:

- Collision handling mechanism between uplink resource and PDSCH, between downlink resource and PUSCH are defined, the related HARQ issue is FFS;

- The TDRA table can indicate whether the PDSCH/PUSCHs are in consecutive slots, the maximum value of the gap between two adjacent PDSCH/PUSCHs, and between the first PDSCH/PUSCH and the last PDSCH/PUSCH are FFS;

- More details are defined of Alt-1 and Alt-2 for generating type-2 HARQ-ACK codebook;

For the other issues related to reference signal and timeline enhancement, the progress are made in RAN1#104b-e [2]:

- Type 1 DMRS is supported for new SCSs;

- More evaluations are encouraged, and evaluation results under non-optional assumptions has highest priority;

- A model-based approach is not used to derive the timelines for single PDSCH/PUSCH and multi-PDSCH/PUSCH scheduling for NR operation in 52.6 GHz to 71 GHz.

In this paper, we continue to discuss some key aspects for multi-slot scheduling enhancement, HARQ enhancement, RS enhancement, and timeline definition of new SCSs in the following sections.

Timelines for the new SCSs

General discussion

In RAN1#104-e, the absolute time same as that of 120 kHz was agreed as an upper bound for timelines of new SCSs. Several model-based approaches were also proposed by companies to further reduce the absolute time duration compared with that of 120 kHz SCS, e.g. exponential models, projection based on log-linear regression method and etc. However, these model-based approaches are just derived mathematically from the existing values defined for lower SCS without solid evidence from implementation. On the other hand, the absolute time of 120 kHz SCS timeline has already been verified in FR2 implementation. So we propose to adopt the absolute time of 120 kHz SCS timeline by default, except for the cases identified to be able to reduce from implementation perspective.

Proposal 1: The absolute time of 120 kHz SCS timelines should be adopted by default unless the reduced value for specific timeline(s) can be verified by implementation.

N1, N2, N3

In FR2, PDSCH processing time Tproc,1 defined for feedback of HARQ-ACK information, is determined by N1, the PDSCH decoding time in units of symbol, which is associated with the location of DMRS symbol as shown in Table 1 for capability 1. PUSCH preparation procedure time Tproc,2 defined for PUSCH transmission, is determined by N2, the PUSCH preparing time in units of symbols as shown in

Table 2 for capability 1. The minimum gap between the second detected DCI and the first PUCCH resource for HARQ-ACK multiplexing is defined by N3 in units of symbols, as shown in Table 3.

Table 1. PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 2. PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 3. HARQ multiplexing timing

|  | HARQ multiplex time N3 [symbols] |  |
| --- | --- | --- |
|  | processingType2Enabled is not configured | processingType2Enabled=enable |
| 0 | 8 | 3 |
| 1 | 10 | 4.5 |
| 2 | 17 | 9 |
| 3 | 20 | -- |

For single slot scheduling of 480 kHz and 960 kHz, the symbol duration and slot duration is scaled down with SCS. One should consider that additional algorithms might be introduced to fight against significantly increased phase noise, for example, ICI compensation, which needs convolution in frequency domain, or IFFT and FFT operation in time domain. Therefore, unless there is clear evidence that the processing time can be reduced, we prefer the same absolute time as that of 120 kHz defined in FR2.

For multi-PDSCH scheduling, additional PDSCH decoding time will be needed if the channel is jointly estimated over all the DMRS symbols in the multiple scheduled slots. In this case, UE would need to defer the start of PDSCH decoding for as many as 42 symbols if a single DCI schedules 4 slots. In addition, there are other features under discussion which have impacts on the PDSCH decoding time, such as the number of HARQ processes, the number of PUCCH resource for HARQ feedback of multi-PDSCH scheduled by a single DCI, and UE complexity of the phase noise estimation and compensation associated with the PTRS pattern. If multiple PUCCH resources are configured for multi-PDSCH scheduled by a single DCI, then whether the definition of N1 still refers to the last symbol of the last PDSCH should be considered too. So we propose to postpone the discussion on timeline for multi slot scheduling until the designs as mentioned above has been determined.

Proposal 2:  For single slot scheduling with 480 kHz and 960 kHz SCS, N1, N2 and N3 values providing same absolute processing time as that of 120 kHz SCS in FR2 is preferred.  The values should be further studied for the case of multi slot scheduling after the detailed schemes are clear, which includes the position of DMRS, the number of PUCCHs for HARQ feedback, PTRS pattern.

k0, k1, k2

In FR2-1, ki, i=0,1,2 indicated by DCI and RRC for determining the starting slot of PDSCH, uplink resource carrying HARQ-ACK information corresponding to the PDSCH, and PUSCH, and the maximum values of ki are {32, 15, 32} respectively, with slot as the unit, which means the maximum absolute time durations of the offset are {4, 1.875, 4} ms, leaving sufficient room for UE with different processing capability, or for different scheduling scenarios.

The TDD configuration of 480 or 960 kHz SCS should preferably be aligned with that of 120 kHz SCS, to maximize the resource allocation utilization when 120 kHz is used for initial access. This means the number of consecutive DL slots and number of consecutive UL slots in a TDD configuration period should be linearly scaled up by the ratio of between the SCS used for data and 120 kHz, as shown in Figure 1.

Figure 1. TDD configuration of 480 kHz and 960 kHz that aligns with that of 120 kHz

With the scaled TDD configuration and non-continuous resource mapping in time domain, extended values are needed for k0 and k2. If separate k0 or k2 is signaled for each SLIV for the case of non-continuous resource allocation, then the value range should be extended to 0~128 and 0~256 for 480 kHz and 960 kHz respectively.

Proposal 3:  The value range of k0 and k2 should be extended to 0~128 and 0~256 for 480 kHz and 960 kHz SCS respectively.

As for the value range of k1, different scenarios should be considered. For PDSCH scheduled by DCI format 1_0, the value of k1 is limited in set {1, 2, 3, 4, 5, 6, 7, 8} slots in the current specification. The exact value is indicated by field “PDSCH-to-HARQ_feedback timing indicator” in DCI. For this scenario, a new set with scaled value should be defined for 480 kHz and 960 kHz. DCI format 1_0 won’t support multi-PDSCH scheduling, and the TDD configuration should be aligned among different SCSs. Therefore, the new set should be {4, 8, 12, 16, 20, 24, 28, 32} for 480 kHz, and {8, 16, 24, 32, 40, 48, 56, 64} for 960 kHz, resulting in the same absolute HARQ feedback delay as 120 kHz SCS, based on the assumption that the absolute UE PDSCH processing time is inherited from 120 kHz as proposed in section 2.2. But in DCI 1_1, as for the analysis presented for k0 and k2, a larger value range can make sure each PDSCH can be indicated with a valid uplink resource to carry ACK information. In addition, in case the location of the last PDSCH in multi-PDSCHs can be located anywhere within the most slots, the value of k1 should support finer granularity than that of DCI format 1_0. Then the value ranges of -1~63 and -1~127 are preferred for the two new SCSs, wherein value “-1” corresponds to a non-numerical value, for the case A/N feedback timing is not explicitly included at the time of scheduling PDSCH. For DCI 1_2, a separate information element are defined in Rel-16 for A/N timing determining, with the value range 0~15 for 120 kHz. Then the extended scheme except “-1” can be applied too.

Proposal 4:  The value range of k1 should be defined for DCI format 1_0, 1_1 and 1_2 separately:

For DCI format 1_0: define a new set with scaled values for 480 kHz and 960 kHz respectively;

- {4, 8, 12, 16, 20, 24, 28, 32} for 480 kHz

- {8, 16, 24, 32, 40, 48, 56, 64} for 960 kHz

For DCI format 1_1: define -1~63 and -1~127 as the value range of k1 for 480 kHz and 960 kHz;

For DCI format 1_2: define 0~63 and 0~127 as the value range of k1 for 480 kHz and 960 kHz.

Z1, Z2, Z3

In FR1 and FR2, CSI computation time Tproc,CSI has been specified by Zi, i=1,2,3, which defines a gap between the PDCCH that triggers a CSI report and the uplink resource that carries the CSI report according to different SCS, as shown in Table 4, where Xi is the UE reported capability beamReportTiming with value set to be {14,28,56} symbols for 120 kHz, and KBi is the UE reported capability beamSwitchTiming with value set to be {14,28,48,224,336} symbols for 120 kHz. From the table, we observe that not all the absolute time duration decreases with increasing SCS, for example, values of Z1 for 60 kHz and 120 kHz.

PDSCH demodulation and CSI computing typically share computing resources in the UE. As analyzed in the previous section, PDSCH processing for 480 kHz and 960 kHz may need more time or more computing resource due to the more complex demodulation algorithm e.g. to deal with increased phase noise, which will impact the available CSI computing time. Practically, the absolute time duration for CSI computing time can remain the same as that of 120 kHz SCS.

Table 4.  CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  | Z3 [symbols] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 | Z3 | Z'3 |
| 0 | 22 | 16 | 40 | 37 | 22 | X0 |
| 1 | 33 | 30 | 72 | 69 | 33 | X1 |
| 2 | 44 | 42 | 141 | 140 | min(44, X3X3+ KB1) | X2 |
| 3 | 97 | 85 | 152 | 140 | min(97, X3+ KB2) | X3 |

Proposal 5:  For 480 kHz and 960 kHz SCS, Z1, Z2 and Z3 values providing same absolute processing time as that of 120 kHz SCS in FR2 is preferred.

Enhancement on PDSCH and PUSCH

Multi-PDSCH/Multi-PUSCH scheduling

Resource allocation and collision

In RAN1#104b-e, the TDRA table configuration similar as that of multi-PUSCH scheduling defined in Rel-16 was agreed for 480 kHz and 960 kHz. In the TDRA table, independent mapping type are configured for each SLIV of each row. On top of this agreement, more than one PDSCHs/PUSCHs with mapping type B in a row could be scheduled within the same slot. This seems unnecessary due to the extremely short slot duration of new SCSs.

Proposal 6:  For single TRP operation:

- A UE is not expected to be scheduled more than one PDSCH in a slot with a single DCI or with multiple DCIs for 480 kHz and 960 kHz SCS.

- A UE is not expected to be scheduled more than one PUSCH in a slot with a single DCI or with multiple DCIs for 480 kHz and 960 kHz SCS.

In RAN1#105-e, agreements were made on the resource allocation for multi-PDSCH/PUSCH indication, resource collision between PDSCH and uplink resource and between PUSCH and downlink resource achieved:

Agreement: For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For multi-PDSCH/PUSCH scheduling by a single DCI, non-continuous resource allocation in unit of slot is supported as a new feature in FR2-2. It is used to increase resource utilization in the case of multiple different channels/signals located at different times, such as SSB, PUCCH, CSI-RS, and SRS. In order to achieve flexible resource allocation in time domain to avoid other channels/signals, we prefer not to limit the gap between two adjacent PDSCHs/PUSCHs scheduled by a single DCI.

The maximum number of PDSCHs/PUSCHs scheduled by a single DCI was agreed to be 8 in the previous meeting. On top of this agreement, there are two interpretations for the non-continuous resource allocation. One interpretation is that the whole duration from the first PDSCH to the last PDSCH scheduled by a same DCI is 8 slots, as shown in Figure 2(a). Under this limitation, a best alignment among different scheduling corresponding to different SCS can be ensured. The other interpretation is that no maximum gap will be defined, and each SLIV in each row of TDRA table will configure a separate  k0 or k2. Then the whole duration is limited by the maximum value of  k0 or k2, denoted by X in Figure 2(b). The second interpretation can achieve higher scheduling efficiency and more flexible resource allocation in time domain.

Figure 2. Two interpretations of the whole duration scheduled by a single DCI

Proposal 7: Support not limiting the maximum value of gap between two adjacent PDSCHs/PUSCHs. The maximum value of gap between the first PDSCH/PUSCH and the last PDSCH/PUSCH scheduled by a single DCI is limited by the maximum value of k0 and k2.

It was agreed that the PDSCHs collided with uplink resource and the PUSCHs collided with downlink resource are invalid. Furthermore, the handling method for HARQ-related issues is still FFS, such as the HARQ processing index. In current specification related to multi-PUSCHs scheduling, the HARQ process index of the first PUSCH is indicated by DCI, and the HARQ process index is incremented one by one for the subsequent PUSCHs. For the PDSCH/PUSCH which collides with an occupied uplink/downlink resource, the corresponding HARQ process index should be allocated as usual, considering the following aspects:

- A unified procedure for determining HARQ process index whether the collided resource are known to the UE or not. If the collided resource is known to UE, then the HARQ process index can skip the collided PDSCH to save HARQ process index. But it may lead to inconsistent HARQ process index determination by gNB and UE if the collided resource detection is missed by UE, such as UE missed the DCI format 2_0 that indicated SFI. So the Rel-16 HARQ process index procedure should be reused.

- HARQ process index allocated to the collided PDSCH/PUSCH will be freed in the next scheduling, because the retransmission DCIs signal both HARQ process index and NDIs.

To keep the length of the HARQ-ACK codebook, the UE would feedback an NACK for the collided PDSCH on the corresponding position in the codebook. In this case, the gNB will ignore the NACK corresponding to the collided PDSCH and possibly transmit new data with the same HARQ process index.

Proposal 8: As for Rel-16 multi-PUSCH scheduling, determine the HARQ process ID for each PDSCH/PUSCH by incrementing the HARQ process ID by one starting from the first PDSCH/PUSCH, independently of potential resource collisions with UL/DL symbols. If the resource collides with a pre-configured resource, NACK corresponding to the collided PDSCH should be reported by the UE.

DCI format design

In RAN1#105-e, the following 2 agreements were reached on the DCI format design. We also recall that there are still some aspects FFS on 2nd TB scheduling, CBG rule for new SCSs, VRB-to-PRB mapping, PRB bundling size, rate matching indicator, ZP CSI-RS trigger, FDRA enhancement, and frequency hopping enhancement.

Agreement: - Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling. - Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI. - Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI. Agreement: - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Common fields

Two codewords or two TBs are introduced when the number of layers is larger than 4 for one UE in Rel-15. If the number of transmission layers is no larger than 4, then only one TB is mapped to all layers based on an interleaved method. The demodulation performance of the only TB is averaged among layers.

In the frequency range from 52.6GHz to 71GHz, the beam width becomes narrower with the increasing number of transmit antennas. Hence, there would be fewer co-scheduled UEs in the same beam. Moreover, the difference of channel quality among different layers for the same UE will be larger. On the one hand, fewer co-scheduled UEs creates an opportunity of transmission with more layers (more than 4) per UE. On the other hand, indicating a unified MCS for all layers with different channel quality is not efficient. Therefore, a 2nd TB with a separate MCS can be introduced for multi-slot PDSCH/PUSCH scheduling, even if the number of transmission layers is no more than 4. Similar to the indication of MCS of the 1st TB, the MCS for the 2nd TB can be applied commonly to all the scheduled PDSCHs. Meanwhile, NDI and RV for the 2nd TB are suggested to be indicated individually for each scheduled PDSCH/PUSCH as being done for the 1st TB.

Proposal 9: Support scheduling 2nd TB for multi-slot PDSCH/PUSCH scheduling. MCS for the 2nd TB is applied commonly to all the scheduled PDSCHs/PUSCHs, even if the number of layers is less than 5. NDI and RV are indicated individually for each scheduled PDSCH/PUSCH.

For frequency domain resource allocation, increasing the RBG size has been proposed to decrease the FDRA size in DCI, because fewer UEs will be FDMed on the whole bandwidth due to the narrower beam width in the new frequency range. However, larger RBG size will limit the flexibility of resource allocation in frequency domain, especially for 480 kHz and 960 kHz. Moreover, the overhead of FDRA indication does not increase because the maximum number of PRBs is equal to or less than the value defined in FR2 for 120 kHz due to the same assumption of FFT size per carrier. Therefore, it is not essential to further reduce FDRA indication.

Observation 1: Further enhancements of FDRA are not essential for either multi-slot PDSCH scheduling or multi-slot PUSCH scheduling.

It was agreed that CBGTI field is present only if a single PUSCH is scheduled and a CBG-based (re)transmission is configured for 120 kHz, where the single PUSCH is one configuration of TDRA table containing at least one row with multiple SLIVs. The motivation of CBG-based (re)transmission introduced in Rel-15 is to avoid a whole TB transmission if UE fails to decode only part of a TB interfered by a bursty interference. Considering the short slot duration for 480 kHz and 960 kHz, the probability for a partial TB transmission failure within one slot is relatively small. Moreover, the DCI overhead would increase dramatically if CBGTI is configured in multi PUSCH scheduling. Therefore, we propose to apply the behavior related to CBGTI of 120 kHz to both 480 kHz and 960 kHz.

Proposal 10: CBGTI is not present if multi-PDSCHs is scheduled among a TDRA table including at least one row with multiple SLIVs.

Multi-PDSCH/PUSCH scheduling by a single DCI with non-continuous resource in time domain was agreed in the previous meeting. In this case, the resource allocation determining procedure should be redefined. The simplest way is to introduce multiple values of k0 (k2) for each SLIV, for example, separate k0 (k2) for each PDSCH (PUSCH) in RRC configuration. Considering the largest number of rows in TDRA table is 64 in current specification, the supported combination of k0 (k2) for each SLIV is under a lot of limitations. Note that, typically the slots indicated by k0 (k2) per SLIV are consecutive, and hence the separate k0 (k2) per SLIV introduces a considerable amount of unnecessary RRC configuration payload. It should be possible to omit providing k0 (k2) value in case of consecutive slots, to save RRC overhead.

Proposal 11: For TDRA table configuration supporting non-continuous resource indication, signal a separate k0 (k2) for each SLIV. k0 (k2) can be omitted for a SLIV to indicate that the SLIV corresponds to the next slot from the previous SLIV (i.e. in case of two consecutive slots allocation).

Fields related to multi-slot PDSCH scheduling

An example of interleaved VRB-to-PRB mapping defined in Rel-15/16 is shown in Figure 1, where Li is the bundle size provided by higher layer parameter vrb-ToPRB-Interleaver, can be configured as 2 PRBs or 4 PRBs, N is the bundle number (an even number is assumed). As the maximum number of usable PRBs is not increased compared with that of 120 kHz SCS, the same set of bundle size can be reused for 480 kHz and 960 kHz SCS without increasing the complexity of interleaved VRB-to-PRB mapping.

Figure 3. Interleaved VRB-to-PRB mapping

Observation 2: The interleaved VRB-to-PRB mapping for 120 kHz SCS can be reused for 480 kHz and 960 kHz SCS.

PRB bundling procedure is defined with a PRG size for precoding granularity determination, where PRG size can be configured with one value in {2, 4, wideband}. Within the PRG, the same precoding is applied to consecutive PRBs. In Rel-15, PRG size is not only decided by explicit signaling, but also associated with number of the scheduled PRBs, VRB-to-PRB mapping configuration, to make sure the same precoding is applied to contiguous PRB. As analyzed in previous section, legacy FDRA scheme and interleaved VRB-to-PRB mapping mechanism with existed RBG size can be reused.  Therefore, the PRB bundling procedure defined in Rel-15 including RRC configuration and DCI indication can be a baseline with the same maximum number of PRBs.

Observation 3: PRB bundling mechanism defined in Rel-15 can be reused as a baseline for multi-PDSCH scheduling in this new frequency range.

Rate matching indicator in DCI is used to determine the unavailable resource for PDSCH in Rel-15/16, in units of RB in frequency domain, symbol in time domain. For PDSCH, at most two rate matching patterns can be configured by higher layer parameters RateMatchPattern. Each RRC configuration includes symbol level bitmap with 14 or 28 symbols, periodicity based on a time unit level bitmap, and a subcarrier spacing. The configured rate matching pattern is based on the configured SCS. Then, by adding two more SCSs choice in the configuration, rate match pattern can achieve sufficient flexibility without any enhancement on DCI field.

Observation 4: The existing configuration and indication related to RateMatchPattern can be reused.

ZP CSI-RS resource set is used to measure interference, or decrease the interference for other UE’s measurement. A UE can be configured with one or more ZP CSI-RS resource set configurations for different time domain behaviors by higher layer parameters.  For aperiodic ZP CSI-RS, which is triggered by DCI, it can be applied to all the PDSCHs scheduled by a single DCI. Meanwhile, for semi-persistent and periodic ZP CSI-RS, the maximum periodicity that can be configured is 640 slots based on the existing configuration settings, which corresponds to 80 ms with 120 kHz SCS. This would reduce to 20ms for 480 kHz, and 10ms for 960 kHz SCS, which doesn’t leave much flexibility for the network to adapt the overhead in case of slow channel variations. Therefore, some new periodicities can be considered to increase the configuration flexibility for 480 kHz and 960 kHz SCS, e.g. at least ensuring that a maximum periodicity of 80 ms is configurable.

Observation 5: Triggering scheme defined in Rel-15/16 can be reused directly for aperiodic ZP CSI-RS.

Proposal 12: Support periodic/semi-persistent ZP CSI-RS for 480 and 960 kHz SCS with periodicity up to 80 ms.

Fields related to multi-slot PUSCH scheduling

Considering the absolute time durations of a slot for 480 kHz and 960 kHz are 1/4 and 1/8 to that of 120 kHz respectively, intra-slot frequency hopping for multi-slot PUSCH scheduling causes too high complexity of UE. On the other hand, intra-slot frequency hopping can’t help to increase the decoding performance by providing frequency diversity, because different TBs are transmitted on different PUSCHs, and decoded separately. Moreover, the payload of the related field in DCI format is up to 1 bit, which can be ignored when compared with the whole size of DCI format. This bit could be reserved for something else, to keep the same complexity of PDCCH blind detection.

Observation 6: Further enhancements of frequency hopping for multi-slot PUSCH scheduling are not essential.

DMRS

The traditional DMRS pattern is agreed for 480 kHz and 960 kHz, which is based on the assumption of a single slot scheduling. However, based on this traditional per-slot DMRS pattern, as plotted in Figure 4(a), the channel estimation performance is not the best for multi-slots scheduling in the case that the channel is independently estimated for each slot. As an enhancement, joint channel estimation among slots can be used. But, joint channel estimation increases processing delay and receiving buffer size, because UE must defer the PDSCH decoding until all the DMRS symbols of the multi-PDSCH are received. It may have to be deferred for as long as 3 slots for 480 kHz SCS and 7 slots for 960 kHz SCS.

Figure 4. DMRS pattern in time domain per time unit, (a) reused DMRS pattern in R15 with only FL DMRS per slot (b) bundling DMRS

In addition, even if independent channel estimation is used for each slot with multi-PDSCH scheduling, the available time duration for PDSCH processing is different for different SCSs, as demonstrated in Figure 5. In the figure, the end of the last PDSCH is aligned among different SCSs. With the same absolute time duration defined for PDSCH processing capability, the actual PDSCH processing time difference is determined by T0, which is the time available between the last received DMRS and the end of the PDSCH allocation. Assuming only FL-DMRS are mapped, the time duration of T0 is {88*△t, 22*△t, 11*△t} for {120 kHz, 480 kHz, 960 kHz}, wherein △t is equal to the symbol duration of 960 kHz.

Since Rel-15, the PDSCH processing time capability depends on the location of DMRS, i.e. it depends on T0. For 120 kHz SCS with front-loaded DMRS, N1 is 20 symbols. For 120 kHz SCS with additional DMRS, N1 is 24 symbols, where the last additional DMRS could be as late as symbol 11. The last slot with 960 kHz SCS occurs even later than the equivalent symbol 11 of a 120 kHz slot. Therefore, any DMRS in the last (13-11)*(960/120)=16 symbols for 960 kHz SCS would require the same processing time as 24 symbols of 120 kHz SCS. To reuse the same absolute processing time as 20 symbols of 120 kHz SCS, there should be no DMRS in slots of 480/960 kHz that would occur later than the equivalent symbol 3 of a slot of 120 kHz SCS. In other words there should be no DMRS in the last part of the allocation corresponding to 10 symbols of 120 kHz, i.e. 40 symbols of 480 kHz (last 2.8 slots) and 80 symbols of 960 kHz (last 5.7 slots).

This could pose a problem if the multi-slot allocation schedules less than 2 slots or 5 slots for 480/960 kHz SCS, respectively. In this case, an alternative is to reuse the absolute time for PDSCH processing capability corresponding to 120 kHz SCS with additional DMRS, in which case there should be no DMRS in the last slot (for 480 kHz SCS) or in the last two slots (for 960 kHz SCS), approximately.

Alternatively, a longer PDSCH processing delay could be specified for 480 kHz and 960 kHz SCS with multi-slot scheduling, which would result in longer HARQ feedback delay compared to 120 kHz SCS.

Figure 5. DMRS location impact on PDSCH processing delay

To achieve both the best channel estimation performance and to avoid a higher HARQ feedback delay for 480/960 kHz SCS compared with 120 kHz SCS, the DMRS symbols can be concentrated at the beginning of the scheduled multi-slot PDSCHs/PUSCHs, referred to as bundling DMRS. For example, the DMRS are mapped on symbol l0 to l0+S*L-1, as shown in Figure 4 (b), where, l0 can be set to the first symbol after the PDCCH, L is the DMRS length indicated by RRC and DCI, and S is a linear scaling factor determined by SCS.

As shown in Figure 6, the BLER performance of the two DMRS patterns are compared. In the evaluations, the channel estimation is performed independently per slot for the legacy pattern (Figure 4 (a)), denoted by “Pat(a) + independent”, and the channel is jointly estimated based on all the consecutive DMRS symbols for the proposed pattern (Figure 4 (b)), denoted by “Pat(b)+joint”. From the results, we can see that gains about 0.2dB~0.6dB are provided by the proposed DMRS pattern.

Figure 6. BLER performance comparison with different time domain pattern of DMRS

Observation 7: For 480 kHz and 960 kHz, bundling DMRS in one slot per multi-slot outperforms the legacy DMRS pattern mapped per slot.

Proposal 13: Support DMRS mapped on symbol l0 to symbol l0+S*L-1 within the first PDSCH slot for the multi-slot scheduling, when only front-loaded DMRS is configured, where L is the length and l0 the starting symbol of legacy front-loaded DMRS.

PTRS enhancement

PTRS for CP-OFDM enhancement

- PTRS pattern

In FR2-1, 120 kHz SCS is supported for both data/control channel.  CPE dominates the negative effect of phase noise, which can be effectively estimated on the PTRS designed in Rel-15. However, for the frequency band above 52.6GHz, the influence of phase noise is larger than that below 52.6 GHz, and hence both the CPE and ICI become more severe, which in turn lead to a decrease in the demodulation performance of high MCS.

Enhanced PTRS designs, such as block-based or chunk-based PTRS, can be considered for ICI estimation. A new PTRS sequence is introduced to increase the performance of ICI estimation, composed of a base sequence and a circular sequence as illustrated in Figure 7. The base sequence Si, i=1,…,N has a constant modulus after IFFT operation to provide a better estimation on ICI coefficients. ZC sequence can be a candidate for the base sequence. The circular sequence consists of a head circular part and a tail circular part. Specifically, the head circular part is composed of the tail of the base sequence, and the tail circular part is composed of the head of the base sequence. The length of the head part and the tail part are determined by the number of estimated ICI coefficients J, which should be no less than (J-1)/2 to ensure the received base sequence is not contaminated by unknown data.

Figure 7. Sequence and pattern of block-based PTRS

With ZC sequence as a base sequence and additional circular sequence design for block PTRS, the ICI coefficients can be estimated based on the method applied to CPE estimation, in which matrix inversion can be avoided. ICI estimation based on the PTRS and received signal are formulated as follows:

rp1rp2rp3…=SN-1 SN    S1SN S1    S2S1 S2    S3         S2   S3S3   S4S4   S5…SN-4SN-3SN-2SN-3SN-2SN-1SN-2SN-1SN     SN-1SNSNS1S1S2E-2E-1E0E1E2+n                                        (1)

The  Si’s (i=1,…,N) are the PTRS of base sequence, and the ICI order equal to 5 is assumed in the equation. Both the length of the head circular part and the tail circular part are 2. Denote by rpi the received signal corresponding to Si. The Ei’s,  i=0,±1,±2, are the most significant ICI coefficients to be estimated.

Applying the auto-correlation property of ZC sequence, we can estimate Ei directly by multiplying SH/N at both sides of the equation, where S is the matrix of PTRS transmission signal, as follows:

E-2E-1E0E1E2=SN-1 SN    S1SN S1    S2S1 S2    S3         S2   S3S3   S4S4   S5…SN-4SN-3SN-2SN-3SN-2SN-1SN-2SN-1SN     SN-1SNSNS1S1S2Hrp1rp2rp3…                                       (2)

The performance of the algorithm with lower complexity are evaluated in Figure 8 based on block PTRS with same PTRS overhead. In the figures, “Alg-1” denotes the algorithm discussed in [3] with a higher complexity, and “Alg-2” denotes the algorithm corresponding to format (2) with a lower complexity. From the results we can see, Alg-2 performs a little worse than the Alg-1, due to less number of PTRS per OFDM symbol used in the equation for ICI estimation. And it is worth noting that, the performance loss decreases as the number of PTRS increases, because the proportion of invalid PTRS is smaller.

Figure 8. BLER performance comparison between two algorithms

Moreover, power boosting for PTRS is introduced in Rel-15 to increase the accuracy of CPE estimation. But, power boosting will not work if distributed PTRS is used for ICI estimation based on the algorithm introduced in [3]. The formula for ICI estimation is shown in equation (3), where pi (i=1,2,3,…) is the subcarrier index of PTRS, rj is the received signal on subcarrier j, Ik(k=-2,…,2) is the ICI coefficient, and Spi (i=1,2,3,…) is the transmitted signal at subcarrier pi. For distributed PTRS, only the middle column  of the first matrix on the left-hand side of (3) are from PTRS with power boosting, while the rest of the received signals are without power boosting. Thus, the effect of power boosting is quite limited in the ICI estimation.

rp1-2rp1-1rp1rp2-2rp2-1rp2rp3-2rp3-1rp3     rp1+1rp1+2rp2+1rp2+2rp3+1rp3+2…I-2I-1I0I1I2=Sp1Sp2Sp3…+n	(3)	Based on multiple consecutive subcarriers of block PTRS, the received signal in each row of matrix r are mainly from PTRS with power boosted, improving the ICI estimation accuracy.

Before performance evaluation among different patterns, the influence of ICI order is simulated. For different ICI order and different length of block, the detailed parameters of sequence are shown in Table 5. The results are shown in Figure 9 for different number of scheduled RBs. We can observe that a higher ICI order performs better only when number of PTRS per OFDM symbol used for ICI coefficients estimation is sufficient. Considering the amplitude of ICI coefficients is different for different SCS, the best ICI order should be decided by number of PTRS, SCS and power boosting conditions.

Table 5. Length of base sequence and circular sequence for block PTRS

| Block size CS | 64 | 41 | 41 | 33 | 33 | 17 | 17 | 65 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ICI order estimated | 5 | 5 | 3 | 5 | 3 | 5 | 3 | 7 | 5 |
| Length of base sequence L | 59 | 37 | 37 | 29 | 31 | 13 | 13 | 59 | 61 |
| Length of head circular sequence q | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 3 | 2 |
| Length of tail circular sequence p | 3 | 2 | 2 | 2 | 1 | 2 | 2 | 3 | 2 |

Figure 9. Influence of ICI order to BLER performance

The BLER performance with 3 dB power boosting for both PTRS patterns are shown in Figure 10~Figure 12, wherein the best ICI order are chosen separately for each PTRS, each scheduled bandwidth, and each SCS. In the evaluations, Rel-15 PTRS with K=4 are used as a baseline. For block PTRS, CN denotes number of blocks, and CS denotes block size, as shown in Figure 7. At receiver side, the algorithm (denoted by Alg-1) discussed in [3] is applied to both PTRS patterns. All curves include power boosting for PTRS where the total power for RS and data in one OFDM symbol is the same for all schemes.

From the results, we observe that block PTRS with Alg-1 outperforms its counterparts in all cases in terms of BLER and spectral efficiency. The gains are {0.3~3dB, 0.2~1.5dB and 0.4~1.8dB} for {120 kHz, 480 kHz and 960 kHz} respectively, when compared with distributed PTRS.

Figure 10. BLER and spectral efficiency comparison between block PTRS and Rel-15 PTRS of 120 kHz with 3dB power boosting and same average power per OFDM symbol

Figure 11. BLER and spectral efficiency comparison between block PTRS and Rel-15 PTRS of 480 kHz with 3dB power boosting and same average power per OFDM symbol

Figure 12. BLER and spectral efficiency comparison between block PTRS and Rel-15 PTRS of 3dB power boosting and 960 kHz and same average power per OFDM symbol

Since power boosting doesn’t provide any performance benefit for distributed PTRS, additional evaluations with normalized power per OFDM symbol between two PTRS patterns are simulated. In these simulations, average power per RE for both PTRS patterns are “1”. Power boosting is only set for block PTRS, and the power of corresponding data is reduced to keep the average power per OFDM symbol same as that of distributed PTRS without power boosting. From the results shown in Figure 13 we can see that, block PTRS with 3dB power boosting and normalized power per OFDM symbol, performs better than distributed PTRS. The gain is at least 0.7dB when scheduled bandwidth is 128RB.

Figure 13. BLER and spectral efficiency comparison between power boosted block PTRS and Rel-15 PTRS without power boosting, with normalized power.

In addition, block-based PTRS allows PTRS sharing among different UEs with a localized pattern. Block-based PTRS can therefore be used to guarantee the ICI estimation performance for UE with narrow scheduled bandwidth, while maintaining a low overall PTRS overhead.

Observation 8: Block PTRS with the designed circular sequence has better BLER and spectrum efficiency than distributed PTRS when power boosting is applied.

Proposal 14: Support block PTRS as one type of PTRS pattern for 120 kHz, 480 kHz and 960 kHz SCS of CP-OFDM. The use of block PTRS can be indicated by MCS, scheduled bandwidth, and power boosting scheme. The PTRS sequence is composed of ZC sequence and circular part based on ZC sequence for block PTRS.

PTRS for DFT-s-OFDM

- Parameters of PTRS pattern

For DFT-s-OFDM, due to more severe phase noise than below 52.6 GHz, the interpolation distance between two adjacent PTRS groups of the legacy PTRS patterns is too long with large bandwidth and high MCS. An example of Rel-15 PTRS is shown in the Figure 14 when scheduled bandwidth is 256 RBs, in the case of NgroupPT-RS=8 and Nsamplegroup=4. Take scheduled bandwidth equal to 160 RBs as another example, the maximum interpolation distance is 360, meanwhile the minimum interpolation distance is 240, both are still too large to track the phase noise.

Figure 14. Interpolation distance with 256RB when PTRS pattern is (CN, CS) = (8, 4)

To reduce the phase noise impact with large scheduled bandwidth and high MCS, more PTRS groups within one DFT-s-OFDM symbol should be considered, for example with NgroupPT-RS=16 PTRS groups. Furthermore, considering the high SNR region of high MCS, the PTRS samples in each group responsible for noise reduction can be decreased accordingly, e.g. Nsamplegroup=2. Additional, NgroupPT-RS=16  and Nsamplegroup=2 brings no extra PTRS overhead meanwhile. The locations of the PTRS groups within DFT-s-OFDM symbol is illustrated in Figure 15.

The BLER performances with PTRS patterns of (NgroupPT-RS, Nsamplegroup) = (8, 4), (NgroupPT-RS, Nsamplegroup) = (16, 2) and (NgroupPT-RS, Nsamplegroup)=(16,4) for 120 kHz and 480kHz SCS are compared in Figure 16 and Figure 17 respectively. In the legend, CN denotes NgroupPT-RS, CS denotes Nsamplegroup. The BLER performance with PTRS patterns of (NgroupPT-RS, Nsamplegroup) = (4,4), (NgroupPT-RS, Nsamplegroup) = (8,4) and (NgroupPT-RS, Nsamplegroup) = (16,2) for 960 kHz SCS are compared in Figure 18. We can see that PTRS patterns with more PTRS groups improve the BLER performance significantly for MCS22 of all SCSs, while they perform almost the same as (NgroupPT-RS, Nsamplegroup) = (8, 4) for MCS16. It is worth noting that the BLER can be less than 0.01 only with PTRS pattern (NgroupPT-RS, Nsamplegroup) = (16, 2) for MCS26, due to the uniform interpolation distance.

Figure 15. PTRS location for the new PTRS patterns

Figure 16. BLER performance of different PTRS pattern for 120 kHz with 400 MHz

Figure 17. BLER performance of different PTRS pattern for 480 kHz and 960 kHz

Figure 18 	BLER performance of different PTRS pattern for 960 kHz

Observation 9: With more PTRS groups and less PTRS samples per PTRS group denoted as (NgroupPT-RS, Nsamplegroup) = (16, 2), the BLER performance of all the SCSs are improved significantly.

Proposal 15: A new PTRS pattern with more PTRS groups and less PTRS samples per PTRS group denoted as (NgroupPT-RS, Nsamplegroup) = (16, 2) within one DFT-s-OFDM symbol should be supported for large scheduling bandwidth and high scheduled MCS.

- Placement of PTRS groups

Figure 19.  The resulting problem due to Rx window timing shift when Nsamplegroup=4

In this section, we would like to draw attention to an insofar overlooked problem related to the placement of PTRS groups within an OFDM symbol in the case of Nsamplegroup=4. The problem is demonstrated graphically below in Figure 19, which depicts the placement of PTRS groups when Nsamplegroup=4 (for NgroupPT-RS=2, 4, 8), from both the transmitter and receiver perspectives. In a practical implementation, receiver typically sets some intentional timing margin for the CP removal and FFT windowing in units of the pre-DFT QAM symbol, called “advance shift” λ, in order to cope with imperfect synchronization and various multipath channel profiles of the UL transmissions from simultaneously FDM-ed UEs.

The impact of the advance shift depends on the size of λ, measured in units of the pre-DFT QAM symbol duration, which in turn depends on the allocation (or DFT) size M. The situation depicted in Figure 19 corresponds to the case of λ > NgroupPT-RS (= 4), which may happen when M is relatively large.

As aforementioned, the original “tail group” (in the red box) is wrapped-around and located next to the “head group”. We consider the following two to estimate the phase noise corresponded to the tail PTRS group, which only cause difference between the last group and the second last group:

- Estimate phase noise as with no receiver advance shift, as illustrated by orange circle and orange arrow in Figure 20. It suffers from a maximum estimation error, because the phase noise used for the last group are those carried on the head of the symbol.

- Drop the last PTRS group in the receiver, and copy the estimated value from the second last group to the last group, as illustrated by blue circle and blue arrow in Figure 20. It results in a long interval to extrapolate the estimated PN to the end of a DFT-S-OFDM symbol.

Figure 20.  The influence of phase noise estimation introduced by Rx advance shift

With increased carrier frequency, the phase noise variance is more severe, as shown by the CDF curves of phase noise for 28GHz and 60GHz in Figure 21. Then the estimated error introduced by the two methods of phase noise estimation in case of Rx advance shift cannot be ignored, and becomes the bottleneck of performance under high MCS.

Figure 21.  The CDF of phase noise under different carrier frequency

The influence of the Rx advance shift are evaluated with different shift values, whose results are demonstrated in Figure 22. The shift value is the ratio of advance shift duration to CP duration, and (NgroupPT-RS, Nsamplegroup) = (4, 4) are applied for all SCSs while (NgroupPT-RS, Nsamplegroup) = (8, 4) is only applied to 120 kHz. As can be observed, the influence caused by Rx advance shift can’t be ignored especially for high MCS, and it decreases with SCS due to the decreased sample number before DFT.

Figure 22.  Influence of BLER performance caused by Rx advance shift for different SCSs

Observation 10: Due to Rx timing shift, (at least part of) a PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol, may wrap-around to the head of the symbol from the receiver’s perspective, thus spoiling the original intention of the design and unnecessarily increasing Rx complexity, as well as deteriorating PN compensation performance, especially for the high MCS.

It should be emphasized that the actual value of λ, whether set statically or dynamically, is unknown to the transmitter, and it is tuned or decided by the receiver, based on considerations of imperfect synchronization,  and various multipath channel profiles of multiplexed UL transmission. Thus, the placement of PTRS groups should be robust with respect to the choice of advance shifts within a reasonable range, say, up to Z% of the CP duration (Z FFS, presumably larger than 10). Otherwise, the PNC performance and implementation complexity may both degrade.

As a simplest solution, each PTRS group can be located at the middle of interval regardless of PTRS group size, to avoid locating PTRS group at the tail of DFT-s-OFDM symbols, and provides uniform interpolation distance between adjacent PTRS groups as an additional advantage meanwhile. The evaluation results shown in Figure 23 are the BLER performance of existed PTRS placement and proposed PTRS placement. Just similar as demonstrated in Figure 15, from which we can see the proposed placement of PTRS groups provides a convincing benefit when Rx advance shift is exist for all SCSs, especially with high MCS.

Figure 23.  BLER performance comparison between different PTRS locations for different SCSs

Observation 11: New PTRS location which is in the middle of each interval can solve the influence on BLER performance caused by Rx advance timing shift.

Proposal 16: For PTRS with Nsamplegroup=4, the mapping of last PTRS group should consider potential Rx timing shift and avoid the last X pre-DFT symbol(s).

HARQ

Type-1 HARQ-ACK codebook

In RAN1#105e meeting, discussion is continued on how to determine the candidate PDSCH reception occasions according to the TDRA table and K1 set for type-1 HARQ-ACK codebook, and the following agreement was made and three FFS(s) are left for further discussion:

Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - FFS: details of further pruning of the set of SLIVs - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot - FFS impact of time domain bundling, if supported

Note that there is another agreement on how to deal with the issue when a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated as following:

Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. ……

For the first FFS, based on the second agreement, it is clear that if a SLIV among the set of SLIVs corresponding to a DL slot(belonging to the set of DL slots) is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, it will be pruned from the set of the SLIVs.

For the second FFS, when the collision happens among the SLIVs from different rows of TDRA table or same rows of TDRA table with different k0 if more than one PDSCH in a slot is allowed, which is scheduled by different DCIs. Considering the redundancy perspective, the number of candidate PDSCH reception occasion within a slot is determined not only by the none-overlapped SLIVs in that slot but also by the overlapped SLIVs in other slot.

For the third FFS, we suggest that time domain bundling should be considered when selecting a proper mapping between the HARQ-ACK feedback information and the final candidate PDSCH reception occasions.

Based on the above discussion, following are the steps on how to determine a type-1 HARQ-ACK codebook for one serving cell. We assume 1 bit HARQ-ACK information for one candidate PDSCH reception.

Two different TDRA tables are provided for example as below:

Table 6. TDRA table_1

| Row index | SLIV |
| --- | --- |
| 0 | {S=1,L=13}, slot-gap, {S=0,L=6} |
| 1 | {S=1,L=13}, slot-gap, {S=7,L=7}} |

Table 7. TDRA table_2

| Row index | SLIV |
| --- | --- |
| 0 | {S=1,L=13}, {S=0,L=6} |
| 1 | {S=1,L=13}, slot-gap, {S=7,L=7}} |

Assuming each TDRA table has two rows, each row has two SLIVs with a slot-based gap or not. Assuming the K1 sets has two values as {3, 2}. Assuming the slots from N-5~N-1 indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are all DL slots. Assuming the same PUCCH resource scheduled by DCIs for the transmission of HARQ-ACK feedback information is allocated in slot N.

Step 1, the set of DL slots are determined by the TDRA table and K1 set includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. If a SLIV among the set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, it will be pruned from the set of the SLIVs. As show in Figure 23 and Figure 24. The set of DL slots for both TDRA tables are {N-5, N-4, N-3, N-2}.

Step 2, for each pair of {row of SLIVs, k1}, if at least one SLIV are overlapped in symbols between two pairs, these pairs will be grouped together.

For example of TDRA table 1, there are two groups as below:

Group1: pair_1 {row1, 3} and pair_2 {row2, 3}; Group2: pair_3 {row1, 2} and pair_4 {row2, 2};

And for example of TDRA table 2, only one group as below:

Group1: pair_1 {row1, 3}, pair_2 {row2, 3}, pair_3 {row1, 2}, pair_4 {row2, 2}

Within each group, for each two pairs with non-overlapped SLIVs in the same DL slot, if overlapping of SLIVs occurs in other DL slots, those non-overlapped SLIVs will be considered as one candidate PDSCH reception, and the index of the candidate PDSCH reception is indicated by the earliest non-overlapped SLIV. For each two pairs with overlapped SLIVs in the same DL slot, those overlapped SLIVs will be considered as one candidate PDSCH reception. After looping all combinations of two pairs within the group, the size of the HARQ-ACK codebook is determined by the number of remaining PDSCH receptions. The order of HARQ-ACK bits within each group is indexed in ascending order of the start time of the remaining PDSCH receptions.

Table 8. Candidate PDSCH receptions of TDRA table_1

| Group index | Number of candidate PDSCH receptions | Position of candidate PDSCH receptions |
| --- | --- | --- |
| Group 1 | 2 | {N-5, N-3} |
| Group 2 | 2 | {N-4, N-2} |

Table 9. Candidate PDSCH receptions of TDRA table_2

| Group index | Number of candidate PDSCH receptions | Position of candidate PDSCH receptions |
| --- | --- | --- |
| Group 1 | 6 | {N-5, N-4, N-3, N-3,N-2,N-2} |

Step 3, the HARQ-ACK information is generated group by group. The MSB to LSB of the codebook is sequenced in group according to the start time of first PDSCH reception of each group.

For example of TDRA table 1, the earliest start time of Group 1 is slot N-5, and for Group 2, the earliest start time is slot N-4. So the HARQ-ACK codebook is grouped from Group 1 to Group 2 in sequence of {N-5, N-3, N-4, N-2} with size of 4 bit HARQ-ACK information.

For example of TDRA table 2, since only one group was obtained by step 2, the HARQ-ACK codebook is in sequence of {N-5, N-4, N-3, N-3, N-2, N-2}.

Figure 24. Example of TDRA table_1 and K1 set of type-1 HARQ-ACK codebook

Figure 25. Example of TDRA table_2 and K1 set of type-1 HARQ-ACK codebook

In the above examples, more than one PDSCH is allowed to be scheduled within the same slot. However, considering the large SCS above 52.6GHz, there is probably little benefit in allowing a UE to receive more than one PDSCHs in the same slot. Even for the latency-sensitive traffic a delay of one slot may not be critical with such short slot duration. If only one PDSCH is allowed to be scheduled in the same slot, it will reduce the complexity of the procedure in step 2, the number of candidate PDSCHs can simply be determined by the unique number of DL slots that can be scheduled by all the pairs after step 1.

Proposal 17: If more than one PDSCH is allowed to be scheduled in the same slot by different DCI, for enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, for each pair of {row of SLIVs, k1}, if at least one SLIV from a pair is overlapped with one SLIV from another pair, these pairs will be grouped together, the number and position of the candidate PDSCH receptions for each group are determined according to the procedure in step 2, the HARQ-ACK information is generated in group based sequence according to the start time of the first candidate PDSCH reception of each group.

Proposal 18: If only one PDSCH is allowed to be scheduled in the same slot, for enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, for cells belonging to FR2.2, the number and position of the candidate PDSCH receptions are determined by the unique number of DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing, the HARQ-ACK information is generated according to the index of DL slots.

Type-2 HARQ-ACK codebook

In the RAN1#105e meeting, two alternatives were continued discussed on how C-DAI/T-DAI is counted for the multi-PDSCH scheduling, and the following two agreement was made:

Agreement: If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

For Alt1, when C-DAI/T-DAI is counted per DCI, it is observed that HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-) codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group. If CBG transmission for single PDSCH scheduling is also supported in some serving cells, and the maximum configured number of CBG of PDSCH is bigger than the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells, the HARQ-ACK payload size will be increased further if we support single sub-codebook for multi-PDSCH and single PDSCH with CBG transmission. If we support separate sub-codebook for multi-PDSCH and single PDSCH with CBG, as some companies discussed in last meeting we need to discuss how to indicate the UL DAI for 3 sub-codebook.

For Alt 2, when C-DAI/T-DAI is counted per PDSCH, then the missing DCI issue can be avoided because UE knows exactly how many PDSCHs are missing, except when the last DCI is missing. The only downside of Alt 2 is the increased number of DAI bits, including C-DAI, T-DAI and UL DAI. The Bit-width of DAI for multi-PDSCH DCI could be configured by RRC signaling or according to the maximum size of SLIVs of each rows within configured TDRA table. If the maximum number of PDSCHs scheduled by a single DCI is N, to align with current capability of up to 3 consecutive DCI mis-detections (except the last DCI before the PUCCH), (2+log2 (N)) bit width is needed. There is an agreement made in last meeting that the bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR and not using fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling. To solve the ambiguity issue of DAI indication comes from not aligned size of DAI field in fall back and none full back DCI, separate sub-codebook could be supported for single PDSCH scheduling without CBG transmission and multi-PDSCH scheduling without CBG transmission. For at least one serving cells, if CBG transmission is configured for single PDSCH scheduling scheduled by fullback DCI or none-full back DCI configured with a TDRA table in which each row contains a single SLIV, then three sub-codebooks are needed.

For example, assuming a UE is configured with a cell group consists of three cells, one activated BWP for each cell, and each BWP is configured with a specific TDRA table. The UE is configured to search DCI 1_0 in cell 1 and cell 2, DCI 1_1 in cell 3. In cell 2, CBG transmission is configured with maxCodeBlockGroupsPerTransportBlock = 4. Assuming that gNB transmitted only one DCI in each BWP scheduled the same PUCCH resource in the primary cell. In cell 3, row 0 of TDRA table_2 is scheduled by DCI 1_1. For simplification, we assume 1 bit HARQ-ACK information for one PDSCH.  The overhead of HARQ-ACK codebook by Alt1 and Alt2 can be found in table 6. For Alt1 with two sub-codebook, due to the HARQ-ACK information of CBG transmission and multi-scheduling per DCI transmission are put together in the same sub-codebook, the size of HARQ-ACK bit information for one DCI is equals = max(maxCodeBlockGroupsPerTransportBlock, maxLengthofRowsOfTDRA) = 8. As a results, the redundancy of Alt1 with two sub-codebook is 10 bits which is even larger than the size of none-redundant HARQ-ACK codebook of Alt2 with three sub-codebooks.

Based on the above discussion, the pros and cons of all candidate solutions for Type-2 HARQ-ACK codebook are summarized in table 7. Huge redundancy in HARQ-ACK codebook will hurt the reliability for PUCCH with limited PRB number especially for low power device. However, the downside of Alt2 is only needed to extend the current DAI field in none fullback DCI, while the PRB number of CORESET could be configured by gNB in order to maintain the reliability and coverage, and the transmission power in gNB is not a bottleneck. Considering the performance of the network, we prefer Alt2 for type-2 HARQ-ACK codebook.

Table 10. TDRA table_1 for DCI 1_0 in cell 1/2

| Row index | SLIV |
| --- | --- |
| 0 | SLIV_1 |
| 1 | SLIV_1 |
| 2 | SLIV_1 |

Table 11. TDRA table_2 for DCI 1_1 in cell 3

| Row index | SLIV |
| --- | --- |
| 0 | SLIV_1, SLIV_2 |
| 1 | SLIV_1, SLIV_2, SLIV_3, SLIV_4, |
| 2 | SLIV_1, SLIV_2, SLIV_3, SLIV_4, SLIV_5, SLIV_6, SLIV_7, SLIV_8 |

Table 12. Overhead of candidate solutions for Type-2 HARQ-ACK codebook

| Candidate solutions for Type-2 HARQ-ACK codebook | Size of sub-codebook 1 | Size of sub-codebook 2 | Size of sub-codebook 3 | Total size of HARQ-ACK codebook |
| --- | --- | --- | --- | --- |
| Alt 1 with two sub-codebooks | 1 bit | 16 bits | / | 17 bits |
| Alt 1 with three sub-codebooks | 1 bit | 4 bits | 8 bits | 13 bits |
| Alt2 with three sub-codebooks | 1 bit | 4 bits | 2 bits | 7 bits |

Table 13. Pros and cons of candidate solutions for Type-2 HARQ-ACK codebook

| Assuming CBG transmission is configured at least in one serving cell for all candidate solutions | Pros | Cons |
| --- | --- | --- |
| Alt 1 with two sub-codebooks | No specification impact on DCI format | Huge redundancy in HARQ-ACK codebook among the three Alts. |
| Alt 1 with three sub-codebooks | Smaller redundancy in HARQ-ACK codebook than Alt 1 with two sub-codebooks | Specification impact on UL DAI indication (extension of UL DAI field is need to indicate three sub-codebooks DAI) |
| Alt2 with three sub-codebooks | Smallest redundancy in HARQ-ACK codebook | Specification impact on C-DAI/T-DAI/UL DAI field in non- fallback DCI |

Observation 12: Huge redundancy in HARQ-ACK codebook will hurt the reliability for PUCCH transmission.

Proposal 19: Support Alt 2 (C-DAI/T-DAI is counted per PDSCH) for type-2 HARQ-ACK codebook with separate sub-codebooks for single PDSCH without CBG transmission, for multi-PDSCH scheduling without CBG transmission, and for fallback DCI.

For time domain bundling of HARQ-ACK feedback with Alt 2, because C-DAI/T-DAI is counted per PDSCH not per bundling of PDSCH, UE will not know how many DCI formats are lost according to DAI information, so there will be ambiguity about the codebook size. In R16, harq-ACK-SpatialBundlingPUCCH is configured per cell group (i.e. for all cells within the cell group) for PUCCH reporting HARQ-ACK. If time domain bundling of HARQ-ACK per DCI is supported for multi-PDSCH scheduling, this feature could be configured per cell group as for spatial bundling, then there is no benefit to count C-DAI/T-DAI per PDSCH when bundling of HARQ-ACK feedback is configured for all the serving cells. C-DAT/T-DAI could be counted per DCI as Alt1.

Proposal 20: If time domain bundling of HARQ-ACK is supported for multi-PDSCH scheduling, this feature could be configured per cell group.

Proposal 21: When time domain bundling of HARQ-ACK per DCI is configured for multi-PDSCH scheduling with Alt2, C-DAI/T-DAI could be counted per DCI as Alt1.

For the last FFS of Alt 2, for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion. We think this FFS is talking about how to determine the C-DAI value when more than two DCIs for multi-PDSCH scheduling transmit in the same PDCCH monitoring occasion on a serving cell. Similar as R16， the C-DAI is accumulated in increasing order of the PDSCH reception, for DCI for multi-PDSCH scheduling, the C-DAI could be accumulated in increasing order according to the starting time of  the first PDSCH reception of multi-PDSCH.

Proposal 22: For the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, the C-DAI could be accumulated in increasing order according to the starting time of the first PDSCH reception of multi-PDSCH.

Conclusions

We discussed the required changes of physical layer design using exiting NR waveform for both licensed and unlicensed band with the following observations and proposals.

Observation 1: Further enhancements of FDRA are not essential for either multi-slot PDSCH scheduling or multi-slot PUSCH scheduling.

Observation 2: The interleaved VRB-to-PRB mapping for 120 kHz SCS can be reused for 480 kHz and 960 kHz SCS.

Observation 3: PRB bundling mechanism defined in Rel-15 can be reused as a baseline for multi-PDSCH scheduling in this new frequency range.

Observation 4: The existing configuration and indication related to RateMatchPattern can be reused.

Observation 5: Triggering scheme defined in Rel-15/16 can be reused directly for aperiodic ZP CSI-RS.

Observation 6: Further enhancements of frequency hopping for multi-slot PUSCH scheduling are not essential.

Observation 7: For 480 kHz and 960 kHz, bundling DMRS in one slot per multi-slot outperforms the legacy DMRS pattern mapped per slot.

Observation 8: Block PTRS with the designed circular sequence has better BLER and spectrum efficiency than distributed PTRS when power boosting is applied.

Observation 9: With more PTRS groups and less PTRS samples per PTRS group denoted as (NgroupPT-RS, Nsamplegroup) = (16, 2), the BLER performance of all the SCSs are improved significantly.

Observation 10: Due to Rx timing shift, (at least part of) a PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol, may wrap-around to the head of the symbol from the receiver’s perspective, thus spoiling the original intention of the design and unnecessarily increasing Rx complexity, as well as deteriorating PN compensation performance, especially for the high MCS.

Observation 11: New PTRS location which is in the middle of each interval can solve the influence on BLER performance caused by Rx advance timing shift.

Observation 12: Huge redundancy in HARQ-ACK codebook will hurt the reliability for PUCCH transmission.

Proposal 1: The absolute time of 120 kHz SCS timelines should be adopted by default unless the reduced value for specific timeline(s) can be verified by implementation.

Proposal 2:  For single slot scheduling with 480 kHz and 960 kHz SCS, N1, N2 and N3 values providing same absolute processing time as that of 120 kHz SCS in FR2 is preferred.  The values should be further studied for the case of multi slot scheduling after the detailed schemes are clear, which includes the position of DMRS, the number of PUCCHs for HARQ feedback, PTRS pattern.

Proposal 3:  The value range of k0 and k2 should be extended to 0~128 and 0~256 for 480 kHz and 960 kHz SCS respectively.

Proposal 4:  The value range of k1 should be defined for DCI format 1_0, 1_1 and 1_2 separately:

For DCI format 1_0: define a new set with scaled values for 480 kHz and 960 kHz respectively;

- {4, 8, 12, 16, 20, 24, 28, 32} for 480 kHz

- {8, 16, 24, 32, 40, 48, 56, 64} for 960 kHz

For DCI format 1_1: define -1~63 and -1~127 as the value range of k1 for 480 kHz and 960 kHz;

For DCI format 1_2: define 0~63 and 0~127 as the value range of k1 for 480 kHz and 960 kHz.

Proposal 5:  For 480 kHz and 960 kHz SCS, Z1, Z2 and Z3 values providing same absolute processing time as that of 120 kHz SCS in FR2 is preferred.

Proposal 6:  For single TRP operation:

- A UE is not expected to be scheduled more than one PDSCH in a slot with a single DCI or with multiple DCIs for 480 kHz and 960 kHz SCS.

- A UE is not expected to be scheduled more than one PUSCH in a slot with a single DCI or with multiple DCIs for 480 kHz and 960 kHz SCS.

Proposal 7: Support not limiting the maximum value of gap between two adjacent PDSCHs/PUSCHs. The maximum value of gap between the first PDSCH/PUSCH and the last PDSCH/PUSCH scheduled by a single DCI is limited by the maximum value of k0 and k2.

Proposal 8: As for Rel-16 multi-PUSCH scheduling, determine the HARQ process ID for each PDSCH/PUSCH by incrementing the HARQ process ID by one starting from the first PDSCH/PUSCH, independently of potential resource collisions with UL/DL symbols. If the resource collides with a pre-configured resource, NACK corresponding to the collided PDSCH should be reported by the UE.

Proposal 9: Support scheduling 2nd TB for multi-slot PDSCH/PUSCH scheduling. MCS for the 2nd TB is applied commonly to all the scheduled PDSCHs/PUSCHs, even if the number of layers is less than 5. NDI and RV are indicated individually for each scheduled PDSCH/PUSCH.

Proposal 10: CBGTI is not present if multi-PDSCHs is scheduled among a TDRA table including at least one row with multiple SLIVs.

Proposal 11: For TDRA table configuration supporting non-continuous resource indication, signal a separate k0 (k2) for each SLIV. k0 (k2) can be omitted for a SLIV to indicate that the SLIV corresponds to the next slot from the previous SLIV (i.e. in case of two consecutive slots allocation).

Proposal 12: Support periodic/semi-persistent ZP CSI-RS for 480 and 960 kHz SCS with periodicity up to 80 ms.

Proposal 13: Support DMRS mapped on symbol l0 to symbol l0+S*L-1 within the first PDSCH slot for the multi-slot scheduling, when only front-loaded DMRS is configured, where L is the length and l0 the starting symbol of legacy front-loaded DMRS.

Proposal 14: Support block PTRS as one type of PTRS pattern for 120 kHz, 480 kHz and 960 kHz SCS of CP-OFDM. The use of block PTRS can be indicated by MCS, scheduled bandwidth, and power boosting scheme. The PTRS sequence is composed of ZC sequence and circular part based on ZC sequence for block PTRS.

Proposal 15: A new PTRS pattern with more PTRS groups and less PTRS samples per PTRS group denoted as (NgroupPT-RS, Nsamplegroup) = (16, 2) within one DFT-s-OFDM symbol should be supported for large scheduling bandwidth and high scheduled MCS.

Proposal 16: For PTRS with Nsamplegroup=4, the mapping of last PTRS group should consider potential Rx timing shift and avoid the last X pre-DFT symbol(s).

Proposal 17: If more than one PDSCH is allowed to be scheduled in the same slot by different DCI, for enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, for each pair of {row of SLIVs, k1}, if at least one SLIV from a pair is overlapped with one SLIV from another pair, these pairs will be grouped together, the number and position of the candidate PDSCH receptions for each group are determined according to the procedure in step 2, the HARQ-ACK information is generated in group based sequence according to the start time of the first candidate PDSCH reception of each group.

Proposal 18: If only one PDSCH is allowed to be scheduled in the same slot, for enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, for cells belonging to FR2.2, the number and position of the candidate PDSCH receptions are determined by the unique number of DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing, the HARQ-ACK information is generated according to the index of DL slots.

Proposal 19: Support Alt 2 (C-DAI/T-DAI is counted per PDSCH) for type-2 HARQ-ACK codebook with separate sub-codebooks for single PDSCH without CBG transmission, for multi-PDSCH scheduling without CBG transmission, and for fallback DCI.

Proposal 20: If time domain bundling of HARQ-ACK is supported for multi-PDSCH scheduling, this feature could be configured per cell group.

Proposal 21: When time domain bundling of HARQ-ACK per DCI is configured for multi-PDSCH scheduling with Alt2, C-DAI/T-DAI could be counted per DCI as Alt1.

Proposal 22: For the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, the C-DAI could be accumulated in increasing order according to the starting time of the first PDSCH reception of multi-PDSCH.

- References

R1-2104042, “Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ) [104b-e-NR-52-71GHz-06]”, Moderator (LG Electronics), e-meeting, April 12th –April 20th, 2021.

Chairman…

P. Mathecken, T. Riihonen, S. Werner, and R. Wichman, “Constrained phase noise estimation in OFDM
using scattered pilots without decision feedback,” IEEE Transactions on Signal Processing, vol. 65, no. 9, pp. 2348–2362, May 2017