3GPP TSG-RAN WG1 Meeting #106-e	R1-2106451

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.5.3

Source:	Huawei, HiSilicon

Title:	Enhancement for DL AoD positioning

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the DL-AoD enhancements on the following aspects

First path measurement

Adjacent beams for DL-AoD

2-stage beam sweeping

DL-AoD angle calculation enhancements

Expected Rx angle indication for UE

# First path measurement

In RAN1#105-e [1], we made the following agreement with regard to the first path power reporting for DL-AoD.

Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path

In this section, we discuss other issues for first path measurement reporting.

## Phase reporting

Other alternatives are still being considered, include the phase reporting, and direct DL-AoD reporting based on CSI-like methods.

We think that the phase reporting can be feasible in the following cases

When PRS resources from a TRP is transmitted on consecutive symbols in a single slot or on the same symbols, in which the phase continuity is feasible.

When the positioning reference unit is involved in the DL-AoD calculation, in which the phase discontinuity between different PRS resources (on different symbols), if present, can be calibrated.

- Consider the ULA array with M×1 elements with single polarization, which can be easily extended to UPA case and to the bi-polarization case, as shown in Figure 1.

Figure 1 ULA array with antenna spacing dλ

- The UE is located in the local “ZOA” direction θ, and the LOS path is expected have the following phase shift along each antenna elements.

- hn=aφnθ

- φnθ=expj2πndcosθ

- The vectorized first path gain is h=h0,h1,⋯,hN-1T.

- If the Tx beamforming coefficients for M beams can be known, e.g. w0,w1,…,wM-1, the first path gain over the M Tx beams can be measured by the UE with

- p=w0w1⋯wM-1Th+v

- One way of resolving the angle θ is similar to the way of RSRP based methods.

- θ=argmaxθΦT(θ)w0w1⋯wM-1p
Φθ=φ0θ,φ1θ,⋯,φN-1θT

- For the direct DL-AoD estimation, it first depends on whether the port selection codebook is adopted for DL-AoD angle calculation enhancements. Second we think that the resource-specific phase reporting can also be applied for this method. Introducing AoD measurement reporting by the UE for UE-assisted DL-AoD positioning is thus not necessary, while for UE-based DL-AoD, it is not needed.

- Therefore, we have the following proposal.

- Proposal 1:  Support the phase reporting for the first path in DL-AoD.

## Path measurement window

- Unlike the RSRP measurement which includes the power of all paths, path specific power measurement could be subject to Rx sampling.

- As shown in Figure 2, the actual delay is at 20.1 Ts where Ts=1122.88μsec. The CIR for the upper figure is sampled at integer Ts with the available CFR on Nsubcarrier=3300 subcarriers out of the NFFT=4096 FFT buffer, mimicking the 275 PRB transmission, while the sampling rate is increased to 0.25 Ts in the lower figure for the up-sampling.

Figure 2 CIR sampling 0.1 Ts offset between the delay and the sampling grid

Likewise, in Figure 3, the delay is at 20.5 Ts with other conditions the same as in Figure 2.

Figure 3 CIR sampling 0.5 Ts offset between the delay and the sampling grid

- Figure 2 and Figure 3 demonstrated that when the delay is not in the sampling grid, a single path will be sampled into multiple channel taps. With oversampling, the peak can be better captured, but first it is not expected that all UE should perform oversampling, and second the oversampling also creates multiple taps for a single path. How the path power should be evaluated needs further consideration.

- To illustrate it, we use the following equations. (Note that in Figure 2 and Figure 3, there is an additional amplitude scaling of NFFTNsubcarrier to normalize the peak power, which is not shown in the following deduction.)

- Using the normal IFFT definition to convert CFR to CIR, where D is the delay counting in number of samples, which can be fractional.

- hn=1NFFTk=-NFFT2NFFT2-1Hkexp2πjnkNFFT
Hk=exp-2πjDk4096k∈-1650,-1649,⋯,16490k∈-2048,⋯,-1651,1650,⋯,2047

- According to the Parseval’s theorem, one can found that

- nhn2=1NFFTkHk2=NsubcarrierNFFT

- It means that for a “unit” channel impulse response hct=δt-DTs that corresponds to a pure delay, the power of the delay path observed under the finite-bandwidth system (with Nsubcarrier subcarriers) is scattered in multiple CIR taps, and the total power across all taps is scaled by a factor that is related to the processing (I)FFT size and the number of subcarriers.

Observation 1: The total power for a path scattered in multiple CIR taps, and normalization could be required taking into account the number of subcarriers with CFR and the (over-)sampling FFT size.

On the other hand, a well-defined RSRP measurement in this case is 1, which is constant independent from the time domain sampling behaviour.

1NsubcarrierkHk2=1

We would like to see a path-specific absolute RSRP definition being

the same order as the current RSRP definition

independent from the UE Rx FFT size, and the number subcarriers with CFR

well reflecting the path power

- One method to achieve this, could be via the following equation

- Rp=n∈Wphn2n∈Hhn2×RSRP

- Where

Rp is the RSRP of the path p

hn is the CIR of the channel impulse response at sampling point n

Wp is the set of CIR taps corresponding to the path RSRP measurement evaluation window for path p

W is the set of all CIR taps

RSRP is the currently defined RSRP

- The selection of Wp can be further discussed, since a larger path RSRP measurement window would be affected by additional paths, while a smaller window will be volatile for unaligned sampling time and channel delay.

- Proposal 2:  Adopt the following definition of path RSRP.

Rp=k∈Wphk2k∈Whk2×RSRP

FFS: the path RSRP measurement evaluation window

Send an LS to RAN4

# Adjacent beams for DL-AoD

In RAN1#105-e [1], we made the following agreement regarding enhancement on the adjacent beams for DL-AoD.

Agreement: For UE-assisted DL-AOD positioning method, select one or more of the following to enhance the signaling to the UE for the purpose of PRS resource(s) measurement and reporting: - Option 1: the LMF explicitly identify adjacent beams in the assistance data (AD) - Option 2: the LMF send the beam information in the AD with an order of priority of PRS resources. - Option 3: the LMF includes boresight direction information for each PRS resource in the assistance data. - Option 4: the LMF send the beam information in the AD with indicated subset of PRS resources. - FFS: Detailed signaling and procedure - FFS: How to define adjacent beams

Using adjacent beams is claimed to allow UE to prioritize RSRP measurement and reporting for the PRS with adjacent beams, so that the angle calculation can be more efficient, compared with the existing method entirely based on RSRP values (from the highest to the lowest ones).

We do acknowledge that given limited RSRP reporting budget, reporting the RSRP (or path-specific RSRP) belonging to adjacent PRS resources could be helpful. However, the benefit is not unconditional.

First, the reporting should be on the basis of correctly detecting the PRS resource that is pointing at the UE, or equivalently the PRS resource with the highest RSRP or the first path RSRP. We may refer to it as PRS Reference Resource. In other words, in order to find the PRS Reference Resource, UE needs to do full search/measurement for all PRS resources (within its measurement capability) of a TRP.

Figure 4 PRS reference resource

Second, when UE has determined the PRS Reference Resource, UE may additionally include the RSRP measurements of the “adjacent beams” of the PRS Reference Resource, or more generally, the RSRP measurements from the PRS resources that are associated with the PRS Reference Resource. Even if the spec somehow captures the adjacent beams in the definition, but does not regulate how UE should include the content in the measurement report, the indication itself would be useless.

Observation 2: In order to use the “adjacent beam” information, UE will

Conduct full PRS measurement to first find the PRS Reference Resource, and

Then include the RSRP measurements of PRS resources that are adjacent to/associated with the PRS Reference Resource.

For Option 1 and Option 3, the lack of instruction to the UE in terms of how to generate a measurement report will make both the two options useless.

For Option 2, it depends on whether the PRS resource level priority is defined, and could be discussed later.

We think that Option 4 can work by indicating multiple pairs of candidate “PRS Reference Resource” and associated subset of PRS resources, and further defining UE behaviour that if UE has selected a PRS Reference Resource, UE is required to provide the RSRP of the PRS resources in the subset that is associated with the selected PRS Reference Resource. The PRS Reference Resource and the associated subset of PRS resources may be in the same PRS resource set or in different PRS resource sets. We will discuss different PRS resource sets in section 4.

Proposal 3: For dealing with assistance data to indicate the “adjacent beams”, select Option 4.

Option 4: the LMF sends the beam information in the AD with indicated subset of PRS resources.

Note: Option 2 can be discussed if PRS resource level priority is introduced.

- For signaling details of Option 4, we propose to consider the following signaling structure:

For a PRS resource configured by NR-DL-PRS-Resource, optionally include a field associatedSubset that is composed of a list of NR-DL-PRS-ResourceID, indicating the subset of PRS resources that is associated with the PRS resource.

For a DL-AoD measurement report, if the PRS resource indicated by NR-DL-PRS-ResourceID in the IE NR-DL-AoD-MeasElement corresponds to the PRS resource that has the field associatedSubset, UE shall include in the IE NR-DL-AoD-AdditionalMeasurementElement the RSRP measurements of the PRS resources identified by the associatedSubset.

- Proposal 4: Support indicating for a PRS resource (resource A) a subset of PRS resources (subset B) in the assistance data, and if UE reports RSRP of the resource A as the main result of DL-AoD, UE shall include the RSRP of the resources in the corresponding subset B.

# 2-stage beam sweeping

In RAN1#105-e [1], we made the following agreement regarding 2-stage beam sweeping.

Agreement: - For both UE-based and UE-assisted DL methods, at least for two-stage PRS beam sweeping, study further at least the following: - Enhancements in the association between resources belonging to two DL PRS resource sets of the same TRP - Companies are encouraged to evaluate whether other potential enhancements in this subagenda or other subagendas (e.g. additional beam information, on-demand PRS framework) could be used to enable this feature (potentially by implementation). - Note: Two-stage PRS beam sweeping corresponds to different DL PRS resource sets

In our view, the support of 2-stage beam sweeping can be via two different ways.

Alt.1 Both two DL PRS resource sets are provided to the UE

Alt.2 Reconfiguration of a single DL PRS resource set

## Two DL PRS resource set

For Alt.1, we think that a straightforward case is to support differential beamforming, as proposed by [3][4]. The coefficient of a differential beamformer is characterized by setting negative to the second half of a DFT vector, i.e.,

wdiff=1, ej2πdλsinθ,…,ej2πdλN2-1sinθ,-ej2πdλN2sinθ,…, -ej2πdλN-1sinθ=[W1, -W2]

The resulting relative radiation pattern compared against that of a normal DFT beam will create a sharp transit from infinity to 0, as shown in [4].

Figure 5 Illustration of channel gain of normal beam and differential beam [4]

The differential beam transmission scheme generates the most efficient RSRP reporting, by utilizing only two PRS resources that radiates in the most contrast manner in the spatial domain.

A way of supporting differential RSRP is to allow the first PRS resource set to be associated with the normal beam, while the second PRS resource set to be associated with the differential beam. Then from the UE measurement procedure, similar to the discussion in section 3, UE will do full beam search/measurement for the first set, and then based on the measurement results, only perform measurement on a single PRS resource for the second set that corresponds the maximum RSRP in the first PRS resource set.

The link between the normal beam (first PRS resource set) and the differential beam (second PRS resource) can be implicit, e.g. using the same PRS resource ID.

Observation 3: Differential beam is efficient in that the PRS resources transmitted in the normal DFT beam and the differential DFT beams are the most contrast in the spatial domain.

Proposal 5: Support 2-stage PRS beam sweeping for differential beam where the first PRS resource set corresponds to the normal beam and the second PRS resource set corresponds to the differential beam.

## Reconfiguration of a single resource set

Another alternative is by reconfiguration of a single set. The reconfiguration could follow the on-going on-demand PRS procedure. Consider the following signalling procedure as shown in Figure 6.

- The first PRS resource set could be wide-beam sweeping, and UE is expected to perform full search/measurement as the normal DL-AoD measurement. Then UE reports the PRS measurement to the LMF, triggering the LMF to start LMF-initiated on-demand PRS procedure.

- LMF may request the gNB/TRP to transmit additional PRS in a specific AoD direction, and when confirmed by the gNB/TRP, LMF reconfigures the assistance data by replacing the existing first PRS resource set of a TRP with a second PRS resource set to the UE (Note that the first PRS resource set is released from UE perspective).

- The second PRS resource set could be narrow-beam sweeping, and UE is expected to perform normal measurements on the “customized” PRS resources and report it back to the LMF.

- In case the UE is in motion, LMF may further trigger the on-demand PRS procedure to adjust the AOD direction of the narrow beams, and update the “second” PRS resource set.

Figure 6 Procedure of reconfiguring the single PRS resource set based on on-demand PRS

In this sense, there is no specification impact to support the reconfiguration of the single PRS resource set, besides the necessary steps for on-demand PRS.

Observation 4: There is no spec impact to support the reconfiguration of the single PRS resource set besides the necessary steps for on-demand PRS.

Proposal 6: No need to discuss reconfiguration of a single resource set to enable 2-stage beam sweeping.

# DL-AoD angle calculation enhancements

In RAN1#105-e [1], we made the following agreement on DL-AoD angle calculation enhancement.

Agreement: For the beam/antenna information to be optionally provided to the LMF by the gnodeB, select one or more of the following: - Option 1: the gNB reports the antenna configuration including at least the following parameter: - the number of antenna elements (vertical and horizontal) - antenna spacing dh and dv - FFS: For DFT-based beams, precoder information for each PRS resource - Check whether the already reported boresight directions are sufficient, or whether more information is needed - FFS: Antenna Element pattern Information - FFS: Details - FFS: If additional information about panel/orientation is needed - Option 2: the gNB reports a mapping of angle and beam gains for each of the PRS resources. - FFS: representation of the mapping (e.g. parametric function approximating the beam response, or gain/angle table, beamwidth, intersection point of multiple beams (angle, RSRP)intersection point) - Other options are not precluded - In either option, the gNB beam/antenna information can optionally be provided to the UE by the LMF for UE-based DL-AoD

For Option 1 with DFT beams, there was discussion whether the existing boresight direction can be used and whether the antenna element pattern information is needed.

## Use of existing boresight direction

With the AoD/ZoD for each PRS resource specified in Rel-16, the LMF can model the beam shape of PRS resource k assuming using the DFT coefficients.

Pkθ,φ=sinπd2N2cosθ-cosθ0ksinπd2cosθ-cosθ0k2sinπd1N1sinθsinφ-sinθ0ksinφ0ksinπd1sinθsinφ-sinθ0ksinφ0k2

Where

- N1 and N2 are the number of elements on the horizontal and vertical axis, respectively.

- θ0k and φ0k are the ZoD and AoD for the PRS resource k, represented in the local coordinate system of the gNB antenna panel.

- d1 and d2 are the antenna element spacing on the horizontal and vertical axis, respectively.

The derived beam shape will be used for comparing against the RSRP measurements (per path) measured by the UE for DL-AoD angle calculation,e.g. based on the following optimization equation

θ,φ=argminθ,φRTPRP

Where

- R=Rk0,Rk1,…,RkK-1

- P=Pk0θ,φ,Pk1θ, φ,…,PkK-1θ, φ

- Rk0 is the reported RSRP for PRS resource k0

- K is the number of PRS resources in the reporting

## Antenna element radiation pattern

- Regarding whether antenna element radiation pattern should be included, we would first clarify that it should not be interpreted that the antenna element radiation pattern may be varying across different elements, and then the understanding would be that whether a single antenna element radiation pattern should be used for all antenna elements (at least per a polarization per panel) for a single TRP.

- Observation 5: The provision of antenna element radiation pattern is intended to be applicable for all antenna elements [FFS per polarization per panel] for a TRP.

- Let’s assume a single polarization with a single panel, the antenna radiation pattern could somehow affect the peak direction. Figure 7 shows the radiation pattern of three beams on the horizontal plane, where the straight line corresponds to the intended direction calculated based on DFT coefficients, i.e. -30 degree, 0 degree, and 30 degrees respectively. The actually beamformed radiation pattern is further weighted by the antenna gain, causing the radiation peak direction shifting towards the centre.

Figure 7 Beamformed radiation pattern with a typical gNB antenna model

On the other hand, when the DL-AoD is calculated based on the RSRP relative values, we are actually care about the relative values on a specific radiation angle between different beams. The antenna pattern gain will be cancelled.

For example, the beamforming considering the antenna radiation pattern can be written as following, with Gθ, φ denoting the antenna element radiation pattern along θ, φ.

Pkθ,φ=G(θ,φ)sinπd2N2cosθ-cosθ0ksinπd2cosθ-cosθ0k2sinπd1N1sinθsinφ-sinθ0ksinφ0ksinπd1sinθsinφ-sinθ0ksinφ0k2

The relative RSRP difference between beam k and beam l would be not dependent on G(θ,φ) any more, as shown in the following.

Pkθ,φPlθ,φ=sinπd2N2cosθ-cosθ0ksinπd2cosθ-cosθ0k2sinπd1N1sinθsinφ-sinθ0ksinφ0ksinπd1sinθsinφ-sinθ0ksinφ0k2sinπd2N2cosθ-cosθ0lsinπd2cosθ-cosθ0l2sinπd1N1sinθsinφ-sinθ0lsinφ0lsinπd1sinθsinφ-sinθ0lsinφ0l2

It is worth noting that the since the antenna element radiation pattern may affect the peak direction as shown in Figure 7, the boresight ZoA and AoA provision should be based on the DFT precoder instead of the radiation peak.

Observation 6: Antenna element radiation pattern does not affect the AoD calculation based on relative RSRP values.

Observation 7: To enable angle calculation enhancements for DFT beams, the boresight ZoD/AoD information should be based on DFT beam without considering the spatial shaping of the antenna element radiation pattern.

- Proposal 7:  Support to reuse the existing boresight direction to assist LMF/UE to emulate the beam response for Option 1 DFT-based angle calculation enhancement.

The boresight ZoD/AoD information should be based on DFT precoder without considering the spatial shaping of the antenna element radiation pattern.

# Expected Rx angle indication for UE

In RAN1#104bis-e [2], we reached the agreement regarding DL-AoD angle search window.

Agreement: - For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options: - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP] - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP] - Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced. - FFS: details of signaling - FFS: Applicability of this agreement to other Positioning methods

Three Options were listed, with expected DL-AoD, expected DL-AoA, and no support of any enhancement. In our view, as DL-AoD positioning requires the same Rx beam reception so that the RSRP (or path RSRP) difference corresponds to the deviation between Tx beam direction and the actual AoD direction.

To support this, Rel-16 adopted a spec transparent way of configuring SSB-PRS-PRS QCL-TypeD, and the enhancement on the reporting of the Rx beam index by the UE. However, which Rx beam should be used is not specified or optimized. Normally it would require UE to perform Rx beam sweeping or train the Rx beam for the ultimate source SSB (based on existing RRM).

For IIoT case in Rel-17, the UE may not be inside a mobile phone, but could be on an AGV, a robotic arm, etc, which can have its orientation/bearing information. The incoming angle information defined in GCS can be transformed to the LCS by the device to properly form the Rx beam.

Figure 8 Rx beam selection based on DL-AoD angle search window

This can effectively reduce the PRS measurement period in case the number of the BS stations is small, e.g. 3 or 4, while in Rel-16, RAN4 assumed that UE needs to perform up to 8 Rx beam sweeping. In addition, the angle information can also help UE identify which panel to receive the corresponding PRS.

Observation 8: To assist UE to perform the Rx beamforming and select the Rx panel, providing DL-AoD angle search window is beneficial.

Regarding how LMF set the window, it can be either based on UE location derived from CID/E-CID, or based on best effort estimate of the UE location with other positioning methods, as shown in Figure 9.

Figure 9 Illustration of Expected DL-AoA/DL-AoD based on the coarse UE location

The use of DL-AoA/ZoA and DL-AoD/ZoD can be equivalent, given that we have

AoA=AoD+π

ZoA=π-ZoD

However, the angle information is for the purpose of UE selecting the Rx beam, and using the angle from UE perspective is rather straightforward, and thus we prefer to adopt Option 2.

The expected DL-AoA/ZoA can also be beneficial to reduce to UE Rx beam sweeping and strengthen the LOS path for DL-TDOA and Multi-RTT methods, since from LMF perspective, DL-AoA/ZoA maps to the LOS direction, and UE Rx beamforming will be used to strengthen the LOS path power for a better timing based methods.

| (a) | (b) |
| --- | --- |

Figure 10 CIR observed in 100MHz bandwidth compared against the true CIR: (a) without Rx beamforming, (b) with Rx beamforming

Figure 10 shows a comparison between the CIR without Rx beamforming and with Rx beamforming (having the correct expected angle of arrival), where the following assumptions in Table 1 are made. It can be observed that due to every close NLOS paths to the LOS path, the peak of the 100MHz CIR without beamforming is biased by approximately 1ns prior to the desired TOA, while that with beamforming is quite aligned. The 1ns TOA error will convert to 0.3 metres accuracy loss, making reaching the 0.2/0.5m@90% target extremely difficult.

Table 1 Evaluation assumptions for Rx beamforming

| Parameters | Values |
| --- | --- |
| Channel model | CDL-D 5ns |
| Number of Rx | Non-beamformed: 1 Beamformed: 4x1 |
| Nominal delay | 150ns |

Proposal 8: Support indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

- Note: This is also applicable to DL-TDOA and Multi-RTT methods.

# Conclusion

In this contribution, we have the following observations and proposals regarding DL-AoD enhancement in Rel-17.

Observation 1: The total power for a path scattered in multiple CIR taps, and normalization could be required taking into account the number of subcarriers with CFR and the (over-)sampling FFT size.

Observation 2: In order to use the “adjacent beam” information, UE will

Conduct full PRS measurement to first find the PRS Reference Resource, and

Then include the RSRP measurements of PRS resources that are adjacent to/associated with the PRS Reference Resource.

Observation 3: Differential beam is efficient in that the PRS resources transmitted in the normal DFT beam and the differential DFT beams are the most contrast in the spatial domain.

Observation 4: There is no spec impact to support the reconfiguration of the single PRS resource set besides the necessary steps for on-demand PRS.

- Observation 5: The provision of antenna element radiation pattern is intended to be applicable for all antenna elements [FFS per polarization per panel] for a TRP.

Observation 6: Antenna element radiation pattern does not affect the AoD calculation based on relative RSRP values.

Observation 7: To enable angle calculation enhancements for DFT beams, the boresight ZoD/AoD information should be based on DFT beam without considering the spatial shaping of the antenna element radiation pattern.

Observation 8: To assist UE to perform the Rx beamforming and select the Rx panel, providing DL-AoD angle search window is beneficial.

- Proposal 1:  Support the phase reporting for the first path in DL-AoD.

- Proposal 2:  Adopt the following definition of path RSRP.

Rp=k∈Wphk2k∈Whk2×RSRP

FFS: the path RSRP measurement evaluation window

Send an LS to RAN4

Proposal 3: For dealing with assistance data to indicate the “adjacent beams”, select Option 4.

Option 4: the LMF sends the beam information in the AD with indicated subset of PRS resources.

Note: Option 2 can be discussed if PRS resource level priority is introduced.

- Proposal 4: Support indicating for a PRS resource (resource A) a subset of PRS resources (subset B) in the assistance data, and if UE reports RSRP of the resource A as the main result of DL-AoD, UE shall include the RSRP of the resources in the corresponding subset B.

Proposal 5: Support 2-stage PRS beam sweeping for differential beam where the first PRS resource set corresponds to the normal beam and the second PRS resource set corresponds to the differential beam.

Proposal 6: No need to discuss reconfiguration of a single resource set to enable 2-stage beam sweeping.

- Proposal 7:  Support to reuse the existing boresight direction to assist LMF/UE to emulate the beam response for Option 1 DFT-based angle calculation enhancement.

The boresight ZoD/AoD information should be based on DFT precoder without considering the spatial shaping of the antenna element radiation pattern.

Proposal 8: Support indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

- Note: This is also applicable to DL-TDOA and Multi-RTT methods.

# Reference

3GPP RAN1, Chairman’s Notes RAN1#105-e final, RAN1#105-e, e-Meeting, 17th May  – 29th May, 2021.

3GPP RAN1, Chairman’s Notes RAN1#104b-e final, RAN1#104bis-e, e-Meeting, 12th April  – 20th April, 2021.

3GPP, R1-2101212, Accuracy improvements for DL-AoD positioning solutions, Samsung, RAN1#104-e, e-Meeting, 25th January  – 5th February, 2021.

3GPP, R1-2101213, Discussion on differential beamforming for positioning enhancement, Samsung, RAN1#104-e, e-Meeting, 25th January  – 5th February, 2021.