3GPP TSG RAN WG1 Meeting #106-e	R1-2106456

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.14.1

Source:	Huawei, HiSilicon

Title:	Traffic model for XR and Cloud Gaming

Document for:	Discussion and Decision

Introduction

eXtended Reality (XR) and Cloud Gaming (CG) are important media applications enabled by 5G. For Rel-17 new study item on XR evaluations for NR, agreements on XR traffic model have been made in previous RAN1 meetings [1]-[3].

In this contribution, we provide our views on the remaining issues of XR traffic models based on SA4 outcomes, especially for multi-stream model for DL video and AR UL.

I-stream and P-stream for DL video

In RAN1#105-e, the detailed traffic model for two streams of I-frame and P-frame for DL video was agreed [3]. The average size ratio between one I-frame/slice and one P-frame/slice α and the PER/PDB for each of the I-stream and P-stream are remained as FFS. In this section, we provide our views on these issues.

Agreement: For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed. - FFS: Parameter values of α, A, B, C, D, E, F, G, H - Including the possibility of using multiple set of parameter values - For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2. It is encouraged to evaluate additional baseline/optional scenarios/configurations. Two data streams, i.e. M1 = 2; Option 1A: slice-based; Option 1B: GOP-based. I-stream; P-stream; I-stream; P-stream. Packet modelling; Slice-level; Frame-level. Traffic pattern; Both streams are periodic at 60 fps with the same jitter model as for single stream.; Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream.. Number of packets per stream at a time; 1; N-1; I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet. N = 8: the number of slices per frame.. Average data rate per stream; RI=R*αK-1+αRI=R*αK-1+α; RP=R*K-1K-1+αRP=R*K-1K-1+α. - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3. Packet size distribution; Truncated Gaussian distribution. Mean = RIFPSRIFPS; Mean = RPFPS*(N-1)RPFPS*(N-1); Mean = RI*KFPSRI*KFPS; Mean = RP*KFPS*(K-1)RP*KFPS*(K-1). - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video. PER, PDB; [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms]; [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms].

Average size ratio

Generally, the average size ratio between I-frame/slice and P-frame/slice α may depend on rate control settings, e.g., CBR, VBR, etc. Different configurations may result in different values of α. According to our analysis in Table 1, which is based on SA4 outcome in S4aV200634 [4] and the traces for VR2 [5], α = 1.07 for VR2-5 and α is about 1.86~1.91 for other configurations. More details about each configuration can be found in our previous paper [13]. Therefore, α = 2 can be approximately taken as a good starting point for the average size ratio between I-frame/slice and P-frame/slice. Other values can also be optionally evaluated.

Table 1. Average packet size for I-frame/slice and P-frame/slice

|  | VR2-1 | VR2-2 | VR2-3 | VR2-4 | VR2-5 | VR2-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Mean size of one I-frame/slice (Bytes) | 11982 | 11983 | 12651 | 12651 | 55313 | 11987 |
| Mean size of one P-frame/slice (Bytes) | 6438 | 6437 | 6637 | 6664 | 51656 | 6438 |
| Average size ratio between one I-frame/slice and one P-frame/slice | 1.86 | 1.86 | 1.91 | 1.90 | 1.07 | 1.86 |

Proposal 1: For I/P-frame model for DL video, α = 2 is baseline. Other values can also be optionally evaluated.

Per UE KPI

In RAN1#104b-e, the following agreements of baseline per UE KPI on capacity for DL video stream were achieved, where the packet success rate (PSR) X% and the air interface PDB are jointly used to determine whether a UE is satisfied or not. Some combinations of (X, PDB) have been agreed for capacity evaluation, e.g., (99, 10), (99, 7) and (95, 13) for VR/AR and (99, 15), (99, 12) and (95, 18) for CG. However, the values of (X, PDB) for I-stream and P-stream are remained as FFS. In the following, we discuss the values of (X, PDB) for I-stream and P-stream.

Agreement: In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB. - The baseline X value is 99. - Other values of X can be optionally evaluated, e.g., X < = 95, X=99.9. - Additional combinations of (X, PDB) values can be optionally evaluated, e.g., - (99, 7), (95, 13) for VR/AR - (99, 12), (95, 18) for CG - FFS: Different values for I-frame and P-frame if evaluation of them is agreed.

Generally, for Option 1B: GOP-based model, a P-frame is inter-coded or inter-decoded with reference to other frames, e.g. I-frame. While an I-frame is encoded or decoded without any reference to other frames. In this sense, I-frame is more important than P-frame since it is the reference for other frames. Therefore, I-stream should have higher reliability, i.e. higher packet success rate (PSR) X%, than P-stream. On the other hand, due to same reason, the encoding and decoding time of I-stream may be shorter than that of P-stream. Thus, the PDB requirement for I-stream may be relaxed compared with P-stream given the end-to-end delay constraint.

According to the above analysis, it is meaningful for RAN1 to evaluate different PSR and PDB values since those evaluation results can help RAN1 identify the impact of different network transmission quality on capacity performance. RAN1 does not need to spend too much time discussing which PSR and PDB value is more reasonable. For PSR, the results for different X values can be obtained after one simulation, i.e. there is no additional simulation workload to evaluate different PSR values. For PDB, both same and different PDB values for I-stream and P-stream can be evaluated considering the difference of the encoding and decoding time for I-stream and P-stream.

In summary, we propose that the following multiple combinations of (X, PDB) for VR/AR and CG in Table 2 should be evaluated for DL multiple streams, and other combinations of (X, PDB) can be optionally evaluated. The only difference between CG and VR/AR is that the PDB requirement of CG is relaxed with 5ms, i.e., similar as single stream. Note that in Table 3, we also give equivalent (PER, PDB) values, where PER=1-X, to align with the notations in the FFS of RAN1#105-e agreement.

As shown in Table 2, to reflect the different importance of I-stream and P-stream, (X, PDB) combinations corresponding to Index 1 and 3 are proposed to be evaluated, where the PDB of I-stream and P-stream is assumed the same for simplicity. For these combinations, different PSR values for I-stream and P-stream are considered. For Index 1, the PSR of I-stream is higher than that of single-stream, i.e. 99.5%, and the PSR of P-stream is relaxed to 95%. For Index 3, the PSR of I-stream is the same as that of single-stream, and the PSR of P-stream is relaxed to 95%.

In addition, to reflect the difference of the encoding and decoding time for I-stream and P-stream, (X, PDB) combinations corresponding to Index 2 and 4 are proposed to be evaluated, where the PSR of I-stream and P-stream is assumed the same for simplicity. For Index 2, different PDB values for I-stream and P-stream are considered, in which the PDB of I-stream is 5ms longer than that of single-stream, and the PDB of P-stream is 1ms shorter than that of single-stream. For Index 4, the PDB of I-stream is 5ms longer than that of single-stream, and the PDB of P-stream is the same as that of single-stream.

Finally, both the frame importance difference and encoding/decoding time difference are considered in Index 5, where the PDB of I-stream is 5ms longer than that of single-stream and the PSR of P-stream is relaxed to 95%.

Similar to I-frame and P-frame in Option 1B: GOP-based model, a P-slice in one frame is inter-coded or inter-decoded with reference to its corresponding slice in previous frames. While an I-slice is encoded or decoded without any reference to other frames or slices. Therefore, the PSR and PDB for I-slice/P-slice can be the same as I-frame/P-frame for simplicity. Thus, in this contribution, we propose that the value of (X, PDB) for I-slice/P-slice in Option 1A: slice-based model can be the same as I-frame/P-frame in Option 1B: GOP-based model for evaluation.

Proposal 2: For I-stream and P-stream for DL video, RAN1 agrees on the following multiple combinations of (X, PDB) for evaluation. Other combinations can be optionally evaluated.

Table 2. (X, PDB) values for I-stream and P-stream for DL video

| Index | (Packet success rate X%, PDB (ms)) of {I-stream, P-stream} in Option 1A and Option 1B |  |
| --- | --- | --- |
|  | VR/AR | CG |
| 1 | {(99.5, 10), (95, 10)} | {(99.5, 15), (95, 15)} |
| 2 | {(99, 15), (99, 9)} | {(99, 20), (99, 14)} |
| 3 | {(99, 10), (95, 10)} | {(99, 15), (95, 15)} |
| 4 | {(99, 15), (99, 10)} | {(99, 20), (99, 15)} |
| 5 | {(99, 15), (95, 10)} | {(99, 20), (95, 15)} |

The equivalent (PER, PDB) values for I-stream and P-stream, i.e. {(A, C), (B, D)} and {(E, G), (F, H)} are as follows

Table 3. (PER, PDB) values for I-stream and P-stream for DL video

| Index | {(A, C), (B, D)} and {(E, G), (F, H)} of {I-stream, P-stream} in Option 1A and Option 1B |  |
| --- | --- | --- |
|  | VR/AR | CG |
| 1 | {(0.5, 10), (5, 10)} | {(0.5, 15), (5, 15)} |
| 2 | {(1, 15), (1, 9)} | {(1, 20), (1, 14)} |
| 3 | {(1, 10), (5, 10)} | {(1, 15), (5, 15)} |
| 4 | {(1, 15), (1, 10)} | {(1, 20), (1, 15)} |
| 5 | {(1, 15), (5, 10)} | {(1, 20), (5, 15)} |

In addition, based on the per UE KPI agreed in RAN1#104b-e for UL multiple stream, a UE is declared as satisfied only when each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB. This principle can be directly applied to DL multiple streams as well, i.e., for evaluation of DL multiple streams/flows, a UE is declared a satisfied UE if each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB.

Agreement: - In case multiple steams are evaluated for UL AR, a UE is declared as satisfied only when each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB. - X value for pose/control: follow X values for pose/control for CG/VR - X value for other stream: follow X values for DL video stream.

Proposal 3: For evaluation of DL multiple streams, a UE is declared a satisfied UE if each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB.

I-stream and P-stream for UL video

In RAN1#104b-e [2], the following agreements were achieved for AR UL.

Agreement: - Option 1 (Baseline for power and capacity evaluations): Two streams as defined below - Stream 1: pose/control - Traffic model and QoS parameters are same as for pose/control for UL CG/VR. - Stream 2: A stream aggregating streams of scene, video, data, and audio. - Packet size: Truncated Gaussian distribution with the parameter values same as for DL - Periodicity: 60 fps - Jitter (optional): same model as for DL - Data rate: 10 Mbps (baseline), 20 Mbps (optional) - PDB: [60] ms (baseline), [10/15] ms (optional) - Option 2 (Optional for power evaluation and baseline for capacity evaluation): Single stream as defined below - Packet size: Truncated Gaussian distribution with the parameter values same as for DL - Periodicity: 60 fps - Jitter (optional): same model as for DL - Data rate: 10 Mbps (baseline), 20 Mbps (optional) - PDB: [60] ms (baseline), [10/15] ms (optional) - Option 3 (Optional): Three streams as defined below - Stream 1: pose/control - Traffic model and QoS parameters are same as for pose/control for UL CG/VR. - Stream 2: A stream aggregating streams of scene and video - Packet size: Truncated Gaussian distribution with the parameter values same as for DL - Periodicity: 60 fps - Jitter (optional): same model as for DL - Data rate: 10 Mbps (baseline), 20 Mbps (optional) - PDB: [60] ms (baseline), [10/15] ms (optional) - Stream 3: A stream aggregating streams of audio and data - Periodicity: 10ms - Data rate: 0.756 Mbps/s or 1.12 Mbps - Packet size: determined by periodicity and data rate - PDB: 30 ms - Option 4 (Optional): Three streams as defined below - Stream 1: pose/control - Traffic model and QoS parameters are same as for pose/control for UL CG/VR. - Stream 2: I-stream for video - Stream 3: P-stream for video - Note: For stream 2 and stream 3, the I/P-stream model for DL video can be reused for UL video. Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria to be satisfied UE. - Companies should strive to align the parameter values for the options chosen as much as possible - Note: Above PDB values in [ ] for Stream 2 in Option 1 and 3, and Option 2 are to be further discussed and potentially confirmed in RAN1#105-e, where other values can be also discussed if needed. - In case multiple streams are evaluated for UL AR, a UE is declared as satisfied only when each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB. - X value for pose/control: follow X values for pose/control for CG/VR - X value for other stream: follow X values for DL video stream.

The PDB value of the stream in UL AR aggregating streams of scene, video, data, and audio, i.e., Option 2, Stream 2 in Option 1, and Stream 2 in Option 3 was discussed and agreed in RAN1#105-e as follows [3]. In the following, we discuss the X value and PDB for I-stream and P-stream of AR UL video, i.e., Stream 2 and Stream 3 in Option 4.

Agreement: PDB value of the stream in UL AR aggregating streams of scene, video, data, and audio, i.e., Option 2, Stream 2 in Option 1, and Stream 2 in Option 3. - 30ms (baseline), 10/15/60ms (optional)

According to the above agreements, in case of multiple streams are evaluated for UL AR, the X values for other stream follow X values for DL video stream. Similar with the discussion for DL video in Section 2.2, multiple values of PDB for AR UL video can be considered for the evaluation. However, too many combinations will result in large simulation workload and companies’ results not comparable. Therefore, similar to DL video, we propose that the following multiple combinations of (X, PDB) for AR UL in Table 4 should be evaluated and other combinations of (X, PDB) can be optionally evaluated.

As shown in Table 4, to reflect the different importance of I-stream and P-stream, (X, PDB) combinations corresponding to Index 1 and 2 are proposed to be evaluated, where the PDB of I-stream and P-stream is assumed as 15ms. In these combinations, different PSR values for I-stream and P-stream are considered. For Index 1, the PSR of I-stream is higher than that of single-stream, i.e. 99.5%, and the PSR of P-stream is relaxed to 95%. For Index 2, the PSR of I-stream is the same as that of single-stream, and the PSR of P-stream is relaxed to 95%.

Similar to Index 1 and 2, Index 4 and 5 are also evaluated by assuming the PDB of I-stream and P-stream as 30ms. In addition, to reflect the difference of the encoding and decoding time for I-stream and P-stream, (X, PDB) combinations corresponding to Index 3 can be evaluated, where the PSR of I-stream and P-stream is assumed the same. The PDB of I-stream and P-stream are assumed as 30ms and 15ms, respectively.

Proposal 4: For I-stream and P-stream of AR UL video, i.e., Stream 2 and 3 in Option 4 for AR UL, RAN1 agrees on the following multiple combinations of (X, PDB) for evaluation. Other combinations can be optionally evaluated.

Table 4. (X, PDB) values for I-stream and P-stream for AR UL video

| Index | (Packet success rate X%, PDB (ms)) of {I-stream, P-stream} |
| --- | --- |
| 1 | {(99.5, 15), (95, 15)} |
| 2 | {(99, 15), (95, 15)} |
| 3 | {(99, 30), (99, 15)} |
| 4 | {(99.5, 30), (95, 30)} |
| 5 | {(99, 30), (95, 30)} |

The equivalent (PER, PDB) values for I-stream and P-stream, i.e. {(A, C), (B, D)} and {(E, G), (F, H)} are as follows

Table 5. (PER, PDB) values for I-stream and P-stream for AR UL video

| Index | (PER, PDB (ms)) of {I-stream, P-stream} {(A, C), (B, D)} or {(E, G), (F, H)} |
| --- | --- |
| 1 | {(0.5, 15), (5, 15)} |
| 2 | {(1, 15), (5, 15)} |
| 3 | {(1, 30), (1, 15)} |
| 4 | {(0.5, 30), (5, 30)} |
| 5 | {(1, 30), (5, 30)} |

Conclusions

In this contribution, traffic models for XR and CG are discussed with the following proposals:

Proposal 1: For I/P-frame model for DL video, α = 2 is baseline. Other values can also be optionally evaluated.

Proposal 2: For I-stream and P-stream for DL video, RAN1 agrees on the following multiple combinations of (X, PDB) for evaluation. Other combinations can be optionally evaluated.

Table 2. (X, PDB) values for I-stream and P-stream for DL video

| Index | (Packet success rate X%, PDB (ms)) of {I-stream, P-stream} in Option 1A and Option 1B |  |
| --- | --- | --- |
|  | VR/AR | CG |
| 1 | {(99.5, 10), (95, 10)} | {(99.5, 15), (95, 15)} |
| 2 | {(99, 15), (99, 9)} | {(99, 20), (99, 14)} |
| 3 | {(99, 10), (95, 10)} | {(99, 15), (95, 15)} |
| 4 | {(99, 15), (99, 10)} | {(99, 20), (99, 15)} |
| 5 | {(99, 15), (95, 10)} | {(99, 20), (95, 15)} |

The equivalent (PER, PDB) values for I-stream and P-stream, i.e. {(A, C), (B, D)} and {(E, G), (F, H)} are as follows

Table 3. (PER, PDB) values for I-stream and P-stream for DL video

| Index | {(A, C), (B, D)} and {(E, G), (F, H)} of {I-stream, P-stream} in Option 1A and Option 1B |  |
| --- | --- | --- |
|  | VR/AR | CG |
| 1 | {(0.5, 10), (5, 10)} | {(0.5, 15), (5, 15)} |
| 2 | {(1, 15), (1, 9)} | {(1, 20), (1, 14)} |
| 3 | {(1, 10), (5, 10)} | {(1, 15), (5, 15)} |
| 4 | {(1, 15), (1, 10)} | {(1, 20), (1, 15)} |
| 5 | {(1, 15), (5, 10)} | {(1, 20), (5, 15)} |

Proposal 3: For evaluation of DL multiple streams, a UE is declared a satisfied UE if each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB.

Proposal 4: For I-stream and P-stream of AR UL video, i.e., Stream 2 and 3 in Option 4 for AR UL, RAN1 agrees on the following multiple combinations of (X, PDB) for evaluation. Other combinations can be optionally evaluated.

Table 4. (X, PDB) values for I-stream and P-stream for AR UL video

| Index | (Packet success rate X%, PDB (ms)) of {I-stream, P-stream} |
| --- | --- |
| 1 | {(99.5, 15), (95, 15)} |
| 2 | {(99, 15), (95, 15)} |
| 3 | {(99, 30), (99, 15)} |
| 4 | {(99.5, 30), (95, 30)} |
| 5 | {(99, 30), (95, 30)} |

The equivalent (PER, PDB) values for I-stream and P-stream, i.e. {(A, C), (B, D)} and {(E, G), (F, H)} are as follows

Table 5. (PER, PDB) values for I-stream and P-stream for AR UL video

| Index | (PER, PDB (ms)) of {I-stream, P-stream} {(A, C), (B, D)} or {(E, G), (F, H)} |
| --- | --- |
| 1 | {(0.5, 15), (5, 15)} |
| 2 | {(1, 15), (5, 15)} |
| 3 | {(1, 30), (1, 15)} |
| 4 | {(0.5, 30), (5, 30)} |
| 5 | {(1, 30), (5, 30)} |

- References

Chairman notes, RAN1#104-e, January 25-February 5, 2021.

Chairman notes, RAN1#104b-e, April 12-April 20, 2021.

Chairman notes, RAN1#105-e, May 10-May 27, 2021.

S4aV200634, “Traces and Configurations for VR2, CG and AR2”, SA4-e (AH) Video SWG post 111-e, January 12, 2021

http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Candidate/VR2

S4aV200669, “[FS_XRTraffic] Discussion on the regular IDR setting in Encoding Configuration”, SA4-e (AH) Video SWG post 112-e, Telco, 16 March 2021

R1-2102308, “Reply LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)”, 3GPP TSG SA WG4#112-e, February 01-10, 2021.

R1-2104239, “Evaluation methodology for XR and Cloud Gaming”, RAN1#105-e, May 10 - May 27, 2021, Huawei, HiSilicon.

P. Garrido Abenza, M. P. Malumbres, P. Pinol, and O. Lopez-Granado,“Source coding options to improve HEVC video streaming in vehicular networks”, Sensors, vol. 18, no. 9, pp. 1–18, Sep. 2018.

S4aV200635, “ [FS_XRTraffic] Summary of XR Traffic Models for RAN1 and Open Issues”, SA4-e (AH) Video SWG post 111-e, January 12, 2021

R1-2104023, “LS on Status Update on XR Traffic”, RAN1#104bis-e, 12 - 20 April 2021.

S4-210614, “FS_XRTraffic: Permanent document, v0.6.0”, SA4-e (AH) Video SWG post 113-e, April 6 - 14 2021.

R1-2104238, “Traffic model for XR and Cloud Gaming”, RAN1#105-e, May 10 - May 27, 2021, Huawei, HiSilicon.