3GPP TSG RAN WG1 #106-e	R1-2106469

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon

Title:	Discussion on CSI Enhancements for Rel-17

Document for:	Discussion and Decision

# Introduction

In 3GPP RAN1 #105 e-Meeting, the following agreements are made for CSI enhancement [1].

- FDD CSI enhancement

Agreement

For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32.

Agreement

At least for rank 1, candidate values of K1 for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}.

Working Assumption

At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1

Agreement

For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook

- FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2

- FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8

Agreement

At least for rank 1 and for Mv>1, Minit for the single window with size N is fixed to be 0

Agreement

At least for rank 1 and 2 and Mv > 1, for relationship between N and Mv, study and down-select one alternative from following in RAN1#106-e

- Alt 1: N= Mv always, no UE reporting of Wf

- Alt 2-1: N >= Mv, Wf  is layer-common and reported by UE for N>Mv.

- Alt 2-2: N >= Mv, Wf is layer-specific and reported by UE for N>Mv.

Agreement

Support rank 2 for Rel-17 codebook

Agreement

A polarization-specific bitmap for indication non-zero coefficients should be supported for W2.

Agreement

For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4)

  - Alt 3: it is replaced as (1/2)^(3/8)

Agreement

At least for rank 1 and 2, for the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {[1/4], 1/2, 3/4, 1}

- Note: [1/4] means that 1/4 is also a candidate value for the discussion on reduction of parameter combinations, but has a lower priority compared to other beta values

For future RAN1 meeting:

Study whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- CSI enhancement for MTRP

Agreement

For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact:

- CRI codepoint mapping order with CMRs and CMR pairs

- Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement

- Whether/how to enhance the CSI updating rule to address CPU overbooking

- Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation

- Whether/how to support wideband CSI report

For future RAN1 meeting:

For a CSI report setting with Option 1 and X=1 or 2, study prioritizing CSI associated with reported CSI hypotheses within a CSI Reporting Setting

- FFS potential impact for UCI payload generation

- FFS whether/how to update CSI priority formula, and additional specification impact due to updated formula

- FFS whether/how to update CSI omission rules for Part 2 CSI based on prioritized CSI

- FFS: whether the X+1 CSI hypotheses per CSI Reporting Setting are mapped to a single CSI report or X+1 CSI reports

- Companies are encouraged to discuss and justify purposes of prioritizing CSI associated with reported CSI hypotheses.

In this contribution, we provide our views on CSI enhancement based on FDD angle and delay reciprocity including remaining issues of Rel-17 PS CB enhancement for Rank 1, design of Rank 2 and general design principles for Rank 3/4. Besides CSI enhancement for multi-TRP is also discussed in this contribution for next level details.

# Rel-17 port selection codebook enhancement based on angle and/or delay reciprocity

Up to RAN1#105-e agreements, Rel-17 port selection codebook enhancement are discussed and can be summarized as following. Remaining issues related to codebook structure and quantization for Rank 1 are discussed in section 2.1, Design for Rank 2 are discussed in section 2.2 and section 2.3 provides our considerations on general design principles for Rank 3/4.

W=W1W2WfH , where W1∈NP×K1, Wf∈CN3×Mv and W2∈CK1×Mv, is supported as the codebook structure for R17 PS CB enhancement, in which

- W1∈NP×K1 is a port selection matrix whereas each column of W1 has only one element of “1”. In addition, P is the number of CSI-RS ports and K1 is the number of ports selected by UE.

  - As agreed in previous meetings, for Rank 1 at least, based on polarization-common free-selection with combinatorial coefficient for W1, UE can freely select the same L=K1 /2 ports out of P/2 ports for both polarizations. The maximal value of CSI-RS port number is 32 and the candidate values of K1 are {2, 4, 8, 12, 16, 24, 32}.

- Wf∈CN3×Mv is a DFT based compression matrix, in which N3=NCQISubband*R and Wfi,n=ej2πniN3, i=0,1,⋯,N3-1, n=0,⋯, Mv-1.

  - As agreed in previous meetings, Mv = 1 and Mv = 2 are supported. A single window with size N with Minit fixed to 0 is used to limit the FD bases used for Wf quantization (agreed as WA). Based on this window indication, a window with N consecutive FD bases are applied by the UE for Wf quantization and the UE only needs to search for Mv strongest FD components among N consecutive FD bases. It should be noted that the window configured to UE with Minit is fixed to 0, but UE can still shift the starting point of window by UE implementation.

  - There still have some open issues for Wf, e.g. whether to confirm WA on FD bases window, whether further restriction on the number of CSI-RS ports applied to Mv = 2, relationship between N and Mv, and candidate values of R.

- W2∈CK1×Mv is a linear combination coefficient matrix.

  - As agreed in previous meetings, a polarization-specific bitmap for indicating non-zero coefficients is supported for W2 with compression coefficient beta whereas the values of beta are {[1/4], 1/2, 3/4, 1} by removing negligible coefficients. And the quantization of W2 coefficient reuses Rel-16 quantization mechanism with FFS the reserved state for reference amplitude.

  - In addition, there have a few open issues for W2, e.g. reserved code point for reference amplitude, whether/how the bitmap for indicating non-zero coefficients can be absent and related SCI design.

## Remaining issues of Rel-17 PS CB enhancement for Rank 1

In this section, we will mainly focus on Rank 1 of Rel-17 PS codebook enhancement for remaining issues of codebook components  Wf and W2 since we do not see remaining issues left for W1.

- Remaining issues of codebook structure for Wf

For a DFT based compression matrix Wf∈CN3×Mv, there still have some remaining issues to be discussed as following:

- Whether to confirm WA related to a single window to limit FD bases used for Wf quantization

- Whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv = 2 and value of N

- Values of R

- Whether to confirm WA

There is a working assumption in RAN1 #105e, which is related to a single window with size N configured to the UE to limit FD bases used for Wf quantization.

Based on channel delay reciprocity for TDD/FDD, gNB can limit the FD bases used for Wf within a single window with size N. As shown in the upper figure in Figure 1, capital letters A, B and C represent respectively three strongest and dominant FD components (A>C>B), and their positions in the delay domain are dispersed. The numbers 0 indicate the weakest FD component (to be ignored for the sake of discussion). Although multiple taps of channel are discrete in delay domain, gNB can shift discretely distributed delays in delay domain, to align the same/single FD base window across CSI-RS ports, as shown in the lower figure in Figure 1. Therefore a single FD bases window with a size of N, configured/indicated by gNB, can be sufficient to counteract the effect of non-ideal delay reciprocity.

Figure 1 Illustration of a DFT Window

Therefore, we propose to confirm the working assumption.

Proposal 1: Confirm the working assumption that at least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix.

- Whether further dependence/restriction

To determine whether further dependence/restriction can be applied to Mv = 2, the overhead of R16 eTypeII port selection codebook is assumed as the upper bound of overhead. Figure 2 compares the performance with different number of ports and Mv under different values of β. In the simulations, we assume K1=PCSI-RS for W1, N=Mv for Wf  by varying values of β for W2. It can be observed in Figure 2 that when the feedback overhead of CSI does not exceed the upper bound of R16 eTypeII port selection codebook, the best performance of N=2,Mv=2 outperforms the best performance of N=1,Mv=1 in Table 1. The main benefits of Mv=2 can be summarized as following:

- By utilizing Mv=2, compared with Mv=1, more coefficients can be reported by UE, which is generally helpful to improve overall performance further. Though more coefficients means larger CSI feedback overhead, total CSI feedback overhead does not exceed that of R16 eTypeII PS codebook.

- By utilizingMv=2, compared to Mv=1 by assuming the same reporting overhead with higher compression ratios of β, more candidate coefficients can be selected freely by the UE as shown in Figure 2 (Marked by red cycles). Such freedom of selection provided to the UE can be transferred into a small performance gain also.

| CSI-RS Port number Performance Comparison | 12 | 16 | 24 | 32 |
| --- | --- | --- | --- | --- |
| N=Mv=2 VS N=Mv=1 | 3.58% | 3.21% | 1.88% | 1.32% |

Table 1 Performance gains of Mv versus the number of CSI-RS ports

Based on simulation results, we have the following observation:

Observation 1: Assuming that CSI feedback overhead is upper bounded by R16 eTypeII port selection codebook, Mv=2 can be beneficial for Rel-17 PS codebook.

Figure 2 Performance Comparison of Mv versus the number of CSI-RS port number

Based on above system level simulation results, we have the following proposal.

Proposal 2: At least for rank 1 of R17 port selection codebook, no further restriction or condition is applied for Mv=2. Further reduction of parameter combinations including Mv and other CB codebook parameters are feasible and can be discussed jointly.

- Values of R

In this section, we will discuss the impact of R. For R16 type II codebook, larger R could lead to better performance due to improved PMI with finer frequency quantitation. For R17 type II PS codebook, though gNB can estimate delay information based on angle-delay channel reciprocity, the value of R can also impact the determination of combination coefficients in W2 and consequently give rise to preference gain or loss. Taking  N=Mv=2 and DFT-based beamformed CSI-RS as an example, the impact is analyzed as following.

From spec view, different values of R means different length of FD bases in Wf, which can be used by gNB to indicate UEs for the appropriate FD bases in order to match with the precoding granularity of beamformed CSI-RS when UEs calculate the coefficients. For illustration, R= D* NPRBSB can be used for the precoding granularity with 1RB and R = 1 can be used for the precoding granularity with NPRBSB continuous RBs. Taking the precoding granularity with 1RB and CSI-RS density = 1(D=1) as an example. In this case, from the perspective of the gNB, the coefficient at p-th port and the k-th FD basis is expected to be calculated as  dkHhp where hp is the estimated channel at p-th port, dk=e-j2π(f-1)k/NRB f=1f=NRB is a DFT vector of length NRB and NRB is the number of RBs in the bandwidth, which means R= D* NPRBSB. On the other hand, if R = 1, dkHhp is used to calculate the coefficients at UE side and [2] gives an example to obtain dk where dk=e-j2πk(f-1)R/NPRBSB/N3 f=1f=NRB and can be thought as repeating each element of a DFT vector of length N3 by NPRBSB/R times. An example of dk and dk with R=1 and R= D* NPRBSB  assuming NPRBSB=4 is shown in Figure 3 where dkf and dkf are used to denote the f-th elements of dk and dk, respectively.

Figure 3 The Illustration of weights for calculating coefficients with R=1 versus R=4

If the value of R is mismatched with the precoding granularity of the beamformed CSI-RS, there will have a difference between the expected coefficient and the obtained coefficient at UE when k>0, which may lead to performance loss. And the difference can be expressed as dkHhp-dkHhp=f=1NRB1-dkfdk*f dk*fhpf, which means that the difference is small as soon as the phase of dkfdk*f is close to 0 since dkfdk*f has complex exponential form. In particular, the phase of dkfdk*f can be expressed as -2πk(i-1)/NRB where 1≤i≤NPRBSB/R. Therefore, it implies that the difference can be larger with a smaller NRB if fixing the value of k and i.

To investigate the impact of the mismatch between R and the precoding granularity of the beamformed CSI-RS, Figure 4 compares the performance with different R under the same precoding granularity of beamformed CSI-RS. We assume that the precoding granularity of the beamformed CSI-RS is 1RB at the gNB. However the UE has assumed that R=1, R=8 (for the case of 20MHz), and R=4 (for the case of 5MHz) when deriving PMI. Besides, K1=PCSI-RS=16, Mv=N=2 and β=1. Both 5 MHz and 20 MHz are considered. Because a CC with 5MHz BW has a smaller NRB, about 2.7% performance loss can be observed due to the mismatch.  On the other only 0.4% loss can be observed for a CC with 20MHz BW.

Figure 4 Comparison of the performance with different R

Based on above simulation and analysis, the following proposal should be supported.

Proposal 3: R= D* NPRBSB should be supported for Rel-17 PS codebook.

- Remaining issues of codebook structure for  W2

For linear combination coefficients matrix W2∈CK1×Mv, main remaining issues in our understanding may include:

- Whether/how to update reserved code point for reference amplitude

- Whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- SCI Design

- Reserved code point for reference amplitude

For the reserved state for reference amplitude, down-select one Alt

- Alt 1: it is kept to be reserved

- Alt 2: it is replaced as (1/2) ^ (15/4)

- Alt 3: it is replaced as (1/2) ^ (3/8)

Alt 2 replaces the reserved state as a small value following 1.5dB step size, but in general, the probability of a large difference in the energy of two polarizations, such as the stronger polarization quantizes to 1 while the weaker one to  (1/2) ^ (15/4), is very low. So the value (1/2) ^ (15/4) has a low probability of being selected. Alt 3 with (1/2) ^ (3/8) =0.7711 is a middle value between (1/2)^(1/4)=0.8409 and (1/4)^(1/4)=0.7071. However whether Alt3 can achieve performance gain is related to specific channel characteristics.

In order to investigate the impact of different Alts, Figure 5 compares the performance with different Alts for the reserved code point. In the simulations, we assume PCSI-RS=32 by varying values of K1 for W1, N=Mv=1 for Wf, β=1 for W2. Based on simulation results, it can be found that all 3 alts have nearly the same performance. So we prefer Alt 1.

Observation 2: Assuming the same CSI feedback overhead, three alts for reserved code point for reference amplitude have similar performance.

Based on above simulation and analysis, the following proposal should be supported.

Proposal 4: Considering similarity of performance gain, it is preferred to keep the reserved state for reference amplitude.

Figure 5 Performance with different schemes of the reserved state for reference amplitude

- Whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

Before discussing this issue, we wish to distinguish two concepts of the maximum number of non-zero coefficients (NZC) and the real number of NZC. The maximum number of NZC can be determined according to RRC parameters e.g. K1, Mv, β etc. Taking rank 1 for instance, maximum number of NZC is equal to K1Mv β, whereas the real number of NZC is determined and reported by UE, and cannot be greater than the maximum number of NZC. In Rel-16, since β is always smaller than 1 and a bitmap for indicating NZC is always needed. Whilst in Rel-17, since gNB can implement CSI compression implicitly based on FDD angle/delay reciprocity, β can be set to 1 by allowing UE to feedback all NZC so that the bitmap can be absent. The absence of bitmap has the advantage of reducing the coding rate of UCI to improve the probability of correct UCI decoding by gNB.

Observation 3: Since β = 1 allows UE to feedback all NZC with the maximum number of NZC, if the UE considers it to be necessary, corresponding bitmap can be absent for Rel-17 port selection codebook.

There are three methods in general to design corresponding applicable conditions of being absent:

Method 1: applicable conditions of bitmap being absent is that Mv=1 and β =1 for rank 1 only

Since for higher ranks, gNB mandating UE to report all NZC may cause unnecessary feedback overhead so that it may be more efficient to leave UE implementation when reported rank is greater than 1. Specifically, when Mv=1 and Beta =1 for rank 1, UE is mandated to report K1 NZC to gNB so that the bitmap is absent.

Method 2: applicable conditions of bitmap being absent is that total number of reported NZC across all layers is equal to the maximum number of NZC for rank 1 and 2

In Rel-16, an indication of total number of nonzero coefficients across all layers denoted as KNZ is reported in UCI part I. KNZ≤2K0, where K0=K1Mvβ is the maximal number of NZC per layer. If reusing such a reporting field in Rel-17, the bitmap is absent when KNZ is equal to the maximum number of NZC configured by RRC. From gNB perspective, after detecting KNZ in UCI part I, gNB can know whether the bitmap is reported or not in UCI part II. If the limitation of  KNZ≤2K0 remains in Rel-17 for rank 3 and 4, Method 2 may not be suitable for rank3 and 4 since corresponding bitmap must be reported always. Moreover, Method 2 shall be applied for both layers’ bitmaps being absent or not simultaneously.

Method 3: applicable conditions of bitmap being absent is that total number of reported NZC across all layers is equal to the maximum number of NZC for rank 1 and 2, and the applicable conditions of bitmap being absent for layer 1 is that the number of reported NZC for layer 1 is equal to K1Mv for ranks greater than 1. Denoted the number of NZC for layer l as KlNZ,l=1,…,υ,  the bitmap can be absent for layer l when KlNZ=K1Mv. Considering that the probability of being absent for corresponding bitmap from layer 1 to layer N gradually decreases, the bitmap of layer 1 has the greatest probability, if the rank is greater than 1. So Method 3 is to support additional case on top of Method 2, i.e. the bitmap of layer 1 alone is omitted, but other layer(s) are not. Therefore in addition to KNZ,  extra field number of NZC for layer 1 should be reported to on top of Method 2.

Method 1 may have a performance loss when UE is forced to quantize zeros or negligible coefficients of W2 to a non-zero coefficient. Comparing Methods 2 and 3, the latter can support all ranks so that dominant layer 1’s bitmap can be absent if needed by the UE. Therefore in our understanding, Method 3 is preferred with following proposal:

Proposal 5: The bitmap for indicating non-zero coefficients for W2  is absent when the reported total number of NZC is equal to the maximum number of NZC for Rank 1 and 2, if β = 1 is configured.  Additionally the bitmap for layer 1 alone can be absent, only if reported real number of NZC for layer 1 is equal to corresponding maximal number of NZC for layer 1.

- SCI design

The design for reporting the strongest coefficient indication (SCI) for Rel-17 port selection codebook in W2 includes the following alternatives:

- Alt 0 : Reporting of the position, [il*, fl*], of the strongest coefficient of layer l using ceil(log2(K0)) bits, where K0=Beta*K1*Mv

- Alt 1-1: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits

- Alt 1-2: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits, and shifting of the strongest coefficient to position fl*=0

- Alt 2: shifting the strongest coefficient to fl* = 0, and using ceil(log2(N)) bits to indicate the shift quantity for l-th layer. The strongest coefficient is indicated by il*, using ceil (log2 (K1)) for l-th layer.

- Alt 3: SCI is not needed so that the SCI in R16 codebook is replaced with a strongest polarization indicator (1 bit)

Four alternatives were agreed as candidate design for SCI reporting for Rel-17 PS CB enhancement in RAN1#105e. Based on the last email discussion, we summarized the four alternatives in terms of the overhead of SCI and group design of UCI part II for SCI/ remapping or encryption the FD component, which is shown as Table 2.

|  | Overhead(bits per layer) | Impact of UCI design | Whether to remap or encrypt the FD component |
| --- | --- | --- | --- |
| Alt 0 | log2(βK1Mν) ，up to 6 bits | Group SCI and bitmap together in G0 or G1 | Not to need to shift the strongest coefficient to position fl*=0 |
| Alt 1-1 | log2(K1Mν)， up to 6 bits | Not to need to group SCI and bitmap together in G0 or G1 | Not to need to shift the strongest coefficient to position fl*=0 |
| Alt 1-2 | log2(K1Mν)， up to 6 bits | Not to need to group SCI and bitmap together in G0 or G1 | Shift the strongest coefficient to position fl*=0 |
| Alt 2 | log2(K1N)， up to 7 bits | Not to need Group SCI and bitmap together in G0 or G1 | Shift the strongest coefficient to position fl*=0 |
| Alt 3 | 8bits | No SCI and introduce the strongest polarization indicator | Not to need to shift the strongest coefficient to position fl*=0 |

Table 2   Summary of the four alternatives for SCI reporting

In this section, we will analyze these candidate SCI design with more details:

For the overhead of four candidate SCI design, Alt 0 has lower overhead of SCI with log2(βK1Mν) bits per layer when β<1 and have the maximum overhead of 6 bits when β=1, K1=32 and Mν=2. For Alt 1-1 and Alt 1-2, the SCI per layer is represented with log2(K1Mν) or log2(K1)+ log2(Mν)bits and have the same maximum overhead of 6 bits when  K1=32 and Mν=2. For Alt 1-2, the overhead of SCI is log2(K1N) or log2(K1)+ log2(N)bits per layer. Compared with Alt 1-1 and Alt 1-2, Alt 2 has more SCI overhead if  N>Mν and the overhead is up to 7 bits when β=1, K1=32 and N=4. For Alt 3, SCI is not needed and replaced with a strongest polarization indicator with 1 bit. In addition, the strongest coefficient is also reported with 3 bits of amplitude and 4 bits of phase. Consequently, Alt 3 has the most overhead with 8 bits.

For the impact of UCI design, Alt 0 could have larger impact of UCI design, which means that the SCI and bitmap may need to be grouped together in G0 or G1 because the SCI depends on the bitmap. If the SCI and bitmap are grouped together in G0, the overhead of G0 will significantly increases due to bitmaps in G0. If the SCI and bitmap are grouped together in G1, the gNB cannot determine which port in W1 is the strongest port with only G0 reported. The common advantage of Alt 1-1, Alt 1-2 and Alt 2 is that the SCI and bitmap may not need to be grouped together in G0 or G1 because the SCI does not depend on the bitmap, which has relatively less impact of UCI design. Alt 3 seems to have some impact on UCI design due to replace SCI with the strongest polarization indicator. Other than that, discarding SCI makes it impossible for the gNB to know which port in W1 is the strongest port by only G0 alone.

With regarding to whether to remap or encrypt the FD component, Alt 1-2 and Alt 2 needs to cyclically shift the FD component in a window of length N and make the FD component of the strongest coefficient to position fl*=0, and others needn’t. From the perspective of SCI design, remapping or encrypting the FD component does not reduce SCI overhead or simplify UCI design, because the frequency component indication of the strongest coefficient in SCI always needs to be reported regardless of whether shifting the FD component of the strongest coefficient to position fl*=0.

Based on above analysis, the following proposal is suggested:

Proposal 6: For SCI reporting in R17 PS CB, support the reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits.

## Rel-17 PS Codebook Design for rank 2

- General Design principles

Each layer for a rank 2 PMI based on Rel-17 PS codebook can be quantized independently so that design principles for rank 2 can be similar to rank1 and codebook parameters associated to rank 1 can be reused largely and expended to the second layer of rank 2 PMI, such as candidate value of β={14,1/2,3/4,1}, the window size of N, candidate values of K1={2,4,6,8,12,16,24,32} for W1, and the value of Mv for Wf.

Observation 4: At least for rank 1 and 2, the window size of N,  K1  for W1，and the value of Mv for Mf are layer-common, since these codebook parameters associated to rank1 can be reused and expended to the second layer of a rank2 PMI. The design principle for rank2 is similar to rank1.

To investigate whether port selection/FD basis subset selection and non-zero coefficient selection across two layers may lead to better trade-off between performance and CSI report overhead, following questions are studied and compared:

- whether non-zero coefficient selection is layer-common or layer-specific for rank 2

- whether port and/or FD basis subset selection is layer-common or layer-specific for rank2

With regarding to select non-zero coefficients for a rank 2 PMI, the performance-overhead trade-off among two alternatives are compared as following:

- Non-zero coefficient selection of bitmap for W2 is layer-common

- Non-zero coefficient selection of bitmap for W2 is layer-specific

The simulation results are provided in Figure 6, and relevant simulation assumptions and parameters are enlisted in Table A.2-1 in Appendix A.

From Figure 6, it can be observed that for non-zero coefficient selection for rank 2, layer-specific selection can achieve average gain up to 12% in low overhead regime. As the number of reported non-zero coefficients increases, i.e. values of beta, the difference between layer-specific and layer-common selection starts to reduce until performance gain of Rel-17 PS codebook has saturated. Based on the reciprocity of the uplink and downlink channel angles and delays, the location of non-zero coefficients in W2 across different layers can be roughly the same, but independent non-zero coefficients selection/indication for different layers can still help the robustness of quantization of PMI for higher rank PMI and significantly improve the performance at least within lower payload regime.

Figure 6 Comparison of the performance with different schemes

Observation 5: For the bitmap indicating non-zero coefficient selection for rank 2, layer-specific selection performs better than layer-common up to 12%, especially at low/medium overhead regime, until performance gain of Rel-17 PS codebook saturates.

Based on above analysis and simulation results, we have the following proposal:

Proposal 7: For Rel-17 PS CB for Rank 2, support layer-specific quantization/non-zero coefficient selection of W2, assuming layer-common port selection/FD basis subset and the same maximum number of non-zero coefficients per layer.

- Design of port and FD basis subset selection for rank 2

On selecting port and FD basis subset for Rel-17 PS CB assuming, the performance-overhead trade-off of four alternatives assuming P=32 are compared as following:

- port selection is layer-common and FD basis subset selection is layer-common

- port selection is layer- common and FD basis subset selection is layer- specific

- port selection is layer- specific and FD basis subset selection is layer- common

- port selection is layer- specific and FD basis subset selection is layer- specific

The simulation results are provided in Figure 7, and the relevant simulation assumptions and parameters are enlisted in Table A.2-1 in Appendix A.

Figure 7 Comparison of the performance with different schemes

According to Figure 7, when port or FD basis subset selection is layer-common or layer-specific，layer-specific selection performs slightly better than layer-common selection, since independent basis selection will provide more degree of selection freedom. Moreover, as shown in Figure 7, compared with layer-common port selection, independent port selection has performance gain up to 1% in high overhead regime. However, considering the SD basis for Rel-15 Type II and Rel-16 Type II is common across layers, it may be more UE implementation-friendly to use layer-common port selection, which help to simplify CB design. Furthermore, as shown in Figure 7, independent FD basis has low performance gain about 1% in high overhead regime. In our understanding since the gNB has already selected the range of FD basis in advance through the window and associated frequency-domain beamforming, independent FD basis selection introduces additional overhead with extra bits which unfortunately has offset marginal benefit of selection freedom. Meanwhile, common FD basis can help to reduce the complex of UE implementation.

Observation 6: For port-selection and/or FD basis subset selection for rank 2, the performance of layer-specific selection is slightly better than layer-common selection in high overhead regime around 1%. On the other hand layer-common selection performs similarly or marginally better than layer-specific selection in low overhead regime.

Proposal 8: Support layer-common port selection and FD basis selection for rank 2.

Following we will discuss the relationship between N and Mv with the configuration with Mv>1,

- In the case of exploiting FDD angular/delay reciprocity reasonably with proper gNB implementation, gNB can shift delays in delay domain such that all dominated FD components can be well captured by a window with size of N=Mv>1, and there is no need for UE to report the index of the strongest FD component. Note that a SCI-like coefficient may be still needed in W2 for Rel17 PS codebook, as discussed in Proposal 6.

- In the case of non-ideal FDD angular/delay reciprocity due to some implementation limitations, N > Mv can be used to combat a certain imperfection. For example, from the perspective of gNB, FD components A, C shown in Figure 8 are the strongest components based on UL channel with certain implementation/estimation errors for FD component C. After applying SD/FD beamforming to ports 1 and 2, real beamformed CSI-RS ports suggest that the first and third FD components (two red boxes in Figure 8) are the strongest ones, from the perspective of CSI measurement. So N=4 (Green box in Figure 8) and Mv=2 could be useful to combat such imperfection, in a certain degree. Although some non-ideal factors, such as calibration errors have been incorporated in our simulations based on the agreements in RAN1#102, the performance gain from N>Mv is not observed in our simulations, considering that it is highly related to specific gNB implementation mechanism and characteristics of propagation channels.

Figure 8 Illustration of non-ideal reciprocity with dominant FD components A>C>B

Based on above analysis and simulation results in Figure 8, we have following proposal on relationships between N and Mv.

Proposal 9: For rank 1 and 2 PMI for Rel-17 PS CB, Wf is layer-common and reported by UE if N>Mv, i.e. Alt 2-1.

## General design principles for Rel-17 PS CB with Rank 3/4

In Rel-16, eType II codebook and eTypeII port selection codebook can support up to rank 4 so that it can be straightforward to assume that rank 3 and 4 are supported in Rel-17 PS CB. From NW implementation perspective, supporting SU-MIMO is equally important to MU-MIMO by advanced gNB SD/FD beamforming to improve CSI quantization accuracy. In order to verify the benefits of Rel-17 PS CB with Rank 3/4 for SU-MIMO, compared with R16 PS CB with Rank 3/4, system-level simulation results of rank adaptation for SU-MIMO are illustrated in Figure 9. In the simulation the following codebook design principles/parameters for Rel-17 PS CB has been assumed.

- The overall CSI feedback is upper bounded by CSI overhead of Rank 2, which is archived by limiting the maximum non-zero coefficients number with smaller values of β for rank 3 and 4. For example K1=PCSI-RS=32 for W1, N=Mv=1 for Wf, β={1/4, 1/2, 3/4, 1} for W2 with Rank 1 and Rank 2, in order to keep the CSI feedback of Rank 3/4 upper-bounded taking into account extra bitmap(s),  β34 for rank 3 and 4 PMI is assumed to be {1/8, 3/8, 5/8, 7/8}  in SLS so that

4K1Mv+7*2β34K1Mv ≈ 2K1Mv+7*(2βK1Mv)

- The bitmap of non-zero coefficient selection is layer-specific up to Rank 4.

- 12 ports CSI-RS                                    (b)   32 ports CSI-RS

Figure 9 Comparison of the performance with R17 and R16 under SU MIMO up to Rank 4

Figure 9 gives the performance of Rel-16 and Rel-17 PS CB enhancement up to Rank 4 for SU-MIMO with dynamic rank adaptation taking into account SRS error and calibration error. It can be observed that compared with R16, R17 can provide about 4% performance gain at medium CSI feedback overhead and 8% performance gain at low CSI feedback overhead, if the same reporting overhead is assumed. Alternatively, in the case of 32 ports CSI-RS, both R17 and R16 are near the saturated performance, but compared with R16 PS CB, Rel-17 PS CB enhancement can save significantly CSI overhead at the saturated performance, e.g. up to 45%.

In our understanding by utilizing angle and delay channel reciprocity, gNB can apply finer precoding granularity at both SD and FD domains to “manipulate” beamformed CSI-RS for better PMI quantization based on Rel-17 PS codebook. Also improved port selection freedom may help mitigating across-layer interference for SU-MIMO.

Observation 7: R17 PS CB for Rank 3/4 can provide much better mean UPT up to 8%, especially within low and medium CSI overhead regime. Alternately it may save significantly CSI overhead, e.g. up to 45%.

The main design consideration of Rank 3 and 4 is related to CSI feedback overhead. Similar to Rel-16 eTypeII codebook for Rank 3 and Rank 4, if simply extending rank 2 solution to Rank 3/4, the overhead increases linearly with the value of rank. Therefore we have the following proposal:

Proposal 10: Support Rel-17 PS codebook design with rank ¾ by keeping CSI feedback overhead comparable to rank 2 and also reusing rank ½ CB design as much as possible for higher rank.

# CSI Enhancement for Multi-TRP

In the following, we focus on CSI enhancement for NCJT where CSI measurement is associated to one single reporting setting.

  - CSI measurement Enhancement

In Rel-15/16, to limit and balance gNB/UE implementation complexity, some restrictions are introduced for CSI-RS resource configuration. All CSI-RS resources within one set are configured with:

- R1: same density

- R2: same nrofPorts

- R3: same starting RB and number of RBs

- R4: same cdm-type

- R5: same periodicity

Note that in Rel-15/Rel-16, the slot offset for all CMRs within one set can be same or different for different CMRs. Similarly in Rel-17, some restrictions were confirmed as following:

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level resource association configuration and the corresponding CRI codepoint design

- CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs.

Now the remaining problem is that whether any other restriction is necessary.

In practice, DL/UL switching will cause the undesired phase rotation at receiver so as to impact the estimation of the inter-TRP interference. It may lead to the mismatch between the reported CQI and the proper MCS in NCJT scheduling. Considering an example shown in Figure 10, where one CMR pair consists of 2 CMRs, i.e., CMR1 associated with TRP1 and CMR2 associated with TRP2. For a practical receiver, there is a random phase rotation on each receiving port at the beginning of the DL reception. If there is a UL transmission between CSI-RS transmission occasion on CMR1 and  CSI-RS transmission occasion on CMR2, an independent random phase rotation over each receive antenna port is introduced when UE receives the CSI-RS on CMR2, i.e., ejϕa, a=1,2,…Nr, where ϕa denote the independent random phase rotation on the received CSI-RS on CMR2. H1∈CNr×Ntand ΦH2∈CNr×Nt are the channel response matrices estimated on CMR1 and on CMR2 respectively, where Φ∈CNr×Nr is diagonal matrix, whose diagonal entries are ejϕa. W1 and W2 are the estimated PMIs based on H1 and ΦH2 respectively. The inter-TRP interference can be represented by W2HH2HΦHH1W12=W2HH2He-jϕ1⋯0⋮⋱⋮0⋯e-jϕNrH1W12. In NCJT CSI calculation, there is random phase rotation in inter-TRP interference which has an impact on PMI selection and CQI calculation. However, above random phase rotation does not exist in NCJT scheduling itself, because the transmission of two cooperative TRPs are SDM-based and the same slot is used. Consequently, the inter-TRP interference in CQI calculation may be too conservative or too aggressive comparing the inter-TRP interference in NCJT scheduling PDSCH experienced by UE. It means that the gNB cannot derive the proper MCS relying on the reported CQI. The reported NCJT CSI may be useless for NCJT scheduling.

Figure 10: The impact of random phase rotation on inter-TRP interference estimation

To avoid the above issue caused by possible DL/UL switching, we make the following proposal:

Proposal 11: Two CMRs within the same CMR pair configured for NCJT measurement hypothesis are within the same DL slot.

  - CSI Reporting Enhancement

In Rel-15/16, the UE can be configured with nRI allowed RI values for a UE for a given CSI measurement in order to relax the implementation complexity of UE.

For CSI measurement associated to a reporting setting CSI-ReportConfig for Single-TRP measurement hypothesis, it is slightly preferred that single RI restriction should be configured across TRPs. If the RI restriction is configured differently per TRP, allowed RI values for 2 CMRs may be different with different TRPs. For CSI option 2, when the reported CSI is associated with a Single-TRP measurement hypothesis, the payload of Rank Indication field is then determined by the maximal number of allowed RI values by RI restriction associated with particular TRP. It may give rise to a certain complexity of specification.

It was agreed that when the maximal transmission layers are less than or equal to 4, the indication of RI combinations of {1,1}, {1,2}, {2,1} and {2,2} is supported by a joint RI field for a NCJT measurement hypothesis in CSI part 1. For a Single-TRP measurement, when the maximal transmission layers are less than or equal to 4, candidates of RI restriction can be 1, 2, 3 or 4. It is more straightforward to use two different RI restrictions to indicate corresponding measurement hypothesis type.

Proposal 12: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two RI restrictions can be configured for a given reporting setting whereas:

- One RI restriction corresponds to M CMRs for Single-TRP measurement hypothesis

- Another RI restriction corresponds to N CMR pairs for NCJT measurement hypothesis

To avoid reporting the precoders which may induce the strong interference, the UE can be configured with CBSR to be informed that the reported PMI is not allowed to correspond to the associated precoders. In NCJT, the precoders causing the strong interference on the cooperative TRPs can be very different. It is reasonable to configure CBSR per TRP. Specifically, for CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE is configured with two CBSRs. When the UE derives the PMI over a CMR associated with the first CMR group, the reported PMI is restricted by the first CBSR associated to that CMR group, and similarly for the second CMR group in the same CMR set. Note that the payload of PMI is independent to the configuration of CBSR. For example, for a given NZP CSI-RS resource set for channel measurement with 4 NZP CSI-RS resource, i.e., {{#0 #1}, {#2, #3}} with N=1 so that {#0 #2} is for NCJT measurement hypothesis. {#0 #1} is the first CMR group associated with the first CBSR #1 and {#2, #3} is the second CMR group associated with the second CBSR2. Therefore, for NCJT CSI calculation, both CBSR #1 and #2 configured for corresponding CSI-RS resources are applied jointly when determining PMI. On the other hand, if single-TRP CSI calculation is considered, CBSR #1 and #2 configured for corresponding CSI-RS resources are applied independently.

Proposal 13: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two CBSRs can be configured for a given reporting setting whereas each of them corresponds to one CMR group in a CMR set, i.e. per TRP.

  - CSI processing criteria

In Rel-15/16, each CSI report is associated with a priority value. The priority value is used for Part 2 CSI omission as well as the CPU occupation. In Rel-17, the CSI feedback will be related to multiple measurement hypotheses. The overhead of CSI report and the computation complexity of CSI measurement will increase accordingly. To make the CSI enhancement more feasible, we propose to enhancement Part 2 CSI omission and CPU occupation in Rel-17.

    - Part 2 CSI omission

When the UL resource is compact, Part 2 CSI omission can be applied according to priority reporting levels as shown in Table 3. Priority reporting levels among different reports are determined according to the priority value PriiCSIy,k,c,s of each report. In each report, CSI of even subbands has a higher priority reporting levels than CSI of odd subbands.

Table 3: Rel16 Priority reporting levels for Part 2 CSI

Priority 0: For CSI reports 1 to NRep, Group 0 CSI for CSI reports configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 wideband CSI for CSI reports configured otherwise

Priority 1: Group 1 CSI for CSI report 1, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report 1, if configured otherwise

Priority 2: Group 2 CSI for CSI report 1, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of odd subbands for CSI report 1, if configured otherwise

Priority 3: Group 1 CSI for CSI report 2, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report 2, if configured otherwise

Priority 4: Group 2 CSI for CSI report 2, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'. Part 2 subband CSI of odd subbands for CSI report 2, if configured otherwise

⁞

Priority 2NRep-1: Group 1 CSI for CSI report NRep, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report NRep, if configured otherwise

Priority 2NRep: Group 2 CSI for CSI report NRep, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of odd subbands for CSI report NRep, if configured otherwise

For CSI Option 1 with X=1 or 2, the payload of CSI report further increases because the UE is required to report multiple CSIs for various hypotheses, e.g. one CSI for NCJT hypothesis and X (X=1,2) CSIs for Single-TRP hypothesis. When UL transmission resource is only enough to carry partial CSIs in a report, UE can omit a subset of CSIs targeting for measurement hypotheses with lower priority reporting levels. Therefore R17 needs to define priority reporting levels among different hypotheses, for CSI Option 1.

Similar as Rel-15/16, priority reporting levels among different reports are determined according to the priority value PriiCSIy,k,c,s of each report. In each report, priority reporting levels of different hypotheses are determined as: NCJT hypothesis>1st S-TRP hypothesis>2nd S-TRP hypothesis. In each hypothesis, CSI of even subbands has a higher priority reporting level than CSI of odd subbands. In other words, priority reporting levels of NCJT report with Option 1 are given as below.

Proposal 14: For a given NCJT report with Option 1, priority reporting levels for Part 2 CSI in the report are defined as following orders, if configured

- Part 2 subband CSI of even subbands of NCJT measurement hypothesis

- Part 2 subband CSI of odd subbands of NCJT measurement hypothesis

- Part 2 subband CSI of even subbands of the first Single-TRP measurement hypothesis

- Part 2 subband CSI of odd subbands of the first Single-TRP measurement hypothesis

- Part 2 subband CSI of even subbands of the second Single-TRP measurement hypothesis

- Part 2 subband CSI of odd subbands of the second Single-TRP measurement hypothesis

    - CSI Updating

It was agreed that the calculation of a CSI assuming NCJT measurement hypothesis occupies 2 CPUs. Therefore total number of CPUs required for given CSI report associating with a CSI-RS resource set with M NZP CSI-RS resources for Single-TRP measurement hypotheses and N CMR pairs for NCJT measurement hypotheses, OCPU(n) , is given by:

- 2N, when reporting Option 1 with X=0 is configured

- M+2N, when reporting Option 1 with X=1 or 2 or Option 2 is configured.

Table 4: The number of occupied CPUs

| Ks | N | OCPU(n) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rel-15/16 (N is not applied) | Opt. 1 with X=0 | Opt. 1 with X=1/2 or Opt.2 |
| 2 | 1 | 2 | 2 | 4 |
| 3 | 1 | 3 | 2 | 5 |
| 3 | 2 | 3 | 4 | 7 |
| 4 | 1 | 4 | 2 | 6 |
| 4 | 2 | 4 | 4 | 8 |
| 6 | 1 | 6 | 2 | 8 |
| 6 | 2 | 6 | 4 | 10 |
| 8 | 1 | 8 | 2 | 10 |
| 8 | 2 | 8 | 4 | 12 |

As shown in above table, the minimal number of required CPUs for a NCJT report in Rel-17 is 2 when the UE is configured with reporting Option 1 with X=0. Moreover, the maximal number of required CPUs for a NCJT report is 12 when the UE is configured with reporting Option 1 with N=2 or Option 2.

However, typical chipset can support very limited CPUs per CC, e.g. 2 or 4 CPUs per CC for 16 ports CSI-RS resources for Rel-15 commercial UEs, so that Option 1 with X=1/2 or Option 2 becomes even more challenging in UE implementation, or unfriendly network limitation vice versa. Note that gNB normally has to configure periodic CSI reporting for basic sTRP transmission, which will occupy and reserve a certain amount of CPUs for given UE, from time to time. To avoid above issue caused by CPU overbooking, one possible solution is that when one CSI report is associated with both Single-TRP and NCJT measurement hypotheses, the UE can only update the CSI associated with NCJT measurement hypothesis and is not required to update the CSI associated with Single-TRP hypothesis.

For example, when the CSI report is configured with Option 1 with X=1/2, the CSI associated with NCJT measurement hypothesis can be updated and X CSIs associated with Single-TRP measurement hypothesis may not be required to update, if there is no sufficient available CPU. When the CSI report is configured with Option 2, the UE report the updated CSI associated with NCJT measurement hypothesis. Moreover,  when available CPUs is less than the number of required CPUs associated with all NCJT measurement hypotheses, i.e., 2N, the UE is not required to update whole CSI report.

Proposal 15: For a CSI report associated with both Single-TRP and NCJT measurement hypotheses, the UE is required to update the CSI associated with NCJT measurement hypotheses in the CSI report, if 2N≤NCPU<OCPU, where N is the number of CMR pairs associated with NCJT measurement hypotheses,  NCPU denotes the number of available CPUs on a given OFDM symbol, and OCPU is the number of CPUs required to update whole CSI report.

# Conclusions

This contribution provides our views on CSI enhancement based on angle and delay reciprocity and CSI enhancement for multi-TRP. In summary, the following proposals and observations are made.

Proposal 1: Confirm the working assumption that at least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix.

Proposal 2: At least for rank 1 of R17 port selection codebook, no further restriction or condition is applied for Mv=2. Further reduction of parameter combinations including Mv and other CB codebook parameters are feasible and can be discussed jointly.

Proposal 3: R= D* NPRBSB should be supported for Rel-17 PS codebook.

Proposal 4: Considering similarity of performance gain, it is preferred to keep the reserved state for reference amplitude.

Proposal 5: The bitmap for indicating non-zero coefficients for W2  is absent when the reported total number of NZC is equal to the maximum number of NZC for Rank 1 and 2, if β = 1 is configured.  Additionally the bitmap for layer 1 alone can be absent, only if reported real number of NZC for layer 1 is equal to corresponding maximal number of NZC for layer 1.

Proposal 6: For SCI reporting in R17 PS CB, support the reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits.

Proposal 7: For Rel-17 PS CB for Rank 2, support layer-specific quantization/non-zero coefficient selection of W2, assuming layer-common port selection/FD basis subset and the same maximum number of non-zero coefficients per layer.

Proposal 8: Support layer-common port selection and FD basis selection for rank 2.

Proposal 9: For rank 1 and 2 PMI for Rel-17 PS CB, Wf is layer-common and reported by UE if N>Mv, i.e. Alt 2-1.

Proposal 10: Support Rel-17 PS codebook design with rank ¾ by keeping CSI feedback overhead comparable to rank 2 and also reusing rank ½ CB design as much as possible for higher rank.

Proposal 11: Two CMRs within the same CMR pair configured for NCJT measurement hypothesis are within the same DL slot.

Proposal 12: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two RI restrictions can be configured for a given reporting setting whereas:

- One RI restriction corresponds to M CMRs for Single-TRP measurement hypothesis

- Another RI restriction corresponds to N CMR pairs for NCJT measurement hypothesis

Proposal 13: For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, two CBSRs can be configured for a given reporting setting whereas each of them corresponds to one CMR group in a CMR set, i.e. per TRP.

Proposal 14: For a given NCJT report with Option 1, priority reporting levels for Part 2 CSI in the report are defined as following orders, if configured

- Part 2 subband CSI of even subbands of NCJT measurement hypothesis

- Part 2 subband CSI of odd subbands of NCJT measurement hypothesis

- Part 2 subband CSI of even subbands of the first Single-TRP measurement hypothesis

- Part 2 subband CSI of odd subbands of the first Single-TRP measurement hypothesis

- Part 2 subband CSI of even subbands of the second Single-TRP measurement hypothesis

- Part 2 subband CSI of odd subbands of the second Single-TRP measurement hypothesis

Proposal 15: For a CSI report associated with both Single-TRP and NCJT measurement hypotheses, the UE is required to update the CSI associated with NCJT measurement hypotheses in the CSI report, if 2N≤NCPU<OCPU, where N is the number of CMR pairs associated with NCJT measurement hypotheses,  NCPU denotes the number of available CPUs on a given OFDM symbol, and OCPU is the number of CPUs required to update whole CSI report.

Observation 1: Assuming that CSI feedback overhead is upper bounded by R16 eTypeII port selection codebook, Mv=2 can be beneficial for Rel-17 PS codebook.

Observation 2: Assuming the same CSI feedback overhead, three alts for reserved code point for reference amplitude have similar performance.

Observation 3: Since β = 1 allows UE to feedback all NZC with the maximum number of NZC, if the UE considers it to be necessary, corresponding bitmap can be absent for Rel-17 port selection codebook.

Observation 4: At least for rank 1 and 2, the window size of N,  K1  for W1，and the value of Mv for Mf are layer-common, since these codebook parameters associated to rank1 can be reused and expended to the second layer of a rank2 PMI. The design principle for rank2 is similar to rank1.

Observation 5: For the bitmap indicating non-zero coefficient selection for rank 2, layer-specific selection performs better than layer-common up to 12%, especially at low/medium overhead regime, until performance gain of Rel-17 PS codebook saturates.

Observation 6: For port-selection and/or FD basis subset selection for rank 2, the performance of layer-specific selection is slightly better than layer-common selection in high overhead regime around 1%. On the other hand layer-common selection performs similarly or marginally better than layer-specific selection in low overhead regime.

Observation 7: R17 PS CB for Rank 3/4 can provide much better mean UPT up to 8%, especially within low and medium CSI overhead regime. Alternately it may save significantly CSI overhead, e.g. up to 45%.

# References

“RAN1 #104e Chairman’s Notes”, E-meeting, January 25th – February 5th, 2021.

R1-2105295, Views on Rel-17 CSI enhancements, Samsung.

# Appendix A.

## A.1 Complexity Analysis

In this appendix, we will give the analysis of UE complexity for Rel-17 port selection codebook. Firstly, we will compare the complexity between Rel-16 port selection codebook and Rel-17 port selection codebook. In the comparison, assuming CSI-RS ports P=32, the selected port number for Rel-16 (P=8), the select port number for Rel-17 (K1=32), and  N=Mv=1 for Rel-17. In addition, 20MHz with 104RBs (NRB=104,NSB=13) is assume in the analysis, R=1 for Rel-16 and Rel-17 where R corresponds to the number of PMIs per CQI sub-band, and 2Rx is assumed at UE side. The complex multiplication of R16 and R17 is summarized as following

Illustrative PMI calculating process/UE complexity for R16 PS codebook assuming layer 1 PMI:

- Calculating covariance matrix per sub band with O(NRB*P22) complex multiplication

- Port selection and spatial domain compression. Since most operators are complex addition and selection, corresponding complex is ignored.

- Subband SVD operator with NSB*EVD(P*P), which equals to R*NSB*O(P3) complex multiplication

- Frequency domain compression with O(P*R*NSB2) complex multiplication

Illustrative PMI calculating process for R17 PS codebook assuming layer-common port selection and FD basis subset selection assuming layer 1 PMI:

- N3-point IFFT transform over P channel matrix,  which has O(P*N3logN3) complex multiplication. P is the CSI-RS ports number used by UE to search the timing mismacth, e.g. P=8.

  - Note: N3=R*NSB,

- Obtaining coefficients for all P CSI-RS ports with P*N-1*N3

- Wideband SVD operator with O(K1*Mv*(NRx)2+NRx*K1*Mv2) complex multiplication across obtained coefficients

Based on the following table, it can be observed that compared with R16, R17 needs fewer complex multiplication even with UE searching operator.

| Steps | Calculating covariance matrix | Port selection | Subband SVD | FD compression | Total |
| --- | --- | --- | --- | --- | --- |
| complex multiplication | 53248 | 0 | 20657 | 1352 | 75257 |

- R16 PS codebook

| Steps | UE searching | Obtaining coefficients | Wideband SVD | Total |
| --- | --- | --- | --- | --- |
| complex multiplication | 2632 | 0 | 2184 | 4816 |

- R17 PS codebook

Table A.1-1 Example of UE complexity

In addition, the complexity of R17 with different design of port and FD basis subset selection is compared. In order to compare the complexity between different designs, we assume N=4, Mv=2, P=32, K1=16, and the other assumption is same as above. The impact on complexity of different design of port selection and FD basis subset selection are mainly in SVD operator. Specifically, different design of port selection affects the dimension of SVD operator.

- For layer-common port selection, UE can selection K1 ports from P ports across all layers before SVD operator, e.g. the complex multiplication is O(K1*Mv*(NRx)2+NRx*K1*Mv2)(Assuming layer-common FD basis subset selection).

- For layer-specific port selection, UE should do SVD operator with P ports firstly and then select K1 ports for each layer after SVD operator, e.g. complex multiplication is O(P*Mv*(NRx)2+NRx*P*Mv2) (Assuming layer-common FD basis subset selection).

- Similar to port selection, the FD basis subset selection also affects the dimension of SVD operator. For layer-common FD basis subset selection, UE can selection Mv FD basis from N FD basis across all layers before SVD operator, e.g. the complex multiplication is O(K1*Mv*(NRx)2+NRx*K1*Mv2) (Assuming layer-common port selection).

- For layer-specific FD basis selection, UE should do SVD operator with N FD basis firstly and then select Mv FD basis for each layer after SVD operator, e.g. the complex multiplication is O(K1*N*(NRx)2+NRx*K1*N2) (Assuming layer-common port selection).

## A.2 SLS assumptions for CSI enhancement

Table A.2-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption: - CSI feedback periodicity (full CSI feedback) : 5ms - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| SRS Configuration |  | - SRS periodicity with 10ms - Comb: 2 - Number of OFDM symbols: 2 |
| SRS Error Model |  | SRS error Modelling in Table A.1-2 in 36.897. Δ=9dB and detailed derivation of Δ can be found in R1-144943. |
| Calibration error model at gNB |  | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN amplitude error (expressed in decibels) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation for simulation bandwidth , respectively |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI reporting overhead |