3GPP TSG RAN WG1 Meeting #106-e	R1-2106482

E-meeting, August 16th – August 27th, 2021

Agenda Item:	8.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on timing relationship enhancements for NTN

Document for:	Discussion and Decision

# Introduction

In RAN1#105-e, the following were agreed for timing relationship enhancements for NTN [1]:

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1:	The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac.  How to treat NTA and NTA,offset can be further discussed.

Note 2:	According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3:	The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4:	Other options of determining the estimate of UE-gNB RTT can be further discussed.

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

In this contribution, we discuss the remaining details of timing relationship enhancements for NTN.

# Discussion

## K_offset in initial access

### Signalling of cell-specific K_offset

For the determination of cell-specific K_offset, it was agreed in RAN1#104bis-e to down-select from two options. In option 1, one offset value for K_offset is explicitly signaled, wherein the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point. In option 2, K_offset is the sum of two offset values, wherein the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link. As agreed in RAN1#104bis-e, common TA is indicated to determine the TA for Msg1/MsgA transmission, K_offset can be derived from the common timing offset following Option 2:

K_offset = ⌈ Common_TA/slot_duration ⌉+△K

wherein the Common_TA can be considered as a first offset value, and △K, which is determined by the maximum service link RTD within the cell coverage area, can be considered as a second offset value.

The main difference between option 1 and option 2 is that option 1 is an explicit way to determine K_offset while option 2 is an implicit way. From signaling overhead point of view, option 2 is superior to option 1 as only △K needs to be signaled. It was argued that the signaling overhead saving of option 2 is only 1 bit compared to option 1. This may be true at a glance but one needs to consider the overall procedure related to how the value of K_offset is determined and how it will be updated.

As discussed in our companion paper [2], the validity time of common TA related parameters, e.g. common TA, common TA drift rate or even higher order derivative of common TA are at least in the order of several or tens of seconds. Within one period of validity time for these common TA related parameters, the change of common TA may be of several slots. As an example, if the elevation angle for the satellite gateway is 10 degree, the validity time of common TA related parameters is 15.23s. Within this period, the change in feeder link RTD is 0.67s which corresponds to 6 slots for 120kHz SCS.

In earth-moving cells, the maximum service link RTD within one cell remains unchanged, i.e., △K is valid as long as the UE does not switch to another cell. Given that K_offset is the sum of common TA and △K in option 2, the value of K_offset can be updated even when the UE does not reacquire common TA related parameters. For option 1, K_offset can only be updated when the UE reacquires the SIB, e.g. the common TA related parameters. Therefore, a conservative value of K_offset has to be indicated with option 1, e.g. each time several slots are added to the one decided by the maximum TA. This leads to a longer scheduling delay for the UE.

Base on the above discussion, we propose the following

Proposal 1: For determination of cell-specific K_offset in system information, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in system information and covers the maximum service link RTD within the cell.

### Beam specific K_offset in initial access

In NTN, the maximum RTD within a cell may be larger than the maximum RTD in any beam within the same cell, using a cell specific K_offset can be conservative especially for UEs that are close to the satellite. Applying beam specific K_offset in the initial access can reduce the scheduling delay during RACH stage hence reduce the access delay.

To support beam specific K_offset for initial access, one can extend option 2 for the determination of cell-specific K_offset by using a beam specific △K in Msg2.

- After signalling △K to UE in Msg2, the UE can derive the beam specific initial K_offset by

K_offset = ⌈ Common_TA/slot_duration ⌉+△K,

where △K can be determined by maximum service link RTD within the beam coverage area (Beam_RTD_max), in which the UE locates.

Proposal 2: For determination of beam specific K_offset used in initial access if supported, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in Msg2 and covers the maximum service link RTD within the beam.

Similar to cell-specific K_offset, if a UE is provided with a beam-specific K_offset value in Msg2 or MsgB, the beam-specific K_offset value will be used for

- The transmission timing of RAR/fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

Proposal 3: If a UE is provided with a beam-specific K_offset value, the beam-specific K_offset value will be used for

- The transmission timing of RAR/fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

## Update of K_offset after initial access

For update of K_offset after initial access, it was agreed to at least support either RRC or MAC CE. RRC requires frequent RRC reconfigurations and there is a longer application latency compared to MAC CE.

For UE-specific K_offset, a UE needs to report some information (e.g., UE location or UE-specific TA) to assist the gNB to track the change of the K_offset. From signalling overhead point of view, it is beneficial to report UE location when UE is stationary or with low speed. While TA reporting will be more frequent as it will be impacted by the movement of satellite.

To allow UE-specific K_offset as early as possible, the UE can report UE-specific TA related information in Msg3 or MsgA. For contention based random access, K_offset can be updated from cell-specific to beam-specific for the scheduling of ACK/NACK for Msg4 or MsgB. For non-contention based random access, K_offset can be updated from cell-specific to beam-specific or UE-specific value for the scheduling of ACK/NACK for Msg4 or MsgB.

Proposal 4: K_offset update after initial access is supported via MAC-CE.

Proposal 5: UE-specific TA related information is reported in Msg3 or MsgA for update of K_offset.

To reduce signalling overhead, UE-specific TA can be updated by indicating a differential value, e.g. with granularity of one slot, to the previous value. Moreover, as TA reporting is used to determine the values of UE specific K_offset, it is sufficient to report TA with a same granularity of K_offset. Similarly a differential value with granularity of one slot can be adopted for TA reporting. As a result, only one bit is needed for TA reporting to determine the UE specific K_offset.

Observation 1: It is sufficient to report TA with a same granularity as K_offset.

Proposal 6: Differential indication with granularity of one slot are adopted for both UE-specific K_offset update and TA report to reduce the signaling overhead.

## Timing relationship of Configured Grant Type 1

In NR, the configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of ConfiguredGrantConfig message without the detection of a UL grant in a DCI. Apparently, as shown in Figure 1, the gNB should configure UE’s first transmission occasion (N = 0) with Offset larger than UE-TA.

Figure 1 Illustration for timing relationship of configured grant type 1

According to current specification [4], configured grant type 1 transmission resource is determined by timeReferenceSFN-r16, timeDomainoffset and periodicity contained in ConfiguredGrantConfig as below:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeReferenceSFN-r16 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

where timeDomainoffset ranges from 0~5119 and timeReferenceSFN-r16 can be 0 or 512. Extending the set of  timeReferenceSFN-r16 value can provide sufficient flexibility to fulfill the timing relationship of Offset>UE-TA, there is no need to introduce an extra offset parameter for timing relationship on Configured Grant Type 1.

Proposal 7: By extending the range of timeReferenceSFN-r16, there can be sufficient scheduling flexibility to fulfil the timing relationship for configured grant type 1.

## Exceptional MAC CE timing relationship

The MAC CE timing for aperiodic CSI Trigger State subselection indication is described in TS38.214 section 5.2.1.5.1:

A trigger state is initiated using the CSI request field in DCI.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

According to the above, it can be observed that the timing for the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field has not taken the UE-specific TA into account, i.e. “logic” timing is used. Therefore, both the gNB and UE will assume that the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied from the first slot that is after slot n+3Nslotsubframe,µ. Consequently, the gNB can only trigger CSI based on the updated CSI triggering state from the first slot that is after slot n+3Nslotsubframe,µ since the gNB has no idea whether UE has correctly decoded the PDSCH carrying the updated subselection indication. This is illustrated in Figure 3.

Figure 3 Illustration for MAC-CE timing relationship

The MAC CE timing for updating the spatial relation of the aperiodic SRS is described in TS38.214 section 6.2.1

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ.  The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

Similar to the discussion above, the MAC CE action timing for updating the spatial relation of the aperiodic SRS also has not taken the UE-specific TA into account, i.e. “logic” timing is used. Therefore, both the gNB and UE will assume that the updated spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. Consequently, the gNB can only trigger SRS assuming the updated spatial relation from the first slot that is after slot n+3Nslotsubframe,µ

Proposal 8: The MAC CE action timing for the aperiodic CSI Trigger State subselection indication and updating the spatial relation of the aperiodic SRS are for the CSI request and SRS triggering respectively.

## On K1 range extension

Extending the value range of K1 from (0, 15) to (0, 31) is necessary to support some TDD UL/DL configuration. One candidate option is to extend the DCI field to 4bits, supporting K1 extension while maintaining scheduling flexibility. However, it is not clear whether the expected benefit of scheduling flexibility can justify the extra signalling overhead. Therefore, to minimize the specification impact, DCI field extension or new DCI format is not preferred. Re-interpreting K1 as number of slot groups rather than number of slots shall be prioritized, which avoids modification on DCI format while contributing to compatibility for further enhancement. For example, for a PDSCH reception ending in slot n=5, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot n+k*2=31, where k equals to 13.

Figure 4 Illustration for K1 range

Proposal 9: K1 indication can be enhanced without impact on the size of DCI by re-interpreting PDSCH-to-HARQ_feedback timing indicator field.

## PDCCH ordered PRACH

According to NR spec, gNB can trigger RACH procedure via PDCCH for the purpose of uplink re-synchronization between UE and gNB, described as follow:

TS 38.213, Section 8.1:

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

However, selection of available PRACH occasion within first available mapping cycle does not take the impact of TA into account. Due to the large TA value in NTN scenario, there may be ambiguity between gNB and UE that which PRACH occasion is occupied, consequently, additional blind detections may be need at gNB side.

Similar with other timing issues, K_offset can be introduced for aligning the understanding of PRACH occasion selection to address the ambiguity issue between gNB and UE.

Proposal 10: For PDCCH ordered PRACH, K_offset can be introduced for PRACH occasion selection.

## Beam failure recovery

For beam failure recovery, the timing relationship between PRACH transmission and PDCCH monitoring at UE are defined as follows:

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot  within a window configured by BeamFailureRecoveryConfig.

Figure 5 Illustration of timing relationship for Beam failure recovery

As shown in Figure 5, an additional offset shall be introduced between PRACH transmission and PDCCH monitoring, i.e. the UE shall monitor PDCCH starting from slot n+4+BFR_offset, wherein the BFR_offset equals to RTD-TA=K_mac.

Proposal 11: The timing relationship for beam failure recovery needs to be enhanced with timing offset of K_mac, i.e. a UE monitors PDCCH from slot n+4+K_mac within a window configured by BeamFailureRecoveryConfig.

# Conclusion

In this contribution, we further discuss the issues of timing relationship. The following observations and proposals are presented:

Observation 1: It is sufficient to report TA with a same granularity as K_offset.

Proposal 1: For determination of cell-specific K_offset in system information, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in system information and covers the maximum service link RTD within the cell.

Proposal 2: For determination of beam specific K_offset used in initial access if supported, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in Msg2 and covers the maximum service link RTD within the beam.

Proposal 3: If a UE is provided with a beam-specific K_offset value, the beam-specific K_offset value will be used for

- The transmission timing of RAR/fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

Proposal 4: K_offset update after initial access is supported via MAC-CE.

Proposal 5: UE-specific TA related information is reported in Msg3 or MsgA for update of K_offset.

Proposal 6: Differential indication with granularity of one slot are adopted for both UE-specific K_offset update and TA report to reduce the signaling overhead.

Proposal 7: By extending the range of timeReferenceSFN-r16, there can be sufficient scheduling flexibility to fulfil the timing relationship for configured grant type 1.

Proposal 8: The MAC CE action timing for the aperiodic CSI Trigger State subselection indication and updating the spatial relation of the aperiodic SRS are for the CSI request and SRS triggering respectively.

Proposal 9: K1 indication can be enhanced without impact on the size of DCI by re-interpreting PDSCH-to-HARQ_feedback timing indicator field.

Proposal 10: For PDCCH ordered PRACH, K_offset can be introduced for PRACH occasion selection.

Proposal 11: The timing relationship for beam failure recovery needs to be enhanced with timing offset of K_mac, i.e. a UE monitors PDCCH from slot n+4+K_mac within a window configured by BeamFailureRecoveryConfig.

# References

RAN1#105-e Chairman's Notes

R1-2106483, “Discussion on UL time and frequency synchronization enhancement for NTN”, Huawei, HiSilicon

RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales

3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”