3GPP TSG RAN WG1 Meeting #106-e	R1-2106514

E-meeting, August 16th – 27th, 2021

Agenda Item:	7.2.7

Source:	Huawei, HiSilicon

Title:	Discussion on corrections of SCell dormancy for power saving

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining maintenance issues on Rel-16 SCell dormancy for power saving.

# Discussion

## Correction on “one or both” of DCI format 0_1 and DCI format 1_1 in section 10.3, TS 38.213

In Rel-16, it was agreed when UE is within DRX Active Time, the DCI format 0_1 and 1_1 which schedules data can carry SCell dormancy indication. According to 38.331 [1], if configured, DCI format 0_1 and 1_1 are configured simultaneously for a search space set. According to 38.212 [2], if the higher layer parameter(s) dormancyGroupWithinActiveTime are configured, both DCI format 0_1 and 1_1 should contain the field of "SCell dormancy indication", which has at least 1 bit. Related descriptions are highlighted in the following description in the specification.

### 38.331 ### 6.3.2 Radio resource control information elements SearchSpace ::= SEQUENCE { ..., searchSpaceType CHOICE { ..., ue-Specific SEQUENCE { dci-Formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, ..., } } OPTIONAL -- Cond Setup2 } ### 38.212 ##### 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: ======skipped part======= - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. ======skipped part======= ##### 7.3.1.2.2 Format 1_1 DCI format 1_1 is used for the scheduling of PDSCH in one cell. The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: ======skipped part======= - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. ======skipped part=======

The current specification in 38.213 is inconsistent with the above descriptions. As stated in the specification, the description of "if one or both of" causes misunderstanding that it allows only one of the DCI format 0_1 and DCI format 1_1 includes the SCell dormancy indication field. Related descriptions are highlighted in the following description in the specification.

### 38.331 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

Therefore, we have the following proposal:

Proposal 1: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

## Clarification on whether Case 2 PDCCH does not indicate a SPS PDSCH release

For Case 2 PDCCH, as stated in the following specification (10.3 of TS 38.213), if a detected DCI format 1_1 meets the corresponding conditions, the UE considers the DCI format 1_1 not indicating a SPS PDSCH release. According to the description in section 10.2 of TS 38.213, a SPS PDSCH release indication is carried by a DCI with CRC scrambled by a CS-RNTI [3]. However, the conditions in 10.3 of TS 38.213 include that the CRC of DCI format 1_1 is scrambled by a RNTI which includes C-RNTI and MCS-C-RNTI and does not include CS-RNTI. Therefore, the description of “not indicating a SPS PDSCH release” should be removed to avoid confusion on the understanding.  Related descriptions are highlighted in the following description in the specification.

### 38.213 ## 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0', and - the DFI flag field, if present, in the DCI format is set to '0', and - if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16. ======skipped part======= ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of ======skipped part=======

Therefore, we have the following proposal:

Proposal 2: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of ======skipped part=======

## Clarification on where Case 2 PDCCH applies for each configured SCell

For Case 2 PDCCH, the DCI format 1_1 does not schedule PDSCH and carries SCell dormancy indication for each configured SCell. Specifically, the fields of MCS, NDI, RV fields etc. are repurposed as a bitmap, where each bit corresponds to a configured SCell in an ascending order of the SCell index. This means, for a configured SCell, the corresponding bit is a placeholder for this SCell even when the SCell is not activated or the SCell is not configured with a dormant BWP. Related descriptions are highlighted in the following description in the specification.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

It is clear that if an activated SCell is configured with higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id, the UE will carry out the specified procedure, i.e., the UE sets the active DL BWP to the indicated active DL BWP. However, for a configured SCell, if the higher layer parameters dormantBWP-Id and/or firstWithinActiveTimeBWP-Id are not configured, the UE cannot execute the specified procedure considering the UE does not have dormant BWP and first non-dormant BWP on the SCell.

In order to correct this issue, we propose to restrict that the specified procedure only applies to the activated SCell(s) configured with the higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id. Hence, we have the following proposed corrections:

Proposal 3: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where, for an activated SCell configured with dormantBWP-Id and firstWithinActiveTimeBWP-Id, - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding the activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding the activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding the activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

# Conclusions

According to the above discussion, the following updates are needed in 38.213:

Proposal 1: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

Proposal 2: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of ======skipped part=======

Proposal 3: Adopt the following change for SCell dormancy in TS 38.213.

### 38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where, for an activated SCell configured with dormantBWP-Id and firstWithinActiveTimeBWP-Id, - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding the activated SCell - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding the activated SCell, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for a corresponding the activated SCell, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part=======

# References

- 3GPP TS38.331 v16.5.0

- 3GPP TS38.212 v16.6.0

- 3GPP TS38.213 v16.6.0