3GPP TSG-RAN WG1 Meeting #106	R1-2106515

# E-meeting, August 16th – August 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of Scell dormancy for power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 38.331, if configured, DCI format 0_1 and 1_1 are configured simultaneously for a search space set. According to 38.212, if the higher layer parameter(s) dormancyGroupWithinActiveTime are configured, both DCI format 0_1 and 1_1 should contain the field of "SCell dormancy indication" which has at least 1 bit. The current description of "if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field" causes misunderstanding that it allows only one of the DCI format 0_1 and DCI format 1_1 includes a SCell dormancy indication field. - Considering a SPS PDSCH release indication is carried by a DCI with CRC scrambled by a CS-RNTI, and a PDCCH is considered as a Case 2 PDCCH only if the CRC of DCI format 1_1 is scrambled by a RNTI which includes C-RNTI and MCS-C-RNTI, therefore, the description of “the UE considers the DCI format 1_1 as not indicating a SPS PDSCH release” should be removed to avoid confusion on the understanding. - For a configured SCell, if the higher layer parameters dormantBWP-Id and/or firstWithinActiveTimeBWP-Id are not configured, when a Case 2 PDCCH is detected, the UE cannot execute the specified procedure, considering the UE does not have dormant BWP and first non-dormant BWP on the SCell. In order to correct this issue, it is proposed to restrict that the specified procedure only applies to the activated SCell(s) configured with the higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that if configured, then both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field. - Delete the description that Case 2 PDCCH does not indicate a SPS PDSCH release. - If Case 2 PDCCH is detected, correct that the specified UE procedure only applies to the SCell(s) configured with the higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The description of "if one or both of" causes misunderstanding that it allows only one of the DCI format 0_1 and DCI format 1_1 includes a SCell dormancy indication field. - Description in the specification causes confusion. - Incorrect UE behaviour for a SCell which is not configured with higher layer parameters dormantBWP-Id and firstWithinActiveTimeBWP-Id when a Case 2 PDCCH is detected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

========================= Unchanged parts =========================

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field,

-	the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime,

-	each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells

-	if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	a one-shot HARQ-ACK request field is not present or has a '0' value, and if

-	the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of

-	modulation and coding scheme

-	new data indicator

-	redundancy version

and of

-	HARQ process number

-	antenna port(s)

-	DMRS sequence initialization

as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where, for an activated SCell configured with dormantBWP-Id and firstWithinActiveTimeBWP-Id,

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for the activated SCell

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for the activated SCell, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for the activated SCell, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

========================= Unchanged parts =========================