3GPP TSG RAN WG1 Meeting #106-e                                                               R1-2106548

e-Meeting, August 16th–27th, 2021

Source:              ZTE

Title:                   Further details on Multi-beam and Multi-TRP operation

Agenda Item:     8.1.5

Document for:   Discussion and Decision

Introduction

In Rel-17 NR, in order to enhance the performance of multi-beam and multi-TRP operation, some solutions have been designed and agreed accordingly. Regarding the enhancement on multi-beam operation, the aspects of unified TCI framework for both DL and UL transmission, fast UE panel switching and FR2 MPE mitigation have been discussed and progressed. Regarding the enhancement on multi-TRP operation, PDCCH/PUCCH/PUSCH repetition, inter-cell mTRP enhancement, beam management for multi-TRP operation and multi-TRP based HST enhancement have been introduced for better performance and reliability. In this contribution, we provide our views on some further details of multi-beam and multi-TRP operation.

Unified TCI framework for downlink/uplink transmission with mTRP

Unified TCI is one of essential enhancement for beam management in Rel-17, as well as uplink transmission with mTRP. Whether unified TCI should be supported in mTRP scenario has been discussed during latest RAN1 meetings, and a conclusion and agreement were reached as follows.

Conclusion (in RAN1#104-e) On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1: - DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1: - UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. - Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1: - DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1: - UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N. - Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used. Agreement (in RAN1#105-e) On Rel-17 unified TCI, in RAN1#106-e, for M>1 and/or N>1: - Identify and agree on use cases - Decide whether to support M>1 and/or N>1, and if so, the maximum value of M and/or N - If supported, identify feasible candidate schemes for beam indication signaling mechanism (including TCI state activation) Note: - Previous agreement in RAN1#104b-e that remaining unused DCI fields and codepoints are reserved in R17 are not to be reverted. - The use case of simultaneous UL transmission from multiple UE panels are not to be considered in Rel-17 as it is out of scope

M>1 and/or N>1 is supposed to apply M DL or N UL indicated TCI states to at least one DL or UL channel or signal respectively. In our views, M>1 and/or N>1 represent multiple TCI states indicated for updating source reference RS for DL or UL channels or signals corresponding to different TRPs.

In this section, MAC CE design for activating DL/UL TCI states in mTRP, details on how to apply the indicated TCI state to the target channels/signals in mTRP scenario and power control enhancements for unified TCI based mTRP are analyzed.

MAC CE design for activating DL/UL TCI states in mTRP

A UE can be configured “joint TCI state” or “separate DL/UL TCI state” via RRC signaling for property of DL/UL for an indicated TCI state. This would affect the type of codepoint of TCI states in MAC CE.

- For “joint TCI state”, a TCI state can be used for both DL and UL, and the type of codepoint of TCI states can be unified DL and UL;

- For “separate DL/UL TCI state”, a TCI state can be used for either DL or for UL, and the type of codepoint of TCI states can be one of: DL, UL, combined DL and UL.

For MAC CE design, the type for each codepoint of TCI sates should be indicated. There are 3 values for “separate DL/UL TCI state” case, thus 2 bits are needed for each codepoint. In order to reduce overhead, numbers of each type can be indicated before the TCI state list, and the TCI state list is organized in a predefined order of types of codepoint of TCI states. For example, as shown in table 1, number of TCI state codepoints indication is used to indicate a number of type 1 (combined DL and UL) N1, a number of type 2 (DL) N2 and a number of type 3 (UL) N3. Number of TCI state codepoints indication can comprise 3 values for numbers for type 1, type 2 and type 3, or a joint-coded value for numbers for type 1, type 2 and type 3. Alternatively, number of TCI state codepoints indication can also indicate numbers of type 1 and type 2 only, but the number of type 3 can be deduced using the length of this MAC CE.

Table 1 MAC CE for for activating DL/UL TCI states

| bit #7 | #6 | #5 | #4 | #3 | #2 | #1 | bit #0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R | Serving Cell ID |  |  |  |  | BWP ID |  |
| Number of TCI state codepoints indication (type1: N1, type2: N2, type3: N3) |  |  |  |  |  |  |  |
| R | TCI state type1_1_DL |  |  |  |  |  |  |
| R | TCI state type1_1_UL |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| R | TCI state type1_N1_DL |  |  |  |  |  |  |
| R | TCI state type1_N1_UL |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| R | TCI state type2_1_DL |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| R | TCI state type2_N2_DL |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| R | TCI state type3_1_UL |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| R | TCI state type3_N3_UL |  |  |  |  |  |  |

In mTRP scenario, the MAC CE comprises information to indicate S-DCI (single-DCI) or M-DCI (multiple-DCI). In the case of M-DCI, the MAC CE may comprise information to indicate CORESET pool ID. That means one MAC CE can be used for activating DL/UL TCI states for one TRP, as in Rel-16. In the case of S-DCI, the MAC CE may comprise information to indicate presence of additional TCI state for a second link in a codepoint. Link can be associated with TRP. The information of presence of additional TCI state for a second link in a codepoint indicates presence of additional TCI state for a second link in a codepoint for a same usage (DL or UL) as the TCI state for a first link.

For example, as shown in figure 1, in the case that “separate DL/UL TCI state” is determined, and mTRP is supported with S-DCI, a TCI state entry (byte) may comprises a “C” or “R” bit and corresponding TCI state indication (7-bit). A TCI state can be for DL or for UL. The type of a codepoint of TCI states may comprise DL, UL, or combined DL and UL. A codepoint of TCI states may comprise one DL or UL TCI state, two DL or UL TCI states for two links (e.g. TRP0 and TRP1), two DL and one UL TCI states, two UL and one DL TCI states, or two DL and two UL TCI states. “C” bit in the TCI state entry for a first link indicates presence of additional TCI state for a second link in a codepoint with the same usage (for DL or UL) as the TCI state for the first link.

Figure 1 TCI state list for mTRP with S-DCI

Proposal 1: Regarding MAC CE for activating DL/UL TCI states,

- When separate DL/UL TCI state is configured, 3 types of codepoint of TCI states, such as DL, UL, combined DL and UL, should be supported in the MAC CE;

- The TCI state list should be organized in a predefined order of types of codepoint of TCI states in the MAC CE;

- Two independent values or one joint-coded value for numbers of two types of codepoint of TCI states should be indicated in the MAC CE.

Proposal 2: Regarding unified TCI state scheme in mTRP,

- M>1, N>1 should be supported for mTRP as a starting point;

- mTRP requirement should be considered in MAC CE design for activating DL/UL TCI states;

- Information of presence of additional TCI state for another TRP for same DL/UL in a codepoint should be supported.

Unified TCI state applied to DL/UL channel/signal in mTRP

When a unified TCI state (source TCI state) indicated by a DCI format can be applied to (target) channel/signal of downlink and/or uplink in mTRP, the TRP related to the unified TCI state and the TRP related to the applied (target) channel/signal should be aligned. In the case of mTRP, a unified TCI state indicated by a TCI codepoint value in DCI format comprises one TCI state corresponding to one TRP (but no information on which TRP) or two TCI states corresponding to two TRPs. Meanwhile, the target channel/signal is also related to one or two TRPs.

Then, there are the following cases which are needed to be considered for TCI state updating for the target channel/signal:

- Case 1: Unified TCI codepoint with a single TCI state is applied to one given TRP, and then target channel/signal related to a single TCI state transmission corresponds to the same TRP.

- Case 2: Unified TCI codepoint with two TCI states is applied to two given TRPs, and then target channel/signal related to a single TCI state transmission corresponds to one of the two given TRPs.

- Case 3: Unified TCI codepoint with a single TCI state is applied to one given TRP, and then target channel/signal related to two-TCI-state transmission corresponds to two TRPs that include the given TRP.

- Case 4: Unified TCI codepoint with two TCI states is applied to two given TRPs, and then target channel/signal related to two-TCI-state transmission corresponds to those two TRPs.

Straightforwardly, in mTRP scenario, a TCI state can be explicitly or implicitly associated with a TRP ID information, and the resource (set) of DL or UL control channels or reference signals can also be explicitly or implicitly associated with a TRP ID information, and then multiple (i.e. M/N) TCI states can be applied to the target DL or UL channels or signals according to the associated TRP ID information.

Proposal 3: Regarding the unified TCI state/codepoint applied to the target channel/signal,

- TRP ID information should be tied to a unified TCI state as well as a target channel/signal;

- When indicated, the unified TCI state can be applied to the target channel/signal that shares the same TRP ID information as the unified TCI state.

For the sake of presentation, we elaborate the detailed solution for each type of channels/signals as follows.

- For UL channels/signals,

- Regarding SRS, up to N SRS resource sets for codebook or non-codebook can be supported/configured. Each SRS resource set is associated with a TRP ID information. An indicated TCI state which is associated with a TRP ID among N TCI states is applied to all the SRS resources in the SRS resource set which is associated with the same TRP ID.

- Regarding PUSCH, a DCI scheduling a PUSCH transmission indicates one or two SRIs and their SRS resource set IDs, and the corresponding details can be found in our companion contribution [1]. In our views, this approach can also be used for PUSCH transmission in the case of unified TCI. Alternatively, especially for mDCI-mTRP case, unified TCI can be used to select SRS resource set, and only SRI field in the scheduling DCI is interpreted based on the selected set.

- Regarding PUCCH, each PUCCH resource (group) can be associated with a TRP ID information. An indicated TCI state which is associated with a TRP ID among N TCI states is applied to all PUCCH resources which are associated with the same TRP ID. Alternatively, especially for sDCI-mTRP case, each PUCCH resource can be associated with up to two TRP ID information, such as two PUCCH spatial relations corresponding to two TRPs, as which is under discussion in MTRP topic for PUCCH repetitions [1].

- For DL channels/signals,

- Regarding PDCCH, a CORSET can be associated with a TRP ID information for both mDCI-mTRP and sDCI-mTRP cases, such as CORESET pool ID in Rel-16. An indicated TCI state which is associated with a TRP ID among M TCI states is applied to the CORESET(s) which are associated with the same TRP ID.

- Regarding PDSCH, the indicated unified TCI states can be applied to a PDSCH transmission after application time. Note that, if the offset between DL DCI and the PDSCH is less than the threshold timeDurationForQCL, the current/latest effective unified TCI state for PDSCH is used for determining the DM-RS ports of the PDSCH, instead of a default beam approach specified as in Rel-15/Rel-16.

- Regarding CSI-RS, whether a unified TCI can be applied to CSI-RS is still FFS. if it is supported, CSI-RS resource set can be associated with TRP ID information. An indicated TCI state which is associated with a TRP ID among M TCI states is applied to the CSI-RS resources in the CSI-RS resource sets which are associated with the same TRP ID.

Power control for uplink transmission

As for power control framework, open-loop power control parameters, e.g. P0 and alpha, closed-loop power control parameter, and PL-RS can be associated with UL TCI state or DL and UL TCI state in RRC. PL-RS can be configured by RRC, the number of configured PL-RSs can be up to 64. When the TCI state is activated by a MAC CE, the total number of activated TCI states may be up to 8, then the number of PL-RSs could be at most 8. But a UE can only support at most 4 active PL-RSs to be monitored by a UE. To solve this problem, it should be supported to update PL-RS for an activated TCI state in MAC CE.

Proposal 4: Regarding power control framework for unified TCI for mTRP,

- The association between TCI state and power control parameters, e.g. PL-RS, should be maintained by MAC CE.

- If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state.

Further details on beam indication/update cross carriers

In RAN1#105-e, a general framework for common DL/UL beam indication cross carriers (CCs) under the unified TCI framework has been agreed as follows. In this subsection, we further discuss the details on common beam update cross CCs.

Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

As mentioned in above, sharing a single RRC TCI state pool for the set of CCs has been agreed as WA. Then, based on the WA, we have the following analysis on the pros and cons of agreed WA solution over legacy one, i.e., configuring RRC TCI state pool per individual BWP/CC.

- For common beam indication of DL and UL channel/RS, the (unified) TCI state may include QCL assumption of DL channel/RS, spatial relation of UL chnanel/RS and power control parameters of UL channel/RS. As elaborated in Table 2, for common beam indication of DL and UL channel/RS per BWP in a CC, RRC configuration of 128 TCI states requires up to 9089 bits (corresponding to 1.12 kB). Due to up to 4 BWPs are supported in a CC, up to 1.12*4=4.48 kB are required per CC. Further, for multiple CCs and multiple BWPs, the signaling overhead of RRC configuration of TCI states is summarized in Figure 1.

- It can be observed that with the increase of the number of CCs, RRC overhead in Opt-2 rises sharply. For instance, in the worst case, the RRC overhead can be up to 143.36 kB when all of 32 CCs have been configured with respective TCI state pools. But, on the other hand, due to sharing the same RRC pool, the RRC overhead in Opt-1 can be kept unchanged, and only the RRC overhead of 1.12 kB is needed. It means that compared with Opt-2, the RRC overhead in Opt-1 can be saved significantly.

- However, from the perspective of RRC configuration flexibility, it is no doubt that the Opt-2 outperforms the Opt-1 due to the fact that the RS ID in the TCI state can be configured per BWP/CC. But, considering the unified TCI state framework, the same beam is applied to both DL and UL channel across a CC group, and it is natural that the identical RS configuration (e.g., TRS) is configured for the CC group. Therefore, gNB only need to guarantee that the RS (e.g., TRS) with same ID share the same Tx beam across the CC group, and so the loss related to RRC configuration flexibility should be acceptable.

In a word, we identify the benefit of sharing a single RRC TCI state pool for the set of CCs, and suggest to confirm the above WA and support that RRC-level PL-RS pool can be shared cross multiple BWP/CC by reference BWP/CC ID. Some more details can be found in our companion contribution [2].

Observation 1:

- Due to sharing a single RRC TCI state pool for a CC group rather than respective pools per CC/BWP can significantly save RRC overhead, e.g., 1.12kB vs 143.36 kB.

- Configuration flexibility of sharing a single RRC TCI state pool for a CC group is sufficient for unified TCI indication, considering the identical RS configuration (e.g., TRS) across multiple CCs under the requirement of updating a common beam for all RS/channels, although losing some flexibility compared with legacy one.

Table 2 Bit(s) required for RRC configuration of TCI state(s) applicable to common beam indication

| Element in TCI state |  | The number of required bits |
| --- | --- | --- |
| TCI-StateId |  | 7 |
| QCL-Info | ServCellIndex (OPTIONAL) | 5+1 |
|  | BWP-Id (OPTIONAL) | 2+1 |
|  | NZP-CSI-RS-ResourceId or SSB-Index | 8 or 6 |
|  | qcl-Type | 2 |
| QCL-Info (OPTIONAL) | ServCellIndex (OPTIONAL) | 5+1 |
|  | BWP-Id (OPTIONAL) | 2+1 |
|  | NZP-CSI-RS-ResourceId or SSB-Index | 8 or 6 |
|  | qcl-Type | 2 |
| ServCellIndex (OPTIONAL) |  | 5+1 |
| Spatial relation | NZP-CSI-RS-ResourceId, SSB-Index or SRS-ResourceId | 8 or 6 or 8 |
| PC parameters | PathlossReferenceRS-Id | 6 |
|  | p0-Id | 3 |
|  | closedLoopIndex | 2 |
| Bits required for RRC configuration of 1 TCI state |  | Up to 71 |
| Bits required for RRC configuration of 128 TCI states (OPTIONAL) |  | Up to 71*128+1=9089 |

Figure 1 Overhead of RRC configuration required for different numbers of CCs

Evaluation results for Option 1 panel-specific reporting

In this section, we evaluate reporting quantities in Option 1 panel-specific reporting that has been identified as a candidate solution for facilitating inter-TRP beam pairing. In order to well evaluate different scenarios, we provide our SLS (indoor hotspot and dense urban) and LLS evaluation results for Option 1, respectively.

According to the grouping criteria of Option 1 (antenna group based reporting), we evaluate SLS performance for the case that a UE with 2 Rx panels is served by 2 TRPs in the indoor hotspot and dense urban scenarios under the UE blockage model of TS 38.901. In dense urban, we consider the high-mobility case, i.e., speed of UE = 60 km/h. The evaluation assumptions can be found in Table 4 and Table 5 in the appendix.

The simulation results for indoor hotspot and dense urban are provided as a function of number of beams per group in a report instance and can be found in Table 3 and Table 4 respectively. Rel-15 group based reporting (up to 1 beam for each of 2 UE panels is reported) is considered as baseline. From the evaluation results, it can be observed that increase number of beams per group can bring performance benefit due to combat blockage and UE mobility. Specifically, we have the further observations as follows.

- With the increase of the number of beams to be reported M from only 1 to 4, the performance gain of 7.7% and 5.6% in terms of average UPT can be observed in indoor hotspot and dense urban.

- For 5% cell edge UE in dense urban, we can find the performance gain of 36.8% compared with Rel-15 group based reporting, which proves that the increase of candidate beams for each UE panel can distinctly enhance the performance of cell-edge UE with high mobility and be against the blockage and UE mobility.

Table 3	SLS for Option 1 (panel-specific reporting) with M beams per group to be reported and N groups in indoor hotspot

| UPT (Mbps) | mean | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| (N = 2 group, M = 1 beam) i.e., Rel-15 group reporting | 89.48 (100%) | 52.02 (100%) | 79.20 (100%) | 176.47 (100%) |
| (N = 2 group, M = 2 beams) | 93.34 (+4.3%↑) | 55.50 (+6.7%↑) | 84.19 (+6.3%↑) | 182.06 (+3.2%↑) |
| (N = 2 group, M = 4 beams) | 96.36 (+7.7%↑) | 55.82 (+7.3%↑) | 84.82 (+7.1%↑) | 183.05 (+3.7%↑) |

Table 4	SLS for Option 1 (panel specific reporting) with M beams per group to be reported and N groups in dense urban

| UPT (Mbps) | mean | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| (N = 2 group, M = 1 beam) i.e., Rel-15 group reporting | 83.42 (100%) | 13.47 (100%) | 77.65 (100%) | 165.66 (100%) |
| (N = 2 group, M = 2 beams) | 86.19 (+3.3%↑) | 15.15 (+12.5%↑) | 80.86 (+4.1%↑) | 170.71 (+3.0%↑) |
| (N = 2 group, M = 4 beams) | 88.09 (+5.6%↑) | 18.42 (+36.8%↑) | 81.40 (+4.8%↑) | 170.07 (+2.7%↑) |

Besides, up to 2 groups (e.g., UE panels) in Rel-15 group reporting are also unsuitable for the enhanced UE in Rel-16. Three or four UE panels may be applied to a handheld device, e.g., mobile phone. Regarding EVM for multi-beam operation in Item-1 of this FeMIMO, evaluation assumption of 3 UE panels (left, right, and back) has also been agreed for subsequent evaluation. That means that the number of groups N and the number of beams in a group M should also be enhanced, and it can be assumed as a starting point that N and M are up to 4, i.e., to consider up to 4 panels, and up to 4 candidate beams per UE panel (for Option 1).

Observation 2: From the evaluation results for Option 1 panel-specific reporting, it can be observed that:

- Increase number of beams per group for Option 1 can bring significantly higher performance gain to combat blockage and UE mobility, especially for cell-edge UE in dense urban, through improving system flexibility with more candidate beams.

- Increase number of groups for Option 1 can be more suitable for advanced UE with 3, 4 or more panels.

UE initialized TRP switching for multi-TRP operation

The movement and rotation of the UE will cause continuous changes in the channel quality between the gNodeB and the UE. Especially in the high-speed train scenario, the channel quality may be poor in some cases. In these cases, a better TRP or multi-TRP group should be selected for transmission to enhance communication reliability. At present, the transmission selection of Single-TRP or Multi-TRP is usually indicated by the gNodeB. However, if the backhaul between TRPs is not ideal, a large latency between TRPs may be experience and so the serving TRP can NOT obtain the channel quality of other TRP(s) in time. Therefore, if the quality of other TRP(s) is better, the handover can NOT be performed in time.

In the high-speed train (HST) scenario, when a UE moves away from the connected TRP, the increase of path loss and beam changes may affect the channel quality between the TRP and the UE. Figure 2 shows the channel quality changes with UE movement. Here, RSRP (Reference Signal Receiving Power) is used as the channel quality evaluation indicator.

Figure 2 Channel quality in the high-speed train scenario

As shown in Figure 2, the RSRP curves of the TRP1 and TRP2 are dotted and solid, respectively. When the UE is at point A, it has established a connection with the TRP1. Then, when the UE moves from point A to point B or C, the RSRP of TRP1 increases gradually at the beginning. After the UE continues to move, the UE gradually keeps away from the TRP1, and the RSRP of TRP1 starts to decrease gradually. Conversely, as the UE approaches TRP2, the corresponding RSRP increases gradually. Therefore, to ensure the transmission performance, the UE needs to report a TRP handover mode from the following options.

- Mode 1: switch from current TRP to another TRP with better channel quality.

- Mode 2: switch from current TRP to multi-TRP with better channel quality.

According to the above description, multi-TRP transmission may be configured for UE, but the following situation may occur during the UE movement: When a UE moves away from one TRP and approaches another TRP at the same time, the RSRP of one TRP will be obviously better than that of the other TRP. In this case, the transmission of multi-TRP cannot provide obvious transmission reliability, compared with the transmission of single-TRP with better channel quality. Therefore, the UE needs to report a TRP handover mode as the following option.

- Mode 3: switch from current multi-TRP to single-TRP with better channel quality.

As shown in Figure 2, when the UE moves from point B to point C, the RSRP measurement value of TRP1 reduces gradually, and the RSRP measurement value of TRP2 increases gradually. At this time, the difference between them increases gradually, and the communication with the UE can be completed through the TRP2 only. The UE can report to the gNodeB and recommend single-TRP transmission of the TRP2 (Mode 3) to reduce unnecessary overhead.

Proposal 5: According to the measurement of channel quality by the UE,

- The UE actively reports to TRP, the report explicitly or implicitly indicates TRP handover mode from the following options:

- Mode 1: switch from a TRP to another TRP.

- Mode 2: switch from single-TRP to multi-TRP.

- Mode 3: switch from multi-TRP to single-TRP.

Further details on L1/L2-centric intra/inter-cell mobility

NW initiated L1/L2 reporting of neighboring cell has been agreed. Besides NW initiated L1/L2 reporting of neighboring cell, the UE initiated L1/L2 reporting should also be supported The L1 reporting of neighboring cell can be based on SSB/CSI-RS for mobility or SSB/CSI-RS from the neighboring cell. Further, it should be considered how the UE knows which L1 reporting of the neighboring cell should be reported. There are the following candidate methods.

- NW initialed reporting by RRC: The gNB configures the UE with a list of candidate neighboring cell for an L1 reporting by RRC. The gNB gets the information of neighboring cell(s) according to the location of the UE. The number of the neighboring cell(s) may be large, the UE performs L1 measure for all the candidate neighboring cells to select some good cells. The complexity of UE is too high.. It is inefficient and unnecessary.

- NW initialed reporting by MAC-CE: The gNB configures UE with a list of candidate neighboring cell for L1 reporting by MAC-CE after the gNB receives an L3 mobility reporting which includes a few of selected neighboring cells. It can reduce the overhead of RRC and reduce the power of UE as the UE only need to perform L1 measurement and L1 reporting for a few of selected neighboring cells.

- UE initialed reporting: The UE automatically initiates an L1 reporting for a neighboring cell which is selected by an L3 mobility measurement as shown in Figure 3. The L3 mobility measurement is used to monitor the quality of serving cell and select neighboring cell(s) once the L3 mobility event occurs. The UE can automatically initiate an L1 reporting for a neighboring cell when the condition of L3 mobility measurements event is satisfied. The UE can report the L3 reporting or not report the L3 reporting while the UE reports the L1 reporting for the selected neighboring cells. In addition, since the neighboring cell is selected by the L3 mobility measurement, it can avoid the ping-pang effect of selecting neighboring cell. The UE reports an L1 reporting for a neighboring cell in time and on demand.

Figure 3 UE initiates an L1 reporting for a neighboring cell if an L3 mobility measurement event occurs

For NW initialed L1/L2 reporting for neighboring cell, we prefer to adopt NW initialed L1/L2 reporting by MAC-CE instead of by RRC to reduce the latency and overhead of signaling. Compared with NW initialed L1/L2 reporting, the latency of UE initialed L1/L2 reporting can be smaller because the UE can initiate the L1 reporting for a neighboring cell without waiting for the signaling from gNB.

Proposal 6: UE automatically initiates an L1 reporting of neighboring cell when it detects that an L3 mobility event happens.

Proposal 7: gNB configures neighboring cell information for an L1 reporting using MAC-CE.

When the UE reports a new neighboring cell in a L1 reporting, the serving cell trigger the UE to transmits a RACH to the neighboring cell by a DCI. So the DCI should tell the UE PCI information of the triggered PRACH. As shown in Figure 4, the gNB can configures the UE with N sets of PRACH configuration. Each of the N sets corresponds to one physical cell. The UE can get PRACH resource triggered by a DCI according to PCI indicated in the DCI.

Figure 4  DCI triggering PRACH includes PCI

Proposal 8: The UE can be configured with multiple sets of PRACH configurations, each of which corresponds to one PCI.

Proposal 9: The DCI triggering PRACH should includes PCI, then the UE get the PRACH resources according to a set of PRACH configuration corresponding to the PCI.

In addition, when a serving cell is associated with more than one PCIs, the serving cell will be associated with more than one mobility measurement signal sets configured in MeasObject. The UE will determine whether a L3 mobility event happens based on quality of a serving cell which is an average value of qualities of a mobility RS set, but the serving cell is associated with two mobility RS sets each of which is associated with a PCI now. The PCI of a measurement signal set on which the quality of the serving cell is measured should be clarified.

Proposal 10: When a serving cell is associated with multiple mobility RS sets corresponding to multiple PCIs, the PCI of a measurement signal set on which the quality of the serving cell is got should be clarified.

Further details on TRP-Specific BFR

As discussed in our contribution [3], when the UE reports two q_news each of which is for a TRP, both separate BFR MAC-CE and joint BFR MAC-CE should be supported. In this section, we give more details of the separate BFR MAC-CE and joint BFR MAC-CE.

- A separate BFR MAC-CE includes beam failure information of only one TRP, and beam failure information for different TRPs are in different BFR MAC-CEs as shown in Figure 5. Only one of TRP index, such as CORESET pool index, needs to be included in the separate BFR MAC-CE. If the failed TRP doesn’t fail on the SPCell, the separate BFR MAC-CE can be successfully transmitted to the failed TRP on SPCell, otherwise the separate BFR MAC-CE can be in msg3/msgA. It requires no communication between TRPs, and the failed TRP can be recovered quickly.

- A joint BFR MAC-CE includes beam failure information of more than one TRPs as shown in Figure 6. There are two serving cell indication bitmaps. The two bitmaps corresponds two TRPs, such as two CORESET pool indexes. Joint BFR MAC-CE requires communication between TRPs and is suitable when the failed TRP fails on all serving cells of an MCG/SCG. Further, it requires only one SPCell PRACH if both SPCell TRPs fail.

Figure 5 Separate MAC-CE for TRP-Specific BFR

Figure 6: Joint MAC-CE for TRP-Specific BFR

Proposal 11:  Support separate BFR MAC-CE and joint BFR MAC-CE, wherein only one CORESET pool index is in the separate BFR MAC-CE and two serving cell indication bitmaps each of which corresponds to a CORESET pool index are in a joint BFR MAC-CE. Apply separate BFR MAC-CE or joint BFR MAC-CE according to a received signaling.

Considering that the UE can report L1 measurement for neighboring cell, the beams of the neighboring cell can be also included in a BFR candidate RS set q_1 that comprises two candidate RS subset q11 and q12 for each of TRPs.  Even without multi-TRPs, a q_0 can be associated with two candidate RS sets each of which is associated with a PCI respectively. When beam failure is detected for a serving cell, the UE first selects qnew from a first candidate RS subset q11 with a first PCI, then selects qnew from a second candidate RS subset q12 with PCI 2 if channel qualities of all RSs in q11 are lower than a threshold.

Proposal 12: PCI can be configured for TRP-specific BFR candidate RS set(s).

Conclusion

In this contribution, we discuss some further details and remaining issues of multi-beam and multi-TRP operation. The following observations and proposals are given as follows.

Observation 1:

- Due to sharing a single RRC TCI state pool for a CC group rather than respective pools per CC/BWP can significantly save RRC overhead, e.g., 1.12kB vs 143.36 kB.

- Configuration flexibility of sharing a single RRC TCI state pool for a CC group is sufficient for unified TCI indication, considering the identical RS configuration (e.g., TRS) across multiple CCs under the requirement of updating a common beam for all RS/channels, although losing some flexibility compared with legacy one.

Observation 2: From the evaluation results for Option 1 panel-specific reporting, it can be observed that:

- Increase number of beams per group for Option 1 can bring significantly higher performance gain to combat blockage and UE mobility, especially for cell-edge UE in dense urban, through improving system flexibility with more candidate beams.

- Increase number of groups for Option 1 can be more suitable for advanced UE with 3, 4 or more panels.

Proposal 1: Regarding MAC CE for activating DL/UL TCI states,

- When separate DL/UL TCI state is configured, 3 types of codepoint of TCI states, such as DL, UL, combined DL and UL, should be supported in the MAC CE;

- The TCI state list should be organized in a predefined order of types of codepoint of TCI states in the MAC CE;

- two independent values or one joint-coded value for numbers of two types of codepoint of TCI states should be indicated in the MAC CE.

Proposal 2: Regarding unified TCI state scheme in mTRP,

- M>1, N>1 should be supported for mTRP as a starting point;

- mTRP requirement should be considered in MAC CE design for activating DL/UL TCI states;

- Information of presence of additional TCI state for another TRP for same DL/UL in a codepoint should be supported.

Proposal 3: Regarding the unified TCI state/codepoint applied to the target channel/signal,

- TRP ID information should be tied to a unified TCI state as well as a target channel/signal;

- When indicated, the unified TCI state can be applied to the target channel/signal that shares the same TRP ID information as the unified TCI state.

Proposal 4: Regarding power control framework for unified TCI for mTRP,

- The association between TCI state and power control parameters, e.g. PL-RS, should be maintained by MAC CE.

- If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state.

Proposal 5: According to the measurement of channel quality by the UE,

- The UE actively reports to TRP, the report explicitly or implicitly indicates TRP handover mode from the following options:

- Mode 1: switch from a TRP to another TRP.

- Mode 2: switch from single-TRP to multi-TRP.

- Mode 3: switch from multi-TRP to single-TRP.

Proposal 6: UE automatically initiates an L1 reporting of neighboring cell when it detects that an L3 mobility event happens.

Proposal 7: gNB configures neighboring cell information for an L1 reporting using MAC-CE.

Proposal 8: The UE can be configured with multiple sets of PRACH configurations, each of which corresponds to one PCI;

Proposal 9: The DCI triggering PRACH should includes PCI, then the UE get the PRACH resources according to a set of PRACH configuration corresponding to the PCI.

Proposal 10: When a serving cell is associated with multiple mobility RS sets corresponding to multiple PCIs, the PCI of a measurement signal set on which the quality of the serving cell is got should be clarified.

Proposal 11:  Support separate BFR MAC-CE and joint BFR MAC-CE, wherein only one CORESET pool index is in the separate BFR MAC-CE and two serving cell indication bitmaps each of which corresponds to a CORESET pool index are in a joint BFR MAC-CE. Apply separate BFR MAC-CE or joint BFR MAC-CE according to a received signaling.

Proposal 12: PCI can be configured for TRP-specific BFR candidate RS set(s).

References

R1-2106542, Multi-TRP enhancements for PDCCH, PUCCH and PUSCH, ZTE

R1-2106541, Enhancements on multi-beam operation, ZTE

R1-2106544, Enhancements on beam management for multi-TRP operation, ZTE

Appendix

Table 5 Simulation assumptions for indoor-hotspot system-level evaluation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 80 MHz |
| Subcarrier Spacing | 120 kHz |
| Channel Model | Indoor in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weight for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | Two UE panels are attached with two respective TRP. |
| ISD | 20 m |
| BS Tx power | 23 dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1), (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3 m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5 dBi |
| Noise figure for BS | 7 dB |
| UE receiver noise figure | 10 dB |
| UE distribution | 100% Indoor, 3 km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| Transmission scheme | SU, Rank adaptation |

Notes: Any other parameters not specified here remain the same as those in NR evaluation assumption for beam management in Rel-16 eNR-MIMO.

Table 6 Simulation assumptions for dense urban system-level evaluation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 80 MHz |
| Subcarrier Spacing | 120 kHz |
| Channel Model | Dense Urban Micro layer only in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weight for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | Two UE panels are attached with two respective TRP. |
| ISD | 200 m |
| BS Tx power | 43 dBm |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT,= 0 degree |
| BS antenna pattern | According to TR38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25 m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5 dBi |
| Noise figure for BS | 7 dB |
| UE receiver noise figure | 10 dB |
| UE distribution | 100% Indoor, 60 km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| Transmission scheme | SU, Rank adaptation |

Notes: Any other parameters not specified here remain the same as those in NR evaluation assumption for beam management in Rel-16 eNR-MIMO.