3GPP TSG RAN WG1 Meeting #106-e	R1-2106558

E-meeting, August 16th – August 27th, 2021

Agenda Item:	8.9.1

Source:	Huawei, HiSilicon

Title:	Support of 16QAM for unicast in UL and DL in NB-IoT

Document for:	Discussion and Decision

- Introduction

Based on the agreements achieved so far, this paper discusses DCI design, downlink power allocation and channel quality reporting to support 16-QAM for DL and UL.

- DCI design

In RAN1#105-e, the WA was confirmed that the DCI size is not increased to indicate 16-QAM in uplink and downlink, and for DL 16QAM one reserved state in MCS field is utilized to indicate the use of 16QAM and the repetition field is utilized to indicate the 16QAM MCS indices. The working assumption and the agreement reached are copied below:

Agreement Confirm the working assumption. The DCI size is not increased to support 16-QAM in uplink and downlink. Agreement For the indication of 16-QAM in downlink: - The “Modulation and coding scheme” field in DCI Format N1 is utilized as in legacy for scheduling QPSK. - One reserved state in the “Modulation and coding scheme” field in DCI Format N1 is utilized to indicate the use of 16QAM. - The “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices for 16-QAM in DL when the reserved state in MCS field is indicated. - FFS: The manner of distinguishing the different ranges of TBS indices for “Stand-alone/Guard-band” (i.e., I_TBS indices from 14 to 21) and “In-band” (i.e., I_TBS indices from 11 to 17) deployments. Agreement On the indication of downlink 16-QAM, when the reserved state in MCS field is indicated, the “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices - From 14 to 21 for standalone/guardband deployments, - From 11 to 17 for inband deployment. - FFS: How UE distinguishes the deployment Working Assumption For the indication of 16-QAM in uplink - The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK. - One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM. - The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL.

For the FFS on how UE distinguishes the deployment, it can be the same as legacy, where it is configured by IE operationmodeInfo. For stand-alone or guard-band deployment, 3-bits in the “Repetition number” field are re-interpreted as I_TBS indices spanning from index 14 to 21. For in-band deployment, 3-bits “Repetition number” field are re-interpreted as I_TBS indices spanning from index 11 to 17.

Proposal 1: The deployment is indicated by the legacy signaling operationmodeInfo, i.e., no enhancement is needed on distinguishing the deployment.

The DL DCI indication method for 16QAM agreed in last meeting can allow eNB to schedule any Rel-16 QPSK TBS and repetition numbers for the UE, which is beneficial for the performance when channel condition becomes worse. Thus the UL 16QAM indication can follow this method and the working assumption for UL 16QAM indication should be confirmed.

Proposal 2: Confirm the working assumption for the indication of 16-QAM in uplink.

Working Assumption

For the indication of 16-QAM in uplink

- The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM.

- The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL.

Furthermore, since UL PUR is supported since Rel-16 UE and the “1110” state in the MCS field for DCI format N0 scrambled by PUR-RNTI has already been utilized to indicate ACK or fallback for PUR transmission, thus the reserved state “1111” in the MCS field should be utilized for the indication of 16QAM in DCI format N0. To align the DL and UL 16QAM indication, the reserved state to indicate the use of 16QAM should be “1111”.

Proposal 3: The reserved state to indicate the use of 16QAM in DCI format N0 and DCI format N1 should be “1111”.

- Channel quality reporting

In RAN1#104bis-e, it has been agreed that the channel quality report for 16-QAM is based on NPDSCH transport block with the target error probability BLER≤10%. The agreement reached is copied below:

Agreement When configured with downlink 16-QAM, the channel quality can be reported in MAC CE. - FFS on support in Msg3 in connected mode Agreement For CQI table for downlink 16-QAM, down-select between following options in RAN1#106-e: - Option 1: More than three candidate values for 16-QAM are added in the legacy table. - FFS: Which of the legacy entries are removed - Option 2: Three candidate values for 16-QAM are added in the legacy table. - Option 3: A new CQI table is defined for 16-QAM based on the eMTC table (CQI Tables in 36.213) as a starting point

In connected mode, the eNB has already been able to trigger the channel quality report by MAC CE. There is no benefit or necessity to support CQI reporting in Msg3 in connected mode.

Proposal 4: The CQI reporting in Msg3 in connected mode should not be supported.

From performance perspective, the SNR corresponding to the CQI table should be evenly spaced, which can efficiently indicate the current channel quality. The SNR for selected entries from legacy CQI table and 16QAM MCS is evaluated, according to simulation assumptions listed in Table 3 in Appendix. The target BLER is 1% for the legacy NPDCCH CQI reporting and the target BLER is 10% for the NPDSCH CQI reporting. The simulation results are summarized in Table 1..

Table 1 The simulation results of NPDCCH and NPDSCH

|  | SNR | Target BLER |
| --- | --- | --- |
| NPDCCH Repetition 1 | -0.6dB | 1% |
| NPDSCH TBS index 14 with 16QAM | 9.2dB | 10% |

It can be observed that the SNR gap between the NPDCCH repetition 1 and 16QAM MCS is very large (>>3dB), and the entries for QPSK CQI reporting and 16QAM CQI reporting can not be evenly distributed. Thus for Option 2, utilizing the 3 reserved states in the legacy CQI table may not be efficient for the CQI reporting for DL 16QAM. Consequently, more than 3 candidate values for 16QAM are required for the 16QAM CQI table.

For option 1, UE has to calculate the CQI assuming two different channels, i.e., NPDCCH and NPDSCH, which increases UE complexity.

Therefore, option 3 is preferred in terms of performance and UE complexity.

For option 3, the eMTC CQI table can be used as a start point. Then, the newly defined CQI table can be the following table as an example. For MAC CE feedback, one bit can be used to indicate whether the legacy CQI table or the new CQI table is used.

Table 2 An example 16QAM CQI table of NB-IoT

| CQI index | modulation | TBS index |
| --- | --- | --- |
| 0 | QPSK | 0 |
| 1 | QPSK | 2 |
| 2 | QPSK | 4 |
| 3 | QPSK | 6 |
| 4 | QPSK | 8 |
| 5 | QPSK | 10 |
| 6 | QPSK | 12 |
| 8 | 16QAM | 14 |
| 9 | 16QAM | 16 |
| 10 | 16QAM | 18 |
| 11 | 16QAM | 20 |
| 12 | Reserved | Reserved |
| 13 | Reserved | Reserved |
| 14 | Reserved | Reserved |
| 15 | Reserved | Reserved |

Proposal 5: Option 3 should be supported for CQI table for downlink 16-QAM CQI reporting.

- 4. DL power allocation

In RAN1#105-e, it has been agreed that the NPDSCH EPRE in symbols with NRS can be different or same with the NPDSCH EPRE in symbols without CRS and NRS. The agreement reached is copied below:

Working Assumption For downlink power allocation to support 16QAM: - For standalone and guard-band deployments: - One power ratio is signaled optionally - NPDSCH EPRE to NRS EPRE in symbols without NRS - The same transmit power is assumed across different symbols. - If the signaling is not indicated, the legacy power allocation is used. - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports - UE specific signaling is used Agreement For downlink power allocation to support 16QAM: - For inband deployments, a power ratio is signaled in addition to the signaling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”. Down-select from the following options in RAN1#106-e: - Alt 1: - The existing parameter nrs-CRS-PowerOffset is reused for same PCI case, and is signaled for different PCI case. - The same transmit power of different symbols is assumed as to derive from nrs-CRS-PowerOffset the NPDSCH EPRE in symbols with CRS. - Alt 2: - the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signaled - FFS: NPDSCH EPRE to NRS EPRE in symbols with NRS FFS: Whether UE specific or cell-specific or carrier-specific signaling is used

For standalone deployment or guard-band deployment, the working assumption in RAN1#105-e should be confirmed. The NPDSCH EPRE to NRS EPRE in symbols without NRS can be signalled optionally. If the signalling is not indicated, the ratio of NPDSCH EPRE to NRS EPRE before Rel-17 can be followed. If the signalling is indicated, considering the constant transmit power, the NPDSCH EPRE to NRS EPRE in symbols with NRS can be obtained implicitly.

Proposal 6: The working assumption for DL power allocation to support 16QAM for guard-band and standalone deployments should be confirmed.

Working Assumption

For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - One power ratio is signaled optionally

    - NPDSCH EPRE to NRS EPRE in symbols without NRS

  - The same transmit power is assumed across different symbols.

  - If the signaling is not indicated, the legacy power allocation is used.

    - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports

- UE specific signaling is used

For inband mode, with Alt-1, the existing NRS to CRS power offset parameter can only be reused for the same PCI and a new signaling should be needed for different PCI case. Even though the new parameter can be signaled, the same transmit power of different symbols should be assumed to obtain the NPDSCH EPRE to NRS EPRE in symbols with CRS. For in-band deployment, if there is power boosting for CRS, the constant transmit power limitation is restrictive, which has impact on eNB scheduling flexibility for in-band deployment mode.

For Alt-2, if the NPDSCH EPRE in symbols with NRS is the same with the NPDSCH EPRE in symbols without NRS, only the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS and NPDSCH EPRE to NRS EPRE in symbols with NRS should be additionally signaled explicitly. For in-band deployment mode, Alt-2 should be supported.

Proposal 7: Support Alt-2 for in-band deployments in the DL power allocation signaling.

- Others

In RAN1#105-e, the agreement has been reached that 16QAM is supported for NPUSCH in PUR procedure and FFS on support of 16-QAM for NPDSCH in PUR procedure, as copied below.

Working Assumption Support 16-QAM for NPUSCH in PUR procedure. - FFS on support of 16-QAM for NPDSCH in PUR procedure.

In PUR, the UL resource is preconfigured in connected mode before RRC release. And the UE can inform gNB about the capability of UL 16-QAM and DL 16QAM. Thus 16QAM can be supported for NPDSCH in PUR procedure without any extra implementation from UE perspective if the UE already implements DL 16QAM. Furthermore, even if the channel condition becomes worse, the agreed DCI indication method can still schedule the NPDSCH with QPSK modulation. Thus, it is technically possible to support 16-QAM for NPDSCH in PUR procedure.

Proposal 8: Support 16-QAM for NPDSCH in PUR procedure.

In RAN1#105-e, the agreement has been reached that introducing a new term in uplink power control of NPUSCH using 16-QAM and FFS details, as copied below.

Agreement Introduce a new term in uplink power control of NPUSCH using 16-QAM. FFS on the details.

One remaining issue is the uplink power control for 16-QAM. The LTE uplink power control is shown below.

The term of uplink power control of NPUSCH using 16QAM should follow LTE principle. In LTE, the factor ∆TF,c is introduced to quickly adjust the transmit power spectral density of the UE according to the indicated MCS by the eNB. As shown in TS 36.213 5.1.1.1, if the UE transmits PUSCH without a simultaneous PUCCH for the serving cell c, then the UE transmit power PPUSCH,ci for PUSCH transmission in subframe/slot/subslot i for the serving cell c is given by [dBm] where ∆TF,ci=10log102BPRE∙Ks-1∙βoffsetPUSCH for Ks=1.25 and 0 for Ks=0 where Ks is given by the parameter deltaMCS-Enabled provided by higher layers for each serving cell c. BPRE and βoffsetPUSCH for each serving cell c, are computed as below. Ks=0 for transmission mode 2. BPRE= OCQINRE for control data sent via subframe-PUSCH without UL-SCH data or slot/sublot-PUSCH without UL-SCH data if the UE is configured with a higher layer parameter uplinkPower-CSIPayload, - BPRE= OCQI'NRE and OCQI'=maxOCQI,RI with OCQI,RI defined as the number of CQI/PMI bits including CRC for a given RI value for slot/subslot-PUSCH without UL-SCH data if the UE is not configured with a higher layer parameter uplinkPower-CSIPayload, and BPRE= r=0C-1KrNRE for other cases. - where C is the number of code blocks, Kr is the size for code block r, OCQI is the number of CQI/PMI bits including CRC bits and NRE is the number of resource elements determined as NRE= MscPUSCH-initial∙NsymbPUSCH-intial. - βoffsetPUSCH= βoffsetCQI for control data sent via PUSCH without UL-SCH data and 1 for other cases.

For NB-IoT, the same term of ∆TF,c in LTE can be reused. For the calculation of ∆TF,c, Ks=1.25 and 0 for Ks=0 where Ks is given by the parameter deltaMCS-Enabled provided by higher layers for each serving cell c. Since there is no control data sent via subframe-PUSCH, thus BPRE= r=0C-1KrNRE, where C is the number of code blocks, Kr is the size for code block r, and NRE is the number of resource elements determined as NRE= MscPUSCH-initial∙NsymbPUSCH-intial.

Thus, the following proposal of NB-IoT uplink power control is given.

Proposal 9：A new term ∆TF,c is introduce to support 16QAM in NPUSCH, which reuses the LTE term.

- The uplink power control factor ∆TF,c is calculated as ∆TF,ci=10log102BPRE∙Ks-1

- Conclusion

In this contribution, our views on support of 16-QAM for unicast in UL and DL in NB-IoT are provided. The following proposals are made.

Proposal 1: The deployment is indicated by the legacy signaling operationmodeInfo, i.e., no enhancement is needed on distinguishing the deployment.

Proposal 2: Confirm the working assumption for the indication of 16-QAM in uplink.

Working Assumption

For the indication of 16-QAM in uplink

- The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM.

The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL.

Proposal 3: The reserved state to indicate the use of 16QAM in DCI format N0 and DCI format N1 should be “1111”.

Proposal 4: The CQI reporting in Msg3 in connected mode should not be supported.

Proposal 5: Option 3 should be supported for CQI table for downlink 16-QAM CQI reporting.

Proposal 6: The working assumption for DL power allocation to support 16QAM for guard-band and standalone deployments should be confirmed.

Working Assumption

For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - One power ratio is signaled optionally

    - NPDSCH EPRE to NRS EPRE in symbols without NRS

  - The same transmit power is assumed across different symbols.

  - If the signaling is not indicated, the legacy power allocation is used.

    - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports

- UE specific signaling is used

Proposal 7: Support Alt-2 for in-band deployments in the DL power allocation signaling.

Proposal 8: Support 16-QAM for NPDSCH in PUR procedure.

Proposal 9：A new term ∆TF,c is introduce to support 16QAM in NPUSCH, which reuses the LTE term.

- The uplink power control factor ∆TF,c is calculated as ∆TF,ci=10log102BPRE∙Ks-1

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.

R1-2008697, “Discussion on UL and DL 16-QAM for NB-IoT”, ZTE, RAN1#103-e, October 26th-November 13th, 2020.

- Appendix

Table 3 The simulation assumptions of NPDCCH repetition 1

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T, 1R |
| Channel model | AWGN |
| Frequency Resource | 12-tone |
| Number of repetitions | 1 |
| Modulation Order | QPSK, 16-QAM |
| Noise Estimation | Ideal |
| Channel Estimation | Realistic |
| DCI Size | 23 bits |
| NPDCCH Target BLER | 1% |
| NPDSCH Target BLER | 10% |
| Deployment Mode | Standalone/Guard-band |