3GPP TSG-RAN WG1 Meeting #106-e	R1-2106568

e-Meeting, 16th – 27th August 2021

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Potential RedCap solutions for avoiding or minimizing PUSCH resource fragmentation

Document for:	Discussion, Decision

# 1	Introduction

In [1], the potential problem of PUSCH resource fragmentation is discussed. Many MNOs support the view that if the support for RedCap devices is enabled in a cell, there needs to be a configuration option that allows the cell to avoid PUSCH resource fragmentation. The issue of PUSCH resource fragmentation is captured in the RAN1#105e agreement and working assumption as follows [1]:

Agreements: - Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed. - Working assumption: Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - FFS: whether/how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmission for the above case - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD. - FFS whether or not to additionally support the case when the centre frequency is different; if so, how to minimize centre frequency retuning

One important issue for further study is whether and how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmissions when a separate initial UL BWP is configured or defined for RedCap UEs. To address the aforementioned PUSCH resource fragmentation issue, there are a number of relevant open issues that need to be discussed jointly. One of these open issues is highlighted in the working assumption above. Additional relevant open issues are highlighted in the additional agreement and working assumptions below:

Working assumption: - At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access - FFS the details of the configuration/definition - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB. - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s) - whether part of the configuration can be defined instead of signaled - If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment). - FFS during the initial access - FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring - FFS: supported bandwidths in the separate initial DL BWP - FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs - FFS: FDD case Agreements: Take the following as an agreement, revised from the RAN1#104bis-e working assumption: - A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability. - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs. Working assumption: - For enabling/supporting that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth). - FFS: whether/how the specification also supports separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation of the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping)

In this contribution, we discuss how the solutions for these open issues can be used to avoid or minimize the PUSCH resource fragmentation. We use FR1 scenarios as examples.

# 2	Peak user data rate is a KPI adopted by many MNOs

Peak user data rate is an important key performance indicator (KPI) that most mobile network operators (MNOs) adopt for evaluating a technology or equipment. When a new feature is introduced to a network, a key consideration is whether and how the important KPIs are impacted. Such consideration is aligned with the RedCap work item objective for ensuring good coexistence performance between RedCap and non-RedCap UEs [3]:

- Coexistence with non-RedCap UEs is to be ensured.

In our view, the RedCap work item should ensure that the RedCap introduction does not impact the MNO KPIs such as UL peak data rate of eMBB UEs.

UL peak data rate of eMBB UEs is an important KPI when an MNO evaluates the potential introduction of a new technology or equipment. The RedCap work item should ensure that the RedCap introduction does not considerably impact UL peak data rate of eMBB UEs.

One important coexistence issue that has impact on the UL peak data rate of eMBB UEs is PUSCH resource fragmentation caused by PUCCH transmission, as mentioned in our companion contribution [4]. PUSCH resource fragmentation can result in a significant UL peak data rate reduction for non-RedCap UEs not supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) or resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) in the uplink. Note that both these features are optional with capability signaling.

In Section 3, we first illustrate how the existing Rel-15/16 bandwidth part (BWP)  framework currently allows a network configuration to avoid PUSCH resource fragmentation and ensure excellent UL peak user data rate KPI performance while supporting the adaptation of BWP configurations for UE power saving purposes. Then, in Section 4, we illustrate a potential problem due to the introduction of a separate initial UL BWP for supporting RedCap UEs which is currently a working assumption in the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum bandwidth of a RedCap UE [1]. Finally, in Section 5, we present several potential solutions, within the framework of existing RAN1 agreements, for avoiding or minimizing PUSCH resource fragmentation in order to support both non-RedCap and RedCap UEs in the same cell and ensure good coexistence performance.

# 3	Introducing Rel-15 based BWP adaptation does not compromise the UL peak user data rate KPI

UE power consumption has a dependency on the BWP bandwidth. For example, according to the power consumption model in [5], a scaling factor is used to capture such a dependency. For DL in FR1, the power consumption scaling factor Y, for a DL BWP bandwidth of X [MHz], is Y = 0.4 + 0.6 * (X - 20) / 80. Based on this model, the scaling factors for 20 MHz and 100 MHz BWPs are 0.4 and 1.0, respectively. In other words, by switching a UE from a 100 MHz DL BWP to a 20 MHz DL BWP, a 60% power saving can be achieved when all the other parameters are kept the same. For the UL, power consumption is not affected by the bandwidth of UL BWP according to the model in [5]. This makes sense considering that when the UE transmits in the UL, the power consumption is dominated by the power amplifier.

To help reduce UE power consumption, 5G New Radio (NR) introduced the BWP framework from the start in Release 15 for supporting UE bandwidth adaptation. To this end, a UE may be configured with multiple BWPs, but it operates in one active BWP at a given time. If the UE has a large amount of data to transmit or receive, the UE operates best in an active BWP configured with a wide bandwidth. On the other hand, if the UE does not have much traffic in RRC_CONNECTED, or when the UE is in RRC_INACTIVE or RRC_IDLE, the UE can operate in a BWP configured with a small bandwidth to be more energy efficient.

Many of the first deployed 5G NR networks do not adopt the BWP adaptation framework. Rather, they use BWP#0 configuration option 2 as described in TR38.331, Appendix B2, to configure only one BWP that spans the entire carrier bandwidth. However, it is foreseen that most of these networks will evolve toward supporting BWP adaptation using BWP#0 configuration option 1 as described in TR38.331, Appendix B2. In the discussion of adopting the BWP adaptation framework, an important issue is whether introducing BWP adaptation might have an impact on the important KPIs such as the UL peak user data rate. It can be shown that it is possible to support BWP adaptation between multiple BWP configurations for UE power saving benefits while avoiding any reduction in the UL peak user data rate KPI. An example is illustrated in Figure 1, where it is assumed that the carrier bandwidth is 100 MHz. This example solution achieves the following.

- A UE can be configured with a narrow initial DL BWP of 20 MHz and a wider non-initial DL BWP of 100 MHz. UE power saving can be achieved by switching the UE to the narrow initial DL BWP if the UE does not have much traffic in RRC_CONNECTED, or when the UE is in RRC_INACTIVE or RRC_IDLE.

- A contiguous PUSCH resource is preserved even when there is PUCCH transmission in any of the configured UL BWPs. This helps achieve the highest UL peak user data rate KPI.

Figure 1: An example of BWP adaptation for UE power saving while avoiding any impact on the UL peak user data rate KPI (100 MHz carrier bandwidth in FR1 is assumed here).

In Rel-15/16 networks, BWP adaptation between a narrow initial DL BWP and a wider non-initial DL BWP can be enabled to help achieve UE power saving without any impact on the UL peak user data rate KPI.

# 4	Potential impact on the UL peak user data rate KPI due to RedCap support

To support RedCap UEs, which have a maximum UE bandwidth of 20 MHz in FR1, the working assumption in RAN1 is to limit the initial UL BWP to 20 MHz.

An example of possible BWP configurations is illustrated in Figure 2. According to this example:

- The initial DL BWPs (BWP#0) is configured to 20 MHz and can be shared by non-RedCap and RedCap UEs.

- The initial UL BWP for non-RedCap UE is configured to the carrier bandwidth (a 100 MHz TDD carrier in this example). This is advantageous for avoiding PUSCH resource fragmentation caused by PUCCH frequency hopping (FH) during initial access of a non-RedCap UE.

- Since the initial UL BWP for non-RedCap UEs is configured to have a bandwidth wider than the maximum RedCap UE bandwidth, a separate initial UL BWP of 20 MHz is configured for RedCap UEs. Since this example is based on a TDD carrier, this separate initial UL BWP is in this example assumed to have the same center frequency as the initial DL BWP, and therefore is also configured to position at the middle of the carrier.

- Non-initial DL BWPs (e.g., BWP#1) are separately configured for non-RedCap and RedCap UEs according to their different bandwidth capabilities.

- Non-initial UL BWP (e.g., BWP#1) is also configured for RedCap UEs. It is advantageous to place this BWP at the carrier edge and disable PUCCH FH to avoid PUSCH resource fragmentation caused by the UL traffic from a RedCap UE.

As illustrated in Figure 2, one drawback with this configuration is that the UL resources are fragmented into 3 contiguous segments with each contiguous segment limited to 40 MHz or less when a RedCap UE transmits PUCCH in the initial UL BWP during initial access. Note that a Rel-15/16 UE cannot be expected to support non-contiguous PUSCH resource allocation as the features of supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) and supporting resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) are optional with capability signaling. This means that whenever there is a PUCCH transmission from a RedCap UE during the initial access in the initial UL BWP, the uplink peak data rate of a non-RedCap UE is reduced by up to 60% (see [4]). The fragmented PUSCH resources may be left unused if the gNB scheduler cannot schedule another UE due to timing constraint or other reasons. This, in turn, results in an under-utilization of UL resources.

Figure 2: An example of PUSCH resource fragmentation. (a 100 MHz TDD carrier in FR1 is assumed.)

One may argue that one solution to mitigate the problem illustrated in Figure 2 is to shift the initial BWPs to the carrier edge and disable the PUCCH frequency hopping in the non-initial UL BWP for RedCap, as illustrated in . However, even with such an improved configuration, since according to the current specifications the PUCCH FH is always enabled during the initial access (Msg4HARQ ACK), the overall PUSCH resource fragmentation resulting from the PUCCH transmissions from RedCap UEs may still be very significant in some scenarios. One example is illustrated in , which shows that there could be a 50% reduction in UL peak data rate on a 40 MHz carrier. Clearly, without any enhancement, the UL peak data rate reduction due to the PUSCH resource fragmentation can be significant in most cases. A thorough study on UL peak data rate reduction in all possible scenarios is presented in our companion paper in [4].

According to the experiences observed in deployed LTE networks, the random access procedure can generate significant traffic. We expect this will also be the case for deployed NR networks. Thus, PUCCH transmissions during random access procedures cannot be assumed to be infrequent events in a cell. Thus, it is of great importance that RAN1 addresses the issue of PUSCH resource fragmentation caused by PUCCH transmissions.

Figure 3: An example of PUSCH resource fragmentation (a 40 MHz TDD carrier in FR1 is assumed).

A Rel-15/16 UE cannot be expected to support non-contiguous PUSCH resource allocation as the features of supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) and supporting resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) are optional with capability signaling.

Without enhancing the existing BWP or PUCCH solutions, PUSCH resource fragmentation due to PUCCH transmissions from RedCap UEs may result in a significant reduction in UL peak user data rate KPI.

Without enhancing the existing BWP or PUCCH solutions, PUSCH resource fragmentation due to PUCCH transmissions from RedCap UEs may also result in under-utilization of UL resources.

PUCCH transmissions during random access procedures for Msg4 acknowledgement can be frequent events in a cell. Thus, it is of great importance that RAN1 addresses the issue of PUSCH resource fragmentation caused by PUCCH transmissions.

The introduction of RedCap support needs to allow the network to have an option of configuring the UL and DL bandwidth parts to avoid reduction in the UL peak user data rate KPI.

# 5	Enhancements needed for minimizing impacts on the UL peak user data rate KPI

In this section, we illustrate how the impact on UL peak user data rate KPI can be minimized when the support for RedCap UEs is enabled in the network. The relevant aspects are already captured in the RAN1 agreements (as highlighted in Section 1) as open issues for further studies. These aspects are:

- (A) Whether or not to additionally support the case when the centre frequency is different between paired DL and UL BWPs in TDD bands; if so, how to minimize centre frequency retuning.

- (B) When a separate initial DL BWP for RedCap UEs is configured/defined, whether this separate initial DL BWP for RedCap UEs can be used during initial access.

- (C) Whether a RedCap UE should support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

- (D) Whether PUCCH frequency hopping for Msg4 HARQ acknowledgement can be disabled.

These aspects can be respectively mapped to potential Rel-17 features as follows.

- (Feature A) Additionally support the case where the centre frequency is different between paired initial DL and UL BWPs in TDD.

- (Feature B) When a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used during initial access.

- (Feature C) A RedCap UE should support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

- (Feature D) PUCCH frequency hopping for Msg4 HARQ acknowledgement can be disabled.

When the Features A, C, and D above are supported, PUSCH resource fragmentation can be avoided, and in that case introducing RedCap support in the cell will not result in any degradation in the UL peak user data rate KPI. This is illustrated in Figure 4. First, a separate initial UL BWP is configured for RedCap UEs and can be flexibly placed at the cell edge. This results in different center frequencies between the initial DL and UL BWPs (Feature A). In our view, the required switching time between DL and UL is very relaxed during initial access. Thus, frequency retuning between DL and UL center frequencies is not expected to be an issue as far as the UE implementation is concerned. Furthermore, frequency retuning between DL and UL center frequencies is anyway needed for half-duplex FDD UEs. It is expected that the same RedCap UE may support operation in both FDD and TDD bands. Therefore, the same RedCap UE would have the capability of frequency retuning between DL and UL center frequencies, regardless whether it is an HD-FDD or TDD operation. We conclude that frequency retuning between DL and UL center frequencies during initial access within a TDD band is not expected to impose new challenges from the UE implementation point of view.

Then according to Figure 4, PUCCH frequency hopping is disabled for RedCap UE in the initial UL BWP during initial access (Feature D). Additionally, non-initial BWPs need to be configured for RedCap UEs. In our view, there might be more incentives to align the center frequencies between non-initial DL and UL BWPs, as switching between DL and UL occurs more frequently and it might be also advantageous to avoid an extended gap between DL and UL transmissions during RRC_CONNECTED. However, to avoid PUCCH transmissions in the non-initial UL BWP to cause PUSCH resource fragmentation, the UL BWP needs to be placed at the carrier edge. Then as a result of center frequency alignment, the non-initial DL BWP would also need to be placed at the carrier edge. To facilitate the possibility of placing a non-initial DL BWP for RedCap UEs at the carrier edge, it would require a RedCap UE to support FG 6-1a (Feature C). For example, if the DL BWP is placed at the carrier edge, it may not necessarily contain both SSB and CORESET #0.

Considering that the required switching time between DL and UL is very relaxed during initial access, frequency retuning between DL and UL center frequencies during initial access is not expected to be an issue as far as the UE implementation is concerned.

It is expected that the same RedCap UE would support operation in both FDD and TDD bands. Therefore, the same RedCap UE would have the capability of frequency retuning between DL and UL center frequencies, regardless whether it is a HD-FDD or TDD operation.

Frequency retuning between DL and UL center frequencies during initial access within a TDD band is not expected to impose more challenges from the UE implementation point of view than in the half-duplex FDD case.

Figure 4: An example solution for avoiding any reduction in UL peak data rate KPI. This is achieved by allowing different center frequencies in the initial DL and UL BWPs within a TDD carrier (Feature A), allowing the PUCCH frequency hopping to be disabled for RedCap UE during initial access (Feature D), and requiring RedCap UEs to support FG 6-1a (Feature C).

Another possible solution is illustrated in Figure 5. Unlike the solution illustrated in Figure 4, this solution maintains the same center frequency of the initial DL and UL BWPs, but it requires a separate initial DL BWP to be configured for RedCap UEs (Feature B). For RedCap UEs, the locationAndBandwidth of the separate (SIB-configured) initial DL BWP should be available to use during initial access, unlike for legacy UEs where the locationAndBandwidth of the SIB-configured initial DL BWP is used only after initial access. Like the previous solution, to avoid PUSCH resource fragmentation, the network needs to be able to disable PUCCH frequency hopping for RedCap UE during initial access (Feature D) and the RedCap UEs need to support of FG 6-1a (Feature C).

Figure 5: An example solution for avoiding any reduction in UL peak data rate KPI. This is achieved by allowing a separate initial DL BWP configuration for the use during initial access (Feature B), allowing the PUCCH frequency hopping to be disabled for RedCap UE during initial access (Feature D), and requiring RedCap UEs to support FG 6-1a (Feature C).

Based on the discussion, we make the proposal below.

To address the PUSCH resource fragmentation issues and to avoid any reduction in UL peak data rate KPI, down select between the two options below.

- Option 1:

- Support initial DL and UL BWPs with different center frequencies in TDD. To minimize center frequency retuning, the center frequencies of the non-initial DL and UL BWPs are still required to be the same.

- Require RedCap UEs to support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

- Support PUCCH frequency hopping for Msg4 HARQ acknowledgement to be disabled.

- Option 2:

- A separate initial DL BWP for RedCap UEs can be configured/defined and used during initial access.

- Require RedCap UEs to support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

- Support PUCCH frequency hopping for Msg4 HARQ acknowledgement to be disabled.

# 6	Conclusion

To ensure a commercial success of the RedCap features, it is necessary to ensure good coexistence with non-RedCap UEs. Such an objective is also clearly stated in the RedCap WID [1]. One important coexistence aspect is the potential impact on UL peak user data rate KPI. In this contribution, we illustrate how PUSCH resource fragmentation may occur when support for RedCap UEs is introduced. We discuss potential solutions for avoiding such a problem in order to avoid impact on the UL peak user data rate KPI. Other solutions can also be considered.

In the previous sections we made the following observations:

Observation 1	UL peak data rate of eMBB UEs is an important KPI when an MNO evaluates the potential introduction of a new technology or equipment. The RedCap work item should ensure that the RedCap introduction does not considerably impact UL peak data rate of eMBB UEs.

Observation 2	In Rel-15/16 networks, BWP adaptation between a narrow initial DL BWP and a wider non-initial DL BWP can be enabled to help achieve UE power saving without any impact on the UL peak user data rate KPI.

Observation 3	A Rel-15/16 UE cannot be expected to support non-contiguous PUSCH resource allocation as the features of supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) and supporting resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) are optional with capability signaling.

Observation 4	Without enhancing the existing BWP or PUCCH solutions, PUSCH resource fragmentation due to PUCCH transmissions from RedCap UEs may result in a significant reduction in UL peak user data rate KPI.

Observation 5	Without enhancing the existing BWP or PUCCH solutions, PUSCH resource fragmentation due to PUCCH transmissions from RedCap UEs may also result in under-utilization of UL resources.

Observation 6	PUCCH transmissions during random access procedures for Msg4 acknowledgement can be frequent events in a cell. Thus, it is of great importance that RAN1 addresses the issue of PUSCH resource fragmentation caused by PUCCH transmissions.

Observation 7	Considering that the required switching time between DL and UL is very relaxed during initial access, frequency retuning between DL and UL center frequencies during initial access is not expected to be an issue as far as the UE implementation is concerned.

Observation 8	It is expected that the same RedCap UE would support operation in both FDD and TDD bands. Therefore, the same RedCap UE would have the capability of frequency retuning between DL and UL center frequencies, regardless whether it is a HD-FDD or TDD operation.

Observation 9	Frequency retuning between DL and UL center frequencies during initial access within a TDD band is not expected to impose more challenges from the UE implementation point of view than in the half-duplex FDD case.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The introduction of RedCap support needs to allow the network to have an option of configuring the UL and DL bandwidth parts to avoid reduction in the UL peak user data rate KPI.

Proposal 2	To address the PUSCH resource fragmentation issues and to avoid any reduction in UL peak data rate KPI, down select between the two options below.

Option 1:

	Support initial DL and UL BWPs with different center frequencies in TDD. To minimize center frequency retuning, the center frequencies of the non-initial DL and UL BWPs are still required to be the same.

	Require RedCap UEs to support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

	Support PUCCH frequency hopping for Msg4 HARQ acknowledgement to be disabled.

Option 2:

	A separate initial DL BWP for RedCap UEs can be configured/defined and used during initial access.

	Require RedCap UEs to support FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822).

	Support PUCCH frequency hopping for Msg4 HARQ acknowledgement to be disabled.

# References

R1-2104184, “Ensuring coexistence between RedCap and non-RedCap UEs”, Ericsson, Deutsche Telekom, NTT DOCOMO, Softbank, Telecom Italia, Telstra, Verizon Wireless, Vodafone, 3GPP TSG RAN1 #105e, May 2021.

R1-2106213, “RAN1 agreements for Rel-17 NR RedCap”, Rapporteur (Ericsson), 3GPP TSG RAN1 #105-e, May 2021.

RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN #92e, June 2021.

R1-2106563, “Reduced maximum UE bandwidth for RedCap”, Ericsson, 3GPP TSG RAN1 #106e, Aug. 2021.

TR 38.840 V16.0.0, “Study on User Equipment (UE) power saving in NR (Release 16)”, June 2021.