3GPP TSG RAN WG1 #106-e		R1-2106592

e-Meeting, August 16th – August 27th, 2021

Source:	vivo

Title:	Discussion on UL time and frequency synchronization enhancements for NR-NTN

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #104 bis and #105 e-Meeting [1], the following agreements regarding the uplink time and frequency synchronization were made：

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

In this contribution, we further discuss on common TA drift parameters, time alignment timers, and common frequency compensation offset.

- Indication of common TA drift parameters

According to the agreement listed above, there are three relevant issues remained to be addressed, which are feeder link timing handling, common TA drift parameters and common TA calculation formula, respectively.

For feeder link timing handling, one approach is to perform the feeder link timing compensation at UE, the other is to perform the feeder link timing compensation at network. In our point of view, feeder link timing compensation at UE is more feasible. UE receives the broadcast information including common TA indication and update the common TA value correspondingly. While for feeder link timing compensation at network, gNB needs to deal with a time-varying feeder link delay and compensates it to avoid misalignment between DL and UL signals, which might lead to implementation complexity.

Proposal 1: Support feeder link time compensation at UE.

In the previous RAN1 e-meeting, there was no consensus on the parameters of indicating the derivation of common TA. In our view, whether high-order derivatives of common delay drift rate are needed depends on the requirement of UL time synchronization and can be determined based on simulation. If supporting high-order derivatives, when and how to apply delay drift second or third order derivative would be challenging, and signalling overhead and UE complexity will be increased.

However, the main concern is how to use these common TA parameters in some situations. For example, when the satellite switch or the handover occurs, whether these common TA parameters are valid or not and how to re-calculate common TA value. When a satellite switches beam, whether these common TA parameters can be applicable depends on that these parameters are beam-level or cell-level.

Observation 1: Further discussion on how to use common TA parameters is needed.

Last mentioned issue is the formulas to be used for UE-specific TA and common TA. The UE-specific TA is UE self-estimated TA to pre-compensate for the service link delay between the satellite and UE. UE can calculate it based on the obtained satellite ephemeris and GNSS location before UL transmission. Regarding common TA, it can be discussed after common TA drift parameters are determined.

Proposal 2: Support common TA calculation discussion after common TA drift parameters are determined.

- NTN UE Validity Timers

Validity timers for open loop TA update was discussed in the last RAN1 e-meeting, and moderator’s proposal was summarized as follows：

From RAN1 perspective validity timers for satellite ephemeris and common TA are needed to maintain UL

synchronization in NTN:

- A validity timer configured by the network for satellite ephemeris defines the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris.

- A validity timer configured by the network for Common TA defines the maximum time during which the UE can apply the Common TA without having acquired new Common TA parameters to be used for Common TA calculation.

FFS: Whether only one validity timer is configured for both Common TA and satellite ephemeris information.

In our opinion, if satellite ephemeris or common TA are not updated for a long time, the estimated error on UE-specific TA and common TA may exceed the maximum tolerable error, and it may lead to UL asynchronization. Hence, a validity timer for satellite ephemeris or common TA is needed. In our view, if satellite ephemeris information is updated, common TA parameters needs to be re-calculated and informed to UE. Then common TA is updated based on latest common TA parameters. Satellite ephemeris and common TA are closely related. Hence, only one timer is needed. This timer can be used for both common TA and satellite ephemeris. Once this timer expires, UE shall assume UL asynchronization and update satellite ephemeris and common TA simultaneously. Thus, there is no need to introduce separate timers for common TA and satellite ephemeris information.

Proposal 3: Support one validity timer configured for both Common TA and satellite ephemeris information.

- Indication of common frequency compensation offset

On common DL frequency compensation for the service link doppler, the following recommendations were proposed in previous RAN1 meeting.

Is support of common DL frequency compensation for the service link doppler beneficial and needed?

WF1: Similar to Earth moving cell, the amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE.

WF2: Indicate the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB.

WF3: Deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

First of all, considering the movement in NTN scenarios, especially for LEO, DL frequency compensation for the service link doppler should be supported. Without this feature, UE would deal with a very large frequency offset in DL, which would increase the complexity of DL synchronization and potentially delay the initial access procedure. On WF1, it is acceptable for earth-moving cells, which can be discussed together with UL common compensation. The detail of indicating parameters require further discussion, including the value, usage, and signaling overhead. On WF2, it is more suitable for earth-fixed cells, and UE can self-calculate DL pre-compensation based on ECEF co-ordinates. While the remaining issue is the indication of co-ordinates regarding security aspect.

Proposal 4: Support common DL frequency compensation for the service link doppler, considering the following aspects.

- For earth-moving cells, indicate DL common frequency pre-compensation offset to UE.

- For earth-fixed cells, indicate the beam-specific ECEF co-ordinates of a fixed reference point to UE.

- Conclusion

In this contribution, the indication of common TA drift parameters, time alignment timers for NTN UE, and indication of common frequency compensation offset were discussed with the following observation and proposals:

Observation 1: Further discussion on how to use common TA parameters is needed.

Proposal 1: Support feeder link time compensation at UE.

Proposal 2: Support common TA calculation discussion after common TA drift parameters are determined.

Proposal 3: Support one validity timer configured for both Common TA and satellite ephemeris information.

Proposal 4: Support common DL frequency compensation for the service link doppler, considering the following aspects.

- For earth-moving cells, indicate DL common frequency pre-compensation offset to UE.

- For earth-fixed cells, indicate the beam-specific ECEF co-ordinates of a fixed reference point to UE.

References

- Chairman Notes, 3GPP TSG RAN WG1 #104 bis and #105 e-meeting.

- TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks.