3GPP TSG RAN WG1 #106-e		                 R1-2106593

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on HARQ enhancements for NR-NTN

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In RAN1#105 e-meeting, the following agreements on HARQ enhancement were achieved [1]:

Agreement: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2 Agreement: For Type-2 HARQ codebook in NTN, - For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes - FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI Agreement: Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process. Agreement: For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion. Agreement: Discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN.

In this contribution, we provide some considerations on HARQ enhancement for HARQ process ID indication and disabling/enabling HARQ feedback in NR-NTN.

- Discussion

  - Enhancement for HARQ process ID indication

According to the agreements of RAN1 #103 e-meeting, 32 is taken as the maximal supported HARQ processes number for both UL and DL. In previous e-meetings, some solutions to indicate the extended HARQ process ID for different DCI formats were also discussed:

- Option 1: Slot index as the MSB

- Option 1-a:Slot index as the LSB

- Option 2: Reusing one bit from other bit field

- Option 3: Extending the HARQ process ID field up to 5 bits

Option 3 was agreed for DCI 0-2/1-2 and Optinon 1/1a were deprioritied for DCI 0-1/1-1.

For non-fallback DCI format 0-1/1-1, the bits number of some fields have been determined by relevant configured high layer parameters. Hence, the straightforward way to indicate the 32 HARQ processes is to extend the HARQ process ID field up to 5 bits if the maximum number of HARQ processes is configured to 32 by the high layer parameters. In other words, the size of HARQ process number is related to configured higher layer parameters,  which minimizes the impact on the specification and performance, e.g. the scheduling and PDCCH detection.

For fallback DCI format 0-0/1-0, considering the backward compatibility, a straightforward way is that do not support 32 HARQ processes, if the network want to enhance the throughput using more HARQ processes, the network should configure non-fallback DCI foramts. The other alternative is, if the network would like to schedule 32 HARQ processes using DCI 0-0/1-0, the option 2 can be considered. For example, LSB of the MCS indication field can be used to indicate the MSB or LSB of the HARQ process ID. Meanwhile, there is negligible impact on the link adaptation performance, and even no impact on the scheduling flexibility and DCI detection performance. Another solution is to extend the number of scrambled CRC parity bits to indicate the MSB of the HARQ process ID. For example, the CRC parity bits are scrambled with the corresponding RNTI and additional one bit d, to form the sequence of bits  for the DCI transmissions. The relation between ck and bk is:

for k = 0, 1, 2, …,

for k =

for k = , ,,..., .

Proposal 1：Support to indicate enhanced HARQ process ID by the following:

- For DCI 0-1/1-1, extend the HARQ process ID field to 5 bits if the maximum number of HARQ processes is configured to 32 by the high layer.

- For DCI 0-0/1-0, at least the following alternatives can be considered:

  - Alt 1: do not support 32 HARQ processes.

  - Alt 2: re-interpret existing DCI field to indicate the extension of HARQ process ID, e.g. the LSB of the MCS indication.

  - Alt 3：extend the number of scrambled CRC parity bits to indicate the MSB of the HARQ process ID.

  - Restriction on the HARQ feedback disabling

In previous e-meetings, some issues about the potential misalignment between UE’s and gNB’s behavior due to the disabling HARQ feedback were discussed, such as 1) MAC signals timing, and 2) SPS activation/deactivation. Some solutions were also discussed, such as 1) UE expects that at least one HARQ process is configured with enabled HARQ feedback, and 2) UE expects that MAC-CEs are transmitted using HARQ processes with feedback enabled. In our opinions, the scheduling with a given HARQ process to enable or disable HARQ feedback to send MAC CE could be up to gNB implementation. Considering the scheduling flexibility, some dynamic configuration schemes can be considered, such as using dynamic indication, e.g. DCI scrambled by some dedicated RNTIs to switch the enabling/disabling HARQ feedback configured by RRC for some HARQ processes. That is, some or all of the HARQ processes can be configured as disabled HARQ feedback, and can be dynamically switched to enable HARQ feedback if necessary. Thus, gNB can schedule the needed HARQ process with enabling or disabling HARQ feedback.

Proposal 2: The scheduling with a given HARQ process to enable/disable HARQ feedback to send MAC CE is up to gNB implementation.

Proposal 3: The functionality of enabling/disabling HARQ feedback per HARQ process can be semi-static configured and dynamically switched.

  - HARQ-ACK codebook enhancement for disabling HARQ

Due to the disabling/enabling HARQ feedback per HARQ process, the HARQ-ACK codebook scheme with disabled HARQ feedback should be clarified.

For Type-1 HARQ-ACK codebook, a straightforward way is that UE does not change the HARQ-ACK determination scheme, but there are two different situations for one HARQ-ACK reporting occasion. One is that, within the occasions, enabled HARQ processes or SPS PDSCH release are transmitted. One simple approach is without change on the HARQ-ACK determination scheme and to also insert ACK or NACK depending on the decoding results of the PDSCHs in the positions corresponding to the PDSCHs associated with the disabled HARQ processes. In our understanding, one of the motivations of disabling HARQ feedback is to reduce the transmission latency and then enhance the system throughput, thus the ACK information is also helpful for the network to adjust the scheduling strategies, such as MCS and aggregation factor. The other situation is that, within the  occasion, only disabled HARQ processes are transmitted, then this HARQ-ACK feedback can be omitted.

For Type-2 HARQ-ACK codebook, a straightforward way is that DAI is not counted for a PDCCH which is scheduling the disabled HARQ process. In current specification, the value of T-DAI in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) and SPS PDSCH release associated with the DCI formats up to the current PDCCH monitoring occasion, and the C-DAI in a DCI format denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with the DCI formats up to the current serving cell and the current PDCCH monitoring occasion. Regardless of the disabled HARQ processes, DAI counts only PDSCH with enabled HARQ processes and SPS PDSCH release. Then the value of T-DAI in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ and SPS PDSCH release associated with the DCI formats up to the current PDCCH monitoring occasion, and the value of C-DAI in a DCI format denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ or SPS PDSCH release associated with the DCI formats up to the current serving cell and the current PDCCH monitoring occasion,  one example of (C-DAI, T-DAI) is shown in Figure 1.

Figure 1 Example of (C-DAI, T-DAI) with disabled HARQ processes

For Type-3 HARQ-ACK codebook, HARQ-ACK codebook size is determined based on all the configured HARQ processes, which will cause large unnecessary redundancy for HARQ-ACK feedback if many HARQ processes are configured with disabled HARQ feedback. A straightforward way is that UE does not feedback the disabled HARQ processes to reduce the size of HARQ-ACK codebook, so UE only needs to feedback HARQ-ACK information in the type-3 HARQ-ACK codebook for all enabled HARQ processes in one shot.

Proposal 4：Support to enhance HARQ-ACK codebook as below:

- For Type-1 HARQ-ACK codebook:

  - If enabled HARQ processes or SPS PDSCH release are transmitted in the occasions, no enhancement.

  - If only disabled HARQ processes are transmitted in the occasions, omit the HARQ-ACK report.

- For Type-2 HARQ-ACK codebook:

  - DAI counts only PDSCH with enabled HARQ processes and SPS PDSCH release.

    - The value of T-DAI in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ and SPS PDSCH release associated with the DCI formats up to the current PDCCH monitoring occasion.

    - The value of C-DAI in a DCI format denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ or SPS PDSCH release associated with the DCI formats up to the current serving cell and current PDCCH monitoring occasion.

- For Type-3 HARQ-ACK codebook:

  - HARQ-ACK codebook includes HARQ-ACK of all the enabled HARQ processes in one shot.

- Conclusion

In this contribution, we provide our views on HARQ enhancement for NR-NTN with the following proposals:

Proposal 1：Support to indicate enhanced HARQ process ID by the following:

- For DCI 0-1/1-1, extend the HARQ process ID field to 5 bits if the maximum number of HARQ processes is configured to 32 by the high layer.

- For DCI 0-0/1-0, at least the following alternatives can be considered:

  - Alt 1: do not support 32 HARQ processes.

  - Alt 2: re-interpret existing DCI field to indicate the extension of HARQ process ID, e.g. the LSB of the MCS indication.

  - Alt 3：extend the number of scrambled CRC parity bits to indicate the MSB of the HARQ process ID.

Proposal 2: The scheduling with a given HARQ process to enable/disable HARQ feedback to send MAC CE is up to gNB implementation.

Proposal 3: The functionality of enabling/disabling HARQ feedback per HARQ process can be semi-static configured and dynamically switched.

Proposal 4：Support to enhance HARQ-ACK codebook as below:

- For Type-1 HARQ-ACK codebook:

  - If enabled HARQ processes or SPS PDSCH release are transmitted in the occasions, no enhancement.

  - If only disabled HARQ processes are transmitted in the occasions, omit the HARQ-ACK report.

- For Type-2 HARQ-ACK codebook:

  - DAI counts only PDSCH with enabled HARQ processes and SPS PDSCH release.

    - The value of T-DAI in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ and SPS PDSCH release associated with the DCI formats up to the current PDCCH monitoring occasion.

    - The value of C-DAI in a DCI format denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) with enabled HARQ or SPS PDSCH release associated with the DCI formats up to the current serving cell and current PDCCH monitoring occasion.

- For Type-3 HARQ-ACK codebook:

  - HARQ-ACK codebook includes HARQ-ACK of all the enabled HARQ processes in one shot.

References

- RAN WG1 #105 e-meeting Chairman’s Notes.