3GPP TSG RAN WG1 #106-e		R1-2106595

e-Meeting, August 16th –  27th, 2021

Source:	vivo

Title:	Discussion on methods for Rx/Tx timing delay mitigating

Agenda Item:	8.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#105-e meeting [1], some issues for Rx/Tx timing error mitigating were discussed and some agreements and conclusions were achieved as follows.

Agreement:

- For mitigating UE Tx timing errors for UL TDOA, support  one of the following options:

  - Option 1:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

  - Option 2:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling

- Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

Agreement:

Send an LS to RAN2/RAN3 (cc SA2), including the following content:

- RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning.

- Notes:

  - The term “positioning reference unit (PRU)” is only used as a terminology in this discussion.  PRU does not necessarily mean an introduction of a new network node.

  - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2.  The positioning functionalities may include, but not limited to, the following:

    - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences)

    - Transmit the UL SRS signals for positioning

  - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF.

Agreement:

- For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE

  - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

- In either option, a Tx TEG ID is associated with (downselection needed)

  - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

  - Alt. 2: the Tx timing of the Rx-Tx measurement

  - Alt. 3: one or more UL SRS resources for positioning

- Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement

- FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2).

- FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

In this contribution, we present our views on potential enhancements for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays.

Rx/Tx timing delay mitigating for positioning methods

Rx/Tx timing delay mitigating for DL-TDOA method

In last meeting, some issues related to Rx/Tx timing delay mitigating for DL-TDOA were discussed with the following proposal.

### Proposal 3.1-3 (Revision 4)(H) - Subject to UE’s capability, support a UE to - measure the same DL PRS resource from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements. - measure different DL PRS resources from a TRP with the same UE RX TEG, and report corresponding RSTD measurements. - measure different DL PRS resources from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements. - FFS: indicate whether the measured RX TEG timing error difference has been compensated at UE side. - Note: All RSTD measurements are relative to a single reference timing - FFS: Whether the TRP can be both “RSTD” reference TRP and neighbor TRP - FFS: details of the Signaling, procedures, and UE capability

In DL-TDOA method, RSTD measurement is easily affected by UE Rx timing error. For example, when calculating RSTD, PRS Resource A is associated with Rx TEG 0, but ‘the reference PRS resource’ is associated with RX TEG1, this will introduce additional ‘Rx timing error difference’ cause by Rx TEG0 and Rx TEG1 in RSTD measurement results. In general, the UE can provide the association information of RSTD measurements with UE Rx TEG(s) to LMF in RSTD measurements report, which helps the LMF further exclude some of the measurement results containing additional ‘Rx timing error difference’ for higher accuracy.

However, in some cases, there is no need for UEs to report the association information of RSTD measurements with UE Rx TEG(s). For UEs that only support 1 Rx TEG, ‘Rx timing error difference’ natural does not exist. For scenarios with not very high positioning accuracy requirement, the ‘ns-level timing error’ does not have too much impact on the positioning service. In other words, whether the UE should provide Rx TEG information and how to provide Rx TEG information is related to the positioning accuracy requirement. For example, when the positioning accuracy requirements are not tight, the UE may not provide Rx TEG information or only need to divide the Rx timing errors into a few rough Rx TEGs; when the positioning accuracy requirements are high, the UE can be requested to provide Rx TEG information and needs to divide the Rx timing error into more precise Rx TEGs. Therefore, we propose

- The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF.

Moreover, even if the LMF can obtain the association information between RSTD measurements and Rx TEGs, it is also difficult to mitigate the impact of UE Rx timing error without the estimation of ‘Rx timing error difference’.  However, when measuring PRS(s) from a TRP, different UE Rx TEGs are associated, it is possible to estimate ‘Rx timing error difference’. In this case, ‘Rx timing error difference’ between different Rx TEGs will result in mainly differences between ToA measurements. For example, for a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs), e.g. a PRS resource from a TRP associated with with 2 Rx panels (or 2 Rx TEGs) or 2 PRS resources from a TRP associated with with 2 Rx panels (or 2 Rx TEGs) respectively, the difference between ToA measurements from each Rx panel (or Rx TEG) can be simply written as:

ToA_difference=(t1+e1)-(t2+e2)=(t1-t2)+(e1-e2)

where t1, t2 are propagation delays between Rx panels and the TRP respectively, and e1, e2 are Rx timing errors for 2 Rx panels respectively. The ToA_difference can be divided to 2 parts, the first part is (t1-t2) which represents the difference in propagation delay, and the second part is (e1-e2) which represents the  ‘Rx timing error difference’. Moreover, since the propagation distance difference of (t1-t2) shall be less than the distance between the 2 Rx panels, when the distance between the 2 Rx panels is assumed as 10 cm, the value of (t1-t2) can be considered less than 0.33 ns, which is almost negligible compared to ‘Rx timing error difference’ (e1-e2). Therefore, we can treat ‘ToA_difference’ as (e1-e2) approximately, that is, the difference between ToA measurements associated with different Rx TEGs can be regarded as ‘Rx timing error difference’. The related evaluation results are shown below.

Figure1 The performance of ‘Rx timing error differences’ estimation in SH scenario

Figure2 The performance of ‘Rx timing error differences’ estimation in DH scenario

Figure 1 shows the performance of ‘Rx timing error differences’ estimation in SH scenario. In the left figure, it is observed that the ‘estimated Rx timing error differences’ by calculating ‘ToA_difference’ according to a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs) are very close to the real Rx timing error differences. Since in SH scenario, the probability of both Rx panels (or 2 Rx TEGs) being in LOS is high, so the estimation of ‘Rx timing error differences’ is more accurate. In the right figure, we compare the positioning performance for the cases of ‘Rx timing error elimination by timing error estimation’, ‘Rx timing error elimination by extracting measurements from same TEG’ and ‘no Rx timing error elimination’. For the case of ‘Rx timing error elimination by timing error estimation’, the UE location is derived with compensating for the ‘estimated Rx timing error differences’. For the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, we first extracted the RSTD measurement results associated with the same Rx panel (or Rx TEG), then calculated the UE position based on these measurement results. While for the case of ‘no Rx timing error elimination’, we assume that the LMF does not know any Rx TEG information of RSTD measurements and still calculates the UE position in a conventional manner. It can be seen that withcompensating forthe ‘estimated Rx timing error differences’, the accuracy is largely improved compared with the case of ‘no Rx timing error elimination’, for example: from 3.2m@90% to 0.3m@90%; however, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, the accuracy improvement is relatively small, e.g., from 1.9m@90% to 0.3m@90%. This is because in SH scenario, LOS path is relatively abundant, even if we use the measurements from only one Rx panel (or Rx TEG), the accuracy will not be reduced too much.

Figure 2 shows the performance of ‘Rx timing error differences’ estimation in DH scenario. In the left figure, it is observed that the ‘estimated Rx timing error differences’ by calculating ‘ToA_difference’ according to a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs) are close to the real Rx timing error differences, but compared with the SH scenario, the error difference estimation accuracy is reduced. This is because in DH scenario, the probability of both two Rx panels (or 2 Rx TEGs) being in LOS is smaller than in the SH scenario. In addition, in the right figure, it is observed that with compensating forthe ‘estimated Rx timing error differences’, the accuracy is greatly improved compared with the case of ‘no Rx timing error elimination’, for example: from 30m@90% to 3m@90%; furthermore, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, the accuracy is also significantly improved, e.g. from 13m@90% to 3m@90%. The reason is that in the DH scenario, the probability of LOS is lower. For the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, if we use the measurements from only one Rx panel (or Rx TEG) to calculate the UE position, the number of LOS paths cannot be guaranteed, so the accuracy is greatly reduced.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the difference between ToA measurements associated with different UE Rx TEGs can be regarded as ‘Rx timing error difference’, which can be used for mitigating UE Rx timing errors in DL-TDOA method.

- With compensating for the ‘estimated Rx timing error differences’, the positioning accuracy is improved, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’ in which only using the measurements associated with only one Rx TEG to calculate the UE position.

- The accuracy is improved in both SH and DH scenarios, especially in the DH scenario, the accuracy improvement is large.

We think there are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP. For example, the UE can measure the PRS resource associated with different Rx TEGs at the same time, or measure the PRS resource of different repetitions associated with different Rx TEGs, or measure the PRS resource of different time stamp.

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the TRP. For example, the UE can measure at least 2 PRS resources, and different PRS resources are associated with different Rx TEGs. Furthermore, considering that different PRS resources from the same TRP may correspond to different frequency layers (e.g. frequency layers of different bandwidth), which will introduce additional measurement error, we can further limit that the different PRS resources here come from the same PRS resource set.

- There are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP.

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the same PRS resource set of the TRP.

Furthermore, when the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, it can compensate ‘Rx timing error difference’ by itself first, and then report compensated RSTD measurements to the LMF without Rx TEG association information, or it can report original RSTD measurements (uncompensated measurements) and related Rx TEG information to the LMF. The latter is our preference, the reasons are as follows

- On the LMF side, the measurement results of different Rx TEGs for the same TRP can be used as part of the original TDOA positioning equation. In the process of solving equations, based on multiple iterations, the unreasonable Rx timing error difference estimate and TRPs can be excluded. However, on the UE side, without the information of TRP location, the UE cannot rule out unreasonable Rx timing error difference estimate, which may introduce new errors caused by unreasonable Rx timing error difference estimation after compensation in RSTD measurements. And the requirement for compensating on the UE side is difficult to define and guarantee in the different scenarios. Furthermore, we compared the performance of Rx timing error difference compensated by the LMF and by the UE in DH scenario as following.

Figure3 The performance of ‘Rx timing error differences’ estimation by LMF and by UE in DH scenario

It is observed in Figure3 that the performance of Rx timing error difference estimated and compensated by the LMF (pink) is better than the performance of Rx timing error difference estimated and compensated by the UE (red).

- The UE may estimate ‘UE Rx timing error difference’ based on different PRS resources from one TRP, however the different PRS resources may be associated with different TRP Tx TEGs, which is unknown to the UE for UE-assisted positioning. Therefore, TRP Tx timing error difference may be introduced in ‘UE Rx timing error difference estimate’ based on UE’s estimation. On the contrary, the LMF can obtain TRP Tx TEG information associated with different PRS resources and further eliminate its impact.

Therefore, we propose

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, support the UE to report original RSTD measurements (uncompensated measurements) and related Rx TEGs information to the LMF, rather than compensating ‘Rx timing error difference’ by UE implementation and then reporting compensated RSTD measurements to the LMF.

Therefore, when the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the reported measurements related to different UE Rx TEGs is the key information for the LMF to mitigate UE Rx timing errors. However, in some cases, the key information may be missed. For example, one UE is able to measure multiple PRS resources from a TRP associated with different UE Rx TEGs. But in current specification, it can only select up to 4 RSTD measurements to report, and each RSTD measurement corresponds to a PRS resource. Once there is no rule to restrict UE behavior, the UE may report up to 4 RSTD measurements only associated with the same UE Rx TEG by UE implementation, even if there are other PRS resources associated with another UE Rx TEG and the corresponding RSTD measurements have high quality, so that the key information is missed. In addition, for a UE is able to measure a PRS resource associated with different Rx TEGs, it is also natural for the UE to associate only 1 Rx TEG with the RSTD measurement for this PRS resource, e.g. the UE only provides the Rx TEG information of the first path used to determine the RSTD measurement. Similarly, the key information is missed. Thus, to ensure that the UE can report the key information to the LMF, some reporting rules should be defined. For example, for a UE is able to measure multiple PRS resources from a TRP associated with different UE Rx TEGs, when the RSTD measurements for PRS resources associated with a Rx TEG meet certain conditions, the UE should report at least one RSTD measurement associated with the Rx TEG; for a UE is able to measure a PRS resource associated with different UE Rx TEGs, when the measurements associated with a Rx TEG meet certain conditions, the UE should report the measurements associated with the Rx TEG. Therefore, we propose

- In DL-TDOA method, to eliminate the positioning error caused by the UE Rx timing errors of more than one UE Rx TEGs, the RSTD measurement report of a certain TRP for more than one UE Rx TEGs needs to be guaranteed if the UE is able to measure PRS(s) associated with different UE Rx TEGs.

- FFS the UE reporting rules to guarantee the RSTD measurement report for more than one UE Rx TEGs.

As we mentioned ealier, subject to UE capability, the UE can measure one PRS resource associated with different Rx TEGs from the TRP. In the next step, we need to consider how to include different Rx TEG IDs for the same PRS resource in the DL RSTD measurement report. There may be 2 options as below.

- Option 1: Include one UE Rx TEG ID for one DL RSTD measurement.

- Option 2: Include different UE Rx TEG IDs for one DL RSTD measurement, that is, associate additional UE Rx TEG ID with additional path group.

For Option 1, to associate with different Rx TEGs for the same PRS resource, we need to add multiple additional DL RSTD measurements ‘NR-DL-TDOA-AdditionalMeasurements’ for the same PRS resource.  This will further increase the number of ‘NR-DL-TDOA-Additional Measurements’. Considering that the current UE capability supports up to 4 RSTD measurements per TRP pair to be reported, in order to support Option 1, the maximum number of RSTD measurements per TRP pair needs to be expanded.

For Option 2, we need to associate UE Rx TEG ID with additional path group. One PRS resource still corresponds to one DL RSTD measurement. For different UE Rx TEG ID, different path group is used. The paths in the same path group are measured based on the same PRS resource with the same Rx TEG (e.g. Rx panel)  and derived based on same CIR. For example, we list the potential IE structures as below. The ‘nr-RSTD’ and ‘nr-AdditionalPathList’ in ‘NR-DL-TDOA-MeasElement’ represents the RSTD value and additional paths associated with the Rx TEG1; while the ‘NR-AdditionalPathList2’ represents the paths associated with the Rx TEG2 based on the same PRS resource.

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-RxTEGID NR-RxTEGID OPTIONAL nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-AdditionalPathList2 NR-AdditionalPathList2 OPTIONAL nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ... } NR-AdditionalPathList2-r16 ::= SEQUENCE { nr-RxTEGID NR-RxTEGID OPTIONAL NR-AdditionalPathList-r16 SEQUENCE (SIZE(1..3)) OF NR-AdditionalPath-r16 ... }

From our point of view, both options can work. However, from the perspective of signaling overhead, we think Option 2 is better, since the duplicate bit overhead such as TRP/PRS resource set/PRS resource ID, time stamp etc. for the same PRS resource is saved.

Therefore, we propose

- In DL-TDOA method, to include different Rx TEG IDs for the same PRS resource in the DL RSTD measurement report, two options can be considered:

- Option 1: Include one UE Rx TEG ID for one DL RSTD measurement.

- Option 2: Include different UE Rx TEG IDs for one DL RSTD measurement, that is, associate additional UE Rx TEG ID with additional path group.

- From the perspective of signaling overhead, Option 2 is preferred.

Rx/Tx timing delay mitigating for UL-TDOA method

In UL-TDOA method, positioning accuracy is also easily affected by UE Tx timing error. For example, when the LMF is calculating the difference of the RTOA measured by different TRPs for a UE, when the UE Tx TEG associated with the RTOA is different, an extra ‘UE Tx timing error difference’ will be generated in the ‘RTOA difference results’, so that positioning accuracy is impacted. In general, the UE can provide the association information of SRS resources with UE Tx TEG(s) to LMF, which helps the LMF further exclude some of the ‘RTOA difference results’ containing additional ‘Tx timing error difference’ for higher accuracy.

Similar to DL-TDOA method, in some cases, there is also no need for UEs to provide the association information of SRS resources with UE Tx TEG(s), such as scenarios with not very high positioning accuracy requirement. Therefore, we propose

- The UE can be requested to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF when the UE supports more than 1 UE Tx TEG.

Regarding ‘whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF’, our preference is that UE can provide the Tx TEG association information to the LMF directly, that is, Option 1. We found that some information related to the uplink positioning can also be transmitted via LPP messages, for example, the UE can be requested to report UL related capabilities including UL SRS capabilities to the LMF directely via LPP as following. Moreover, we think UE Tx TEG information is completely different from the SRS configuration. The regular SRS configuration for positioning is determined and configured by the serving gNB, however, the UE Tx TEG information is determined by the UE and there is no use for the serving gNB to obtain this information. Therefore, compared with indirect indication of UE Tx TEG information through gNB, direct Tx TEG information indication to the LMF via LPP message is preferred.

RequestCapabilities-r9-IEs ::= SEQUENCE { commonIEsRequestCapabilities CommonIEsRequestCapabilities OPTIONAL, -- Need ON a-gnss-RequestCapabilities A-GNSS-RequestCapabilities OPTIONAL, -- Need ON otdoa-RequestCapabilities OTDOA-RequestCapabilities OPTIONAL, -- Need ON ecid-RequestCapabilities ECID-RequestCapabilities OPTIONAL, -- Need ON … [[ nr-ECID-RequestCapabilities-r16 NR-ECID-RequestCapabilities-r16 OPTIONAL, -- Need ON nr-Multi-RTT-RequestCapabilities-r16 NR-Multi-RTT-RequestCapabilities-r16 OPTIONAL, -- Need ON nr-DL-AoD-RequestCapabilities-r16 NR-DL-AoD-RequestCapabilities-r16 OPTIONAL, -- Need ON nr-DL-TDOA-RequestCapabilities-r16 NR-DL-TDOA-RequestCapabilities-r16 OPTIONAL, -- Need ON nr-UL-RequestCapabilities-r16 NR-UL-RequestCapabilities-r16 OPTIONAL -- Need ON ]] } ProvideCapabilities-r9-IEs ::= SEQUENCE { commonIEsProvideCapabilities CommonIEsProvideCapabilities OPTIONAL, a-gnss-ProvideCapabilities A-GNSS-ProvideCapabilities OPTIONAL, otdoa-ProvideCapabilities OTDOA-ProvideCapabilities OPTIONAL, ecid-ProvideCapabilities ECID-ProvideCapabilities OPTIONAL, … [[ nr-ECID-ProvideCapabilities-r16 NR-ECID-ProvideCapabilities-r16 OPTIONAL, nr-Multi-RTT-ProvideCapabilities-r16 NR-Multi-RTT-ProvideCapabilities-r16 OPTIONAL, nr-DL-AoD-ProvideCapabilities-r16 NR-DL-AoD-ProvideCapabilities-r16 OPTIONAL, nr-DL-TDOA-ProvideCapabilities-r16 NR-DL-TDOA-ProvideCapabilities-r16 OPTIONAL, nr-UL-ProvideCapabilities-r16 NR-UL-ProvideCapabilities-r16 OPTIONAL ]] }

Besides, we believe the way the UE provides the TEG association information to the LMF should be consistent with the way the LMF requests the UE to provide the TEG association information. For example, if the TEG association information requesting signalling is LPP signalling directly (LMF→UE), the corresponding providing signalling should also be LPP signalling directly (UE→LMF); if the TEG association information requesting signalling is sent through the serving gNB (LMF→serving gNB→UE), the corresponding providing signalling should also be sent through the serving gNB (UE→serving gNB→LMF).

Therefore, we propose that

- Support the UE to directly provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF via LPP message.

- Note: The way the UE provides Tx TEG association information to the LMF should be consistent with the way the LMF requests UE Tx TEG association information.

In addition, during the time when the UE transmits the SRS resources, as the UE flips or is blocked, it is up to UE implementation to switch Tx panel for better uplink transmission. Thus, for the same SRS resources for positioning, the associated Tx panel and Tx TEG will also change accordingly, the following figure shows a simple example.  That is, the UE Tx TEG information of SRS resource(s) for different SRS instances may be not always fixed. The UE not only needs to report the initial Tx TEG information to the LMF, but also needs to report the Tx TEG change information to the LMF. Therefore, the report of UE Tx TEG information should be continuous multiple reports, instead of a single report at the beginning of SRS transmission. There may be two ways for UE to report Tx TEG: periodic reporting and event-triggered reporting. Considering that the UE may change Tx TEG information of SRS resource(s)(e.g. switch panels) at any time, to save signaling overhead, we prefer event-triggered reporting. For example, UE should report Tx TEG change information to the LMF when Tx TEG change happens compared with the last Tx TEG report. The LMF can assume that Tx TEG information associated SRS resource(s) is relatively stable between the two triggered reports. Therefore, we propose

Figure4 UE Tx TEG(s) change associated with SRS resource(s)

- The UE should report the information of the UE Tx TEG(s) change associated with transmission of SRS resource(s) to the LMF.

- The report of UE Tx TEG information should be continuous multiple reports, instead of a single report.

- The continuous multiple reports should be event-triggerred report, e.g. UE should report Tx TEG change information to the LMF when Tx TEG change happens compared with the last Tx TEG report.

Furthermore, we think the information of the UE Tx TEG(s) change should also be provided to the gNB which performs SRS measurements. For example, after one gNB measuring SRS, it may report RTOA measurements to the LMF. To obtain the RTOA measurements of a SRS resource, the gNB may derive SRS measurements from multiple SRS occasions/instances for joint processing (e.g. averaging). However, without the information of the UE Tx TEG(s) change, it is possible for the gNB to perform joint processing on multiple SRS occasions associated with different UE Tx TEGs, which will introduce unnecessary errors. Therefore, after the LMF obtains the information of UE Tx TEG change, it can further transmit this information to the gNB performing UL positioning measurement. So we propose that

- Support LMF to forward the UE Tx TEG information associated with SRS resource(s) provided by the UE to the serving and neighboring gNBs.

Then, if a gNB is able to measure SRS resoures related to different UE Tx TEGs, the reported RTOA measurements related to different UE Tx TEGs is the key information for the LMF to migitate UE Tx timing errors. Therefore, it is better to define gNB reporting rules to guarantee the key information to be reported, which is similar to what we mentioned in DL-TDOA method.

- In UL-TDOA method, to eliminate the positioning error caused by the UE Tx timing errors of more than one UE Tx TEGs, the RTOA measurement report for more than one UE Tx TEGs needs to be guaranteed if the gNB is able to measure SRS resoures associated different UE Tx TEGs.

- FFS the gNB reporting rules to guarantee the RTOA measurement report for more than one UE Tx TEGs.

Rx/Tx timing delay mitigating for DL and UL positioning method

For Multi-RTT method, the round-trip-time is calculated by the use of the UE Rx-Tx time difference measurements and gNB Rx-Tx time difference measurements. The measurement accuracy of UE Rx-Tx time difference measurements may be impacted by UE Rx and/or Tx timing errors, and the measurement accuracy of gNB Rx-Tx time difference measurements may be impacted by gNB Rx and/or Tx timing errors. If UE Tx and Rx timing errors and gNB Tx and Rx timing errors cannot be compensated, the accuracy of the multi-RTT positioning may be significantly impacted. The similar problem will also appear in the ‘DL TDOA+UL-TDOA’ method, which can be supported by implementation to reduce some common errors from TRPs and UEs.

In RAN1# 105-e, the issues related Rx/Tx mitigating for DL and UL method were discussed intensely. For mitigating UE Rx/Tx timing errors, the following conclusion on UE side is achieved.

Agreement: - For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

We think both Option1 and Option2 can be supported, which is up to UE capability. Compared with Option2, Option1 seems more comprehensive. Option2 only provides the information of {Rx TEG, Tx TEG} which denotes the information of ‘Rx timing error’ and ‘Tx timing error’, but lacks the information of ‘Rx timing error+Tx timing error’ which facilitates differential Multi-RTT. Option1 not only provides the information of ‘Rx timing error+Tx timing error’ via RxTx TEG, but also doesn’t rule out the provision of Rx TEG and Tx TEG. However, for UEs not able to obtain the information of ‘Rx timing error+Tx timing error’ , Option2 may be needed.

Therefore, we propose,

- For mitigating UE Rx/Tx timing errors for DL+UL positioning, up to UE capability, both of the following options can be supported.

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

Another issue is what a Tx TEG here is associated with. Let's analyze the above 3 alternatives.

- Alt.2: the Tx timing of the Rx-Tx time difference measurement.

Alt.2 is an alternative to associate Tx TEG with the Tx timing of the Rx-Tx measurement. In Rel-16, we have defined the reference point for Rx-Tx time difference measurement at RF, it seems the Tx TEG can be associated with the ‘Tx timing’ at RF of the Rx-Tx time difference measurement. We believe that the definition of ‘Rx-Tx time difference’ and related ‘reference point’ are introduced based on the signal propagation delay, since the propagation delay can accurately reflect the actual distance between the UE and the TRP. However, in practice, the ‘Rx-Tx time difference’ is generally calculated from the baseband, which is why we introduced the ‘group delay’ at the beginning of Rel-17. If the UE report ‘Rx-Tx time difference’ based on the calculation on baseband, UE can determine the ‘Tx timing’ based on the timing of the UL subframe at the baseband, wherein, the actual Tx timing error will not be added. In other words, the Tx timing of the Rx-Tx time difference measurement in baseband is independent of UE Tx TEG. But if the UE report ‘Rx-Tx time difference’ based on RF, e.g. UE has capability to estimate and compensate ‘UE Rx+Tx timing error’, the ‘Tx timing’ of the Rx-Tx time difference may not be independent of UE Tx TEG. In this case, the ‘Rx-Tx time difference’ can be Tx TEG specific, that is, for the same PRS resource, the corresponding ‘Rx-Tx time difference’ can be different for different Tx TEGs. But even so, since Tx TEG is not associated with SRS Resource(s), it is difficult to handle with the potential mismatch between UE and gNB Rx-Tx time difference measurements.

- Alt.1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement.

Alt.1 is another alternative to associate Tx TEG with the Tx timing of the Rx-Tx measurement. It is also a scheme to couple Rx-Tx time difference measurement with SRS resource and actual SRS transmission.

Firstly, if the UE report ‘Rx-Tx time difference’ based on the calculation on baseband, UE can determine the ‘Tx timing’ based on the timing of the UL subframe at the baseband. In this case, even if the ‘Tx timing’ can be associated with the baseband UL subframe of SRS resources, for the same PRS resource, UE can only report one Rx-Tx time difference measurement. In other words, in this case, the Tx timing of the Rx-Tx measurement is not associated with SRS resource(s) and Tx TEG(s).

Then, if the UE report ‘Rx-Tx time difference’ based on RF, as mentioned above, for the case where UE has capability to estimate and compensate ‘UE Rx+Tx timing error’, the ‘Tx timing’ of the Rx-Tx time difference may not be independent of UE Tx TEG. However, in Multi-RTT positioning, according to current definition of ‘Rx-Tx time difference’, the ‘Tx time’ is not associated with actual SRS transmission. Therefore, coupling Rx-Tx time difference measurement with SRS transmission is needed. One way is to associate certain SRS instance to derive Tx TEG, e.g. the most recent SRS instance in advance of the Rx-Tx measurementas shown in Figure 5. Another way is to change the definition of ‘Rx-Tx time difference’, for example, the ‘Tx time’ can be changed to the uplink subframe associated with certain SRS instance. Both ways require UE to compensate ‘Rx+Tx timing error’ based on the Tx TEG(s) related to the SRS instance and include corresponding Tx TEG(s) information in the measurement report. But there are still some problems as follows.

- We still need to consider how to handle with the potential mismatch between UE and gNB Rx-Tx time difference measurements. It is difficult to ensure that the SRS resources or instance associated with UE Rx-Tx time difference measurement can be measured by the TRP.

- For different SRS resources with different Tx TEG, UE may report different Rx-Tx time difference measurement, that is, the measurement is TEG-specific/SRS resource-specific measurement. Once the number of TEGs/SRS resources to be reported is large, additional overhead cannot be ignored.

- It needs to associate the transmission of SRS resources with Rx-Tx time difference measurement, which may cause potential complexity. The association with SRS transmission or potential modifications to the Rx-Tx measurement definition will change current UE behavior and RAN4 measurement period requirement. For example, the UE needs to find suitable SRS instances to derive Tx TEG corresponding to each PRS measurement rather than perform DL measurement only.

- UE compensating ‘Rx+Tx timing error’ to get a measurement at RF side is not the typical scenario/situation in Multi-RTT positioning, where only certain UEs with specific capabilities can achieve the requirement.

Figure5 The most recent SRS instance in advance of the Rx-Tx time difference measurement to derive Tx TEG

- Alt.3: one or more UL SRS resources for positioning.

The difference between Alt.3 and the other two alternatives is that the Tx TEG is only associated with SRS transmission and independent of Rx-Tx time difference measurement. This is more in line with the current definition of Rx-Tx time difference measurement, since the ‘Tx time’ in definition is determined by the time of the UL subframe that is closest in time to the receiving time of PRS resource instead of the time of SRS transmission.

In addition, Alt.3 also makes it easier to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements. Assuming a general case where the UE reports ‘Rx-Tx time difference’ based on the calculation on baseband is applied. In this case, the UE can provide the association information of Rx-Tx difference measurements with UE Rx TEG and the association information of SRS resource(s) with Tx TEG respectively, considering the decoupled relationship, the LMF can freely combine Rx/Tx TEG and no need to care about DL+UL mismatch. Furthermore, the UE can provide additional information of UE ‘Rx timing error+Tx timing error’ associated with the combination of Rx TEGs and Tx TEGs to the LMF, that is, the UE provides full set mapping relationship between {Rx TEGs, Tx TEGs} and RxTx TEGs, so that the LMF can select suitable mapping relationship when differential Multi-RTT enabled.

Therefore, based on the discussion above, regarding what a Tx TEG is associated with, we support Alt.3: a Tx TEG ID is associated with one or more UL SRS resources for positioning.

Then, we propose,

- Regarding association information of Tx TEG for mitigating UE Tx/Rx timing errors in DL+UL positioning, support Alt.3: a Tx TEG ID is associated with one or more UL SRS resources for positioning.

In addition, regarding ‘potential mismatch between UE and gNB Rx-Tx time difference measurements’ in the FFS, to make it easier to handle with, we propose the following, which is also mentioned in the discussion for Alt.3.

- For mitigating UE Rx/Tx timing errors for DL+UL positioning, up to UE capability, the following should be supported.

- UE providing the association information of UE Rx TEG(s) with each UE Rx-Tx time difference measurements to LMF (similar to DL-TDOA).

- UE providing the association information of UE Tx TEG(s) with all UL Positioning SRS resources to LMF (similar to UL-TDOA).

- UE providing the mapping information of UE {Rx TEG ID, Tx TEG ID} to UE RxTx TEG IDs to LMF.

UE and TRP measurements enhancement

In RAN1#104e, the following agreements related to UE and TRP measurements enhancement were achieved.

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

In current specification, when UE to report DL measurements to the LMF, it is up to UE implementation to derive PRS instances to obtain RSTD, RSRP and/or Rx-Tx time difference measurements for reporting. For better matching measurements of UE and TRP or tracking non-idealities related to time-drifts for the LMF, we support a UE or a TRP to report one or more measurement instances in a single measurement report.

However, in some cases, there is no need to support above reporting behavior. For example, when the UE is configured to periodically report DL measurements with small period, it is automatically equivalent to ‘reporting one or more measurement in a single report’. In addition, for DL or UL only positioning without combination of DL and UL measurements, UE reporting one or more measurement instances in a single report may not always be needed. Thus, this reporting behavior should be treated as an optional reporting behavior, which is suitable for both UE and gNB/TRP.

- The UE or the TRP can be configured to report one or more measurement instances in a single measurement report to the LMF.

In addition, about ‘FFS: The measurement instances are within a [configured] measurement time window’, our preference is not to introduce a ‘measurement time window’, as this window can be replaced by other ways. Take the UE side as an example, the UE can be configured to report all the measurement instances with corresponding time stamps, or the UE can be configured to report the most recent measurement instances before measurement report. The same method can also be applied to gNB. We think these two methods are sufficient for positioning report, and it is also clear enough for the LMF to match the measurement results from the UE and the gNB.

Regarding UE to report the most recent measurement instances before measurement report, we think it is benefit for low latency positioning scenarios, as in these scenarios, the latest UE location is strongly required. Furthermore, considering PRS processing time, the most recent measurement instance should be in advance of a certain time before the measurement report, which is related to PRS processing capability.

Figure6 The most recent measurement instance to be report

- Support to enable the UE to report PRS measurements derived from the most recent measurement instances in advance of a certain time before the measurement report.

- The certain time before the measurement report is related to PRS processing capability.

Moreover, regarding ‘FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set’, we think its relationship with ‘Nsample’ defined for measurement period requirement by RAN4 needs to be clarified. In TS38.133[3], in RSTD measurement requirement, ‘Nsample’ is defined as the number of PRS RSTD samples (or instances of DL-PRS Resource Set here) and  ‘Nsample’= 4; in Rx-Tx time difference measurement requirement,  ‘Nsample’ is the number of UE Rx-Tx time difference measurement samples (or instances of DL-PRS Resource Set here) and  ‘Nsample’= 4; in PRS-RSRP measurement requirement, ‘Nsample’ is the number of PRS-RSRP measurement samples  (or instances of DL-PRS Resource Set here) and Nsample= 4. The motivation behind allowing more than one PRS sample in defining the measurement period is to ensure that with channel variations or deep fading, there is at least one PRS sample that satisfies the side conditions. That is, the UE needs to measure 4 PRS samples to get a measurement result. Similarly, we understand that one ‘measurement instance’ also corresponds one instance for measurement results to be report. However, if N<4, the measurement results cannot be ensured in one measurement instance. Compared with the requirement of RAN4, RAN1 seems to be more inclined to configure the number of DL-PRS Resource Set instances in a measurement instance flexibly. Therefore, we suggest to ask RAN4 to reach a consensus, and then we can determine the value of N and whether N can be configured by the network.

- The relationship between ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ and ‘the number of PRS samples for RSTD/Rx-Tx time difference/PRS-RSRP measurements’ defined by RAN4’ should be clarified.

- Send an LS to RAN4 for consistent understanding.

Furthermore, for N instances of the DL PRS resource set on a PRS resource in one UE measurement instance, in order to obtain relatively robust measurement results, the UE may perform joint processing (e.g. average) on the PRS of these N instances, then obtain a RSTD/Rx-Tx measurement for the PRS resource and associate it with a time stamp. However, considering potential TEG variation across different instances, if the UE performs joint processing on multiple PRS instances associated with different TEGs, unnecessary measurement error will be introduced. Therefore, we propose that for N instances of the DL PRS resource set on a PRS resource in one UE measurement instance, the same TEG across N instances should be ensured, wherein, the TEG includes UE Rx TEG and TRP Tx TEG. Corresondingly, for M SRS measurement time occasions in one TRP measurement instance, the same TEG across M instances should be ensured, wherein, the TEG includes TRP Rx TEG and UE Tx TEG.

- For N instances of the DL PRS resource set on a PRS resource in one UE measurement instance, the same TEG across N instances should be ensured, wherein, the TEG includes UE Rx TEG and TRP Tx TEG.

- For M SRS measurement time occasions in one TRP measurement instance, the same TEG across M instances should be ensured, wherein, the TEG includes TRP Rx TEG and UE Tx TEG.

Based on the discussion above, to ensure the same TEG across N PRS instances, the size of N is difficult to be fully controlled by the LMF, since the UE Rx TEG information is determined by the UE. Therefore, we propose that N can be recommended by the LMF, but determined by the UE. If the TEG information of the N PRS instances configured by the network are the same, the UE can determine the measurement intance based on the configured N PRS instances; otherwise, the UE can determine the number of PRS intances in one measurement instance by itself, and report related information(e.g. the number of PRS instances in the measurement instance, time stamp etc.). For the gNB side, the similar behavior can be considered.

- For N instances of the DL-PRS Resource Set within one UE measurement instance, N can be recommended by the LMF and determined by the UE.

- For M SRS measurement time occasions within one TRP measurement instance, M can be recommended by the LMF and determined by the TRP.

Conclusion

In this contribution, we discuss methods for Rx/Tx timing mitigating with the following observations and proposals.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the difference between ToA measurements associated with different UE Rx TEGs can be regarded as ‘Rx timing error difference’, which can be used for mitigating UE Rx timing errors in DL-TDOA method.

- With compensating for the ‘estimated Rx timing error differences’, the positioning accuracy is improved, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’ in which only using the measurements associated with only one Rx TEG to calculate the UE position.

- The accuracy is improved in both SH and DH scenarios, especially in the DH scenario, the accuracy improvement is large.

- There are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP.

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the same PRS resource set of the TRP.

- The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, support the UE to report original RSTD measurements (uncompensated measurements) and related Rx TEGs information to the LMF, rather than compensating ‘Rx timing error difference’ by UE implementation and then reporting compensated RSTD measurements to the LMF.

- In DL-TDOA method, to eliminate the positioning error caused by the UE Rx timing errors of more than one UE Rx TEGs, the RSTD measurement report of a certain TRP for more than one UE Rx TEGs needs to be guaranteed if the UE is able to measure PRS(s) associated with different UE Rx TEGs.

- FFS the UE reporting rules to guarantee the RSTD measurement report for more than one UE Rx TEGs.

- In DL-TDOA method, to include different Rx TEG IDs for the same PRS resource in the DL RSTD measurement report, two options can be considered:

- Option 1: Include one UE Rx TEG ID for one DL RSTD measurement.

- Option 2: Include different UE Rx TEG IDs for one DL RSTD measurement, that is, associate additional UE Rx TEG ID with additional path group.

- From the perspective of signaling overhead, Option 2 is preferred.

- The UE can be requested to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF when the UE supports more than 1 UE Tx TEG.

- Support the UE to directly provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF via LPP message.

- Note: The way the UE provides Tx TEG association information to the LMF should be consistent with the way the LMF requests UE Tx TEG association information.

- The UE should report the information of the UE Tx TEG(s) change associated with transmission of SRS resource(s) to the LMF.

- The report of UE Tx TEG information should be continuous multiple reports, instead of a single report.

- The continuous multiple reports should be event-triggerred report, e.g. UE should report Tx TEG change information to the LMF when Tx TEG change happens compared with the last Tx TEG report.

- Support LMF to forward the UE Tx TEG information associated with SRS resource(s) provided by the UE to the serving and neighboring gNBs.

- In UL-TDOA method, to eliminate the positioning error caused by the UE Tx timing errors of more than one UE Tx TEGs, the RTOA measurement report for more than one UE Tx TEGs needs to be guaranteed if the gNB is able to measure SRS resoures associated different UE Tx TEGs.

- FFS the gNB reporting rules to guarantee the RTOA measurement report for more than one UE Tx TEGs.

- For mitigating UE Rx/Tx timing errors for DL+UL positioning, up to UE capability, both of the following options can be supported.

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

- Regarding association information of Tx TEG for mitigating UE Tx/Rx timing errors in DL+UL positioning, support Alt.3: a Tx TEG ID is associated with one or more UL SRS resources for positioning.

- For mitigating UE Rx/Tx timing errors for DL+UL positioning, up to UE capability, the following should be supported.

- UE providing the association information of UE Rx TEG(s) with each UE Rx-Tx time difference measurements to LMF (similar to DL-TDOA).

- UE providing the association information of UE Tx TEG(s) with all UL Positioning SRS resources to LMF (similar to UL-TDOA).

- UE providing the mapping information of UE {Rx TEG ID, Tx TEG ID} to UE RxTx TEG IDs to LMF.

- The UE or the TRP can be configured to report one or more measurement instances in a single measurement report to the LMF.

- Support to enable the UE to report PRS measurements derived from the most recent measurement instances in advance of a certain time before the measurement report.

- The certain time before the measurement report is related to PRS processing capability.

- The relationship between ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ and ‘the number of PRS samples for RSTD/Rx-Tx time difference/PRS-RSRP measurements’ defined by RAN4’ should be clarified.

- Send an LS to RAN4 for consistent understanding.

- For N instances of the DL PRS resource set on a PRS resource in one UE measurement instance, the same TEG across N instances should be ensured, wherein, the TEG includes UE Rx TEG and TRP Tx TEG.

- For M SRS measurement time occasions in one TRP measurement instance, the same TEG across M instances should be ensured, wherein, the TEG includes TRP Rx TEG and UE Tx TEG.

- For N instances of the DL-PRS Resource Set within one UE measurement instance, N can be recommended by the LMF and determined by the UE.

- For M SRS measurement time occasions within one TRP measurement instance, M can be recommended by the LMF and determined by the TRP.

- References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 105, e-Meeting, May 10th –27th, 2021

- TS 38.455, “NR Positioning Protocol A (LPPa)”

- 3GPP TS38.133, V17.2.0, 2021.6