3GPP TSG RAN WG1 #106-e	           R1-2106606

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Paging enhancements for idle/inactive mode UE power saving

Agenda Item:	8.7.1.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #105bis e-meeting [1], the following agreements were made for potential paging enhancements:

Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. Conclusion: To down-select one solution for PEI physical-layer channel/signal in RAN1 #106-e, using below as a starting point: - PDCCH-based PEI - SSS-based PEI - TRS/CSI-RS-based PEI Note: Additional details for each of the above 3 solutions are encouraged for more informed down-selection Note: further refinement of the above list is possible, e.g., by merging/further splitting, depending on significance of the commonality and/or differences Agreement: Observation: Dynamically sharing PDCCH resources of Rel-15 UEs (whether or not this is an important aspect to consider for PEI is FFS) - For PDCCH-based PEI, - PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates - Exact number of multiplexed/impacted Rel-15 PDCCH candidates depends on AL used for PDCCH-based PEI and relative size of PDCCH CORESET, etc. - For SSS-based PEI and for the case of partial overlap of CORESET and PEI - For interleaved CORESET (such as CORESET#0), SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity - For non-interleaved CORESET, SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates - Exact number of impacted Rel-15 PDCCH candidates depends on relative size and location of PDCCH CORESET, etc. - For TRS/CSI-RS-based PEI and for the case of partial overlap of CORESET and PEI - For interleaved CORESET (such as CORESET#0), TRS/CSI-RS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity - For non-interleaved CORESET, TRS/CSI-RS-based can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at candidate level granularity - Exact number of impacted Rel-15 PDCCH candidates depends on CSI-RS mapping pattern, relative size and location of PDCCH CORESET, etc.) Agreement: For paging indication to the subgroups in a PO, - For PDCCH-based PEI, subgroups in a PO are indicated by one PEI - One bit in the DCI payload indicating one UE subgroup is supported - FFS: Whether code-point based mapping is utilized, and, if so, how to map to the subgroups in a PO - For SSS-based PEI, subgroups in a PO are indicated by a set of sequence realizations - FFS: Sequence mapping design for supporting up to 8 subgroups per PO - Physical-layer configuration(s) and sequence generation design are subject to no impact to initial access and RRM measurements of legacy UEs - For TRS/CSI-RS-based PEI, subgroups in a PO can be indicated by the following alternatives - Alt 1: One TRS sequence with orthogonal cover as PEI transmitted in the PEI monitoring occasion where one orthogonal cover of the PEI indicates one subgroup or combination of subgroups - FFS: Design details for the orthogonal cover - Alt 2: A set of TRS sequences indicating the subgroups with one selected sequence transmitting in one TRS resource - FFS: Sequence mapping design for supporting up to 8 subgroups per PO and combination of subgroups - Alt 3: Multiple TRS/CSI-RS resources FDMed/TDMed /CDMed in the same monitoring occasion where one TRS/CSI-RS resource indicates one subgroup - Reuse Rel-15/16 CSI-RS FDM/TDM/CDM patterns for supporting up to 8 subgroups per PO - Note : It is RAN1 understanding that Physical-layer configuration(s) for paging early indication to the subgroups is subject to the same idle-mode reception bandwidth as CORESET-0 frequency span

In this contribution, sub-grouping indication carried by PEI or paging PDCCH is discussed. Grouping method with a common sequence is proposed. And we further give the comparisons and evaluations for different PEI candidate designs in terms of performance, power saving gain, resource overhead etc. Besides, the feasibility for Behavior A/B is analyzed.

- Sub-grouping paging indication

RAN1 has given the evaluation results for sub-grouping indication by using paging PDCCH or PEI in RAN1 #103e-meeting. An LS [2] has been sent to inform RAN2 the observations of power saving gain for different candidate paging enhancement methods and the recommendation from RAN1 perspective. So, we further study the design on how to provide the indications for UE subgroups over PEI and/or paging PDCCH in the following section.

  - 2.1 Sub-grouping method

For evaluation of the additional power saving gain by sub-grouping paging indication, we adopt the sequenced-based UE grouping method specified for Rel-16 NB-IoT Group WUS. First of all, based on this sub-grouping method, the network will only transmit one certain sequence for each PEI occasion to indicate its associated PO(s). Hence, the resource overhead for sequence-based PEI will only depend on the number of RE that the sequence (e.g. SSS-like sequence or TRS-like sequence) will occupy. This assumption is applied in the following resource overhead analysis in section 4.3. There are two kinds of sequence. One is the sub-group specific sequence, which is linked to one UE sub-group. The other one additional sequence aims to wake up all the UE sub-groups for this PO, named as “common sequence” in this contribution. UE should wake up to receive paging PDCCH, if either the sequence associated with the UE subgroup or the common sequence is detected. For an instance, assuming there are four sub-groups associated with one PO, from the network perspective, there are totally five candidate sequences, of which four sub-group specific sequences and one common sequence. And each UE belongs to only one of the sub-groups. If a UE receives the common sequence or the sub-group specific sequence indicating the sub-group of this UE, then the UE needs to monitor its paging PDCCH. Otherwise, the UE will skip the corresponding paging PDCCH and PDSCH. To this end, each UE only need to detect two candidate sequences and the performance of sequence-based PEI will not degrade which will be proven in section 4.1.

Table 1 shows a typical example of the grouping method adopted in Rel-16 NB-IoT when per PO paging rate is assumed as 10%. The per sub-group paging rate Y can be calculated according to Taylor Expansion i.e., 1- (1-Y) N=X, assuming the number of paging group is N, per PO paging rate is X. Taken sub-groups = 8 as an example, Sequence 0 is the common sequence, and Sequence 1 to Sequence 8 are sub-group specific sequences linked to different sub-groups. Sequence 9 can be a virtual sequence and the network will actually do DTX (no sequence is transmitted i.e., behavior A) when the case happens. Also, sequence 9 will be sent when behavior B is assumed. The probability that no less than two sub-groups need to be woke up is only 0.0046 and the additional false alarm rate (FAR) caused by the common sequence is marginal i.e., about 0.34%.

Table 1: Grouping method adopted in Rel-16 NB-IoT for sub-grouping indication carried by sequence-based PEI-only.

| Characteristics for different cases | Sequence index | Probability |
| --- | --- | --- |
| >= 2 sub-groups need to be woke up | 0 | 0.0046 |
| Only to wake up sub-group 1 | 1 | 0.0119 |
| Only to wake up sub-group 2 | 2 | 0.0119 |
| Only to wake up sub-group 3 | 3 | 0.0119 |
| Only to wake up sub-group 4 | 4 | 0.0119 |
| Only to wake up sub-group 5 | 5 | 0.0119 |
| Only to wake up sub-group 6 | 6 | 0.0119 |
| Only to wake up sub-group 7 | 7 | 0.0119 |
| Only to wake up sub-group 8 | 8 | 0.0119 |
| All sub-groups go to sleep | 9 | 0.9 (1 - per PO paging rate) |

Observation 1: Sub-grouping method introduced in Rel-16 NB-IoT can save the number of candidate sequences by defining a “common sequence” representing the case that no less than two sub-groups need to be paged.

Observation 2: The additional false alarm rate and power consumption caused by the common sequence is marginal.

Observation 3: Based on the sub-grouping method introduced in Rel-16 NB-IoT, the network only needs to transmit one certain sequence for each PEI occasion to indicate its associated PO(s).

As discussed above, for sub-grouping indication, the grouping method by defining N sub-group specific sequence and 1 common sequence (i.e., representing the case that no less than two sub-groups need to be paged) is effective, which is similar as the UE grouping method specified for Rel-16 NB-IoT Group WUS. N refers to the total number of sub-groups. Additionally, a UE only need to correlate two sequences, i.e., one is the UE’s sub-group specific sequence and the other is the common sequence. Hence, the number of sequences required to detect by each UE does not increase with the sub-group number, and the performance of sequence-based PEI will not degrade meanwhile.

Proposal 1: Adopt the sequence-based grouping method introduced in Rel-16 NB-IoT as described in Table 1. With this method, UE only needs to detect the following two sequences of PEI per PEI occasion, if sub-groups are configured.

- The sub-group specific sequence, to indicate only the sub-group which the UE belongs to receive paging, and

- The common sequence, to indicate no less than two sub-groups to receive paging.

  - 2.2 Sub-grouping indication carried by PEI-only

Compared with the scheme of PEI without sub-grouping, the additional power saving gain by introducing sub-grouping paging indication in PEI is presented in Table 2. Based on the assumption shown in Table 1, up to 5.35% additional power saving gain can be achieved by introducing the sub-grouping paging indication with PEI when the per PO paging rate is 10%. Clearly, the larger number of the sub-groups can be divided into, the more power saving gain is achieved. And the higher per PO paging rate, the more power saving gain can be achieved.

Table 2: Additional power saving gain by introducing sub-grouping paging indication carried by PEI compared with the scheme of PEI without sub-grouping.

| Additional power saving gain | The number of sub-groups | For Low SINR case with 3 SSBs before PO | For High SINR case w/o RRM relax |
| --- | --- | --- | --- |
| Per PO paging rate=10% | 2 | 2.80% | 0.90% |
|  | 4 | 4.26% | 1.37% |
|  | 8 | 5.01% | 1.62% |
| Per PO paging rate=20% | 2 | 5.11% | 1.71% |
|  | 4 | 7.94% | 2.66% |
|  | 8 | 9.36% | 3.14% |
| Per PO paging rate=40% | 2 | 8.59% | 3.08% |
|  | 4 | 13.75% | 4.93% |
|  | 8 | 16.59% | 5.95% |

Observation 4: Up to 5.01% or 9.36% additional power saving gain can be achieved by introducing the sub-grouping paging indication carried by PEI-only when the per PO paging rate is 10% or 20% respectively.

Proposal 2: Sub-grouping indication only carried in PEI should be supported for paging enhancement.

  - 2.3 Sub-grouping indication carried by paging PDCCH

From the agreed evaluation results, sub-grouping indication by using paging PDCCH can only obtain 1% power saving gain when the per PO paging rate is 10%. It is negligible benefit to the paging enhancement. Hence, the sub-grouping indication by paging PDCCH should be excluded. Besides, to avoid ambiguity understanding and further save the cost for cross-layer communication, we need to reply the LS [3] from RAN2 to declare that from RAN1 perspective the sub-grouping indication by paging PDCCH should not be considered.

Proposal 3: The sub-grouping indication by using paging PDCCH should be excluded. And reply the LS sending from RAN2 as follow:

- From RAN1 perspective, the sub-grouping indication by using paging PDCCH should not be supported.

- Behavior A/B

  - 3.1 Analysis of resource overhead and power saving gain

As evaluated in our companion contribution [4], for the purpose of CFO (carrier frequency offset) compensation, AGC and T/F tracking and RRM measurement etc., an idle/inactive UE needs to receive 2 or 3 SSB bursts before PO reception in Low/medium SINR case and 1 SSB burst before PO reception in High SINR case. Figure 1 describes the timeline of behaviors for Rel-16 UEs in idle state, including SSB and paging reception.

Figure 1: The paging processing timeline of the baseline scheme

In RAN1 #103e-meeting, PEI before paging occasion has been agreed from RAN1 perspective and then two candidate UE behaviors for PEI detection were provided in RAN1 #104e-meeting. The difference is whether a UE is required to monitor PO when it does not detect PEI. And for Behavior A, the UE will not do PO reception when it does not detect PEI, but the opposite for Behavior B.

So, by adopting Behavior B, if a UE is not required to receive PO, the network shall always transmit PEI with no paging indication explicitly in order to save power.

Figure 2 describes the timeline of UE behaviors when PEI is introduced for Behavior A and B respectively. For Behavior A, PEI is NOT needed to be transmitted for 90% probability from gNB perspective, so that system overhead is largely saved. However, for Behavior B, PEI is needed to be transmitted for 90% probability from gNB perspective.

Figure 2: The paging processing timeline of Behavior A/B (assuming PO rate =10%)

Table 3: Power saving gain compared to baseline paging schemes when the per PO paging rate is higher than 50%

|  |  |  | Power saving gain compared to baseline paging schemes |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Per PO paging rate=50% | Per PO paging rate=60% | Per PO paging rate=80% |
| Gap [1] =3ms | Low SINR case |  | 11.27% | 8.10% | 1.88% |
|  | Medium SINR case |  | 5.25% | 3.17% | -0.90% |
|  | High SINR case |  | -3.70% | -5.07% | -7.79% |
| Gap =0 | Low SINR case |  | 14.55% | 11.35% | 5.08% |
|  | Medium SINR case |  | 9.04% | 6.92% | 2.80% |
|  | High SINR case |  | 2.40% | 1.00% | -1.76% |
| Overall power saving gain in the system |  | Gap=3ms | 1.979% | 0.036% | -3.789% |
|  |  | Gap =0 | 6.822% | 4.846% | 0.976% |
| Note: [1] refers to the gap between PEI and the nearest SSB. |  |  |  |  |  |

Before discuss Behavior A and B, what needs to be mentioned is that as shown in Table 3 when per PO paging rate is higher than 50%, there is almost no or even negative power saving gain for High SINR case and Medium SINR case. Hence, the UEs under High SINR or Medium SINR condition are not expected to do PEI monitoring since there is no power saving benefit when per PO paging rate is higher than 50%. Exemplary, we might assume a possible ratio of High SINR case to Medium SINR case to Low SINR case in the system is 5: 3: 2. Then, the overall power saving gain can be calculated as presented in Table 3, which further proves that it is unnecessary for the system to configure PEI when per PO paging rate is high.

Observation 5: When per PO paging rate is higher than 50%, there is almost no or even negative power saving gain for High SINR case and Medium SINR case.

Proposal 4: PEI makes no power saving benefit in the case of per PO paging rate is higher than 50%.

To comprehensively comparison Behavior A and B, we adopt the control variate method with respect to the two variables i.e., resource overhead and power saving gain. Note that dynamic rate-matching in PDCCH or PDSCH to co-exist with legacy signals/channels is assumed. As shown in Table 4, the results of resource overhead are given by assuming the same power saving gain can be achieved of Behavior A and B. And the power saving gain compared to baseline scheme is obtained while ensuring the same resource overhead between Behavior A and Behavior B. In addition, multiple POs associated with one PEI is also considered in our evaluation. The per PO paging rate is configured as 10%.

By observing the results of resource overhead, Behavior B will consume 64.6%~80% more PEI resources than Behavior A. From power saving perspective, Behavior B will result in more power consumption even than baseline scheme, and it is even inferior to Behavior A, under the same assumption of resource overhead. The reasons are that the introducing of PEI will increase the power consumption when UE need to receive paging compared to the baseline scheme. And for the sake of saving resource, the network would not send PEI, which will sacrifice the power saving gain by using PEI if UE is not paged. Hence, there is no benefit if UE behavior B is adopted, from both network resource overhead and UE power saving perspective.

Table 4: Comparison of Behavior A and Behavior B

| PEI design | Resource overhead [4] |  | Power saving gain (* compared to baseline paging scheme and based on the same resource overhead of Behavior A) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 to 1 [1] | 4 to 1 [2] | Cases |  | Gap[3] =3ms | Gap =0 |
| Behavior A | Network needs to transmit PEI in only 10% cases | Network needs to transmit PEI in only 34.4% cases | Low SINR |  | 24.43% | 27.80% |
|  |  |  | Medium SINR |  | 13.93% | 17.84% |
|  |  |  | High SINR |  | 1.89% | 8.06% |
| Behavior B | Network needs to transmit PEI in 90% cases | Network needs to transmit PEI in 99% cases | Low SINR | 1 to 1 | -6.97% | -3.60% |
|  |  |  |  | 4 to 1 | 2.61% | 5.98% |
|  |  |  | Medium SINR | 1 to 1 | -9.72% | -5.81% |
|  |  |  |  | 4 to 1 | -2.51% | 1.40% |
|  |  |  | High SINR | 1 to 1 | -12.02% | -5.84% |
|  |  |  |  | 4 to 1 | -7.78% | -1.60% |
| Notes: [1] Each PO associated with one PEI. [2] Four POs associated with one PEI. [3] The gap between PEI and the nearest SSB. [4] Dynamic rate-matching in PDCCH or PDSCH to co-exist with legacy signals/channels is assumed. |  |  |  |  |  |  |

Observation 6: Behavior B will consume 64.6%~80% more PEI resources than Behavior A when assuming the same power saving gain achieved by Behavior A/B when PO rate is 10%.

Observation 7: Behavior B will result in more power consumption than Behavior A, and it is even worse compared with baseline paging scheme, when assuming its resource overhead is the same as that of Behavior A.

For both Behavior A and Behavior B, there is no doubt that PEI reception will increase the power consumption when UE need to monitor paging reception. But the power saving gain by using PEI derives from the most cases that PEI will indicate UE there is no paging. Hence, for Behavior B, the network shall transmit PEI with no paging indication to guarantee the total power consumption of it will at least no larger than that of baseline paging scheme i.e., no PEI is configured. To this end, we evaluate the critical probability of transmitting PEI with no paging indication for Behavior B to achieve the same total power consumption with baseline paging scheme. As shown in Table 5, the critical probability is 79%, which means the network shall guarantee the probability of sending PEI with no paging indication is larger than 79%. Otherwise, there is no power saving gain achieved by using Behavior B.

Table 5:  The critical probability of sending PEI with no paging indication (i.e. the value of X) for Behavior B to achieve the same total power consumption with baseline paging scheme

| Different cases |  | Average power consumption per slot |  |  |  | The value of X, i.e. the probability of sending PEI to indicate go to sleep by network |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline i.e., no PEI is configured go to sleep (90%) ¦ wake up (10%) |  | PEI scheme PEI with go to sleep indication (X) ¦ PEI with wake-up indication or without sending PEI (1-X) |  | Paging rate = 10% | Paging rate = 20% | Paging rate = 40% |
| Gap between PEI and the nearest SSB is 3ms | Low SINR | 3.035 | 3.254 | 2.19 | 3.39 | 28% | 26% | 22% |
|  | Medium SINR | 2.613 | 2.832 | 2.19 | 2.969 | 43% | 40% | 35% |
|  | High SINR | 1.663 | 1.718 | 1.608 | 1.898 | 79% | 77% | 74% |
| Gap between PEI and the nearest SSB is 0 | Low SINR | 3.035 | 3.254 | 2.087 | 3.287 | 19% | 18% | 14% |
|  | Medium SINR | 2.613 | 2.832 | 2.087 | 2.866 | 30% | 27% | 21% |
|  | High SINR | 1.663 | 1.718 | 1.505 | 1.795 | 44% | 42% | 38% |

Observation 8: For Behavior B, the network shall guarantee the probability of sending PEI with go to sleep indication is larger than 79%, otherwise there is no power saving benefit to configure PEI for idle/inactive UE.

Proposal 5: Only Behavior A should be adopted for UE behavior of PEI detection.

Besides, on the basic of that only behavior A should be adopted for UE behavior of PEI detection, it will be no doubt that UE is not required to monitor the target PO if UE does not detect any PEI from all associated PEI MO(s) for the target PO.

Proposal 6: Adopt that UE is not required to monitor the target PO if UE does not detect any PEI from all associated PEI MO(s) for the target PO.

- PEI signal/channel designs

In this section, the performance and resource overhead evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS have been given based on the assumptions agreed in the last e-meeting.

  - 4.1 Performance of PEI signals/channels

- Sequence-based PEI

As specified for Rel-15 NB-IoT/eMTC wake up signal, sequence based signaling has been adopted for PEI.

For NR, SSS-like sequence is one candidate solution for paging early indication signal. Multiplexing with existing channels and signals, e.g. PDSCH/PDCCH, CSI-RS, etc., should be defined if SSS-like sequence is introduced. CSI-RS is already existing signal, and the multiplexing rule with other channels has already been defined in Rel-15. However, TRS/CSI-RS for power saving purpose may be different with the existing TRS/CSI-RS configuration, thus additional overhead is needed.

As mentioned earlier, SSB and TRS (multiple CSI-RS resources) can be used for carrier frequency error calibration. If SSS-like sequence or TRS-like sequence are also supported for PEI, UE can perform CFO calibration based on the PEI sequence itself. Considering this benefit own to sequence-based PEI, the SSB measurement before PO reception can be replaced/assisted by PEI. Thus, higher power saving gain can be achieved by supporting sequence based PEI, compared with DCI based PEI.

The premise for CFO calibration for sequence-based PEI is that it should be transmitted in more than 1 symbols, and UE can estimate the frequency error based on the phase difference among the RS symbols. The sequence pattern can be carefully designed to facilitate CFO calibration. Furthermore, with more symbols allocated for PEI, the reliability of PEI can be further improved. An example is shown in Figure 3.

Figure 3: Illustration of PEI mapped to multiple symbols

For sequence-based PEI, co-existence with legacy UEs should be considered. SSS like sequence is mapped consecutive in frequency domain, while TRS-like sequence is mapped in comb pattern and mapped to wide bandwidth. However, gNB can properly schedule the resources to make sure the resource for the PEI sequence is orthogonal to legacy signals and channels. If the PEI is not transmitted due to no paging arrived, network can schedule the resources for other purposes dynamically. Besides, network can configure rate matching pattern for UE to rate matching around the resources for PEI. Hence, sequence-based PEI can be well co-existed with legacy signals/channels.

Observation 9: Sequence-based PEI can be well co-existed with legacy signals/channels.

When sub-grouping indication is delivered through sequence-based PEI, there should be at least one candidate sequence associated with all of sub-groups. According to the grouping method described in section 2.1, UE should wake up to receive paging PDCCH, if either the sequence associated with the UE subgroup or the common sequence is detected. Hence, each UE only need to correlate two sequences, although N+1 sequences are required from network’s perspective, where N refers to the number of sub-groups. In this regard, the performance of sequence-based PEI will not degrade with the increasing of number of subgroups.

Observation 10: Performance of sequence-based PEI will not degrade with the increasing of number of subgroups, if a common sequence is introduced to wake up no less than two sub-groups.

- DCI-based PEI

DCI-based PEI can be used to indicate UE whether to monitor PDCCH in the subsequent PO, which is similar to DCI format 2-6 for C-DRX in Rel-16. Compared with sequence based signaling, more power consumption is needed for PDCCH decoding even if blind decoding on multiple PDCCH candidates is not considered, since channel estimation, detection, and polar decoding is needed for one blind decoding attempt.

For PDCCH, the minimum DCI size is 12bits. To obtain better PEI performance, it is preferred to limit the DCI size for PEI. However, in current mechanism, an idle UE only needs to decoding one DCI size, i.e. DCI 1-0/0-0 size, which is typically around 40bits. If a PDCCH with new DCI size needs to be monitored by idle UE, it may increase the power consumption. However, if the DCI size of DCI-based PEI is aligned with DCI format 1-0/0-0, it may degrade reliability of paging.

Observation 11: If the DCI size of DCI based-PEI is less DCI format 1-0/0-0, UE need to decode two DCI size in idle state. If the DCI size of DCI based-PEI is aligned with DCI format 1-0/0-0, the performance of DCI-based PEI is less reliable.

We evaluated the performance of sequence-based and DCI-based PEI. Non-coherent detector is used for sequence detection, UE performs non-coherent detection for two sequences, i.e., the sequence for UE subgroup and the common sequence. If the correlation value is greater than certain threshold, UE considers the sequences detected. The threshold is selected to ensure that false alarm rate is lower than 0.01/0.001.

For TRS-like PEI, the TRS resource in 48 RBs (initial DL BWP size) is segmented to 8 subbands, and each subband contains 6 RBs, the sequence correlation is performed per subband basis, and correlation values of the 8 subbands are summed up and compared with the threshold to determine whether the sequence is transmitted.

Based on the agreements, joint MDR for paging PDCCH and MDR of PEI under both Behavior A&B are provided in Figure 4. And the simulation parameters are given in Table 6.

Table 6 Simulation Assumptions for PEI

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C 300ns 100Hz |
| Antenna number at BS | 1 |
| Antenna number at UE | 4 |
| Subcarrier spacing | 30 kHz |
| SSS like sequence-based PEI | Length: 127 (REs per symbol) Sequence: same as SSS Number of symbols: 1 or 2 or 4 |
| TRS/ CSI-RS like sequence-based PEI | Number of RBs: 48 Number of symbols: - 1 symbol in a slot - 2 symbols in a slot - 4 symbols in 2 consecutive slots Density: 3 REs/RB |
| PDCCH based PEI | Aggregation level: [4] or 8 or 16 DCI size: 12 (minimum) or 40 bits (DCI format 1-0) |
| CFO for signal/channels | Uniformly distributed in [-X X] ppm X=0.5 for PEI seq & PDCCH X=0.1 for paging PDCCH and PDSCH |
| Note: For PDCCH based PEI, the false alarm rate is extremely low, and zero false alarm rate is assumed. |  |

- Alt-1 Behaviour -A                                                          (b) Alt-1 Behaviour-B

- Alt-2 Behaviour -A                                                          (d) Alt-2 Behaviour-B

Figure 4: Performance of PEI signals and channels

As shown in Figure 4, for evaluation metric Alt-1 of Behavior A, if joint MDR performance of paging PDSCH with MCS#0 w/o TB scaling is considered as benchmark, all PEI options are qualified. Note that PDCCH-based PEI with 4 AL cannot be applied because of its very poor performance. While, if compared with perfoemance of PDSCH with MCS#0 w/ TB scaling=0.25, no PEI signals/channels are qualified. For joint MDR of Behavior B, DCI-based PEI outerperforms sequence based PEI due to almost zero FAR.

For Alt-2, i.e. to compare the MDR of PEI directly, the performance difference will be more obvious compared with that in Alt-1. Compared with paging PDCCH with AL=16, SSS-like or TRS-like PEI with 2 or 4 symbols and PDCCH with AL16, are quanlified PEI design options.

Observation 12: The performance of SSS-like based PEI and TRS-like based PEI outperforms that of DCI-based PEI.

  - 4.2 Power saving gain

Another crucial metric for the comparison of the two types of PEI is power saving gain. As previously discussed in our companion paper [4], the residual CFO is expected to be limited within about -/+ 0.1ppm, which is -/+ 400Hz for 4GHz carrier frequency. Similar to paging PDCCH, if a DCI-based PEI is introduced, UE need to process one or more SSBs before it for correcting the frequency error. As shown in Figure 5, DCI-based PEI should be placed after an SSB burst. While for sequence-based PEI such as TRS-based PEI which has better detection performance and measurement function itself, UE can replace SSB reception by TRS-based PEI and less power will be consumed per DRX cycle. As such, compared to DCI-based PEI, adopting sequence-based PEI such as TRS-liked based PEI will bring more significant power saving gain as shown in Table 7. In addition, to capture the whole picture of the power saving gain, Table 8 obviously shows the obtained power saving gain by using different PEI candidate designs compared to the baseline scheme (as shown in Figure 1). And it is note that Table 8-b is presented by adopting the table form agreed in the last meeting for power saving gain comparison.

Note that we assume no sub-grouping during the following evaluation. And the gap between the nearest SSB and PEI is assumed as 3ms. There is one SSB burst assumed before PEI detection. And the number of SSB burst between PEI and PO when UE is paged is: For DCI-based PEI, two for low SINR case, one for medium SINR case and zero for high SINR case; For sequence-based PEI, zero for low/medium/high SINR case.

Figure 5: The paging processing timeline of different PEI candidate designs.

Table 7: The additional power saving gain by comparing TRS-based PEI vs DCI-based PEI

| Cases | Average power consumption per slot |  |  |  | Additional power saving gain (TRS-based PEI vs DCI-based PEI) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TRS-based PEI (no paging/paging) |  | DCI-based PEI (no paging/paging) |  | PO paging rate: 10% | PO paging rate: 20% | PO paging rate: 40% |
| Low SINR | 2.15 | 2.47 | 2.19 | 3.39 | 5.54% | 8.89% | 14.68% |
| Medium SINR | 2.15 | 2.47 | 2.19 | 3.10 | 4.34% | 6.66% | 10.81% |
| High SINR | 1.61 | 1.84 | 1.61 | 2.05 | 1.38% | 2.57% | 4.77% |

Table 8-a: The power saving gain by adopting three different PEI candidate compared to baseline

| Cases |  | Average power consumption per slot |  |  |  | Additional power saving gain (Three PEI candidate designs vs baseline scheme) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | With PEI (no paging/paging) |  | Baseline scheme i.e., without PEI (no paging/paging) |  | PO paging rate: 10% | PO paging rate: 20% | PO paging rate: 40% |
| TRS or SSS-based PEI | Low SINR | 2.15 | 2.47 | 3.035 | 3.254 | 28.62% | 28.09% | 27.05% |
|  | Medium SINR | 2.15 | 2.47 | 2.769 | 2.988 | 21.82% | 21.29% | 20.25% |
|  | High SINR | 1.61 | 1.84 | 1.817 | 1.873 | 10.50% | 9.51% | 7.54% |
| DCI-based PEI | Low SINR | 2.19 | 3.39 | 3.035 | 3.254 | 24.43% | 21.07% | 14.49% |
|  | Medium SINR | 2.19 | 3.1 | 2.769 | 2.988 | 18.27% | 15.67% | 10.59% |
|  | High SINR | 1.61 | 2.05 | 1.817 | 1.873 | 9.25% | 7.12% | 2.90% |

Table 8-b: Summary of average power saving gains for the comparison of PEI candidate designs

| UE subgroups in a PO | PEI candidate type (PDCCH, SSS, TRS/CSI-RS) | #SS burst(s) before PO in Rel-16 baseline | PO paging rate | Power Saving Gain | Assumed #SS burst(s) before PEI | Assumed #SS burst(s) between PEI and PO when UE is paged |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PDCCH | 1 | 10% | 9.25% | 1 | 0 |
|  |  |  | 20% | 7.12% | 1 | 0 |
|  |  |  | 40% | 2.90% | 1 | 0 |
|  |  | 2 | 10% | 18.27% | 1 | 1 |
|  |  |  | 20% | 15.67% | 1 | 1 |
|  |  |  | 40% | 10.59% | 1 | 1 |
|  |  | 3 | 10% | 24.43% | 1 | 2 |
|  |  |  | 20% | 21.07% | 1 | 2 |
|  |  |  | 40% | 14.49% | 1 | 2 |
|  | SSS or TRS/CSI-RS (same results) | 1 | 10% | 10.50% | 1 | 0 |
|  |  |  | 20% | 9.51% | 1 | 0 |
|  |  |  | 40% | 7.54% | 1 | 0 |
|  |  | 2 | 10% | 21.82% | 1 | 0 |
|  |  | 2 | 20% | 21.29% | 1 | 0 |
|  |  |  | 40% | 20.25% | 1 | 0 |
|  |  | 3 | 10% | 28.62% | 1 | 0 |
|  |  |  | 20% | 28.09% | 1 | 0 |
|  | SSS or TRS/CSI-RS (same results) | 1 | 40% | 27.05% | 1 | 0 |

Observation 13: Up to 14.68% more power saving gain can be achieved by using sequence-based PEI, compared with DCI-based PEI.

Observation 14: Sequence-based PEI is superior to DCI-based PEI in terms of power saving gain, compared to baseline paging scheme.

Proposal 7: Capture the power saving gain results given in Table 8-b for the comparison of PEI candidate designs.

  - 4.3 Resource overhead

For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, we provide the evaluation results for PEI candidate designs based on the following assumptions:

- Assumption of Behavior A/B;

- Required #REs from performance evaluation described in section 3.1;

- Per PO paging rate is 10%;

- Dynamic rate-matching in PDCCH or PDSCH to co-exist with legacy signals/channels;

- PO and PEI can be one to one mapping or four to one mapping;

- As identified and analyzed in section 2.1, 8 UE sub-groups are configured with PEI-only for each PO and sequence-based grouping method introduced in Rel-16 NB-IoT is adopted;

As we debated in the last e-meeting for coexistence between different PEI candidate designs and legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1. When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource. Additionally, we strongly hold the view that the network can definitely avoid the collision between PEI and other signals/channels especially for light load system (i.e., per PO paging rate is 10% or 20%). When the system is under a heavy load e.g. per PO paging rate is higer than 50%, the configuration of PEI seems not necessary for power saving purpose, so the collision handling by gNB in those cases need no consideration.

As per the assumptions above, Table 9 shows the two sets of resource overhead (unit: RE) with different PEI candidate designs. Please note that the resource overhead is for per PEI occasion and per PO. One set of the resource overhead is a baseline configuration with both one to one mapping for DCI-based PEI and sequence-based PEI. Another set is given due to the consideration for the different capacity of sequence and DCI. For fairness, we assume one to one mapping for PO and sequence-based PEI, and 2 POs associated with one DCI-based PEI. Additionally, the DCI size with 16 bits can be configured with a bitmap to indicate all sub-groups respectively (i.e., 8 sub-groups per PO*2 POs = 16 sub-groups).

Given the results below, it can be observed that even if 2 POs associated with one PEI is configured, DCI-based PEI design still consume the most resource. Besides, Behavior B will consume more PEI resources than Behavior A, which has been proved in section 3.1.

Table 9: Resource overhead of each PEI occasion with different PEI candidate designs

| PEI design | Parameters | Baseline: Resource overhead for per PEI occasion and per PO (unit: RE) PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 1 to 1 for DCI based PEI; |  | Resource overhead for per PEI occasion and per PO (unit: RE) PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 2 to 1 for DCI based PEI; |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Behavior A | Behavior B | Behavior A | Behavior B |
| SSS like sequence | - Length: 127 - Sequence: same as SSS - Number of symbols (symbol): 2 or 4 | 25.4/50.8 for 2/4 symbols | 228.6/457.2 for 2/4 symbols | 25.4/50.8 for 2/4 symbols | 228.6/457.2 for 2/4 symbols |
| TRS like sequence | - Number of RBs: 48 - Number of symbols (symbol): - 2 symbols in a slot - 4 symbols in 2 slots - Density: 3 REs/RB | 28.8/57.6 for 2/4 symbols | 259.2/518.4 for 2/4 symbols | 28.8/57.6 for 2/4 symbols | 259.2/518.4 for 2/4 symbols |
| DCI | - Aggregation level (AL): 8 or 16 - DCI size: 12 (minimum) or 40 bits (align with DCI 1-0) | 57.6/115.2 for AL 8/16 | 518.4/1036.8 for AL 8/16 | 54.72/109.4 for AL 8/16 | 466.56/933.12 for AL 8/16 |

Observation 15: In term of resource overhead for per PEI occasion and per PO, SSS-like sequence-based PEI will consume the least resource, TRS-like sequence-based PEI followed and DCI-based PEI comes last.

Given the analyses for coexistence issue of different PEI candidate designs in the last several meetings, there are still many concerns shown by companies on whether PEI can well coexist with legacy signals/channels. For DCI-based PEI, the current specification can support to do dynamic rate-matching, while for sequence-based PEI especially for SSS-like PEI, companies cannot reach the consensus for whether dynamic rate-matching can be applied or not. However, it is no doubt that semi-static rate-matching method is applicable for all PEI candidate designs. In this regard, we further perform the following resource overhead evaluation by means of the above two rate-matching methods. The related assumptions are given below.

- The paging cycle is 1280ms.

- The system bandwidth is 100M or 20M with 30KHz SCS.

- The PEI and PO mapping manner is assumed as 1 to 1 and 1 to 2.

- Assume that each PF contains one PO.

- one PEI occasion for each PEI.

- The total number of PO for one paging cycle is 128.

- The total number of RE for both 100M and 20M bandwidths are 117,411,840 and 23,482,368 respectively.

Table 10: The resource overhead ratio to the overall system resource of the three PEI candidate designs by using different rate-matching methods.

| PEI design | Rate matching methods | Baseline: resource overhead ratio for 100MHz/20MHz PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 1 to 1 for DCI based PEI; |  | Resource overhead ratio for 100MHz/20MHz PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 2 to 1 for DCI based PEI; |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Behavior A | Behavior B | Behavior A | Behavior B |
| SSS like sequence for 2 symbols | Dynamic rate matching | 0.0028%/0.014% | 0.025%/0.125% | 0.0028%/0.014% | 0.025%/0.125% |
|  | Semi-static rate matching | 0.028%/0.14% | 0.028%/0.14% | 0.028%/0.14% | 0.028%/0.14% |
| TRS like sequence for 2 symbols | Dynamic rate matching | 0.0031%/0.016% | 0.028%/0.14% | 0.0031%/0.016% | 0.028%/0.14% |
|  | Semi-static rate matching | 0.031%/0.16% | 0.031%/0.16% | 0.031%/0.16% | 0.031%/0.16% |
| DCI for AL 8 | Dynamic rate matching | 0.0063%/0.032% | 0.057%/0.285% | 0.006%/0.03% | 0.051%/0.26% |

As shown in Table 10, the resource overhead ratio to the overall system resource is marginal for all the PEI candidate design no matter which rate matching method is adopted. Even when the dynamic rate matching is using for DCI-based PEI and the semi-static rate matching method is using for sequence-based PEI, the difference of resource overhead ratios between them will be negligible. Therefore, resource overhead and coexistence are not the critical factors for the final decision on PEI physical-layer channel/signal, compared with the performance and power saving.

Observation 16: The difference of between DCI-based PEI and sequence-based PEI in the proportion of resource overhead to the overall system resource is negligible, no matter adopting dynamic rate-matching or semi-static rate-matching method.

Observation 17: Compared to performance and power saving, the resource overhead and coexistence are not the critical factors for the final decision on PEI physical-layer channel/signal.

  - 4.4 Summary of PEI signal/channel designs

To conclude, from performance perspective, SSS-like sequence and TRS-like sequence are both superior to PDCCH; And for power saving gain perspective, there is no doubt that sequence-based PEI is better than DCI-based PEI; In terms of resource overhead perspective, SSS-like sequence-based PEI requires the least physical resource compared to the other two PEI candidate design. Note that the resource overhead of two sequence-based PEI designs is very close; And for both sequence-based PEI and DCI-based PEI, there is no coexistence issue with legacy signals/channels by adopting dynamic PDCCH or PDSCH resource occupation.

What needs to be illustrated is that, the resource overhead and coexistence are not the critical factors for the final decision on PEI physical-layer channel/signal, since no significant difference for the whole system among three PEI candidate designs as our analyzed in section 4.3.

Intuitively, Table 11 summarizes the results of the three PEI candidate designs from aforementioned metrics, in which the red represents ‘bad’ result, the yellow represents ‘good’ result and the green represents ‘excellent’ result. Hence, it is no doubt that sequence is preferred for PEI design comprehensively.

Table 11: Summary of different PEI candidate designs

| PEI designs Metrics | SSS-like sequence-based PEI | TRS-like sequence-based PEI | DCI-based PEI |
| --- | --- | --- | --- |
| Performance (Alt1-behavior A) Critical factor 1 | SSS 1 symbol | TRS 1 symbol | PDCCH AL 8 (40bits); PDCCH AL 4 (12 or 40 bits) |
|  |  |  | PDCCH AL 8 (12bits) |
|  | SSS 2, 4 symbols | TRS 2, 4 symbols | PDCCH AL 16 (12 or 40 bits) |
| Performance (Alt2-behavior A) Critical factor 1 | SSS 1 symbol | TRS 1 symbol | PDCCH AL 16 (40bits) PDCCH AL 8 (12 or 40bits); PDCCH AL 4 (12 or 40 bits) |
|  | SSS 2 symbols | TRS 2 symbols | PDCCH AL 16 (12bits) |
|  | SSS 4 symbols | TRS 4 symbols |  |
| Power saving gain (among different PEIs) Critical factor 2 | Up to 14.68% more power saving gain can be achieved by sequence-based PEI |  | - |
| Resource overhead (behavior A) sub-critical factor 3 | 25.4/50.8 for 2/4 symbols for 1 PO to 1 PEI mapping | 28.8/57.6 for 2/4 symbols for 1 PO to 1 PEI mapping | 57.6/115.2 for AL 8/16 for 1 PO to 1 PEI mapping; |
|  |  |  | 54.72/109.4 for AL 8/16 for 2 POs to 1 PEI mapping |
| Coexistence Sub-critical factor 4 | Semi-static or dynamic PDSCH occupation | Semi-static or dynamic PDSCH occupation | Semi-static or dynamic PDCCH occupation |

Proposal 8: Sequence (i.e., SSS-like or TRS-like) should be adopted for PEI design.

- Other considerations for PEI signal/channel designs

  - 5.1 Configuration of PEI occasion

Figure 6: The configuration for PEI occasion.

Regarding the configuration of PEI occasion, the gap between PEI and the first indicated PO should be considered to contain M SSB burst(s) for the purpose of doing some preparation work before PO reception. As shown in Figure 6, if PEI is configured between the first and second SSB burst, then the minimum gap needs to contain one or two residual SSB burst(s) before PO reception for T\F tracing and/or RRM measurement purpose.

Proposal 9: The configuration of PEI occasion should satisfy that the gap between PEI and the first indicated PO contains M SSB bursts, where the value of M can be 1, 2, 3 etc.

  - 5.2 TRS availability

As discussed in our companion paper [5], the availability indication for TRS/CSI-RS can also be transmitted in PEI, if connfigured. However, PEI for paging reception and TRS/CSI-RS are decoupled features for UE power saving, if the support/configuration for PEI is the precondition for the vailability indication for TRS/CSI-RS, which means TRS/CSI-RS availability can not be indicated when PEI is not configured. For an instance, some R17 UE may not support PEI feature, but it has the capability of supporting TRS/CSI-RS feature, thus TRS/CSI-RS availablity indication carried by PEI is unflexible and restricted for some cases. Idle/inactive UEs may need to support different implementations with or without PEI and with/without TRS/CSI-RS availability indication when PEI is configured, leading to additional complexity for UE implementation. Besides, whether the indication can be delievered in PEI also depends on the signal/channel design of PEI. If the PEI is a sequence, the number of bits can be delievered through a sequence is limited. Similarly, whether SI update indication can be also delievered in PEI should be further studied after the PEI signaling/channel design is settled.

Observation 18: TRS/CSI-RS availability indication through PEI is not unified solution since PEI and TRS/CSI-RS for idle/inactive UEs are decoupled features for UE power saving.

Proposal 10: Defer the discussion for whether TRS/CSI-RS availability indication can be carried by PEI until the PEI candidate design is settled.

- Conclusion

In this contribution, we have discussed sub-grouping indication and PEI candidate physical signal/channel design from the aspects of Behavior A/B, power saving gain, resource overhead and coexistence, with the following observations and proposals:

Observation 1: Sub-grouping method introduced in Rel-16 NB-IoT can save the number of candidate sequences by defining a “common sequence” representing the case that no less than two sub-groups need to be paged.

Observation 2: The additional false alarm rate and power consumption caused by the common sequence is marginal.

Observation 3: Based on the sub-grouping method introduced in Rel-16 NB-IoT, the network only needs to transmit one certain sequence for each PEI occasion to indicate its associated PO(s).

Observation 4: Up to 5.01% or 9.36% additional power saving gain can be achieved by introducing the sub-grouping paging indication carried by PEI-only when the per PO paging rate is 10% or 20% respectively.

Observation 5: When per PO paging rate is higher than 50%, there is almost no or even negative power saving gain for High SINR case and Medium SINR case.

Observation 6: Behavior B will consume 64.6%~80% more PEI resources than Behavior A when assuming the same power saving gain achieved by Behavior A/B when PO rate is 10%.

Observation 7: Behavior B will result in more power consumption than Behavior A, and it is even worse compared with baseline paging scheme, when assuming its resource overhead is the same as that of Behavior A.

Observation 8: For Behavior B, the network shall guarantee the probability of sending PEI with go to sleep indication is larger than 79%, otherwise there is no power saving benefit to configure PEI for idle/inactive UE.

Observation 9: Sequence-based PEI can be well co-existed with legacy signals/channels.

Observation 10: Performance of sequence-based PEI will not degrade with the increasing of number of subgroups, if a common sequence is introduced to wake up no less than two sub-groups.

Observation 11: If the DCI size of DCI based-PEI is less DCI format 1-0/0-0, UE need to decode two DCI size in idle state. If the DCI size of DCI based-PEI is aligned with DCI format 1-0/0-0, the performance of DCI-based PEI is less reliable.

Observation 12: The performance of SSS-like based PEI and TRS-like based PEI outperforms that of DCI-based PEI.

Observation 13: Up to 14.68% more power saving gain can be achieved by using sequence-based PEI, compared with DCI-based PEI.

Observation 14: Sequence-based PEI is superior to DCI-based PEI in terms of power saving gain, compared to baseline paging scheme.

Observation 15: In term of resource overhead for per PEI occasion and per PO, SSS-like sequence-based PEI will consume the least resource, TRS-like sequence-based PEI followed and DCI-based PEI comes last.

Observation 16: The difference of between DCI-based PEI and sequence-based PEI in the proportion of resource overhead to the overall system resource is negligible, no matter adopting dynamic rate-matching or semi-static rate-matching method.

Observation 17: Compared to performance and power saving, the resource overhead and coexistence are not the critical factors for the final decision on PEI physical-layer channel/signal.

Observation 18: TRS/CSI-RS availability indication through PEI is not unified solution since PEI and TRS/CSI-RS for idle/inactive UEs are decoupled features for UE power saving.

Proposal 1: Adopt the sequence-based grouping method introduced in Rel-16 NB-IoT as described in Table 1. With this method, UE only needs to detect the following two sequences of PEI per PEI occasion, if sub-groups are configured.

- The sub-group specific sequence, to indicate only the sub-group which the UE belongs to receive paging, and

- The common sequence, to indicate no less than two sub-groups to receive paging.

Proposal 2: Sub-grouping indication only carried in PEI should be supported for paging enhancement.

Proposal 3: The sub-grouping indication by using paging PDCCH should be excluded. And reply the LS sending from RAN2 as follow:

- From RAN1 perspective, the sub-grouping indication by using paging PDCCH should not be supported.

Proposal 4: PEI makes no power saving benefit in the case of per PO paging rate is higher than 50%.

Proposal 5: Only Behavior A should be adopted for UE behavior of PEI detection.

Proposal 6: Adopt that UE is not required to monitor the target PO if UE does not detect any PEI from all associated PEI MO(s) for the target PO.

Proposal 7: Capture the power saving gain results given in Table 8-b for the comparison of PEI candidate designs.

Proposal 8: Sequence (i.e., SSS-like or TRS-like) should be adopted for PEI design.

Proposal 9: The configuration of PEI occasion should satisfy that the gap between PEI and the first indicated PO contains M SSB bursts, where the value of M can be 1, 2, 3 etc.

Proposal 10: Defer the discussion for whether TRS/CSI-RS availability indication can be carried by PEI until the PEI candidate design is settled.

References

- Chairman’s Notes of RAN1 #105e final (Online meeting, May 10th – 27th, 2021).

- R1-2009801, “LS on Paging Enhancement”, RAN1 #103-e, NR_UE_pow_sav_enh-Core, MediaTek.

- R2-2010884, “LS on Paging Enhancement”, RAN2 #112-e, NR_UE_pow_sav_enh-Core.

- R1-2100452, “Paging enhancements for idle/inactive mode UE power saving”, RAN#1 104e, vivo.

- R1-2106607, “TRS/CSI-RS occasion(s) for idle/inactive UEs”, RAN#1 106e, vivo.