3GPP TSG RAN WG1 #106-e	                                                                                       R1-2106611

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on enhancement on PUSCH repetition type A

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

- Introduction

In previous RAN1#105 e-meeting [1], the following agreements about enhancement on PUSCH repetition type A were achieved.

Agreement: - Down-selection in RAN1#106-e: - Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method, - Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots. Agreement: - In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17. - {20, 24, 28} Conclusion: - The following agreement in RAN1#104-e is applied to all slots including special slots. Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details. Agreement: - RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17 Agreement: - Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. Agreement: - If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot. Agreement: Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded) - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - Alt 1-B’ consisting of two steps - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: handling of dynamic signalling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability - Alt 2-A consisting of a single step - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signalling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Alt 2-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - FFS timeline for the dynamic signalling - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

In this contribution, we will analyze and discuss the remaining issues of enhancements on PUSCH repetition type A.

- Maximum number of repetitions

The maximum 32 repetitions has been agreed for the type-A PUSCH repetition counted on physical slots. However, there was no consensus on whether the maximum value can be applied to repetition number counted on available slots. In previous meeting, two alternatives are provided, and the main difference between the two alternatives is whether the two counting methods can be enabled at the same time, or only one can be enabled.

Agreement:

- Down-selection in RAN1#106-e:

  - Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

  - Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

However, enhancements on the maximum number of repetitions and the determination of available slots can be regarded as independent issues on enhancement on repetition type A. From flexibility perspective, restriction on the enabling of the maximum number of repetition and available slots is not necessary, which can be up to NW configuration. That is, simultaneously enabling the maximum 32 repetitions and the counting based on available slots should not be excluded.

Besides, whether the available slots are determined according to semi-static configuration and/or dynamic indication is still under discussion. It is less likely to consider all semi-static configurations and dynamic indications which lead to PUSCH dropping as not available in available slot determination mechanism. If one or more UL slots determined as available slots based on RRC signalling became unavailable due to dynamic signals, such as dynamic SFI, CI, the expected number of repetitions cannot be achieved. As a result, the maximum 16 repetitions for the counting based on available slots may be also insufficient in some scenarios. Thus, no matter the counting based on physical slots or available slots, the configured number of repetitions should be left to NW implementation and there seems no necessity to design different maximum number of repetitions due to the different counting methods. In other words, the maximum 32 repetitions should be supported by Rel-17 PUSCH repetition Type A, irrespective of counting method.

Proposal 1: For PUSCH repetition Type-A, the maximum 32 repetitions should be supported, irrespective of counting method.

In Rel-15, the maximum number of PUSCH repetition type A can be semi-statically configured to 8 via repK for configured grant PUSCH and via pusch-AggregationFactor for dynamic grant PUSCH. In Rel-16, the maximum number of PUSCH repetition type A has been increased to 16, and if numberofRepetitions-r16 is present in the resource allocation table, the number of repetitions K is equal to numberofRepetitions-r16 for configured grant PUSCH and dynamic grant PUSCH.

Since the increased maximum number of repetitions would not be a mandatory feature either in Rel-17, extension on number of repetitions for all of these RRC parameters in Rel-15 and Rel-16 is not necessary. And due to higher resource reservation, network should cautiously configure large number of repetitions in order to strike balance between coverage enhancement and system efficiency. While for dynamic indication of number of repetitions, NW can schedule the number of repetitions properly, and the impact to resource overhead can be minimized. Hence, a new RRC parameter numberofRepetitions-r17 with candidate number of repetitions {1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32} can be introduced. Similar to that for numberofRepetitions-r16, if numberofRepetitions-r17 is present in the resource allocation table, the number of repetitions is equal to numberofRepetitions-r17, for configured grant PUSCH and dynamic grant PUSCH. And a row index for the resource allocation table is dynamically indicated by TDRA field in a DCI for DG PUSCH and CG type 2 PUSCH.

Proposal 2: For PUSCH repetition Type-A, support the increased maximum number of repetitions with the repetition factor configured by only RRC parameter numberofRepetitions.

- Further increasing the maximum number for RRC parameter repK for CG-PUSCH and pusch-AggregationFactor is not supported.

- How to determine available UL slots

In the last meeting, how to determine available slots has been further discussed and 4 alternatives are provided and listed above. Alt 1-B includes RRC configurations to determine the available slots, excluding dynamic signalling; while Alt 1-B’ only considers tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, and it is FFS on how to handle dynamic signalling. Alt 2-A takes dynamic signalling and RRC configurations into consideration on the determination of the available slots; while Alt 2-B only considers dynamic SFI and RRC configurations, precluding other dynamic signalling.

For available slots determination by RRC configurations, the available slots can be derived once the configuration is provided, which does not change frequently once configured. While the PUSCH transmission can also be cancelled by dynamic signalling, e.g. SFI, UL CI, higher priority transmissions, and etc. For the schemes based on dynamic signalling, there is no HARQ-ACK for these transmissions, and NW is not aware of whether these PDCCH is received by UE. Hence, NW and UE may have different understandings on how the available slots are counted, if dynamic signalling is considered in determination of available slots. Hence, available slots for PUSCH repetition type A are preferred to be semi-statically determined, and does not depend on dynamic signalling. And there are RRC configurations other than tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, which would lead to some slots not available for PUSCH transmission, which can also be considered in available slot determination. Thus, Alt 1-B is preferable.

Proposal 3: For PUSCH repetition Type-A, Alt 1-B is preferred for determining the available slots.

In Rel-16, RRC configurations, such as TDD frame structure, SSB burst, monitoring occasions for type-0 CSS, which could cause the unavailable symbols for UL transmission are used to determine whether the slot is available for the PUSCH repetition on the scheduled consecutive slots. Similarly, the same mechanism can be used for determination of available slots. However, the motivation for determination of available slots is to provide more transmission opportunities for PUSCH repetitions. Although RAN1 has excluded DL resources configured by frame structure, SSB occasions, PDCCH monitoring occasions for type-0 CSS, there are still some RRC configurations and corresponding UE behaviours that would lead to the transmission occasion for the PUSCH repetition not available, which should be also considered for the determination of available slots.

For RRM measurement based on SSB, SMTC can be configured for UE to determine the measurement resource. The SMTC configuration is provided per frequency layer, which may cover SSB occasions from multiple serving cells in the frequency layer. Hence, the SSB occasions configured by SMTC configuration is not necessary the same as the SSBs that are transmitted by the serving cell for the UEs, as shown in Figure 1.

Figure 1. Illustration of measurement on flexible symbols with no SSB transmitted in the serving cell

In section 9.2.5.3 of TS 38.133, on the symbols for SSB occasions provided by SMTC configuration, UE is not expected to transmit PUSCH, as shown below.

##### 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

Hence, even if an SSB with certain index is not transmitted on the serving cell, and the SSB occasions overlaps with flexible symbols, UE can not transmit PUSCH on the flexible symbols if the SSB index is indicated by SMTC. The SMTC configuration is also semi-statically provided by RRC signalling, both NW and UE are aligned on the SMTC configuration and the corresponding UE behaviour.

Observation 1: The flexible symbols configured with SSB based measurement by SMTC is not available for PUSCH repetitions.

Proposal 4: SSB based measurement by SMTC  should be considered to determine the available slots.

- Conclusion

In this contribution, we analyze and discuss the relevant enhancement on PUSCH repetition type A, and have the following observations and proposals:

Observation 1: The flexible symbols configured with SSB based measurement by SMTC is not available for PUSCH repetitions.

Proposal 1: For PUSCH repetition Type-A, the maximum 32 repetitions should be supported, irrespective of counting method.

Proposal 2: For PUSCH repetition Type-A, support the increased maximum number of repetitions with the repetition factor configured by only RRC parameter numberofRepetitions.

- Further increasing the maximum number for RRC parameter repK for CG-PUSCH and pusch-AggregationFactor is not supported.

Proposal 3: For PUSCH repetition Type-A, prefer Alt 1-B used for determining the available slots.

Proposal 4: SSB based measurement by SMTC  should be considered to determine the available slots.

References

- Chairman notes for 3GPP TSG RAN WG1 #105-e, May, 2021

- TS 38.101, User Equipment (UE) radio transmission and reception.

- R1-2104376, Discussion on enhancement on PUSCH repetition type A, vivo