3GPP TSG RAN WG1 #106-e	                                              R1-2106631

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Performance evaluation results for XR

Agenda Item:	8.14.3

Document for:	Discussion and Decision

- Introduction

In the previous RAN1 meeting, the evaluation methodologies/assumptions and the characteristics of XR traffic model were discussed, the corresponding agreements can be found in [1][2][3]. Based on these, we provided our updated simulation results of capacity, power consumption, coverage under various scenarios and traffic conditions for Cloud Gaming and XR (i.e. VR/AR). The overall simulation assumptions can be found in Appendix A. The detailed simulation parameters and results also can be found in the accompanying template excel table.

- Performance evaluation results of capacity

Capacity is one of the most important metrics for XR evaluation. In this section, the results of downlink and uplink capacity for both Cloud Gaming and XR are presented and summarized in the form of table separately, where the resource utilization (RU) is also provided to assist the analysis of system capacity. Besides, based on the agreements of evaluation methodologies/assumptions, the following assumptions are applied in the performance evaluation of system capacity.

- A packet is considered as lost when it has exceeded the PDB, such that it will be added to the PER and discarded at the transmitter (including the non-transmitted part).

- The unequal number of UEs per cell is assumed, which means even average load per cell.

- In the following tables, the value of #UEs /cell represents an integer "A", where at least 90% UEs are satisfied UEs with "A" UEs per cell but less than 90% with "A+1" UEs per cell.

- The system capacity is defined as the maximum number of UEs per cell where at least 90% UEs are satisfied UEs.

- A UE will be declared as the satisfied UE if more than X% of packets are successfully delivered within a given air interface PDB.

- UE% denotes the specific percentage of satisfied UE per cell.

The traffic model and scenario combinations in Table 1 are evaluated for system capacity in this section.

Table 1. Traffic model and scenario combinations used for system capacity evaluation

| DL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - CG: 30Mbps, 60FPS, PDB=15ms - VR/AR: 30/45Mbps, 60/120FPS, PDB=10ms | DL FR2 - Indoor Hotspot/Dense Urban scenario: - CG: 30Mbps, 60FPS, PDB=15ms - VR/AR: 30/45Mbps, 60/120FPS, PDB=10ms |
| --- | --- |
| UL FR1 - Indoor Hotspot/ Dense Urban scenario: - Single stream: Pose/control stream or video stream - Two streams: Pose/control stream + video stream - Urban Macro scenario: - Single stream: Pose/control stream or video stream | UL FR2 - Indoor Hotspot/Dense Urban scenario: - Pose/control stream - Video stream : 10Mbps, 60FPS, PDB=30ms |

  - Baseline capacity results

    - DL

Based on the agreements made in RAN1#104b-e, Table 2 and Table 3 provide the traffic models for different applications (i.e., CG and VR/AR). The packet arrival interval and packet size are determined by the corresponding data rate and frame rate. The video frame rate is assumed to be 60 FPS when dual-eye buffers are bundled transmitted by the application server. And the video frame rate is assumed to be 120 FPS for separate packet arrivals in time for dual-eye buffer, which means that dual-eye buffer is sent staggered in time.

For simplicity, one video frame is modeled as one packet and the packet size distribution follows the truncated Gaussian distribution. Besides, the jitter with respect to the packet arrival time at RAN is also considered to model the random delay caused by different encoding delays and core network transmission delays. For F FPS (e.g., F = 60 or 120), the arrival time of packet k is (k / F + J), where the jitter size J is a random variable drawn from a truncated Gaussian distribution. Considering that the key differences between different traffic models are data rate, frame rate and PDB, we use the combination of data-rate_frame-rate_PDB to identify different traffic conditions in the following sections.

Table 2. DL traffic models of Cloud Gaming (CG) and XR with 60 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 62500 | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

Table 3. DL traffic models of Cloud Gaming (CG) and XR with 120 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 31250 | 31250 | 46875 |
| STD of packet sizes (Bytes) | 3281 | 3281 | 4922 |
| Maximum packet size (Bytes) | 46875 | 46875 | 70313 |
| Minimum packet size (Bytes) | 15625 | 15625 | 23438 |
| Packet arrival interval (ms) | 8.33 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

      - FR1

The baseline simulation results of DL capacity for FR1 in Indoor Hotspot, Dense Urban and Urban Macro deployment scenarios are presented in Table 4. Based on the notes given at the beginning of section 2, a UE will be declared as a satisfied UE if X% or more of the packets are successfully delivered within PDB and X=99 is assumed. For the simulation results provided in this section, the MU-MIMO scheduler is assumed to schedule more UEs by utilizing different layers on the same frequency resource. And proportional-fair (PF) algorithm is applied to determine the scheduling order of UEs.

Table 4. FR1 DL capacity simulation results with X=99

| Traffic Model | Indoor Hotspot |  |  | Dense Urban |  |  | Urban Macro |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #UEs / cell | UE% | RU | #UEs / cell | UE% | RU | #UEs / cell | UE% | RU |
| CG 30Mbps _60FPS_15ms | 16 | 91.15% | 92.12% | 19 | 92.56% | 95.14% | 14 | 91.33% | 92.18% |
| VR/AR 30Mbps_ 60FPS_10ms | 10 | 92.50% | 68.53% | 13 | 92.43% | 78.51% | 8 | 93.75% | 63.34% |
| VR/AR 30Mbps_ 120FPS_10ms | 16 | 92.71% | 91.42% | 20 | 92.54% | 95.98% | 14 | 92.06% | 91.88% |
| VR/AR 45Mbps _ 60FPS_10ms | 5 | 96.67% | 52.95% | 6 | 95.63% | 59.08% | 4 | 94.05% | 48.14% |
| VR/AR 45Mbps_ 120FPS_10ms | 9 | 91.36% | 85.14% | 11 | 91.77% | 90.16% | 8 | 90.87% | 84.84% |

From Table 4, it can be observed that the system capacity is degraded with the increase of data rate due to the large packet size. For an instance, for VR/AR traffic, the system capacity could decrease by 4 to 7 as the data rate increase from 30Mbps to 45Mbps in FR1 DL. Compared with VR/AR applications, the capacity of Cloud Gaming shows significant improvement because of the relatively relaxed PDB requirement, and higher resource utilization can be achieved. Besides, for FR1 DL transmission, the capacity in Dense Urban is larger than others due to smaller coupling loss compared with UMa and more BS antennas than Indoor Hotspot.

Moreover, separate packet arrivals in time for dual-eye buffer is an effective way to improve the system capacity. As observed from Table 4, the system capacity improvement by 4 to 7 could be obtained compared to bundling transmission of dual-eye buffers. For the model of separate packet arrivals, the PDB of each frame transmission is independent, implying that the total PDB for dual-eye buffer transmission is longer.

Observation 1：The system capacity is degraded with the increase of data rate.

Observation 2：The capacity of Cloud Gaming outnumbers VR/AR applications due to the relatively relaxed PDB requirement.

Observation 3: Compared to bundling transmission of dual-eye buffers, separate packet arrivals in time for dual-eye buffers can increase the system capacity.

Observation 4: For FR1 DL transmission with 99% reliability requirement, the capacity results are summarized as follows:

| Traffic Model |  | System capacity in FR1 DL |  |  |
| --- | --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban | Urban Macro |
| CG | 30Mbps_60FPS_15ms | 16.20 | 19.65 | 14.33 |
| VR/AR | 30Mbps_60FPS_10ms | 10.80 | 13.59 | 8.82 |
|  | 30Mbps_120FPS_10ms | 16.53 | 20.78 | 14.59 |
|  | 45Mbps _60FPS_10ms | 5.91 | 6.91 | 4.68 |
|  | 45Mbps_120FPS_10ms | 9.22 | 11.42 | 8.12 |

      - FR2

In this section, the system capacity of CG and VR/AR traffic is evaluated in FR2 with 100MHz bandwidth. The results in Indoor Hotspot and Dense Urban deployment scenarios are provided in Table 5, where SU-MIMO scheduler is assumed. Comparing the evaluation results in section 2.1.1.1 and section 2.1.1.2, it can be observed that the number of satisfied UEs per cell in FR2 is less than that in FR1 for XR applications, which is caused by the link fragility at high-frequency band.

Table 5. FR2 DL capacity simulation results with 100M bandwidth and X=99

| Traffic Model |  | Indoor Hotspot |  |  | Dense Urban |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs /cell | UE% | RU | #UEs /cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 9 | 95.37% | 57.80% | 16 | 92.36% | 78.30% |
| VR/AR | 30Mbps_60FPS_10ms | 8 | 92.01% | 50.12% | 13 | 95.24% | 61.67% |
|  | 30Mbps_120FPS_10ms | 10 | 91.94% | 68.19% | 16 | 93.55% | 78.89% |
|  | 45Mbps _60FPS_10ms | 4 | 94.44% | 34.47% | 8 | 93.25% | 56.08% |
|  | 45Mbps_120FPS_10ms | 6 | 90.28% | 58.58% | 10 | 93.97% | 73.66% |

Observation 5: For FR2 DL transmission with 100MHz bandwidth, the capacity results on 99% reliability requirement are summarized as follows:

| Traffic Model |  | System capacity with 100MHz bandwidth in FR2 DL |  |
| --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban |
| CG | 30Mbps_60FPS_15ms | 9.91 | 16.16 |
| VR/AR | 30Mbps_60FPS_10ms | 8.72 | 13.44 |
|  | 30Mbps_120FPS_10ms | 10.23 | 16.28 |
|  | 45Mbps _60FPS_10ms | 4.67 | 8.20 |
|  | 45Mbps_120FPS_10ms | 6.03 | 10.32 |

    - UL

For the UL evaluations, the packet size distribution of the aggregating streams (scene/video/data/audio) is assumed to follow truncated Gaussian distribution and the data rate is assumed as 10 Mbps. Pose/control stream is periodically sampled and delivered with a fixed size. No jitter is assumed for UL evaluations. The traffic characteristics and requirements for the s pose/control stream and scene/video/data/audio stream are presented in Table 6. Only SU-MIMO scheduler is evaluated for UL transmission.

For UL AR application, both two-stream (pose/control stream and scene/video/data/audio stream) and single- stream (scene/video/data/audio) are simulated in FR1 for capacity evaluation. And for UL CG/VR applications, single stream (pose/control) is assumed for capacity evaluation.

Table 6. UL traffic models of Cloud Gaming (CG) and XR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 4 | 10 | No |
| scene/video/data/audio stream (10Mbps) | 20833 | 2187 | 10416 | 31249 | 16.67 | 30 | No |

      - FR1

In this section, CG/VR and AR applications in FR1 UL transmission are evaluated, and the results are presented in Table 7. For UL CG/VR applications, single-stream (pose/control) is assumed for capacity evaluation and only 20 UEs per cell are simulated at most. Under the maximum number of UEs per cell in the simulation, UE% is still much more than 90%. So, ‘>’ indicates that the system capacity is larger than the value listed in #UEs/cell, and UE% corresponds to #UEs/cell listed in Table 7.

Table 7. FR1 UL capacity simulation results with X=99

| Traffic Model | Indoor Hotspot |  |  | Dense Urban |  |  | Urban Macro |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #UEs/cell | UE% | RU | #UEs/cell | UE% | RU | #UEs/cell | UE% | RU |
| CG/VR pose/control stream | >20 | 100% | 63.37% | >20 | 99.99% | 64.69% | >20 | 97.70% | 65.84% |
| AR video stream | 13 | 93.59% | 64.28% | 9 | 92.95% | 78.34% | <1 | 74.60% | 11.16% |
| AR pose/control+video | 12 | 93.29% | 65.24% | 7 | 92.29% | 80.33% | - | - | - |

For pose/control stream of CG and VR applications in UL transmission, more than 20 UEs per cell can be supported in all deployment scenarios due to small packet size. Compared to the results of DL transmission in section 2.1.1.1, UL transmission of CG and VR can satisfy more UEs and is not the bottleneck that limits system capacity.

For UL video stream of AR application, in Indoor Hotspot scenario, compared with downlink transmission with 10ms PDB, more UEs can be accommodated in FR1 UL due to relatively smaller packet size and relaxed PDB (30ms). However, in Dense Urban scenario, fewer UEs can be supported in UL than that in DL transmission, because with the increase of coupling loss, the limited transmission power will limit the system capacity. As for UL video stream of AR application in Urban Macro scenario, the percentage of satisfied UEs is less than 90% even with only 1 UE per cell. This is because cell-edge UEs in Urban Macro suffer large coupling loss and limited transmission power, such that only a small amount of data can be transmitted on each transmission occasion, resulting in a large number of packets exceeding the PDB and added to the PER.

For AR application with two streams, a UE is considered as a satisfied UE only when the two streams of data can both meet the reliability requirements. Compared to the results of single stream, due to the limited UL resources on the TDD structure of “DDDSU”, dense pose/control stream scheduling will occupy part of system resources and decrease the system capacity.

Observation 6: For CG and VR applications in FR1, DL is the bottleneck of system capacity because of the high data rate.

Observation 7: For FR1 UL transmission with 99% reliability requirement, the capacity results are summarized as follows:

| Traffic Model |  | System capacity in FR1 UL |  |  |
| --- | --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban | Urban Macro |
| CG/VR | pose/control stream | >20 | >20 | >20 |
| AR | video stream@10Mbps | 13.95 | 9.49 | <1 |
|  | pose/control+video@10Mbps | 12.71 | 7.43 | - |

      - FR2

The simulation results of FR2 UL with 100MHz bandwidth are provided in Table 8. Similar to the representation of FR1 UL simulation results, ‘>’ is used to indicate that the system capacity is larger than the value listed in #UEs /cell in table. And only single-stream is assumed for XR evaluation in FR2 uplink.

Table 8. FR2 UL capacity simulation results with 100MHz bandwidth and X=99

| Traffic Model | Indoor Hotspot |  |  | Dense Urban |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #UEs/cell | UE% | RU | #UEs/cell | UE% | RU |
| CG/VR pose/control stream | >20 | 97.69% | 20.92% | >20 | 96.51% | 20.68% |
| AR video stream | 8 | 95.14% | 71.96% | 8 | 92.66% | 83.33% |

Observation 8: For FR2 UL transmission with 100MHz bandwidth, the capacity results on 99% reliability requirement are summarized as follows:

| Traffic Model |  | System capacity with 100MHz bandwidth in FR2 UL |  |
| --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban |
| CG/VR | pose/control stream | >20 | >20 |
| AR | video stream@10Mbps | 8.59 | 8.30 |

  - Traffic characteristics-oriented evaluation

    - Impact of relaxed PDB

Figure 1 shows the system capacity results of CG and VR/AR applications with 30Mbps and 60FPS for FR1 DL in Dense Urban scenario. The PDB is 15ms and 10ms for CG and VR/AR respectively, and MU-MIMO scheduler is assumed. By relaxing PDB from 10ms to 15ms, the system capacity can increase 6 UEs per cell in CG application compared to VR/AR application under the same data rate and frame rate.

| - UE% | - RU |
| --- | --- |

Figure 1. Impact of relaxed PDB in FR1 DL in Dense Urban scenario

Observation 9: The system capacity can be increased by relaxing PDB.

    - Impact of scheduling algorithm

Considering stringent PDB requirement relative to high data rate, delay-aware (DA) scheduler taking time factor into account is proposed. Based on the traditional PF scheduler, the DA scheduler prioritizes UEs approaching the deadline by adding time factor to the weight, at the cost of using more system resources.

As listed in Table 9, the capacity performance is simulated in FR1 DL and FR2 DL to evaluate different types of scheduling algorithm. In addition to SU-MIMO scheduler simulated both in FR1 and FR2, MU-MIMO scheduler is evaluated in FR1 downlink transmission. The corresponding results are presented in Table 10 through Table 14, where CG application with 30Mbps_60FPS_15ms and VR/AR application with 30Mbps_60FPS_10ms are evaluated. As shown from the results, the DA scheduler can support a larger percentage of satisfied UEs and improve system capacity, especially when the #UE per cell is small and the system resources are sufficient. In addition, compared to SU-MIMO scheduler, MU-MIMO scheduler can satisfy more UEs by multiplexing different layers in FR1 DL.

Table 9. The scheduler used for system capacity performance comparison

| DL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - SU-MIMO PF - SU-MIMO DA - MU-MIMO PF - MU-MIMO DA | DL FR2 - Indoor Hotspot/Dense Urban scenario: - SU-MIMO PF - SU-MIMO DA |
| --- | --- |

- FR1 DL - Indoor Hotspot

Table 10. FR1 DL capacity simulation results in Indoor Hotspot scenario with X=99

| Traffic Model |  | Scheduler | SU-MIMO |  |  | MU-MIMO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #UEs /cell | UE% | RU | #UEs /cell | UE% | RU |
| CG | 30Mbps_ 60FPS_15ms | PF | 10 | 91.67% | 75.92% | 16 | 91.15% | 92.12% |
|  |  | DA | 11 | 96.06% | 88.12% | 16 | 92.01% | 96.22% |
| VR/AR | 30Mbps_ 60FPS_10ms | PF | 8 | 92.71% | 57.62% | 10 | 92.50% | 68.53% |
|  |  | DA | 10 | 95.20% | 80.33% | 12 | 93.06% | 83.94% |

- FR1 DL - Dense Urban

Table 11. FR1 DL capacity simulation results in Dense Urban scenario with X=99

| Traffic Model |  | Scheduler | SU-MIMO |  |  | MU-MIMO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #UEs /cell | UE% | RU | #UEs /cell | UE% | RU |
| CG | 30Mbps_ 60FPS_15ms | PF | 11 | 94.81% | 75.12% | 19 | 92.56% | 95.14% |
|  |  | DA | 13 | 94.90% | 89.08% | 19 | 92.86% | 98.41% |
| VR/AR | 30Mbps_ 60FPS_10ms | PF | 9 | 94.18% | 59.75% | 13 | 92.43% | 78.51% |
|  |  | DA | 12 | 95.12% | 83.02% | 14 | 91.84% | 87.52% |

.

- FR1 DL - Urban Macro

Table 12. FR1 DL capacity simulation results in Urban Macro scenario with X=99

| Traffic Model |  | Scheduler | SU-MIMO |  |  | MU-MIMO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #UEs /cell | UE% | RU | #UEs /cell | UE% | RU |
| CG | 30Mbps_ 60FPS_15ms | PF | 10 | 91.90% | 77.91% | 14 | 91.33% | 92.18% |
|  |  | DA | 11 | 93.78% | 86.24% | 14 | 91.73% | 96.90% |
| VR/AR | 30Mbps_ 60FPS_10ms | PF | 7 | 92.48% | 56.81% | 8 | 93.75% | 63.34% |
|  |  | DA | 8 | 92.64% | 73.87% | 9 | 92.30% | 73.32% |

- FR2 DL - Indoor Hotspot

Table 13. FR2 DL capacity simulation results in Indoor Hotspot scenario with X=99

| Traffic Model |  | Scheduler | SU-MIMO |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #UEs/cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | PF | 9 | 95.37% | 57.80% |
|  |  | DA | 10 | 91.11% | 67.06% |
| VR/AR | 30Mbps_60FPS_10ms | PF | 8 | 92.01% | 50.12% |
|  |  | DA | 8 | 92.36% | 50.20% |

- FR2 DL - Dense Urban

Table 14. FR2 DL capacity simulation results in Dense Urban scenario with X=99

| Traffic Model |  | Scheduler | SU-MIMO |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #UEs/cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | PF | 16 | 92.36% | 78.30% |
|  |  | DA | 16 | 96.73% | 79.69% |
| VR/AR | 30Mbps_60FPS_10ms | PF | 13 | 95.24% | 61.67% |
|  |  | DA | 14 | 91.27% | 66.70% |

Observation 10: Delay-aware scheduler can increase the system capacity compared to typical PF scheduler.

Observation 11: MU-MIMO scheduler can increase the system capacity in FR1 DL.

    - Impact of 400MHz and 100MHz bandwidth in FR2

The impact of bandwidth increasing from 100MHz to 400MHz is presented in Figure 2. The simulation is performed for VR/AR application with 45Mbps_60FPS_10ms for FR2 DL in Dense Urban scenario. As expected, increasing bandwidth can make a significant improvement in system capacity because of increased resources for transmission. For instance, with 99% reliability requirement, the capacity is 8 for 100MHz and larger than 16 for 400MHz bandwidth, which can be observed from Figure 2.

| - UE% | - RU |
| --- | --- |

Figure 2. Impact of bandwidth in FR2 DL in Dense Urban scenario

Observation 12: Increase system bandwidth can significantly increase the system capacity.

    - Impact of size ratio and QoS requirements between I frame and P frame

In the video and image coding field, the video frames are usually encoded into I-frame and P-frame (or B-frame), which can be modelled as two streams. And there are two traffic patterns for I-stream and P-stream, Group-Of-Picture (GOP) based traffic model and slice-based traffic model. Due to different traffic characteristics and QoS requirements of I/P-frame, different PDB and PER requirements for I/P-frame are evaluated. Details on the traffic model and the strategy of setting PDB and PER are described in our contribution [4].

In this sub-section, the capacity performance results of GOP-based traffic model and slice-based traffic model are provided. The single DL video stream with 10ms PDB and 99% reliability requirements is also evaluated as the baseline for comparison purposes. For single_stream_baseline model, I-frame and P-frame are not modeled, and the traffic characteristics correspond to the 30Mbps data rate of VR/AR in Table 2.

Moreover, the scenarios and traffic to be evaluated in this section include Dense Urban for FR1 and InH for FR2, and VR/AR, 30Mbps. And α representing the ratio of the average frame size of I-frame and P-frame is assumed as 1.5 and 3 both for GOP-based model and slice-based model.

      - GOP based

The system evaluation results of capacity for GOP-based I/P-frame traffic model are presented as follows, where MU-MIMO scheduler is applied and GOP size K = 8. The combinations of different PDB and PER values for GOP-based traffic model are shown in Table 15.

Table 15. The combinations of different PDB and PER values for GOP-based traffic model

| Case | PER and PDB value |
| --- | --- |
| Single_stream_baseline | [PER] = [1%], [PDB] = [10ms] |
| Case 1 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 2 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 3 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [20ms, 10ms] |
| Case 4 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 5 | [PER_I, PER_P] = [5%, 1%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 6 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [20ms, 10ms] |
| Case 7 | [PER_I, PER_P] = [5%, 1%], [PDB_I, PDB_P] = [20ms, 10ms] |

- FR1 DL - Dense Urban

Figure 3. Capacity results of GOP-based I/P-frame traffic model with α = 1.5 for FR1 Dense Urban

Figure 4. Capacity results of GOP-based I/P-frame traffic model with α = 3 for FR1 Dense Urban

- FR2 DL - Indoor Hotspot

Figure 5. Capacity results of GOP-based I/P-frame traffic model with α = 1.5 for FR2 Indoor Hotspot

Figure 6. Capacity results of GOP-based I/P-frame traffic model with α = 3 for FR2 Indoor Hotspot

For GOP-based I/P-frame traffic model, I-frame and P-frame are not generated at the same time, and the average packet size of I-frame is larger. With limited resources, packet size is a key factor that limits system capacity. For the cases where α is equal to 3, GOP-based I/P-frame traffic model shows a significant degradation compared to single-stream traffic model in Figure 4 and Figure 6, which is caused by the relatively larger data size of I-frame. As for the cases of α=1.5, relaxing PDB of I-frame to 20ms can improve system capacity compared to single-stream traffic model, especially when the PER of P-frame is relaxed to 5% (Case 7). Specially, when PDB of I-frame is relaxed to 15ms, relaxing PER of I-frame can achieve more excellent system capacity than just relaxing PER of P-frame. When PDB of I-frame is relaxed to 20ms, relaxing PER of P-frame can further improve system capacity but no obvious improvement by only relaxing PER of I-frame.

Observation 13: GOP-based I/P-frame traffic model with α=3 shows a significant degradation compared to single-stream traffic model.

Observation 14: GOP-based I/P-frame traffic model with α=1.5 and relaxing PDB of I-frame to 20ms can improve system capacity compared to single-stream traffic model.

Observation 15: Relaxing PDB of I-frame can improve capacity performance for GOP-based I/P-frame traffic model.

      - Sliced based

The system evaluation results of capacity for slice-based I/P-frame traffic models are presented in Figure 7 and Figure 8, where MU-MIMO scheduler is applied and the number of slices per frame equals 8. The combinations of different PDB and PER values for slice-based traffic model are shown in Table 16.

Table 16. The combinations of different PDB and PER values for slice-based traffic model

| Case | PER and PDB value |
| --- | --- |
| Single_stream_baseline | [PER] = [1%], [PDB] = [10ms] |
| Case 1 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 2 | [PER_I, PER_P] = [5%, 1%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 3 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 4 | [PER_I, PER_P] = [5%, 5%], [PDB_I, PDB_P] = [10ms, 10ms] |

| - α = 1.5 | - α = 3 |
| --- | --- |

Figure 7. Capacity results of slice-based I/P-frame traffic model for FR1 Dense Urban

| - α = 1.5 | - α = 3 |
| --- | --- |

Figure 8. Capacity results of slice-based I/P-frame traffic model for FR2 Indoor Hotspot

For slice-based I/P-frame traffic model, I frame and multiple P frames are generated at the same time. For slice-based model, the total data size generated at the same time is equal to single-stream traffic model, regardless of α=1.5 or α=3. So under the limited resource, the capacity performances of case1 with multiple-stream are similar with the single_stream_baseline both in FR1 Dense Urban and FR2 Indoor Hotspot scenarios, where the same PER and PDB requirements are set for multi-stream and single-stream. In addition, since I-frame which is intra-coded is more important compared to P-frame, I-frame is prioritized for scheduling in simulation. As observed in Figure 7 and Figure 8, relaxing PER of I-frame (case2) could not improve the capacity performance, but relaxing PER (case3, case4) of P-frame could significantly improve capacity performance.

Observation 16: When I-frame and P-frame adopt the same PER and PDB values as single-stream traffic model, the capacity performance of slice-based I/P-frame traffic model is the same as that of single-stream traffic model, regardless of α=1.5 or α=3.

Observation 17: Relaxing PER of P-frame can improve capacity performance for slice-based I/P-frame traffic model.

- Performance evaluation results of power consumption

As analyzed in our companion’s contribution [5], power consumption is the crucial issue for XR traffic and it is another key metric for XR evaluation, since the specific characteristics of XR traffic i.e., dense traffic period, high traffic data rates and low latency requirement, etc. which is different from eMBB and URLLC. For power consumption evaluation, UE capacity performance should be jointly taken into consideration. Besides, in line with the agreement in RAN1#104b-e and RAN1#105-e, the following assumptions are adopted in performance evaluation of power consumption.

- The power consumption results for both all UEs and only satisfied UEs are counted separately to help comprehensively evaluate XR power performance.

- For XR/CG power consumption evaluation, for DL and UL, both Option 1(i.e., DL and UL performances are evaluated independently…) and Option 3 (i.e., DL and UL performances are evaluated together…) are evaluated.

- The linear interpolation method is adopted by us in a linear scale for UE Tx power values other than 0 dBm and 23 dBm. And the detail of linear interpolation can be found in our companion’s contribution [6].

- For FR2 UL, the fixed value 350 listed in TR38.840[7] is assumed as the relative power of the UL state (long PUCCH or PUSCH).

- The same number of UE per cell are used in baseline and power saving schemes where the max UEs/cell at which UE can meet the capacity KPI is reported as high load case, and approximately 50% of max UEs/cell is evaluated as low load case.

In this contribution, AlwaysOn (i.e., without adopting any power saving mechanism) which can make no capacity loss is the baseline scheme. And CDRX configuration adopted in R15/R16 is evaluated as a power saving scheme. In addition, the potential enhancement of CDRX and the ongoing power saving schemes in R17 power saving WI, such as PDCCH skipping, search space set group switching mechanisms are also evaluated to contrastive analysis of the potential impact on power consumption and corresponding system capacity. In this section, power consumption of different schemes is evaluated by using downlink and uplink traffic models with 60 FPS. Table 17 gives the adopted traffic models and corresponding scenarios for power consumption evaluation.

Table 17. Traffic model and scenario combinations used for power consumption evaluation

| Option 1: DL and UL performances are evaluated independently and DL and UL power consumption results are collected separately. |  |
| --- | --- |
| DL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - VR/AR video stream: 30/45Mbps, 60FPS, PDB=10ms | DL FR2 - Indoor Hotspot/Dense Urban scenario: - VR/AR video stream: 30/45Mbps, 60FPS, PDB=10ms |
| UL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - Single stream: - Pose/control stream - Indoor Hotspot/Dense Urban scenario: - Single stream: - Video stream - Two streams: - Pose/control stream + video stream | UL FR2 - Indoor Hotspot/Dense Urban scenario: - Single stream: - Pose/control stream - Video stream |
| Option 3: DL and UL performances are evaluated together. DL and UL power consumption are counted to obtain the total power consumption |  |
| DL and UL transmission together in FR1 - Indoor Hotspot/Dense Urban scenario: - VR: DL video (30Mbps_60FPS_10ms), UL pose/control stream (0.2Mbps) - AR with UL single-stream: DL video (30Mbps_60FPS_10ms), UL video stream (10Mbps_60FPS_30ms) - AR with UL two-stream: DL video (30Mbps), UL pose/control stream (0.2Mbps) + video stream (10Mbps) |  |

  - DL video stream

For downlink VR/AR application, in addition to no DRX configuration (i.e., AlwaysOn) as the baseline, R15/16 CDRX configurations (i.e., R15/16 CDRX) with different sets of parameters are evaluated, which are captured in Table 18. Note that 10ms is the minimum long DRX cycle configuration supported by the current specification.

Due to non-integer XR traffic periodicity, the existing integer DRX cycle cannot align with the traffic arrival and it will increase data transmission delay. To guarantee system capacity performance, the longer drx-onDurationTimer is preferable to reduce waiting time, which will inevitably damage the power saving gain (PSG). To solve this issue, a straightforward method is to introduce some non-integer values of DRX cycle to align with the XR traffic periodicity. But it is not flexible because of the various traffic period and the jitter effect of DL burst arrival. To solve the impact of jitter and align DRX cycle with traffic arrival, the adjustment to the start offset of CDRX ON Duration could be a potential solution, which is an enhancement of the CDRX scheme (i.e., e-CDRX) and is equivalent to aligning DRX cycle with the period of XR traffic. The parameters of e-CDRX are also given in Table 18. In FR2 transmission, multi-beam is configured and only one beam can be scheduled per slot. Therefore, for e-CDRX scheme in FR2, a longer drx-onDurationTimer is configured to avoid UEs who are not scheduled but with packets waiting for transmission to go into sleep state.

Additionally, PDCCH skipping mechanism can also make a better trade-off between power saving and capacity. Due to the existence of jitter, the packets may arrive at any time within the range of [-4ms, 4ms] around the quasi-periodic time. Therefore, in the simulation we adopt the simplest method for tackling jitter i.e., defining a jitter window to cover the whole or partial jitter range. Besides, within the jitter window, UE must start doing PDCCH monitoring and the PDCCH skipping command will not be transmitted. After the window, gNB can be enabled to send the PDCCH skipping command if no more packets are waiting to be transmitted. As such, the packets jitter can be handled to reduce the potential scheduling delay. The different lengths of jitter window can also make a compromise between system capacity and power consumption. In our simulation, the window length is equal to the maximum positive jitter range of 4ms.

Based on the assumptions and configurations mentioned above, the UE capacity and power consumption results in DL are shown in the following tables, where the PSG is calculated compared to the baseline scheme (without adopting any power saving mechanism). The triplets of numbers (CDRX cycle_ On duration timer_ Inactivity timer) are used to note various CDRX parameters. And the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB. The mean power consumption shown in the following tables is the average power consumption within each slot.

- AlwaysOn (Baseline): Without adopting any power saving mechanism.

- R15/16 CDRX: adopting R15/R16 DRX configuration.

- e-CDRX: aligning the start time of DRX ON Duration with the arrival of XR traffic.

- PDCCH skipping: PDCCH skipping with jitter handling.

Table 18. The configuration for R15/16 CDRX and e-CDRX

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 10 | 8 | 4 |
| R15/16 CDRX | 16 | 14 | 4 |
| e-CDRX (FR1) | 16 | 6 | 4 |
| e-CDRX (FR2) | 16 | 8 | 4 |

    - FR1 VR/AR@30Mbps

- Indoor Hotspot

Table 19. FR1 DL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (10 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 109.98 | - |  | 92.50% | 118.79 | - | all |
|  |  | 109.98 | - |  |  | 117.23 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 103.69 | 5.72% |  | 91.25% | 112.99 | 4.88% | all |
|  |  | 103.69 | 5.72% |  |  | 111.25 | 5.10% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 105.95 | 3.67% |  | 91.81% | 114.94 | 3.24% | all |
|  |  | 105.95 | 3.67% |  |  | 113.17 | 3.46% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 78.77 | 28.38% |  | 91.25% | 90.47 | 23.84% | all |
|  |  | 78.77 | 28.38% |  |  | 88.96 | 24.11% | satisfied |
| PDCCH skipping | 100% | 71.29 | 35.18% |  | 90.70% | 81.56 | 31.34% | all |
|  |  | 71.29 | 35.18% |  |  | 79.84 | 31.89% | satisfied |

- Dense Urban

Table 20. FR1 DL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (7 UEs per cell) |  |  | High load case (13 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 111.80 | - |  | 92.43% | 122.86 | - | all |
|  |  | 111.80 | - |  |  | 120.94 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 105.57 | 5.57% |  | 90.11% | 117.09 | 4.70% | all |
|  |  | 105.57 | 5.57% |  |  | 114.31 | 5.48% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 107.72 | 3.65% |  | 91.58% | 119.14 | 3.03% | all |
|  |  | 107.72 | 3.65% |  |  | 116.76 | 3.46% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 81.07 | 27.49% |  | 91.21% | 96.17 | 21.72% | all |
|  |  | 81.07 | 27.49% |  |  | 92.41 | 23.59% | satisfied |
| PDCCH skipping | 100% | 73.80 | 34.71% |  | 91.21% | 86.13 | 29.90% | all |
|  |  | 73.80 | 34.71% |  |  | 82.83 | 31.51% | satisfied |

- Urban Macro

Table 21. FR1 DL capacity and power consumption results in Urban Macro scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (8 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 98.81% | 109.62 | - |  | 93.75% | 119.74 | - | all |
|  |  | 109.16 | - |  |  | 117.27 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 98.41% | 102.75 | 6.26% |  | 91.47% | 113.73 | 5.02% | all |
|  |  | 101.99 | 6.56% |  |  | 110.74 | 5.57% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 98.81% | 105.18 | 4.05% |  | 92.85% | 115.85 | 3.23% | all |
|  |  | 104.70 | 4.08% |  |  | 113.06 | 3.59% | satisfied |
| e-CDRX 16ms_6ms_4ms | 97.22% | 77.77 | 29.06% |  | 91.87% | 91.81 | 23.33% | all |
|  |  | 76.98 | 29.48% |  |  | 87.68 | 25.23% | satisfied |
| PDCCH skipping | 96.83% | 70.44 | 35.75% |  | 92.06% | 81.45 | 31.98% | all |
|  |  | 69.68 | 36.17% |  |  | 77.40 | 33.99% | satisfied |

Based on the simulation results given above, by using R15/16 CDRX, to ensure achieving the requirement of capacity, the values of drx-onDurationTimer and drx-InactivityTimer should be larger enough to provide sufficient scheduling, transmission opportunities and cover the jitter range (i.e. [-4, 4]ms). Thus, only about 4.70%~6.56% PSG can be obtained by CDRX with 10ms DRX cycle. Moreover, setting a 16 ms DRX cycle that is close to traffic period (e.g.,16.67ms) couldn’t solve the mismatch between the non-integer traffic period and the DRX cycle. A longer drx-onDurationTimer also needs to be set to guarantee the system capacity and only 3.03%~4.08% PSG can be obtained.

Regarding e-CDRX which can align the start time of DRX ON Duration with the quasi-periodic arrival of XR traffic dynamically, the traffic bursts will be scheduled in time and the power consumption due to mismatching will be reduced. Also, a shorter drx-onDurationTimer (i.e. 6ms) can be configured without impacting any system capacity for high load case. Additionally, to handle jitter, the length of drx-onDurationTimer should be no less than the maximum positive jitter range (i.e. +4ms) in our evaluation. Similar effects can be also achieved by adopting PDCCH skipping with jitter handling scheme. Compared to e-CDRX, enhanced PDCCH skipping scheme shows a little more PSG, since the PDCCH skipping command can be sent and applied immediately without waiting for the expiry of drx-InactivityTimer, when there is no traffic buffered on the gNB side. However, it is no doubt that PDCCH skipping with jitter handling scheme will bring more signaling overhead than e-CDRX method, since the PDCCH skipping command is required at least once per DRX cycle.

As observed from the above tables, compared to the R15/R16 CDRX configuration, both enhanced CDRX and enhanced PDCCH skipping scheme can achieve 21.72%~36.17% mean PSG without capacity loss, which is over five times as much as the mean PSG of R15/16 CDRX. Compared to high load case, low load case can obtain larger power saving gain. In addition, the power consumption statistics of only satisfied UEs can bring additional power saving gain, compared with that of all UEs, because unsatisfied UEs are usually those with poor transmission quality and need more transmission opportunity.

Observation 18: By adopting the R15/R16 CDRX configuration, 3.03%~6.56% mean PSG can be obtained without the loss of the system capacity in FR1 DL for VR/AR with 30Mbps data rate.

Observation 19: 21.72%~36.17% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR1 DL for VR/AR with 30Mbps data rate.

Observation 20: Compared to high load case, low load case can obtain larger power saving gain.

Observation 21: Compared to counting the power consumption of all UEs, counting the power consumption of only satisfied users can bring additional power saving gain.

    - FR1 VR/AR@45Mbps

- Indoor Hotspot

Table 22. FR1 DL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (3 UEs per cell) |  |  | High load case (5 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 113.62 | - |  | 96.67% | 121.10 | - | all |
|  |  | 113.62 | - |  |  | 120.38 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 107.58 | 5.32% |  | 92.78% | 115.43 | 4.68% | all |
|  |  | 107.58 | 5.32% |  |  | 113.99 | 5.31% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 109.69 | 3.46% |  | 94.44% | 117.67 | 2.83% | all |
|  |  | 109.69 | 3.46% |  |  | 116.60 | 3.14% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 83.23 | 26.74% |  | 93.89% | 93.72 | 22.61% | all |
|  |  | 83.23 | 26.74% |  |  | 91.95 | 23.62% | satisfied |
| PDCCH skipping | 100% | 74.67 | 34.28% |  | 93.89% | 83.99 | 30.64% | all |
|  |  | 74.67 | 34.28% |  |  | 82.66 | 31.33% | satisfied |

- Dense Urban

Table 23. FR1 DL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (3 UEs per cell) |  |  | High load case (6 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 112.75 | - |  | 95.63% | 120.88 | - | all |
|  |  | 112.75 | - |  |  | 119.77 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 106.49 | 5.56% |  | 93.12% | 115.21 | 4.69% | all |
|  |  | 106.49 | 5.56% |  |  | 113.65 | 5.11% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 108.77 | 3.53% |  | 94.18% | 117.13 | 3.10% | all |
|  |  | 108.77 | 3.53% |  |  | 115.74 | 3.37% | satisfied |
| e-CDRX 16ms_6ms_4ms | 99.47% | 82.02 | 27.26% |  | 94.18% | 93.14 | 22.95% | all |
|  |  | 81.80 | 27.45% |  |  | 91.08 | 23.95% | satisfied |
| PDCCH skipping | 99.47% | 73.69 | 34.64% |  | 93.39% | 83.70 | 30.75% | all |
|  |  | 73.53 | 34.79% |  |  | 82.01 | 31.53% | satisfied |

- Urban Macro

Table 24. FR1 DL capacity and power consumption results in Urban Macro scenario

| Metrics Cases | Low load case (2 UEs per cell) |  |  | High load case (4 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 96.83% | 113.01 | - |  | 94.05% | 119.51 | - | all |
|  |  | 111.76 | - |  |  | 117.46 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 96.83% | 106.45 | 5.81% |  | 92.46% | 113.64 | 4.92% | all |
|  |  | 105.12 | 5.95% |  |  | 111.13 | 5.39% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 96.83% | 108.52 | 3.97% |  | 93.25% | 115.77 | 3.13% | all |
|  |  | 107.24 | 4.04% |  |  | 113.56 | 3.32% | satisfied |
| e-CDRX 16ms_6ms_4ms | 96.83% | 82.13 | 27.33% |  | 91.67% | 91.32 | 23.59% | all |
|  |  | 80.39 | 28.07% |  |  | 87.78 | 25.27% | satisfied |
| PDCCH skipping | 96.83% | 73.77 | 34.73% |  | 91.67% | 81.07 | 32.17% | all |
|  |  | 72.38 | 35.24% |  |  | 77.71 | 33.84% | satisfied |

Observation 22: By adopting the R15/R16 CDRX configuration, 2.83%~5.95% mean PSG can be obtained without the loss of the system capacity in FR1 DL for VR/AR with 45Mbps data rate.

Observation 23: 22.61%~35.24% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR1 DL for VR/AR with 45Mbps data rate.

    - FR2 VR/AR@30Mbps

In this section, the evaluation results of different power schemes in FR2 DL are provided. According to TR38.840, the relative power of PDCCH state in FR1 is 100 while that in FR2 is 175. By reducing PDCCH monitoring, more power can be saved in FR2. As shown from the simulation results, compared to FR1, the larger PSG can be achieved by deploying power saving schemes in FR2. 31.30%~49.05% mean PSG will be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or enhanced PDCCH skipping scheme without capacity loss in FR2 DL for VR/AR with 30Mbps data rate.

- Indoor Hotspot

Table 25. FR2 DL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (8 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 157.20 | - |  | 92.01% | 158.46 | - | all |
|  |  | 157.20 | - |  |  | 157.83 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 99.31% | 141.38 | 10.06% |  | 90.63% | 143.47 | 9.53% | all |
|  |  | 141.39 | 10.06% |  |  | 142.47 | 9.79% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 99.31% | 147.33 | 6.28% |  | 91.67% | 149.08 | 5.81% | all |
|  |  | 147.34 | 6.28% |  |  | 148.30 | 5.91% | satisfied |
| e-CDRX 16ms_8ms_4ms | 98.61% | 102.36 | 34.89% |  | 90.97% | 103.61 | 33.68% | all |
|  |  | 102.08 | 35.06% |  |  | 102.04 | 33.71% | satisfied |
| PDCCH skipping | 100% | 80.65 | 48.70% |  | 91.32% | 82.77 | 47.84% | all |
|  |  | 80.65 | 48.70% |  |  | 81.34 | 48.45% | satisfied |

- Dense Urban

Table 26. FR2 DL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (7 UEs per cell) |  |  | High load case (13 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 99.55% | 156.32 | - |  | 95.24% | 156.36 | - | all |
|  |  | 156.19 | - |  |  | 156.24 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 98.64% | 140.46 | 10.15% |  | 91.82% | 141.51 | 9.50% | all |
|  |  | 140.44 | 10.16% |  |  | 141.21 | 9.62% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 99.32% | 146.32 | 6.40% |  | 93.53% | 147.05 | 5.96% | all |
|  |  | 146.18 | 6.41% |  |  | 146.84 | 6.01% | satisfied |
| e-CDRX 16ms_8ms_4ms | 99.09% | 105.31 | 32.63% |  | 91.94% | 107.42 | 31.30% | all |
|  |  | 104.28 | 33.24% |  |  | 105.88 | 32.23% | satisfied |
| PDCCH skipping | 99.32% | 79.69 | 49.02% |  | 92.67% | 80.55 | 48.48% | all |
|  |  | 79.58 | 49.05% |  |  | 80.07 | 48.75% | satisfied |

Observation 24: By adopting the R15/R16 CDRX configuration, 5.81%~10.16% mean PSG can be obtained without the loss of the system capacity in FR2 DL for VR/AR with 30Mbps data rate.

Observation 25: 31.30%~49.05% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR2 DL for VR/AR with 30Mbps data rate.

Observation 26: Compared to FR1, larger PSG can be achieved by using the same power saving scheme in FR2.

    - FR2 VR/AR@45Mbps

- Indoor Hotspot

Table 27. FR2 DL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (2 UEs per cell) |  |  | High load case (4 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 160.63 | - |  | 94.44% | 161.69 | - | all |
|  |  | 160.63 | - |  |  | 161.04 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 98.61% | 145.34 | 9.52% |  | 91.67% | 146.90 | 9.15% | all |
|  |  | 145.3 | 9.54% |  |  | 146.17 | 9.23% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 98.61% | 151.03 | 5.98% |  | 93.75% | 152.42 | 5.73% | all |
|  |  | 150.97 | 6.01% |  |  | 151.74 | 5.77% | satisfied |
| e-CDRX 16ms_8ms_4ms | 100% | 113.64 | 29.25% |  | 91.67% | 115.82 | 28.37% | all |
|  |  | 113.64 | 29.25% |  |  | 114.35 | 28.99% | satisfied |
| PDCCH skipping | 100% | 84 | 47.71% |  | 93.06% | 85.76 | 46.96% | all |
|  |  | 84 | 47.71% |  |  | 84.82 | 47.33% | satisfied |

- Dense Urban

Table 28. FR2 DL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (8 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 158.79 | - |  | 93.25% | 159.57 | - | all |
|  |  | 158.79 | - |  |  | 159.18 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 144.18 | 9.20% |  | 91.67% | 146.34 | 8.29% | all |
|  |  | 144.18 | 9.20% |  |  | 145.44 | 8.61% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 149.17 | 6.06% |  | 92.26% | 151.62 | 4.98% | all |
|  |  | 149.17 | 6.06% |  |  | 150.28 | 5.57% | satisfied |
| e-CDRX 16ms_8ms_4ms | 100% | 113.42 | 28.57% |  | 91.47% | 116.23 | 27.16% | all |
|  |  | 113.42 | 28.57% |  |  | 115.69 | 27.30% | satisfied |
| PDCCH skipping | 100% | 92.81 | 41.55% |  | 92.06% | 96.38 | 39.60% | all |
|  |  | 92.81 | 41.55% |  |  | 95.26 | 40.14% | satisfied |

Observation 27: By adopting the R15/R16 CDRX configuration, 4.98 %~9.54% mean PSG can be obtained without the loss of the system capacity in FR2 DL for VR/AR with 45Mbps data rate.

Observation 28: 27.16%~47.71% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR2 DL for VR/AR with 45Mbps data rate.

  - UL pose/control stream

According to the agreements in RAN1 #104b-e, the period of UL pose/control stream is assumed as 4ms without jitter. So, an integer DRX cycle can be configured to match the UL traffic arrival. Note that the packet arrival interval of pose/control stream is far below the corresponding PDB requirement (i.e., 10ms), and the packet size is extremely small which can be finished by using only one TTI. Due to these reasons, to further avoid the transition between wake up and go to sleep states, several packets can be bundled together. In our evaluation, considering the potential impact on the system capacity, we assume that two UL pose/control packets can be bundled transmitted. Figure 9 illustrates the packets bundled transmission with corresponding DRX configurations compared to UL pose/control stream transmission without using packet bundling. As analyzed in Figure 9, the PDB of bundled packet should take the minimum PDB of the two single packets, i.e. the PDB of bundled packet is 6ms. And the corresponding DRX configuration parameters for without packet bundling and two packets bundling can be found in Table 29.

- AlwaysOn (Baseline): Without adopting any power saving mechanism.

- R15/16 CDRX: R15/16 CDRX configuration without packet bundling.

- CDRX for packets bundling: packets bundled transmission (i.e., two packets are bundled together for transmission at a time)

Table 29. The DRX configuration for UL pose/control stream

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 4 | 2 | 1 |
| CDRX for packets bundling | 8 | 3 | 1 |

Figure 9. Packets bundled transmission with corresponding DRX configurations

The results of CG or VR application with one stream (pose/control stream) in FR1 UL and FR2 UL is presented in this section. Similar to capacity evaluation of pose/control stream, only the power consumption results of low load case with 20 UEs per cell are provided. As shown in section 3.2.1 and section 3.2.2, compared with the DRX configuration without packets bundling, the packets bundled transmission scheme can obtain additional about 10% PSG.

    - FR1

Table 30. Results of capacity and power consumption in FR1 UL with low load case

| Metrics Cases | Indoor Hotspot scenario (20 UEs per cell) |  |  | Dense Urban scenario (20 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 119.67 | - |  | 99.99% | 121.70 | - | all |
|  |  | 119.67 | - |  |  | 121.70 | - | satisfied |
| R15/16 CDRX 4ms_2ms_1ms | 94.31% | 88.16 | 26.33% |  | 94.84% | 89.31 | 26.62% | all |
|  |  | 88.17 | 26.33% |  |  | 89.27 | 26.65% | satisfied |
| R15/16 CDRX for packets bundling 8ms_3ms_1ms | 93.33% | 75.59 | 36.83% |  | 93.81% | 76.35 | 37.27% | all |
|  |  | 75.59 | 36.83% |  |  | 76.30 | 37.31% | satisfied |

Table 31. Results of capacity and power consumption in FR1 UL with low load case

| Metrics Cases | Urban Macro scenario (20 UEs per cell) |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  |  |
| AlwaysOn | 97.70% | 134.53 | - | all |  |
|  |  | 132.77 | - | satisfied |  |
| R15/16 CDRX 4ms_2ms_1ms | 94.37% | 96.73 | 28.10% | all |  |
|  |  | 94.63 | 28.73% | satisfied |  |
| R15/16 CDRX for packets bundling 8ms_3ms_1ms | 92.94% | 82.17 | 38.93% | all |  |
|  |  | 80.12 | 39.66% | satisfied |  |

Observation 29: For UL pose/control stream in FR1,

- 26.33%~28.73% mean PSG can be achieved with tolerable capacity loss by using R15/16 CDRX without considering packets bundling.

- 36.83%~39.66% mean PSG can be achieved with tolerable capacity loss by using packets bundling transmission.

Observation 30: Compared with the R15/16 CDRX configuration without packet bundling, the packets bundling transmission scheme can obtain additional 10.50%~10.93% PSG for UL pose/control stream in FR1.

    - FR2

Table 32. Results of capacity and power consumption in FR2 UL with low load case

| Metrics Cases | Indoor Hotspot scenario (20 UEs per cell) |  |  | Dense Urban scenario (20 UEs per cell) |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 97.69% | 177.54 | - | 96.51% | 180.6 | - | all |
|  |  | 177.48 | - |  | 179.98 | - | satisfied |
| R15/16 CDRX 4ms_2ms_1ms | 95.90% | 113.64 | 35.99% | 94.13% | 116.87 | 35.29% | all |
|  |  | 113.34 | 36.14% |  | 114.54 | 36.36% | satisfied |
| R15/16 CDRX for packets bundling 8ms_3ms_1ms | 92.82% | 97.52 | 45.07% | 92.30% | 103.83 | 42.51% | all |
|  |  | 97.35 | 45.15% |  | 101.02 | 43.87% | satisfied |

Observation 31: For UL pose/control stream in FR2,

- 35.29%~36.36% mean PSG can be achieved with tolerable capacity loss by using R15/16 CDRX without considering packets bundling.

- 42.51%~45.15% mean PSG can be achieved with tolerable capacity loss by using packets bundling transmission.

  - UL video stream

Similar to the characteristics of XR downlink traffic, AR uplink traffic such as scene/video information also has the non-integer period (e.g., 16.67ms). Differently, there is no need to model jitter for uplink traffic, since it can be left to UE implementation. Therefore, the four power saving schemes without jitter handling and evaluation method evaluated by using DL XR traffic can be reused for uplink AR traffic model with single stream. And the DRX parameters of different schemes are captured in Table 33. The power consumption characteristics under high and low loads are evaluated separately for AR traffic model with single stream (10Mbps_60FPS_30ms).

- AlwaysOn (Baseline): without adopting any power saving mechanism.

- R15/16 CDRX: adopting R15/R16 CDRX configuration.

- e-CDRX: aligning the start time of DRX ON Duration with the arrival of XR traffic.

- PDCCH skipping: PDCCH skipping without jitter handling.

Table 33. The DRX configuration for R15/16 CDRX and e-CDRX

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 10 | 8 | 4 |
| R15/16 CDRX | 16 | 14 | 4 |
| e-CDRX (FR1) | 16 | 6 | 4 |
| e-CDRX (FR2) | 16 | 8 | 4 |

    - FR1

- Indoor Hotspot

Table 34. FR1 UL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (7 UEs per cell) |  |  | High load case (13 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 104.60 | - |  | 93.59% | 105.12 | - | all |
|  |  | 104.60 | - |  |  | 105.09 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 95.82 | 8.39% |  | 92.22% | 96.73 | 7.98% | all |
|  |  | 95.82 | 8.39% |  |  | 96.69 | 7.99% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 99.15 | 5.21% |  | 92.86% | 99.84 | 5.02% | all |
|  |  | 99.15 | 5.21% |  |  | 99.80 | 5.03% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 67.56 | 35.41% |  | 92.38% | 69.86 | 33.54% | all |
|  |  | 67.56 | 35.41% |  |  | 69.72 | 33.66% | satisfied |
| PDCCH skipping | 100% | 63.28 | 39.50% |  | 92.65% | 64.24 | 38.89% | all |
|  |  | 63.28 | 39.50% |  |  | 64.16 | 38.95% | satisfied |

- Dense Urban

Table 35. FR1 UL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (9 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 97.14% | 113.96 | - |  | 92.95% | 113.64 | - | all |
|  |  | 111.09 | - |  |  | 109.47 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 97.14% | 105.84 | 7.13% |  | 91.53% | 105.83 | 6.89% | all |
|  |  | 102.82 | 7.45% |  |  | 102.81 | 7.60% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 97.14% | 108.84 | 4.49% |  | 91.71% | 108.84 | 4.37% | all |
|  |  | 105.90 | 4.67% |  |  | 105.90 | 5.04% | satisfied |
| e-CDRX 16ms_6ms_4ms | 95.56% | 76.94 | 32.48% |  | 91.60% | 76.95 | 29.49% | all |
|  |  | 72.15 | 33.05% |  |  | 74.37 | 31.08% | satisfied |
| PDCCH skipping | 97.14% | 71.43 | 36.32% |  | 91.77% | 72.57 | 34.87% | all |
|  |  | 67.36 | 38.37% |  |  | 68.46 | 36.64% | satisfied |

Similar to the results and analyzes for downlink traffic, only 4.37%~8.39% mean PSG can be achieved by using the R15/R16 CDRX configuration. And 29.49%~35.41% mean PSG can be obtained without loss of the system capacity for enhanced CDRX scheme. To ensure the performance of cell edge UEs, especially for high load cases, e-CDRX has to configure a longer drx-onDurationTimer and drx-InactivityTimer, which increases unnecessary power consumption for high SINR UEs. In fact, gNB can configure per UE DRX configuration to achieve a larger mean PSG for enhanced CDRX scheme. By using PDCCH skipping, there is no need to configure the jitter window and thus, 34.87%~39.50% mean PSG can be obtained without loss of the system capacity under different system loads conditions.

Observation 32: By adopting the R15/R16 CDRX configuration, 4.37%~8.39% mean PSG can be obtained without the loss of the system capacity for video stream with 10Mbps_60FPS_30ms of AR in FR1 UL.

Observation 33: For UL video stream with 10Mbps_60FPS_30ms of AR in FR1, 29.49%~35.41% PSG and 34.87%~39.50% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

    - FR2

- Indoor Hotspot

Table 36. FR2 UL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (8 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 177.92 | - |  | 95.14% | 178.22 | - | all |
|  |  | 177.92 | - |  |  | 178.18 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 159.70 | 10.24% |  | 92.71% | 160.86 | 9.74% | all |
|  |  | 159.70 | 10.24% |  |  | 160.81 | 9.75% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 165.54 | 6.96% |  | 94.10% | 166.49 | 6.58% | all |
|  |  | 165.54 | 6.96% |  |  | 166.44 | 6.59% | satisfied |
| e-CDRX 16ms_8ms_4ms | 100% | 109.69 | 38.35% |  | 92.36% | 112.65 | 36.79% | all |
|  |  | 109.69 | 38.35% |  |  | 112.16 | 37.05% | satisfied |
| PDCCH skipping | 100% | 84.78 | 52.35% |  | 93.06% | 86.76 | 51.32% | all |
|  |  | 84.78 | 52.35% |  |  | 86.70 | 51.34% | satisfied |

- Dense Urban

Table 37. FR2 UL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (8 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 177.69 | - |  | 92.66% | 177.87 | - | all |
|  |  | 177.69 | - |  |  | 177.74 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 99.60% | 161.06 | 9.36% |  | 91.07% | 161.54 | 9.18% | all |
|  |  | 160.97 | 9.41% |  |  | 161.14 | 9.34% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 166.30 | 6.41% |  | 91.67% | 166.88 | 6.18% | all |
|  |  | 166.30 | 6.41% |  |  | 166.36 | 6.40% | satisfied |
| e-CDRX 16ms_8ms_4ms | 99.60% | 119.11 | 32.97% |  | 90.67% | 121.45 | 31.72% | all |
|  |  | 118.77 | 33.16% |  |  | 120.28 | 32.31% | satisfied |
| PDCCH skipping | 100% | 86.30 | 51.43% |  | 91.27% | 95.68 | 46.21% | all |
|  |  | 86.30 | 51.43% |  |  | 93.72 | 47.27% | satisfied |

Observation 34: By adopting the R15/R16 CDRX configuration, 6.18%~10.24% mean PSG can be obtained without the loss of the system capacity for video stream with 10Mbps_60FPS_30ms of AR in FR2 UL.

Observation 35: For UL video stream with 10Mbps_60FPS_30ms of AR in FR2, 31.72%~38.35% mean PSG and 46.21%~52.35% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

  - UL two-stream

The power consumption results of FR1 UL AR traffic with two streams are provided in this section, that one stream is pose/control stream and the other is aggregating streams (scene/video/data/audio) with 10Mbps_60FPS_30ms. Since the aggregating streams with relatively large packet size mainly limit the system capacity, the power saving schemes used for single video stream evaluation in section3.3 are multiplexed, and proper DRX configuration can be found in Table 38. Compared to single stream model, the PSG of two streams by deploying the same PS schemes is smaller, which is closer to the actual operation of UEs. For UL AR traffic with two streams, since scheduling of intensive pose/control stream, UEs have fewer opportunities to go to a light/deep sleep state and more power will be consumed compared to the single-stream model.

Table 38. The DRX configuration for R15/16 CDRX and e-CDRX

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 10 | 8 | 4 |
| R15/16 CDRX | 16 | 14 | 4 |
| e-CDRX | 16 | 6 | 4 |

- Indoor Hotspot

Table 39. FR1 UL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Low load case (6 UEs per cell) |  |  | High load case (12 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 121.58 | - |  | 93.29% | 121.70 | - | all |
|  |  | 121.58 | - |  |  | 121.70 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 117.39 | 3.45% |  | 92.13% | 117.61 | 3.36% | all |
|  |  | 117.39 | 3.45% |  |  | 117.60 | 3.37% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 119.10 | 2.04% |  | 92.59% | 119.46 | 1.84% | all |
|  |  | 119.10 | 2.04% |  |  | 119.45 | 1.85% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 94.64 | 22.16% |  | 91.90% | 95.70 | 21.37% | all |
|  |  | 94.64 | 22.16% |  |  | 95.22 | 21.76% | satisfied |
| PDCCH skipping | 100% | 87.74 | 27.83% |  | 92.36% | 90.56 | 25.59% | all |
|  |  | 87.74 | 27.83% |  |  | 89.84 | 26.18% | satisfied |

- Dense Urban

Table 40. FR1 UL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (7 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 130.12 | - |  | 92.29% | 131.21 | - | all |
|  |  | 130.12 | - |  |  | 130.59 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 126.00 | 3.17% |  | 90.70% | 127.13 | 3.11% | all |
|  |  | 126.00 | 3.17% |  |  | 126.50 | 3.13% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 127.86 | 1.74% |  | 92.06% | 129.35 | 1.42% | all |
|  |  | 127.86 | 1.74% |  |  | 128.71 | 1.44% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 102.90 | 20.92% |  | 90.48% | 105.52 | 19.58% | all |
|  |  | 102.90 | 20.92% |  |  | 104.88 | 19.69% | satisfied |
| PDCCH skipping | 100% | 98.93 | 23.97% |  | 91.16% | 101.49 | 22.65% | all |
|  |  | 98.93 | 23.97% |  |  | 100.95 | 22.73% | satisfied |

Observation 36: For UL pose/control+video(10Mbps_60FPS_30ms) streams of AR in FR1,

- 1.42%~3.45% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 19.58%~22.16% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 22.65%~27.83% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 37: For UL AR traffic, compared with single stream model, less PSG can be obtained for two streams by using the same power saving schemes.

  - DL and UL evaluating together

For XR/CG applications, DL and UL transmission occur simultaneously and both contribute to power consumption. Also, XR/CG has dense UL traffic, such as pose/control stream with 4ms packets period, which may consume extra power when evaluating some power saving schemes. For example, compared to DL independent evaluation, dense UL traffic may activate the Inactivity timer constantly by using R15/16 CDRX when DL and UL performances are evaluated together. In this section, DL and UL performances are evaluated together for different power saving schemes to obtain the total power consumption for VR and AR application in FR1. A single video stream (30Mbps_60FPS_10ms) is assumed for DL transmission. For UL transmission, pose/control stream and video stream (10Mbps_60FPS_30ms) are assumed. Due to high data rate, low latency requirement and jitter of DL transmission, the power saving schemes used for DL XR traffic are reused for the evaluation of DL and UL together transmission. And for e-CDRX scheme, the offset of CDRX ON Duration is aligned with the quasi-periodic time of DL traffic. Corresponding configurations for R15/16 CDRX and e- CDRX can be found in Table 18. Table 41 gives the adopted traffic models and corresponding scenarios for DL and UL evaluation together on power consumption.

Table 41. Traffic model and scenario combinations used for power consumption evaluation

Option 3: DL and UL performances are evaluated together. DL and UL power consumption are counted to obtain the total power consumption

DL and UL transmission together in FR1 - Indoor Hotspot/Dense Urban scenario: - VR: DL video (30Mbps_60FPS_10ms), UL pose/control stream (0.2Mbps) - AR with UL single-stream: DL video (30Mbps_60FPS_10ms), UL video stream (10Mbps_60FPS_30ms) - AR with UL two-stream: DL video (30Mbps), UL pose/control stream (0.2Mbps) + video stream (10Mbps)

    - VR

For VR traffic, DL and UL performances are evaluated together in FR1, and DL and UL power consumption are counted to obtain the total power consumption. The evaluation results are recorded in Table 42 and Table 43.

- Indoor Hotspot

Table 42. FR1 capacity and power consumption results in Indoor Hotspot scenario for DL +UL

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (10 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 136.46 | - |  | 92.50% | 144.85 | - | all |
|  |  | 136.46 | - |  |  | 142.49 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 131.47 | 3.71% |  | 91.25% | 139.85 | 3.45% | all |
|  |  | 131.47 | 3.71% |  |  | 137.40 | 3.57% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 132.91 | 2.64% |  | 91.81% | 141.47 | 2.33% | all |
|  |  | 132.91 | 2.64% |  |  | 139.08 | 2.39% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 102.68 | 25.12% |  | 90.70% | 110.72 | 23.56% | all |
|  |  | 102.68 | 25.12% |  |  | 108.65 | 23.75% | satisfied |
| PDCCH skipping | 100% | 89.09 | 35.23% |  | 91.25% | 98.82 | 31.78% | all |
|  |  | 89.09 | 35.23% |  |  | 95.31 | 33.11% | satisfied |

- Dense Urban

Table 43. FR1 capacity and power consumption results in Dense Urban scenario for DL +UL

| Metrics Cases | Low load case (7 UEs per cell) |  |  | High load case (13 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 144.27 | - |  | 92.43% | 151.16 | - | all |
|  |  | 144.27 | - |  |  | 150.32 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 137.21 | 3.56% |  | 90.11% | 146.16 | 3.31% | all |
|  |  | 137.21 | 3.56% |  |  | 145.13 | 3.45% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 138.80 | 2.44% |  | 91.58% | 147.77 | 2.24% | all |
|  |  | 138.80 | 2.44% |  |  | 146.77 | 2.36% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 108.85 | 23.49% |  | 91.21% | 118.01 | 21.93% | all |
|  |  | 108.85 | 23.49% |  |  | 115.61 | 23.09% | satisfied |
| PDCCH skipping | 100% | 94.51 | 33.57% |  | 91.21% | 107.05 | 29.18% | all |
|  |  | 94.51 | 33.57% |  |  | 104.55 | 30.45% | satisfied |

Figure 10. Relative time fraction of different power consumption states of all UEs (VR DL+UL)

For DL and UL together transmission, Figure 10 illustrates the relative time fraction of different power consumption states by all UEs for different schemes with 7UEs per cell. Compared with only downlink transmission, DL and UL together simulation consumes more energy for each UE due to the existence of the PUSCH state. For R15/16 CDRX scheme, due to the mismatch between the DRX cycle and the packet arrival cycle, the values of drx-onDurationTimer and drx-InactivityTimer have to be set larger enough to provide sufficient time for gNB scheduling. And because the scheduling of uplink dense pose/control stream frequently restarts the drx-InactivityTimer, the UE has no opportunity to go to light sleep and can only obtain a 3.56% PSG.

For e-CDRX scheme which can align the start time of DRX ON Duration with the quasi-periodic arrival of XR traffic dynamically, a shorter drx-onDurationTimer and drx-InactivityTimer can be configured without impacting system capacity. Some UL pose/control packets that arrive in the UE sleep state have to be transmitted by bundling, which can be seen from the decreased ratio of PDCCH/PUCCH and PUSCH states in case 4 and case 5 from Figure 10. In addition, compared to the R15/R16 CDRX configuration, both enhanced CDRX and enhanced PDCCH skipping scheme can achieve a significant mean PSG without capacity loss. Due to the reduced proportion of the PDCCH/PUCCH state and PUSCH state, there is much more chance to enter a light sleep state with lower energy consumption. But for e-CDRX scheme of DL and UL together simulation, dense pose traffic will frequently restart the drx-InactivityTimer, and result in the decrease of mean PSG compared to DL simulation independently.

Observation 38: For DL and UL together simulation of VR application in FR1,

- 2.24%~3.71% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 21.93%~25.12% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 29.18%~35.23% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 39: For DL and UL together simulation of VR application, both enhanced CDRX and enhanced PDCCH skipping schemes can achieve a significant mean PSG without capacity loss due to the reduced ratio of PDCCH and PUSCH states and the increased chance into the light sleep state.

Observation 40: For DL and UL together simulation of VR application, the mean PSG of enhanced CDRX scheme decreases compared to that of DL simulation independently.

    - AR with UL single-stream

In this section, DL and UL performances are evaluated together for AR traffic in FR1. A single video stream (30Mbps_60FPS_10ms) is assumed for DL transmission, and single video stream (10Mbps_60FPS_30ms) is assumed for UL transmission. In consideration of the intense PDB of DL transmission and to avoid UE frequently restarts the drx-onDurationTimer, the offset of CDRX ON Duration is aligned with the quasi-periodic time of DL traffic for e-CDRX scheme. Corresponding configurations for R15/16 CDRX and e-CDRX can be found in Table 18.

- Indoor Hotspot

Table 44. FR1 capacity and power consumption results in Indoor Hotspot scenario for DL +UL

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (10 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 128.45 | - |  | 92.50% | 144.42 | - | all |
|  |  | 128.45 | - |  |  | 143.08 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 123.05 | 4.20% |  | 91.67% | 140.64 | 2.62% | all |
|  |  | 123.05 | 4.20% |  |  | 138.99 | 2.86% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 125.12 | 2.59% |  | 91.94% | 141.96 | 1.69% | all |
|  |  | 125.12 | 2.59% |  |  | 140.42 | 1.86% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 98.11 | 23.61% |  | 90.83% | 123.09 | 14.77% | all |
|  |  | 98.11 | 23.61% |  |  | 121.12 | 15.35% | satisfied |
| PDCCH skipping | 100% | 88.17 | 31.34% |  | 91.39% | 115.68 | 19.90% | all |
|  |  | 88.17 | 31.34% |  |  | 113.41 | 20.74% | satisfied |

- Dense Urban

Table 45. FR1 capacity and power consumption results in Dense Urban scenario for DL +UL

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (9 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 96.51% | 137.62 | - |  | 92.59% | 149.06 | - | all |
|  |  | 137.59 | - |  |  | 148.13 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 96.19% | 132.41 | 3.79% |  | 91.89% | 145.22 | 2.58% | all |
|  |  | 132.37 | 3.80% |  |  | 143.97 | 2.81% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 96.51% | 134.33 | 2.39% |  | 92.06% | 146.64 | 1.62% | all |
|  |  | 134.31 | 2.39% |  |  | 145.72 | 1.63% | satisfied |
| e-CDRX 16ms_6ms_4ms | 95.87% | 109.03 | 20.77% |  | 90.83% | 128.13 | 14.04% | all |
|  |  | 108.93 | 20.83% |  |  | 126.44 | 14.64% | satisfied |
| PDCCH skipping | 96.19% | 100.22 | 27.18% |  | 91.18% | 120.56 | 19.12% | all |
|  |  | 100.18 | 27.19% |  |  | 119.01 | 19.66% | satisfied |

Observation 41: For DL and UL (single-stream) together simulation of AR application in FR1,

- 1.62%~4.20% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 14.04%~23.61% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 19.12%~31.34% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

    - AR with UL two-stream

In this section, DL and UL performances are evaluated together for AR traffic in FR1, which is closer to the actual operation of UEs. A single video stream (30Mbps_60FPS_10ms) is assumed for DL transmission, and two-stream with pose/control stream (0.2Mbps) and video stream (10Mbps_60FPS_30ms) is assumed for UL. As shown in Table 46 and Table 47, compared to the R15/R16 CDRX configuration, both enhanced CDRX and enhanced PDCCH skipping scheme can achieve a significant mean PSG without capacity loss.

- Indoor Hotspot

Table 46. FR1 capacity and power consumption results in Indoor Hotspot scenario for DL+UL

| Metrics Cases | Low load case (5 UEs per cell) |  |  | High load case (10 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 144.02 | - |  | 92.22% | 152.63 | - | all |
|  |  | 144.02 | - |  |  | 150.72 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 141.41 | 1.81% |  | 90.83% | 150.20 | 1.59% | all |
|  |  | 141.41 | 1.81% |  |  | 148.28 | 1.62% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 142.55 | 1.02% |  | 91.67% | 151.36 | 0.83% | all |
|  |  | 142.55 | 1.02% |  |  | 149.41 | 0.87% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 120.04 | 16.65% |  | 90.56% | 131.32 | 13.96% | all |
|  |  | 120.04 | 16.65% |  |  | 129.21 | 14.27% | satisfied |
| PDCCH skipping | 100% | 115.24 | 19.98% |  | 91.11% | 128.01 | 16.13% | all |
|  |  | 115.24 | 19.98% |  |  | 125.84 | 16.51% | satisfied |

- Dense Urban

Table 47. FR1 capacity and power consumption results in Dense Urban scenario for DL+UL

| Metrics Cases | Low load case (4 UEs per cell) |  |  | High load case (7 UEs per cell) |  |  |  | Type of counted UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | Mean_power | Mean PSG |  | UE% | Mean_power | Mean PSG |  |
| AlwaysOn | 100% | 152.36 | - |  | 92.06% | 156.96 | - | all |
|  |  | 152.36 | - |  |  | 154.81 | - | satisfied |
| R15/16 CDRX 10ms_8ms_4ms | 100% | 149.88 | 1.63% |  | 91.16% | 154.59 | 1.51% | all |
|  |  | 149.88 | 1.63% |  |  | 152.39 | 1.56% | satisfied |
| R15/16 CDRX 16ms_14ms_4ms | 100% | 150.97 | 0.91% |  | 91.61% | 155.72 | 0.79% | all |
|  |  | 150.97 | 0.91% |  |  | 153.53 | 0.83% | satisfied |
| e-CDRX 16ms_6ms_4ms | 100% | 130.51 | 14.34% |  | 90.48% | 136.26 | 13.19% | all |
|  |  | 130.51 | 14.34% |  |  | 133.88 | 13.52% | satisfied |
| PDCCH skipping | 100% | 125.50 | 17.63% |  | 90.70% | 131.96 | 15.93% | all |
|  |  | 125.50 | 17.63% |  |  | 129.95 | 16.06% | satisfied |

Observation 42: For DL and UL (two-stream) together simulation of AR application in FR1,

- 0.79%~1.81% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 13.19%~16.65% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 15.93%~19.98% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

- Performance evaluation results of coverage

In our companion contribution, both system-level evaluation and link-level evaluation are all considered for coverage evaluation. For SLS evaluation, the coupling gain of satisfied UEs is used as the metric for the XR coverage evaluation, and the detailed procedure can be found in our companion’s contribution [8]. And for LLS evaluation, the max isotropic loss (MIL) is the XR coverage evaluation metric. Most of the simulation assumptions for capacity evaluation can be reused, and the channel-specific parameters of PDSCH and PUSCH are shown in appendix B, the channel-specific parameters of other channels are described in TR 38.830[9]. The evaluation results of system-level and link-level for different scenarios are shown below.

  - System-level evaluation results

In this section, the coupling gain of satisfied UEs is provided to analyze the coverage performance of XR traffic from the perspective of system-level simulation. For XR/CG in DL or UL, coverage is defined to be the A-percentile point in CDF of coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application in a given deployment scenario. The CDF curves of coupling gain for the “satisfied” UEs with B=1 and B=capacity are provided, and the values at A=5 percentile point are reported. Based on the capacity evaluation results in section 2.1, system capacity equals the minimal value of the DL and UL system capacity results.

    - FR1

The coupling gain results of AR traffic in FR1 Dense Urban scenario and FR1 Urban Macro scenario are provided from Figure 11 to Figure 13. In system-level coverage evaluation, AR traffic is evaluated, where the DL video stream @30Mbps and a UL single stream (scene/video/data/audio) @10Mbps are simulated independently. For Urban Macro scenario, the capacity in FR1 UL transmission is less than 1, so only the results of B=1 are provided. It can be observed that for both Dense Urban and Urban Macro scenarios in FR1, PUSCH is the bottleneck channel for AR traffic.

- 4GHz, DDDSU, 100MHz, Dense Urban

Figure 11. Coupling gain for 4GHz, DDDSU, 100MHz, Dense Urban with 1UE/cell

Figure 12. Coupling gain for 4GHz, DDDSU, 100MHz, Dense Urban with 9UEs/cell

- 4GHz, DDDSU, 100MHz, Urban Macro

Figure 13. Coupling gain for 4GHz, DDDSU, 100MHz, Urban Macro with 1UE/cell (UL capacity <1)

Observation 43: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

    - FR2

Following carrier frequency and frame structures with 100MHz bandwidth are evaluated in FR2. The coupling gain results of AR traffic in Dense Urban scenario and Indoor Hotspot scenario are presented from Figure 14 to Figure 17. In system-level coverage evaluation, AR traffic is evaluated, where the DL video stream @30Mbps and a UL single stream (scene/video/data/audio) @10Mbps are simulated independently.

- 30GHz, DDDSU, 100MHz, Dense Urban

Figure 14. Coupling gain for 30GHz, DDDSU, 100MHz, Dense Urban with 1UE/cell

Figure 15. Coupling gain for 30GHz, DDDSU, 100MHz, Dense Urban with 8UEs/cell

Observation 44: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

- 30GHz, DDDSU, 100MHz, Indoor Hotspot

Figure 16. Coupling gain for 30GHz, DDDSU, 100MHz, Indoor Hotspot with 1UE/cell

Figure 17. Coupling gain for 30GHz, DDDSU, 100MHz, Indoor Hotspot with 8 UEs/cell

Observation 45: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

  - Link-level evaluation results

    - FR1

In the following, carrier frequency FR1 and frame structure DDDSU are assumed, and only NLOS outdoor-to-indoor is considered for Dense Urban scenario and Urban Macro scenario. In coverage evaluation of XR traffic, the target data rate of PDSCH is 30Mbps, 45Mbps and 60Mbps. And the data rate requirements 0.2Mbps, 10Mbps and 20Mbps for PUSCH are evaluated, which correspond to pose information and two cases for scene information respectively. The evaluation results of channels or signals other than the traffic channels are also provided for reference.

The evaluation results of FR1 Dense Urban and Urban Macro scenario are shown from Figure 18 to  Figure 19. It can be observed that PUSCH is the bottleneck channel among all channels, based on MIL, especially the PUSCH with high data-rate requirement.

- 4GHz, DDDSU, Dense Urban

Figure 18. MIL for 4GHz, DDDSU, Dense Urban

- 4GHz, DDDSU, Urban Macro

Figure 19. MIL for 4GHz, DDDSU, Urban Macro

Observation 46: For both Dense Urban and Urban Macro scenarios in FR1, for VR/AR/CG traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

    - FR2

In the following, carrier frequency FR2, frame structure DDDSU, bandwidth with 100MHz and 400MHz are assumed. For Dense Urban scenario, only NLOS outdoor-to-indoor is evaluated. Only NLOS indoor-to-indoor is considered for Indoor Hotspot scenario. The target data rate of PDSCH is 30Mbps, 45Mbps and 60Mbps. And the data rate requirements 0.2Mbps, 10Mbps and 20Mbps for PUSCH are evaluated, which correspond to pose information and two cases for scene information respectively. And the evaluation results for Dense Urban and Indoor Hotspot scenarios in FR2 are shown from Figure 20 to Figure 23.

- 30GHz, DDDSU, 400MHz, Dense Urban

Figure 20. MIL for 30GHz, DDDSU, 400MHz, Dense Urban

- 30GHz, DDDSU, 100MHz, Dense Urban

Figure 21. MIL for 30GHz, DDDSU, 100MHz, Dense Urban

Observation 47: For Dense Urban scenario in FR2, for VR/AR/CG traffic with 400MHz and AR traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

- 30GHz, DDDSU, 400MHz, Indoor Hotspot

Figure 22. MIL for 30GHz, DDDSU, 400MHz, Indoor Hotspot

- 30GHz, DDDSU, 100MHz, Indoor Hotspot

Figure 23. MIL for 30GHz, DDDSU, 100MHz, Indoor Hotspot

Observation 48: For Indoor Hotspot scenario in FR2, for VR/CG traffic with 400MHz bandwidth, and VR/AR /CG traffic with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 49: For Indoor Hotspot scenario in FR2, for AR 10Mbps UL traffic with 400MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

- Conclusion

In this contribution, we provide our updated simulation results on XR capacity, power consumption and coverage with the following observations:

Observation 1：The system capacity is degraded with the increase of data rate.

Observation 2：The capacity of Cloud Gaming outnumbers VR/AR applications due to the relatively relaxed PDB requirement.

Observation 3: Compared to bundling transmission of dual-eye buffers, separate packet arrivals in time for dual-eye buffers can increase the system capacity.

Observation 4: For FR1 DL transmission with 99% reliability requirement, the capacity results are summarized as follows:

| Traffic Model |  | System capacity in FR1 DL |  |  |
| --- | --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban | Urban Macro |
| CG | 30Mbps_60FPS_15ms | 16.20 | 19.65 | 14.33 |
| VR/AR | 30Mbps_60FPS_10ms | 10.80 | 13.59 | 8.82 |
|  | 30Mbps_120FPS_10ms | 16.53 | 20.78 | 14.59 |
|  | 45Mbps _60FPS_10ms | 5.91 | 6.91 | 4.68 |
|  | 45Mbps_120FPS_10ms | 9.22 | 11.42 | 8.12 |

Observation 5: For FR2 DL transmission with 100MHz bandwidth, the capacity results on 99% reliability requirement are summarized as follows:

| Traffic Model |  | System capacity with 100MHz bandwidth in FR2 DL |  |
| --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban |
| CG | 30Mbps_60FPS_15ms | 9.91 | 16.16 |
| VR/AR | 30Mbps_60FPS_10ms | 8.72 | 13.44 |
|  | 30Mbps_120FPS_10ms | 10.23 | 16.28 |
|  | 45Mbps _60FPS_10ms | 4.67 | 8.20 |
|  | 45Mbps_120FPS_10ms | 6.03 | 10.32 |

Observation 6: For CG and VR applications in FR1, DL is the bottleneck of system capacity because of the high data rate.

Observation 7: For FR1 UL transmission with 99% reliability requirement, the capacity results are summarized as follows:

| Traffic Model |  | System capacity in FR1 UL |  |  |
| --- | --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban | Urban Macro |
| CG/VR | pose/control stream | >20 | >20 | >20 |
| AR | video stream@10Mbps | 13.95 | 9.49 | <1 |
|  | pose/control+video@10Mbps | 12.71 | 7.43 | - |

Observation 8: For FR2 UL transmission with 100MHz bandwidth, the capacity results on 99% reliability requirement are summarized as follows:

| Traffic Model |  | System capacity with 100MHz bandwidth in FR2 UL |  |
| --- | --- | --- | --- |
|  |  | Indoor Hotspot | Dense Urban |
| CG/VR | pose/control stream | >20 | >20 |
| AR | video stream@10Mbps | 8.59 | 8.30 |

Observation 9: The system capacity can be increased by relaxing PDB.

Observation 10: Delay-aware scheduler can increase the system capacity compared to typical PF scheduler.

Observation 11: MU-MIMO scheduler can increase the system capacity in FR1 DL.

Observation 12: Increase system bandwidth can significantly increase the system capacity.

Observation 13: GOP-based I/P-frame traffic model with α=3 shows a significant degradation compared to single-stream traffic model.

Observation 14: GOP-based I/P-frame traffic model with α=1.5 and relaxing PDB of I-frame to 20ms can improve system capacity compared to single-stream traffic model.

Observation 15: Relaxing PDB of I-frame can improve capacity performance for GOP-based I/P-frame traffic model.

Observation 16: When I-frame and P-frame adopt the same PER and PDB values as single-stream traffic model, the capacity performance of slice-based I/P-frame traffic model is the same as that of single-stream traffic model, regardless of α=1.5 or α=3.

Observation 17: Relaxing PER of P-frame can improve capacity performance for slice-based I/P-frame traffic model.

Observation 18: By adopting the R15/R16 CDRX configuration, 3.03%~6.56% mean PSG can be obtained without the loss of the system capacity in FR1 DL for VR/AR with 30Mbps data rate.

Observation 19: 21.72%~36.17% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR1 DL for VR/AR with 30Mbps data rate.

Observation 20: Compared to high load case, low load case can obtain larger power saving gain.

Observation 21: Compared to counting the power consumption of all UEs, counting the power consumption of only satisfied users can bring additional power saving gain.

Observation 22: By adopting the R15/R16 CDRX configuration, 2.83%~5.95% mean PSG can be obtained without the loss of the system capacity in FR1 DL for VR/AR with 45Mbps data rate.

Observation 23: 22.61%~35.24% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR1 DL for VR/AR with 45Mbps data rate.

Observation 24: By adopting the R15/R16 CDRX configuration, 5.81%~10.16% mean PSG can be obtained without the loss of the system capacity in FR2 DL for VR/AR with 30Mbps data rate.

Observation 25: 31.30%~49.05% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR2 DL for VR/AR with 30Mbps data rate.

Observation 26: Compared to FR1, larger PSG can be achieved by using the same power saving scheme in FR2.

Observation 27: By adopting the R15/R16 CDRX configuration, 4.98 %~9.54% mean PSG can be obtained without the loss of the system capacity in FR2 DL for VR/AR with 45Mbps data rate.

Observation 28: 27.16%~47.71% mean PSG can be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss in FR2 DL for VR/AR with 45Mbps data rate.

Observation 29: For UL pose/control stream in FR1,

- 26.33%~28.73% mean PSG can be achieved with tolerable capacity loss by using R15/16 CDRX without considering packets bundling.

- 36.83%~39.66% mean PSG can be achieved with tolerable capacity loss by using packets bundling transmission.

Observation 30: Compared with the R15/16 CDRX configuration without packet bundling, the packets bundling transmission scheme can obtain additional 10.50%~10.93% PSG for UL pose/control stream in FR1.

Observation 31: For UL pose/control stream in FR2,

- 35.29%~36.36% mean PSG can be achieved with tolerable capacity loss by using R15/16 CDRX without considering packets bundling.

- 42.51%~45.15% mean PSG can be achieved with tolerable capacity loss by using packets bundling transmission.

Observation 32: By adopting the R15/R16 CDRX configuration, 4.37%~8.39% mean PSG can be obtained without the loss of the system capacity for video stream with 10Mbps_60FPS_30ms of AR in FR1 UL.

Observation 33: For UL video stream with 10Mbps_60FPS_30ms of AR in FR1, 29.49%~35.41% PSG and 34.87%~39.50% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

Observation 34: By adopting the R15/R16 CDRX configuration, 6.18%~10.24% mean PSG can be obtained without the loss of the system capacity for video stream with 10Mbps_60FPS_30ms of AR in FR2 UL.

Observation 35: For UL video stream with 10Mbps_60FPS_30ms of AR in FR2, 31.72%~38.35% mean PSG and 46.21%~52.35% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

Observation 36: For UL pose/control+video(10Mbps_60FPS_30ms) streams of AR in FR1,

- 1.42%~3.45% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 19.58%~22.16% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 22.65%~27.83% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 37: For UL AR traffic, compared with single stream model, less PSG can be obtained for two streams by using the same power saving schemes.

Observation 38: For DL and UL together simulation of VR application in FR1,

- 2.24%~3.71% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 21.93%~25.12% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 29.18%~35.23% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 39: For DL and UL together simulation of VR application, both enhanced CDRX and enhanced PDCCH skipping schemes can achieve a significant mean PSG without capacity loss due to the reduced ratio of PDCCH and PUSCH states and the increased chance into the light sleep state.

Observation 40: For DL and UL together simulation of VR application, the mean PSG of enhanced CDRX scheme decreases compared to that of DL simulation independently.

Observation 41: For DL and UL (single-stream) together simulation of AR application in FR1,

- 1.62%~4.20% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 14.04%~23.61% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 19.12%~31.34% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 42: For DL and UL (two-stream) together simulation of AR application in FR1,

- 0.79%~1.81% mean PSG can be achieved without capacity loss by adopting the R15/R16 CDRX configuration.

- 13.19%~16.65% mean PSG can be achieved without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic.

- 15.93%~19.98% mean PSG can be achieved without capacity loss by adopting PDCCH skipping scheme.

Observation 43: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

Observation 44: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

Observation 45: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in SLS evaluation.

Observation 46: For both Dense Urban and Urban Macro scenarios in FR1, for VR/AR/CG traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 47: For Dense Urban scenario in FR2, for VR/AR/CG traffic with 400MHz and AR traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 48: For Indoor Hotspot scenario in FR2, for VR/CG traffic with 400MHz bandwidth, and VR/AR /CG traffic with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 49: For Indoor Hotspot scenario in FR2, for AR 10Mbps UL traffic with 400MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

# References

- Chairman’s Notes of RAN1 #104-e final (Online meeting, January 25th – February 5th, 2021).

- Chairman’s Notes of RAN1 #104b-e final (Online meeting, April 12th – April 20th, 2021).

- Chairman’s Notes of RAN1 #105-e final (Online meeting, May 10th – May 27th, 2021).

- R1-2106629, Remaining issues on traffic models of XR, vivo.

- R1-2106632, Challenges and potential enhancements of XR, vivo.

- R1-2104396, Discussion on evaluation methodologies for XR, vivo.

- TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)” V16.0.0 (2019-06).

- R1-2106630, Remaining issues on evaluation methodologies for XR, vivo.

- TR 38.830, “Study on NR coverage enhancements (Release 17)” V17.0.0 (2020-12).

# Appendix A – Simulation assumptions

Table I. Simulation assumption for FR1

| Parameter | value |  |  |
| --- | --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Channel model | InH | Uma | Uma |
| Carrier frequency | 4GHz |  |  |
| Bandwidth | 100 MHz, 1.72% Guard Band |  |  |
| Subcarrier spacing | 30 KHz |  |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64T: (8,8,2,1,1;4,8) (dH, dV)=(0.5, 0.5)λ |  |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | DL: For 4R: (1,2,2,1,1;1,2), (dH, dV)=(0.5, N/A)λ UL: For 2T: (1,1,2,1,1;1,1), (dH, dV)=( 0.5, N/A)λ |  |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |  |
| BS Power | 24 dBm per 20MHz | 44 dBm per 20MHz | 49 dBm/20 MHz |
| UE max Power | 23 dBm |  |  |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| ISD | 20 m | 200 m | 500m |
| BS height | 3 m | 25 m | 25m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl~uniform(1,Nfl) where Nfl~uniform(4,8) |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |  |
| Max MCS | 256QAM |  |  |
| Device deployment | 100% indoor | 20% outdoor, 80% indoor |  |
| Down-tilt | 90 degrees | 12 degrees | 6 degrees |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic |  |  |
| Target BLER | 10% |  |  |
| UE speed | 3 km/h |  |  |

Table II. Simulation assumption for FR2

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 30 GHz |  |
| Bandwidth | 100 MHz, 4.96% Guard Band |  |
| Subcarrier spacing | 120 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (16,8,2,1,1;1,1) (dH, dV)=(0.5, 0.5)λ | For 2T/panel: (4,8,2,2,2;1,1) (dH, dV)=(0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R/panel: (1,4,2,1,3;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8dBi |
| UE antenna pattern | Directional antenna panel, 5 dBi |  |
| Min. BE-UT distance(2D) | 0 m | 10 m |
| BS height | 3 m | 25 m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl~uniform(1,Nfl) where Nfl~uniform(4,8) |
| BS Power | 23dBm per 80MHz, EIRP should not exceed 58 dBm | 40dBm per 80MHz, EIRP should not exceed 73 dBm |
| UE Power | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -60, alpha = 0.6) | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -74, alpha = 0.6) |
| Noise Figure | BS:7 dB, UE:13 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 100% outdoor |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% |  |
| UE speed | 3 km/h |  |

# Appendix B – Channel-specific parameters for Coverage

Table Ⅲ. Channel-specific parameters for PDSCH and PUSCH in FR1

| Parameter | value |
| --- | --- |
| Channel model for LLS | TDL-C |
| Number of gNB antenna port in LLS | 2 |
| Number of UE antenna port in LLS | 1Tx/4Rx |
| Frequency hopping | w/o frequency hopping |
| Repetition | w/o repetition |
| BLER | w/o HARQ, 10% iBLER. |
| DMRS configuration | For 3km/h: Type I, 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for uplink CP-OFDM for downlink |
| PRBs/MCS for PDSCH (VR/AR, 30Mbps) | 273RB, Qm = 2, Code rate = 526/1024 |
| PRBs/MCS for PDSCH (VR/AR, 45Mbps) | 273RB, Qm = 4, Code rate = 378/1024 |
| PRBs/MCS for PDSCH (VR/AR, 60Mbps) | 273RB, Qm = 4, Code rate = 490/1024 |
| PRBs/MCS for PDSCH (CG, 30Mbps) | 273RB, Qm = 2, Code rate = 379/1024 |
| PRBs/MCS for PDSCH (CG, 45Mbps) | 273RB, Qm = 2, Code rate = 526/1024 |
| PRBs/MCS for PDSCH (CG, 60Mbps) | 273RB, Qm = 4, Code rate = 340/1024 |
| PRBs/MCS for PUSCH (CG/VR,0.2Mbps) | 273RB, Qm = 2, Code rate = 30/1024 |
| PRBs/MCS for PUSCH (AR, 10Mbps) | 273RB, Qm = 2, Code rate = 379/1024 |
| PRBs/MCS for PUSCH (AR, 20Mbps) | 273RB, Qm = 4, Code rate = 378/1024 |

Table Ⅳ. Channel-specific parameters for PDSCH and PUSCH in FR2

| Parameter | value |
| --- | --- |
| Channel model for LLS | TDL-A |
| Number of gNB antenna port in LLS | 2 |
| Number of UE antenna port in LLS | 1Tx/2Rx |
| Frequency hopping | w/o frequency hopping |
| Repetition | w/o repetition |
| BLER | w/o HARQ, 10% iBLER. |
| DMRS configuration | For 3km/h: Type I, 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for uplink CP-OFDM for downlink |
| PRBs/MCS for PDSCH (VR/AR, 30Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 157/1024 For 100MHz: 66RB, Qm = 2, Code rate = 526/1024 |
| PRBs/MCS for PDSCH (VR/AR, 45Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 193/1024 For 100MHz: 66RB, Qm = 4, Code rate = 434/1024 |
| PRBs/MCS for PDSCH (VR/AR, 60Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 308/1024 For 100MHz: 66RB, Qm = 4, Code rate = 553/1024 |
| PRBs/MCS for PDSCH (CG, 30Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 99/1024 For 100MHz: 66RB, Qm = 2, Code rate = 379/1024 |
| PRBs/MCS for PDSCH (CG, 45Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 157/1024 For 100MHz: 66RB, Qm = 2, Code rate = 526/1024 |
| PRBs/MCS for PDSCH (CG, 60Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 193/1024 For 100MHz: 66RB, Qm = 4, Code rate = 340/1024 |
| PRBs/MCS for PUSCH (CG/VR,0.2Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 30/1024 For 100MHz: 66RB, Qm = 2, Code rate = 30/1024 |
| PRBs/MCS for PUSCH (AR, 10Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 99/1024 For 100MHz: 66RB, Qm = 2, Code rate = 379/1024 |
| PRBs/MCS for PUSCH (AR, 20Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 193/1024 For 100MHz: 66RB, Qm = 4, Code rate = 378/1024 |