3GPP TSG RAN WG1 #106-e	                                            R1-2106632

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Challenges and potential enhancements for XR

Agenda Item:	8.14.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on challenges and potential enhancement areas for XR to achieve optimal user experience.

- Challenges

According to the demands of XR application, a plurality of progress for XR traffic modeling has been achieved in previous RAN1 meetings. The characteristics of XR traffic and challenges are summarized below.

- High data rate

For 3D VR videos with high resolution, based on different frame rates, color codecs, bit-depths, compression rates and etc., the transmission data rate could be up to 60Mbps and above, which would lead to low system capacity.

- Varying frame size

In the field of video compression, three major frame types as depicted in Figure 1 are defined through three different video algorithms with the following characteristics:

- I-frames are the least compressible which can decode independently.

- P-frames can use previous frames to decompress and are more compressible than I-frames.

- B-frames can use both previous and forward frames to get the highest amount of data compression.

A GOP (Group of Pictures) has three types of frames, I (intra) frames, P (predicted) frames and B (bi-directional) frames. The I frame is the least compressed and the most complete frame within the GOP. For I frame, only information contained within the frame itself is compressed, without any compression based on predictions. The first frame in a GOP is always an I frame, it is used as a reference for the P and B frames. The start of a new I frame indicates a new GOP sequence has begun.

P frames are compressed using interpolated data from the previous I or P frames in the GOP. P frames are moderately compressed compared to I frames. B frames are the most compressed frame type. B frames are predicted using interpolated data from both prior and later I and P frames within the GOP. It’s important to note that B frames cannot use other B frames for predictions. Both P and B frames use ‘Inter-frame’ compression, meaning predictions are used to compress data relative to neighboring frames. This type of compression uses a combination of complex algorithms to reduce data and retain quality.

Figure 1. Schematic of video frames

For different frame types, video frame sizes are varying due to different compression rates. Even for a fixed frame type, video frame sizes are also varying over time due to different compression rates based on the content of video frames. An example of online real video frame size varying over time is shown in Figure 2. Based on the capture of the real video frames, truncated Gaussian distribution is adopted in the previous RAN1 meeting for the frame size distribution of video stream for AR/VR/CG.

Figure 2. An example of real video frame size varying over time

- Non-integer period

In RAN1 #104e meeting, it was agreed that the frame per second (fps) for DL video stream for a single UE can be 30, 60 (baseline), 90, 120 FPS. And Table 1 intuitively shows the corresponding non-integer periods of different video frame rates. For comparison, we further give the DRX configuration IE described in TS 38.331 [1], and the values of long/short DRX cycle and related DRX timers are all integers. Hence, it can be observed that the current DRX configuration cannot match with these non-integer XR traffic arrival interval and the mismatching may directly lead to unacceptable transmission delay and even packet loss due to exceeding the PDB limitation. XR traffic cycle mismatch issue is not only for DRX configuration, but also for SPS and CG configurations.

Table 1. Frame rates and period of 3D VR video

| Frame rate (FPS) | 30 | 60 | 90 | 120 |
| --- | --- | --- | --- | --- |
| Period (ms) | 33.33 | 16.67 | 11.11 | 8.33 |

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

…

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

- Packets Jitter

XR DL video frames are usually generated periodically at the server side. However, the packet arrival interval in RAN may not be exactly periodic but quasi-periodic due to jitter resulting from frame rendering, frame encoding, network congestion, routing changes, etc. According to the agreements achieved in RAN1#104-e meeting, jitter distribution is subject to a truncated Gaussian distribution with the range of [-4, 4] ms. It is a very large time range relative to XR traffic period and can have a significant negative impact on scheduling delays or power savings.

- Low latency

To provide good immersive experience, the latency of XR traffic should be as low as possible. As agreed in the previous RAN1 meeting, air interface PDB for DL video stream is as follows. It should be noted that the XR traffic could be much challenging than eMBB or URLLC traffic due that it needs to satisfy the high transmission rate within the short latency bound.

- VR/AR:

  - 10ms

  - Other values, e.g., 5ms, 20ms can be optionally evaluated.

- CG:

  - 15ms

  - Other values, e.g., 10ms, 30ms can be optionally evaluated.

- High reliability

To maintain a reliable registration of the virtual world with the real world, and to ensure accurate tracking of the XR viewer’s pose, the reliability requirement of XR traffic can be 10e-3 or 10e-4, which will press on CSI measurement/reporting.

- Multiple flows/streams

In the previous RAN1 meeting, some discussions focused on the multiple flows/streams of XR traffic. It has been considered that each flow/stream may have individual PDB, frame/packet size and reliability requirements due to the characteristics of video frame. For an instance, the important video frame (e.g. I frame) should be guaranteed with high priority, while the less important video frame (e.g. P frame) might even be dropped conditionally since XR devices could have fault tolerance and recovery capability.

- Low system capacity

The applications of XR traffic generally require high throughput, low latency and high reliability at the same time. When plenty of traffic bursts arriving, it will be no doubt to bring huge challenges for the system capacity of NR networks. As observed in our companion contribution [2], for VR video stream, with the increase of users per cell, system capacity performance is degraded rapidly.

- High power consumption

The applications of XR traffic will bring critical challenges for battery life, due to heavy and high data rate traffic, complicated computation and frequent interaction with the network. In addition, for XR devices, the size and weight should be optimized design so that the devices can be worn for a long duration. It will result in more restrictions for battery capacity and heat dissipation compared to traditional devices such as smartphones. As shown in the histogram below, power consumption for two different video applications on smartphones via NR networks is presented. Based on the measurement data shown in Figure 3, the power consumption proportion of communication modules is around 40% for online video traffic, which is much higher than that of display modules.

Figure 3. Measurement data for power consumption of different video traffic

Notes :

- The data is collected from vivo product teams.

- The measurement is performed in a laboratory environment.

- The test traffic is carried over an NR network.

- The communication modules include baseband, radio frequency (RF), power amplifier (PA) modules, etc.

- The display modules include the liquid crystal module (LCM) panel, etc.

Another analysis from the aspect of power consumption model, we can also derive that the XR traffic will cause more power consumption compared with the existing eMBB traffic models (e.g. FTP3, instant messaging) because of its dense traffic period and high traffic data rate. An intuitive example is that there could be no opportunity for UE to go into the deep sleep state and even the light sleep state to do power saving since the state transition times (i.e. 20ms and 6ms) are even longer than the XR traffic period. Hence, due to the characteristic of XR traffic, high power consumption is an undoubted challenge.

- Potential RAN enhancements for XR

## 3.1 Capacity enhancement

- SPS and CG enhancement

Since XR traffic is quasi-periodic, SPS and CG transmission for XR traffic is a particularly suitable method for data transmission, which can reduce the overhead of control signalling. However, there are two critical issues by using SPS and CG transmission for XR traffic. Firstly, the periodicity configuration of SPS-Config IE and ConfiguredGrantConfig IE as shown below does not support the non-integer period of XR traffic, which means the higher layer configured period and real XR traffic period might mismatch over time. Secondly, due to the scheduling-related parameters (e.g. TDRA, FDRA, MCS) are configured semi-persistently, the amount of data that can be transmitted at one SPS or CG period is fixed and it cannot apply to XR traffic with the characteristic of varying packet size.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                  SEQUENCE {

periodicity                     ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

…

periodicityExt-r16              INTEGER (1..5120)

…

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

…

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

…

periodicityExt-r16                      INTEGER (1..5120)

…

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

Though dynamic grant scheduling can well handle above issues, but the benefit of SPS/CG should not be neglected. It is necessary to solve above issues and accommodate XR traffic for SPS/CG transmission. One possible solution is to configure multiple SPS/CG configurations. The other possible solution is to dynamically adjust SPS/CG parameter(s).

Proposal 1: Study enhancement for SPS and CG to accommodate XR traffic.

- RAN awareness transmission

In principle of service agnostic, NR system architecture and protocols are designed without targeting at any particular service. Such design principle may provide benefits on generality and extensibility. However, service transparent to the network may not work efficiently any longer when new applications/services with different characteristics and requirements from the traditional service show up. To be specific, XR service is transparent in current specification, such that NR network has no way to optimize for the XR service since there is no difference between XR service and traditional multimedia services in IP packets level.

As mentioned above, XR traffic demands high data rate, low latency, etc. requirements. If RAN can acquire some of XR characteristics, the network can perform smart QoS management and adaptive scheduling for the flows of XR traffic, which can improve the immersive experience for user. For instance, XR service-related information e.g., frame boundary identification, identifier of IP packets belonging to a same frame, identifier of multiple flows with different importance, synchronization information among packets or flows, packets latency and etc. can be acquired by RAN so as to RAN can adaptively do packets dropping and delay aware scheduling to enhance the system capacity. Note that user privacy needs to be protected from disclosure in the XR service-related information. So, there might be a trade-off between the benefits of RAN awareness transmission and user privacy.

It is no doubt that RAN awareness transmission needs to be designed cross layers, both SA and RAN should be involved. From RAN perspective, we need to know what XR related information will be delivered to RAN and some potential evaluation outputs from RAN also need to be certified to SA2/4, if necessary. In our point of view, the following two approaches can be considered as starting points for RAN awareness transmission. In line with the above comparison of Table 2, it can be observed that neither of the two approaches is optimal, and both of them are high correlated with the details of XR service-related information.

Table 2. Two possible approaches for XR service awareness transmission

| XR service awareness transmission approaches | Characteristics |
| --- | --- |
| DPI (Deep packet inspection) | - Insert service-related information to PDU header or inspect application data - Might have significant video encoding/decoding, network protocol, core network, RAN impact - Most of XR service-related information can be acquired - Application awareness |
| DFI (Deep/dynamic flow inspection) | - AI based training - Less impact to current system architectures - Some XR service-related information might be unavailable - Application unawareness |

Proposal 2: Study RAN awareness transmission for XR e.g., if RAN can identify the multiple flows with different importance, it can perform adaptive scheduling or, if RAN can obtain the information of frame boundary, it can do packets dropping and delay aware scheduling accordingly.

- New 5QI value(s) for XR service

Assuming XR service-related information is available at RAN side, it is necessary to introduce some new QoS parameters additionally to assist RAN adaptive transmission. Additionally, current 5QI values cannot reflect the characteristic of XR service. For an example, the frame level QoS requirements and synchronization requirement of multiple flows are limit. Hence, RAN1, SA2 and SA4 are all involved and cooperation is required for introducing of new 5QI value(s) for XR services. The new 5QI value(s) is required to be delivered from CN to RAN. Subsequently, RAN can further study some potential improvement accordingly e.g., adaptive scheduling.

Proposal 3: Study and design new 5QI value(s) for XR services, which belongs to the scope of SA.

- Interaction among UE, gNB, CN and application server

The XR-related information interaction among UE, gNB and CN/application server are also beneficial for improving XR user experience. The detailed interaction content can vary depending on the different interaction nodes. For example, UE can report the XR service assistance information (e.g., UL traffic related information, special pose/control information) to assist gNB for UL scheduling and transmission. On the other hand, gNB could also provide some user experience-oriented information to CN and/or application server for QoS related (e.g., data-rate) adjustment/adaptation.

Proposal 4: Study the XR-related information interaction among UE, gNB, CN and application server to assist scheduling/QoS management/source application traffic adjustment e.g., user experience-oriented assistance information reporting.

## 3.2 Power saving enhancement

Based on the analysis in section 2, the enhancement of power consumption is essential. Before discussing the potential enhancement scheme, we need to review the specific characteristics of XR traffic as follow. And all of these characteristics will impact to the XR capacity.

- High data rates

- Dense traffic period (such as 8.33ms)

- Low latency requirement

- High reliability requirement

- Non-integer traffic period and jitter

- Variable file size (i.e. variable packet data size)

Considering that there are already many power saving technologies adopted in R15 and R16, we should firstly reuse them in XR study as the starting point. Going through the existing power saving technologies, none of them takes UE capacity into consideration. A simple example is that when UE is configured with CDRX, the DRX off state can reduce power consumption but at the same time incur delay of DL data transmissions. This absolutely cannot be tolerated by the latency-sensitive service like XR traffic, because the increase of scheduling delay directly results in packet loss and make negative impact on UE capacity. Also, the wake-up signal (WUS) by using DCI format 2_6 and designed for relatively sparse traffic (i.e. eMBB traffic, such as FTP3 and instant messaging) is inapplicable to XR traffic. Other schemes such as BWP switching, MIMO layer adaption having a long application delay are still not beneficial for power saving and capacity.

- CDRX enhancement

As analysed above, we need to consider the potential power consumption reduction scheme to achieve a better trade-off for power saving and capacity. According to TS 38.331 [1], it can be observed that current DRX configurations are not suitable for XR traffic, and enhancement of DRX configuration for XR traffic is needed. To align the start time of DRX cycle with traffic arrival time, the most direct method is to extend the value of DRX cycle to the field of non-integer, such as 8.33ms, 16.67ms. However, the non-integer DRX cycle is not flexible because of the various traffic period and the jitter effect of DL transmission. To solve this problem, DRX start offset adaptation to XR traffic considering jitter effects can be considered for DRX enhancement. One potential method of DRX start-offset adaptation can be dynamically triggered by DCI, such as DCI indicates a certain start-offset applying for the next DRX cycle.

- Jitter handling

Impact of jitter for UE receiving DL transmission should also be studied since the pretty large jitter range (i.e., [-4, 4]ms) will definitely results in either power consumption or scheduling latency. One of the methods to deal with the jitter effect is to extend the length/duration of DRX on-duration or PDCCH monitoring to cover the jitter range. However, by means of this method, the power consumption will be sacrificed for the sake of reducing the scheduling latency. And the bad result will also take if we give up reducing latency to save power. To this end, the design principle of jitter handling method is to make a trade-off between power saving and capacity.

The wake-up indication introduced in Rel-16 power saving WI is applied for pre-indicating UE whether to start the associated DRX on-duration. Inspired by this, we might need to enhance the wake-up indication such as it can be triggered by the network when DL frames arrive at the gNB side during outside DRX active time. And the wake-up indication could be carried by low power wake-up signal (WUS) for the sake of power saving. To flexibly inform UE with the wake-up indication, network may also need to configure enough number of WUS monitoring occasions. Additionally, another method could be to flexibly do sparse/dense PDCCH monitoring switching. For example, when during the jitter range, UE is configured with dense PDCCH monitoring, and once detecting the arrival of a frame, it can be switched to a relative sparse PDCCH monitoring. Surely, the opposite PDCCH monitoring configuration will also makes sense. However, how to achieve a balance between capacity and power saving for jitter handling need more further specifically investigation.

- DCI-based PDCCH adaptation

In addition, DCI-based power saving enhancements can be considered, due to the good flexibility. Considering the quasi-periodic traffic for XR, Layer-1 based dynamic PDCCH monitoring reduction schemes (e.g., PDCCH skipping or search space group switching) can be adopted for trade-off between power saving and UE capacity. However, DCI-based power saving technique is highly correlated with the progress of R17 power saving enhancement.

- UL and DL transmission alignment

Another consideration is that both DL receptions and UL transmissions are quite frequent for XR devices, e.g. DL video data reception and UL pose uploading. If XR device needs to receive DL data and transmit UL data at a short interval, there is no much time for UE to go into sleep for reducing the power consumption. Therefore, UL and DL transmission time alignment to avoid doing transmitting/receiving during DRX off stage or PDCCH skipping duration can be considered for power saving enhancement. For example, SPS/CG configuration relative to DRX cycle, or retransmission cancellation conditionally.

Proposal 5: Study power saving enhancements for XR, at least the following aspects can be considered:

- CDRX enhancement e.g., adaptive DRX configurations/parameters.

- Jitter handling e.g., low power WUS or sparse/dense PDCCH monitoring switching.

- DCI-based PDCCH adaptation refer to R17 power saving WI.

- UL and DL transmission alignment e.g., SPS/CG configuration relative to DRX cycle, retransmission cancellation conditionally.

## 3.3 Mobility and coverage enhancements

Regarding mobility enhancement for XR, Conditional Handover (CHO), Dual-Active Protocol Stack (DAPS) and L1/L2 based mobility etc. can be considered. However, in our view, these mobility enhancement methods can be handled in mobility enhancement rather than in XR enhancement, unless we can identify some XR specific mobility enhancement areas. Additionally, the similar concern is proposed by us for coverage enhancement of XR.

## 3.4 Other enhancements

- CSI measurement/reporting enhancement

In NR Rel-15 and Rel-16, high data rate transmission was not taken into account for URLLC evaluation. Therefore, it is uncertain whether NR can well support XR traffic or not. To support low latency and tight HARQ processing time requirement of XR traffic, CSI measurement should be as accurate as possible, and CSI reporting should be done in time or even in advance, especially for retransmission. Considering the relationship between UL and DL transmission could exist, e.g. UL pose traffic providing the pose update information may trigger the corresponding DL video traffic, some early preparation (e.g. CSI reporting, beam management) according to the UL transmission can be triggered to deal with some upcoming DL traffic burst.

Proposal 6: Study CSI measurement/reporting enhancement for XR traffic.

Except for the high priority enhancements for XR i.e., capacity and power saving, there are still some other enhancements e.g., network coding, interference mitigation, sidelink enhancement etc. In our point of view, these enhancement areas can only be considered when there is still residual discussion room after the study of high priority XR enhancements.

- Conclusion

In this contribution, we provide our views on challenges and potential enhancements for XR with the following proposals:

Proposal 1: Study enhancement for SPS and CG to accommodate XR traffic.

Proposal 2: Study RAN awareness transmission for XR e.g., if RAN can identify the multiple flows with different importance, it can perform adaptive scheduling or, if RAN can obtain the information of frame boundary, it can do packets dropping and delay aware scheduling accordingly.

Proposal 3: Study and design new 5QI value(s) for XR services, which belongs to the scope of SA.

Proposal 4: Study the XR-related information interaction among UE, gNB, CN and application server to assist scheduling/QoS management/source application traffic adjustment e.g., user experience-oriented assistance information reporting.

Proposal 5: Study power saving enhancements for XR, at least the following aspects can be considered:

- CDRX enhancement e.g., adaptive DRX configurations/parameters.

- Jitter handling e.g., low power WUS or sparse/dense PDCCH monitoring switching.

- DCI-based PDCCH adaptation refer to R17 power saving WI.

- UL and DL transmission alignment e.g., SPS/CG configuration relative to DRX cycle, retransmission cancellation conditionally.

Proposal 6: Study CSI measurement/reporting enhancement for XR traffic.

# References

- TS 38.331, “Radio Resource Control (RRC) protocol specification; NR” V16.3.1 (2021-01).

- R1-2106631, Performance evaluation results for XR, RAN1 #106e.