3GPP TSG RAN WG1#106-e                              	                      R1- 2106696

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.2.6

Source:	Spreadtrum Communications

Title:	Discussion on channel access mechanism for above 52.6GHz

Document for:	Discussion and decision

Introduction

In RAN#90-e meeting, the WID of extending current NR operation to 71GHz has been agreed. And the following channel access mechanism has been made: [1]

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this contribution, we focus on the channel access mechanism for operation in the 60GHz unlicensed band.

Discussion

LBT bandwidth

In RAN1#104b-e meeting, the definition of LBT bandwidths was down-selected and the following alternatives were agreed [4]:

Agreement: For LBT for single carrier transmission, continue down selection between - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, continue down selection between - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

In Rel-16 NR-U, LBT can be performed in units of 20MHz, and the NR-U operating bandwidth is also an integer multiple of 20MHz. This is to ensure that the transmission bandwidth matches the LBT bandwidth. To reuse the LBT mechanism of Rel-16 NR-U as much as possible, at least, for single carrier case, Alt SC.1 should be supported.

For carrier aggregation case, performing LBT measurement for each carrier may have benefits over a single wideband LBT measurement from channel access perspective. Because the former can pass the LBT on other carrier even if one carrier is congested. Therefore we prefer to support Alt CA.1.

Proposal 1: Regarding LBT bandwidth, at least Alt SC.1 and Alt CA.1 should be supported.

- For single carrier transmission, at least gNB/UE should perform LBT over the channel bandwidth (or BWP bandwidth)

- For multi-carrier transmission, at least gNB/UE should perform multiple LBT, one for each channel bandwidth separately

Directional LBT

In Rel-16 NR, in order to fairly coexist with Wi-Fi system in low frequency range (e.g. 802.11ac/802.11ax), a channel access mechanism based on Listen-Before-Talk (LBT) is introduced for unlicensed band operation.

The Rel-16 NR-U channel access mechanism is designed only for the low frequency range (i.e. below 7GHz), and the channel access mechanism is similar to LBT mechanism of Wi-Fi system in the low frequency range. As well, Rel-16 NR-U channel access applies the same omni-directional LBT as the Wi-Fi system in the low frequency range. Behind the omni-directional LBT, the omin-directional transmission is assumed.

However, in high frequency range (i.e. above 52.6GHz) the directional transmission, i.e. beam based transmission, should be assumed to combat the large channel fading. In fact, one of key differences between NR-U/Wi-Fi in high frequency range and NR-U/Wi-Fi in low frequency range is the propagation characteristic of the radio. NR-U/Wi-Fi in high frequency range always imposes the use of beamforming to overcome the large propagation loss. Under this assumption, the directional LBT, should be conducted to the beam based transmission. Otherwise, the mismatch between LBT and transmission may cause the unexpected interference. For example, the mismatch between the omni-directional LBT and the subsequent directional transmission may cause the unexpected interference. Specifically, the gNB/UE with omni-directional LBT may suffer from the continuous directional interference and the gNB/UE may lose lots of transmission opportunities. As the counterpart, the directional LBT with the directional transmission can work well like the omni-directional LBT with the omni-directional transmission.

In addition, the directional transmission can be regarded as a way to achieve the spatial reuse (e.g. MU-MIMO), so the directional LBT has the merit of the better spatial reuse.

Proposal 2: The directional LBT should be supported in 60GHz unlicensed band.

A further question is the relationship between the sensing beam and the transmission beam and whether this relationship need to be indicated? There are two views on these issues. One thinks the relationship between the sensing beam and the transmission beam should be left as implementation, while the other thinks the relationship should be defined and at least sensing beam “covers” the transmission beam. From our point of view, if this relationship is not defined, there is a risk that the correspondence between the sensing beam and the transmission beam cannot be ensured.

Furthermore, we believe that the beam correspondence framework or the QCL/TCI indication framework can be extended to define the association between sensing beam and transmission beam.

Proposal 3: The relationship between all the LBT beams and the transmission beam should be defined and at least sensing beam “covers” the transmission beam.

Proposal 4: The beam correspondence framework or QCL/TCI framework can be extended to define “cover”.

Receiver assisted LBT

In RAN1#104-e meeting the receiver assisted LBT had been discussed and the following had been agreed [2]:

Agreement: For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion - Alt 1. Legacy RSSI measurement and reporting with possible enhancements - Alt 2. AP-CSI report with possible enhancements - Alt 3. LBT at receiver - Alt 3.1 eCCA - Alt 3.2 Cat2 LBT

Three receiver assisted LBT alternatives were recommended in RAN1#104-e meeting. Both alt 1 and alt 2 can be readily implemented, as long as some modification need to be made on measurement and reporting. These two alternatives are similar to the Rel-15/Rel-16 mechanism. Regarding to Alt 3, it is likely an approach of performing a handshake between the gNB and the UE. However, it is not clear whether such a handshake is required before each DL transmission. If so, more resources would be consumed and gNB may lose the channel due to frequent DL/UL switching within the COT. Therefore, at least Alt 1 and Alt 2 should be supported for further study.

Proposal 5: Regarding receiver assisted LBT, at least the method of Legacy RSSI measurement and reporting with possible enhancements (Alt 1) and the method of AP-CSI report with possible enhancements (Alt 2) should be supported for further study.

Energy detection threshold enhancement

In RAN1#104-e meeting the ED threshold had been discussed and the following had been agreed [2]:

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

ETSI BRAN 302 567 [3] has provided a simple formula for determination of the ED threshold. The formula only includes the transmission power without considering the impact of LBT bandwidth and the beamforming gain. Given that it can have different LBT bandwidth and transmission power for different RATs operating on the 60GHz unlicensed band, in order to guarantee fairly coexistence among these RATs, the formula of ED threshold should consider the LBT bandwidth and beamforming gain.

Proposal 6: The formula of ED threshold should consider the LBT bandwidth and beamforming gain.

In RAN1#105-e meeting the sensing structure had been discussed and the following had been agreed [5]:

Agreement: For energy measurement in 8us deferral period, continue down-selection between the following alternatives - Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us - Alt 2. One measurement is required - FFS where the measurement is located Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

Regarding to the energy measurement in 8us deferral period, alt 1 is in line with the design of CCA measurement during Rel.16, and Alt 2 is aligned with the ETSI regulation. We believe that Alt 2 can be supported for simplicity and Alt 1 can be left to implementation, since UE can always measure the channel longer if it wants.

Proposal 7: One measurement is required for 8us deferral period.

In RAN1#104b-e meeting the sensing structure had been discussed and the following had been agreed [4]:

Working assumption: For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

In 802.11 ad specification, 5us slot is definition of aSlotTime, and the aSlotTime is composed of CCA measurement, processing delay and RXTX turnaround time and propagation time, of which the CCA measurement is equal to 3us. In order to coexist with 802.11 ad/ay in the 60GHz band, it is reasonable to reuse the design of 3us CCA measurement duration.

Proposal 8: The duration of the measurement should be 3us for 5us observation slot.

COT sharing and Cat 2 LBT

In RAN1#105-e meeting the COT sharing related issues had been discussed and the following had been agreed [5]:

Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

Regarding COT sharing, so far the ETSI EN 302 567 does not mandate any maximum gap for COT sharing [3]. In this sense, alt 1 is acceptable. But ETSI EN 302 567 also requires responding device to transmit immediately after skipping CCA check. However, it is not defined what “immediately” means. If it means that the gap should cover the minimum sum of the processing delay and preparing timing at the gNB or UE side, then the maximum gap is needed, otherwise, the maximum gap is not needed. Based on current situation, unless there is compelling reason to consider the maximum gap, we support Alt 1.

Proposal 9: Regarding COT sharing, no maximum gap is needed.

In RAN1#104-e meeting the COT sharing related issues had been discussed and the following had been agreed [2]:

Agreement: For Cat 2 LBT, down-select from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not.

Regarding Cat 2 LBT, several use cases were recommended in RAN1#104-e meeting. As mentioned in above, ETSI HS EN 302 567 defines a COT including all the gaps and transmissions from both the initiating and responding devices. When the transmission is in the COT, no CCA check is required before transmission. However, for directional LBT, Cat-2 LBT can be used to measure whether the channel is idle before beam switching to mitigate interference from other RATs.

Proposal 10: Cat 2 LBT should be supported for 60GHz unlicensed band operation.

Proposal 11: Cat 2 LBT may be used in case of directional LBT.

Multi-Beam COT

In RAN1#104-e meeting the multi-beam COT related issues had been discussed and the following had been agreed [2]:

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

In RAN1#104b-e meeting more clarifications on Alt 2 and Alt 3 had been discussed and the following had been agreed [4]:

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

For a COT with MU-MIMO (SDM) transmission, both alternatives should be supported, since they can be used for different usage. As for “independent per-beam LBT”, all per-beam LBTs should be performed simultaneously at the beginning of the COT.  In our opinions, Alt A-1 will cause a large latency, which will invalidate the earlier eCCA results. Alt A-2 will cause the transmitter to transmit in one beam direction while performing eCCA in other beam direction, which may cause interference between the transmission beam and LBT beam.  Regarding Alt A-3, it is not aligned with the regulations.

Proposal 12: For a COT with MU-MIMO (SDM) transmission, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported.

Proposal 13: For a COT with MU-MIMO (SDM) transmission, independent per-beam LBT sensing at the start of COT is performed for beams used in the COT should be supported, and the per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams.

In case of a COT with TDM of beams with beam switching, both Alt 1 and Alt 2 should be supported, since they can be used for different usage. As for “independent per-beam LBT”, all per-beam LBTs should be performed simultaneously at the beginning of the COT if the transmitter have the capability to perform per-beam LBT simultaneously. When the transmitter do not have the capability to perform per-beam LBT simultaneously, Alt A-1 can be supported as a baseline, because other alternatives are problematic as we mentioned above.

Proposal 14: Within a COT with TDM of beams with beam switching, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported.

Proposal 15: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT:

- If the transmitter has the capability to simultaneously sense in different beams, the per-beam LBT for different beams is performed simultaneously in parallel

- If the transmitter does not have the capability to simultaneously sense in different beams, Alt A-1 should be supported.

Conclusion

In this contribution, we have discussed LBT bandwidth, directional LBT, receiver assisted LBT, energy detection threshold enhancement mechanism, COT sharing and Cat 2 LBT. Based on the discussion in section 2, we provide the following proposals.

Proposal 1: Regarding LBT bandwidth, at least Alt SC.1 and Alt CA.1 should be supported.

- For single carrier transmission, at least gNB/UE should perform LBT over the channel bandwidth (or BWP bandwidth)

- For multi-carrier transmission, at least gNB/UE should perform multiple LBT, one for each channel bandwidth separately

Proposal 2: The directional LBT should be supported in 60GHz unlicensed band.

Proposal 3: The relationship between all the LBT beams and the transmission beam should be defined and at least LBT beam “covers” the transmission beam.

Proposal 4: The beam correspondence framework or QCL/TCI framework can be extended to define “cover”.

Proposal 5: Regarding receiver assisted LBT, at least the method of Legacy RSSI measurement and reporting with possible enhancements (Alt 1) and the method of AP-CSI report with possible enhancements (Alt 2) should be supported for further study.

Proposal 6: The formula of ED threshold should consider the LBT bandwidth and beamforming gain.

Proposal 7: Two energy measurements are required for 8us deferral period.

Proposal 8: The duration of the measurement should be 3us for 5us observation slot.

Proposal 9: Regarding COT sharing, NO maximum gap is needed.

Proposal 10: Cat 2 LBT should be supported for 60GHz unlicensed band operation.

Proposal 11: Cat 2 LBT may be used in case of directional LBT.

Proposal 12: For a COT with MU-MIMO (SDM) transmission, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported.

Proposal 13: For a COT with MU-MIMO (SDM) transmission, independent per-beam LBT sensing at the start of COT is performed for beams used in the COT should be supported, and the per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams.

Proposal 14: Within a COT with TDM of beams with beam switching, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported.

Proposal 15: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT:

- If the transmitter has the capability to simultaneously sense in different beams, the per-beam LBT for different beams is performed simultaneously in parallel

- If the transmitter does not have the capability to simultaneously sense in different beams, Alt A-1 should be supported.

Reference

- RP-202925 Revised WID: Extending current NR operation to 71GHz, 2020-12

- RAN1#104-e Chairman notes, 2021-02

- ETSI BRAN EN 302 567 v.2.1.20, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum”, June, 2020.

- RAN1#104b-e Chairman notes, 2021-04

- RAN1#105-e Chairman notes, 2021-05