3GPP TSG RAN WG1 #106-e			R1-2106699

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.2

Source: 	Spreadtrum Communications

Title: 	Discussion on enhancements for unlicensed band URLLCIIoT

Document for:	Discussion and decision

Introduction

In WID of Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) [1]，the second objective is about uplink enhancements for URLLC in unlicensed controlled environments, including UE-initiated COT for FBE and UL configured-grant enhancements. The following agreement was achieved in RAN1 105-e [2].

Agreement:

- Option 1 is taken in the following agreement:

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

  - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT..

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

This contribution provides further thoughts on enhancements for unlicensed band URLLC/IIoT.

UE-initiated COT for FBE

Semi-static control of a UE-initiated COT

According to semi-static control of a UE-initiated COT, we had three proposals during RAN1 105-e meeting, which are listed below.

Proposal 6-1:

In semi-static channel access mode when a UE can operate as an initiating device, support disabling UE-initiated COT by RRC for UL transmissions aligned with a set of UE FFP boundaries.

- FFS on details, e.g. configuration of set, etc.

- Note: UE-initiated COT is considered enabled at each UE-FFP boundary once the FFP periodicity and offset are configured.

Proposal 6-2:

In semi-static channel access mode when a UE can operate as an initiating device, support disabling UE-initiated COT by RRC for a UL transmission aligned with a UE FFP boundary.

- FFS on disabling for a configured UL transmission and/or per configuration of the UL transmission

- FFS on disabling for a P-CSI report configuration, or SRS, or CG

- FFS on other details, e.g. configuration, etc.

- Note: UE-initiated COT is considered enabled at each UE-FFP boundary once the FFP periodicity and offset are configured.

Proposal 6-3:

In semi-static channel access mode when a UE can operate as an initiating device, the UE can be RRC configured with a parameter to limit its COT to an indicated duration such that it ends before the idle period/CCA of a subsequent frame in the same channel

- FFF on details, e.g. configurations

From our understanding, the main benefit of proposal 6-1/6-2 is disable some UE-initiated COTs based on RRC parameter. Considering configured UL transmission aligned with a COT, if gNB wants to disable this UE-initiated COT, gNB can send indicator that tell UE to share gNB’s COT, which is already agreed to support and more flexible than this semi-static signaling. Considering dynamic UL transmission aligned with a COT, if gNB prefers to use gNB-initiated COT rather than UE-initiated COT, this indication can be included in DCI. Above all, either configured and dynamic UL transmission can be channeled to initiate a COT, no additional semi-static control of a UE-initiated COT is needed.

Regarding proposal 6-3, the intension is to avoid the collisions between two UEs. It can be solved by proper UE-initiated configurations, including period and offset. Shorter period can achieve similar effect as proposal 6-3. So we don’t see the necessity of the proposal.

- Not support semi-static control of UE-initiated COT.

Dynamic control of a UE-initiated COT

According to dynamic control of a UE-initiated COT, there were two proposals during RAN1 105-e meeting, which are listed below.

Proposal 7-4

In semi-static channel access mode when a UE can operate as an initiating device, for a UL transmission, the UE can be dynamically indicated to change its assumption on the associated COT initiator for the UL transmission.

- FFF on details, e.g. signaling details

Proposal 7-5

In semi-static channel access mode when a UE can operate as an initiating device, for a DL reception, the UE can be dynamically indicated to change its assumption on the associated COT initiator for the DL reception.

- FFF on details, e.g. signaling details

Considering the assumption of the ownership of the COT, both Alt-a for configured and dynamic UL transmission were agreed, UE-initiated COT or sharing a gNB-initiated COT. There are already dynamic indications regarding the ownership, like DCI and DL burst. So according to whether or not dynamic change should be considered additionally, we give our considerations below.

According to the main benefit of proposal 7-4/5, which were provided during the last meeting, includes:

- Case 1: Disallow UL transmission during a gNB idle period

- Case 2: Disable UE-initiated COT at the frame boundary of a UE-FFP

For Case 1, when UL transmission overlapping with a gNB idle period, this UL transmission cannot share a gNB’s COT, a better way for its Tx is using a UE-initiated COT. Thus, a configured UL transmission can be aligned with a COT, and without gNB-initiated COT.  The other case is dynamic UL transmission, which is totally controlled under gNB. So no dynamic change the assumption of owner of COT is needed for this case.

For Case 2, since UE-initiated COT at the frame boundary of a UE-FFP is static. If this UE-initiated COT want to be disabled forever, RRC configured method is more suitable comparing with dynamic indication. Such as no UL transmission aligned with its boundary.

In summary, from the sight of case 1 and case 2, we don't see the necessity of dynamic control of UE-initiated COT.

- Not support dynamic control of UE-initiated COT.

Cross-FFP scheduling

According to cross-FFP scheduling, there were two agreements during RAN1 104b-e meeting, which are listed below. There are still two FFS points.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

For DL/UL transmission, whether or not it can meet the corresponding channel access requirements, depends on channel access. The goal is whether or not introduce specific rules or signalling for UE, especially there are cross-FFP scheduling, in case initiator fails channel access.

From our perspective, assumptions as same as Rel-16 can be used here, consist of DL COT sharing for UL transmission, and DL reception in DL COT. Regarding DL COT sharing for UL transmission, when UE does not detect a DL indicator or burst, UE would drop its UL transmission unless satisfy the condition of its initiating COT. Considering DL reception in DL COT, when UE does not detect a DL burst, UE can just assume there is no DL transmission or error reception. The subsequent operation is same as Rel-16.

- According to cross-FFP scheduling, legacy assumptions can be reused to decide whether or not meet the channel access requirement.

- For UL Tx, when UE does not detect a DL indicator or burst, UE would drop its UL transmission unless satisfy the condition of its initiating COT.

- For DL reception, when UE does not detect a DL burst, UE can just assume there is no DL transmission or error reception.

Conditions on UE-to-gNB COT

Another open issue is UE-to-gNB COT, there was the proposal 9-1 discussed during RAN1 105-e.

Proposal 9-1:

Selection one of the following options:

- Option 1: In semi-static channel access mode, a DL transmission based on a UE initiated COT sharing for an FFP, can be transmitted to any other UE in the cell than the COT initiating UE. It is gNB’s responsibility to ensure the corresponding channel access requirements are fulfilled.

- Option 2: In semi-static channel access mode, a DL transmission based on a UE initiated COT sharing for an FFP, can be transmitted to any other UE in the cell than the COT initiating UE if the DL transmission at least includes data or control intended for the UE that initiated that FFP .

We prefer Option 2. Because UE COT sharing basically aims to serve this UE, like faster HARQ-ACK feedback or scheduling. So Option 1 is more aggressive. There is no reason to share to other UE even if PDCCH only. If another UE need a transmission or a reception at once, gNB can schedule them in gNB COT instead of within one UE COT. The argument is reduce the scheduling delay, like gNB COT boundary still need time to wait. However, the threshold problem can be guaranteed. More problems are introduced by Option 1.

- For UE-to-gNB COT, Option 2 is preferred.

- Option 2: In semi-static channel access mode, a DL transmission based on a UE initiated COT sharing for an FFP, can be transmitted to any other UE in the cell than the COT initiating UE if the DL transmission at least includes data or control intended for the UE that initiated that FFP .

UE-initiated COT in Inactive/Idle mode

Considering the objective of UE-initiated COT for FBE is about uplink enhancements for URLLC in unlicensed controlled environments, the use case should be limited in connected mode.  URLLC traffics cannot be supported by the UEs in IDLE/INACTIVE mode. From this sight, UE in IDLE/INACTIVE mode does not need to use semi-static channel access mode, since it can transmit based on the gNB-initiated COT configuration, DL burst sensing for PRACH and msg3. Thus, UE in IDLE/INACTIVE mode does not use semi-static channel access. We should focused on UE in Connected mode.

- UE in IDLE/INACTIVE mode does not use semi-static channel access mode.

Harmonization of PUSCH repetition

It was agreed that PUSCH repetition type B can be supported for unlicensed band operation when using NR IIoT Rel-16 based CG in RAN1 104e. However, there are two issues of PUSCH repetition type B can should be considered.

- Orphan symbol

- The impact of gNB idle periods

In Rel-16, orphan symbol among the PUSCH repetitions cannot be transmitted. The generation for Orphan symbol can be slot boundary or invalid UL symbols caused the PUSCH segmentation. Orphan symbol is treated as empty symbol duration UL transmission. However, empty symbol in unlicensed band is not allowed. Therefore the improvement of orphan symbol in PUSCH repetition type B can be considered. From our understanding, some padding can be used, like copy of the former end symbol, or later starting symbol, etc.

If PUSCH is within a COT initiated by UE, whether there is a possibility that the PUSCH can be overlapped with idle period of gNB. We think there is a high possibility because UE-initiated COT can be separately configured from gNB-initiated COT. So it is hard to guarantee that all configured grant PUSCH can avoid the overlapping with idle period of gNB. It was not achieved whether a UE is allowed to transmit or not during the idle period of any FFP associated with the serving gNB. If UE is not allowed to transmit, there is some impacts to PUSCH repetition type B if the resources are overlapping with idle periods of gNB. Furthermore, if PUSCH is within a COT initiated by gNB, whether there is a possibility that the PUSCH can be overlapped with idle period of the COT. We don’t think this is a reasonable scenario because gNB can configure its time domain resources properly.

We are fine to discuss this issue later after the decision of transmission during idle periods.

- Enhancements of PUSCH repetition Type B in unlicensed band should consider the impact of orphan symbol and gNB’s idle periods if additional constrain is supported.

Conclusion

In this contribution, we made the following observations and proposals.

- Not support semi-static control of UE-initiated COT.

- Not support dynamic control of UE-initiated COT.

- According to cross-FFP scheduling, legacy assumptions can be reused to decide whether or not meet the channel access requirement.

- For UL Tx, when UE does not detect a DL indicator or burst, UE would drop its UL transmission unless satisfy the condition of its initiating COT.

- For DL reception, when UE does not detect a DL burst, UE can just assume there is no DL transmission or error reception

- For UE-to-gNB COT, Option 2 is preferred.

- Option 2: In semi-static channel access mode, a DL transmission based on a UE initiated COT sharing for an FFP, can be transmitted to any other UE in the cell than the COT initiating UE if the DL transmission at least includes data or control intended for the UE that initiated that FFP .

- UE in IDLE/INACTIVE mode does not use semi-static channel access mode.

- Enhancements of PUSCH repetition Type B in unlicensed band should consider the impact of orphan symbol and gNB’s idle periods if additional constrain is supported.

References

RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR

R1-2103960	Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2106048	Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)