3GPP TSG RAN WG1 #106-e		     R1-2106700

e-Meeting,  August 16th – 27th, 2021

Agenda item:	8.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on intra-UE multiplexing/prioritization

Document for:	Discussion and decision

Introduction

During RAN 88e meeting, a revised WID of Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR was approved. One objective is to further enhance the multiplexing and prioritization scheme in URLLC, the objective is shown below.

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

    - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH

This contribution provides some considerations on intra-UE multiplexing/prioritization for URLLC.

Discussion

- Overlapping between high priority HARQ-ACK and low priority HARQ-ACK

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options:

  - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

  - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

- For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1.

- FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

For the first bullet regarding the coding scheme for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), both options are feasible because current UCI encoder hardware has already supported. We slightly prefer Option 2 since it is a unified solution by using RM coding on PUCCH and has small spec. affect. Thus we have a following proposal:

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, if HP HARQ-ACK or LP HARQ-ACK is of 1-2 bit(s), reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

Another remaining issue is the maxcoderate determination for the LP HARQ-ACK. Since the LP HARQ-ACK is multiplexed on HP PUCCH resource, one simple solution is to determine the coderate of LP HARQ-ACK using the maxcoderate configured for the LP PUCCH-format index, where the index is associated with HP PUCCH resource that carrying the multiplexed HARQ-ACKs. The other alternative solution is using RRC signalling to configure another maxcoderate for each HP PUCCH-format.

For PRB determination and RE mapping issue for PF2/3/4, the minimum PRB number for HP HARQ-ACK and LP HARQ-ACK can be separately determined based on Rel-15 rule, and by using the max coderates configured for HP HARQ-ACK and LP HARQ-ACK, respectively. The final PRB number of the transmitted PUCCH resource is determined by concatenating the PRBs of HP HARQ-ACK and LP HARQ-ACK.

- For separate coding of the two HARQ-ACKs with HARQ-ACK bits being more than 2, the maxcoderate for HP and LP HARQ-ACK can be different.

- The minimum PRB number for HP HARQ-ACK and LP HARQ-ACK can be determined based on Rel-15 rule by using the maxcoderate configured for HP HARQ-ACK and LP HARQ-ACK separately.

- Enable/disable multiplexing of PUCCH

For a mechanism to enable/disable the multiplexing between PUCCHs, compared with the semi-static RRC configuration, dynamically indication via DCI may have DCI miss detection problems and is not feasible for semi-static PUCCHs such as SPS PDSCH HARQ-ACK transmission. So we support to use semi-static RRC configuration at least. In addition, for some urgent URLLC traffics, disabling multiplexing can avoid the negative affect from the LP HARQ-ACK, thus supporting a more flexible switch between enable and disable function may be needed. So RRC+DCI-based enabling/disabling method can be considered as an optional feature.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, support RRC configuration to enable/disable the multiplexing as a baseline. RRC+DCI-based enabling/disabling method can be considered as an optional feature.

- Overlapping between high priority SR and low priority HARQ-ACK.

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, a following proposal was given in the last meeting:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, down-select the following options:

- Opt.1b: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

- Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.3: No enhancement over Rel-16.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, to reuse current spec as much as possible, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. Thus we prefer Opt. 2c here.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Regarding a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, a following proposal was given in the last meeting:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, down-select the following options:

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.5: No enhancement over Rel-16.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In this case, to maintain the latency and performance of HP SR, the SR cannot be transmitted on LP HARQ-ACK resource. For positive SR, LP HARQ-ACK can be dropped or transmitted on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1,   for positive SR, LP HARQ-ACK can be dropped or transmitted on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

About a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, a following proposal was given in the last meeting:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, down-select the following options:

- Opt.2c:  If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: No enhancement over Rel-16.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Rel-15 rule can be reused, thus we support Opt.2c here.

- If a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Overlapping between HARQ-ACK and PUSCH

In last meeting, a following proposal regarding multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUSCH was given：

Proposal:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, further study the following options:

- Option 2: The CSI part 2 is dropped, LP HARQ-ACK is coded separately from HP HARQ-ACK and CSI part 1

  - Option 2a: LP HARQ-ACK has lower priority than LP CSI part 1, and LP HARQ-ACK may be dropped (similar to Rel-15 CSI-part2);

  - Option 2b: LP HARQ-ACK has higher priority than LP CSI part 1, and LP CSI part 1 may be dropped (similar to Rel-15 CSI-part1);

- Option 3: The CSI part 1 is dropped. CSI part 2 is similarly treated as CSI part 1 in Option 2.

- Option 4 No CSI is dropped,

  - LP HARQ-ACK and LP CSI part 1 are jointly encoded and reuse the coding method used for CSI part 1 in Rel-15. CSI part 2 reuses the coding method used for CSI part 2 in Rel-15.

    - Details need to be identified e.g. How to determine code rate for joint coding? Which beta offset should be used?

- FFS for LP CSI consisting of single part

- FFS for HP/LP PUSCH not conveying UL-SCH

In general, dropping CSI part 1 or CSI part 2 may deteriorate the spectrum efficiency of the eMBB traffics, especially for semi-static scheduled PDSCH transmission. On the contrary, Option 4 can maintain the LP CSI/HARQ-ACK transmission without increasing the total encoding chains, thus is preferred here. For joint coding of LP HARQ-ACK and LP CSI part 1, the beta offset of LP HARQ-ACK can be applied as a startpoint.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, Option 4 is supported, i.e., no CSI is dropped.

Conclusion

In this contribution, we made the following observations and proposals.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, if HP HARQ-ACK or LP HARQ-ACK is of 1-2 bit(s), reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

- For separate coding of the two HARQ-ACKs with HARQ-ACK bits being more than 2, the maxcoderate for HP and LP HARQ-ACK can be different.

- The minimum PRB number for HP HARQ-ACK and LP HARQ-ACK can be separately determined based on Rel-15 rule by using the maxcoderate configured for HP HARQ-ACK and LP HARQ-ACK separately.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH, support RRC configuration to enable/disable the multiplexing as a baseline. RRC+DCI-based enabling/disabling method can be considered as an optional feature.

- If a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1,   for positive SR, LP HARQ-ACK can be dropped or transmitted on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- If a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, if SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, Option 4 is supported, i.e., no CSI is dropped.

References

RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR.

Draft Report of 3GPP TSG RAN WG1 #105-e.

Draft Report of 3GPP TSG RAN WG1 #104b-e.