3GPP TSG RAN WG1 #106-e                                                      R1-2106704

E-Meeting,  August 16th – 27th, 2021

Agenda Item:	8.4.4

Source:	Spreadtrum Communications

Title:	Discussion on beam management and other aspects for NTN

Document for:	Discussion and decision

Introduction

In this paper, we discuss our views on enhancements on beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse and signalling of polarization mode.

Beam management

In RAN1#105e, the follow agreement on beam layout in NTN had been achieved [1].

Agreement: Same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported by the specifications for NR-NTN. - FFS: Whether any specification changes are needed specifically to support this functionality

In this section, we share our views on beam management in NR NTN.

NR-NTN beam management issues

In RAN1#104_e, some issues on beam management in NTN have been identified [2].

Conclusion: Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management: - Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching. - Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL. - Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling. - Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP. - Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it. - Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility. - Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

In NR, there is no explicit association between SSB and BWP in current specification, i.e., BWP is not directly associated with a beam. Thus, beam switching will not trigger BWP switching. In NTN FRF>1 case, beam switching may result in a BWP switching. In case of different BWPs associated with different beam, if beam management based on BWP switching is supported, the association between SSBs and BWPs would be determined naturally. The current specifications could be re-used.

Proposal 1: Additional association to map SSB index and BWP index is not needed.

NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL. In our view, BWP switching of UL and DL simultaneously in NTN FRF>1 FDD scenario should be supported.

Proposal 2: BWP switching of UL and DL simultaneously in NTN FRF>1 FDD scenario should be supported.

In NTN, frequency reuse is used to mitigate inter-beam interference. Hence, there is almost inevitable that beam switching would lead to switch another BWP. Current NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, BWP switching triggering without data scheduling is very frequent. In our view, BWP switching without data scheduling should be supported in NTN FRF>1 FDD scenario.

Proposal 3: BWP switching without data scheduling should be supported in NTN FRF>1 FDD scenario.

In RAN1#105e, the follow proposal on beam management in NR-NTN had been achieved [3].

FL proposal DP1-1 For beam management in NR-NTN, support at least one of the following enhancements - Alt-1: UE BWP/beam switching is triggered by gNB relying on prediction on gNB side. - FFS: whether BWP/beam switching can be done for a group of UE. - Alt-2: UE BWP/beam switching is performed by UE automonously relying on assistance information. - FSS: details on assistance information

For Alt-1, gNB can determine the beam for the UE based on the beam layout, satellite ephemeris and UE location information. In this case, UE need to report the location information to gNB. For Alt-2, beam related information of serving cell including the beam size and beam direction should be broadcast in the system information for UE dominant beam/BWP switching. UE does not need to report the location information to gNB. In this way, the signaling overhead of beam management and UE measurement efforts can be significantly reduced. In our view, both Alt-1 and Alt-2 should be supported for predication based beam switch.

Proposal 4: Both Alt-1 and Alt-2 should be supported for predication based beam switch.

In the existing specification, beam and BWP switching is based on UE specific signalling. BWP/beam switching time can be different for different UEs. The same signalling framework of beam/BWP switching as NR can be reused.

Proposal 5: The same signalling framework of beam/BWP switching as NR can be reused in NTN.

Beam measurement and reporting

In RAN1#105e, the follow proposal on beam measurement and reporting in NR-NTN had been achieved [3].

FL proposal DP3-1: For the deployment scenario with multiple beam per cell and frequency reuse >1, NR-NTN should support the following cases for beam measurement performed by a UE: Alt-1: support beam measurement on multiple RS associated with different beams within a same active BWP. Alt-2: support beam measurement on multiple RS associated with different beams within across BWPs.

In RAN1#105e, it has agreed that same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported by the specifications for NR-NTN. For same beam layout in BWP#0 and BWP#x, beam measurement on multiple RS associated with different beams within a same active BWP is prefer. For hierarchical beam for BWP#0, beam measurement on multiple RS associated with different beams within across BWPs should be supported.

Proposal 6: Beam measurement on multiple RS associated with different beams within a same active BWP and within across BWPs should be supported.

Signalling of Polarization

In RAN1#105e, the following agreements on polarization signaling have been achieved [1].

Agreement: For explicit indication of polarization information for DL by the network, support indication in SIB - FFS: Signaling details for indication in SIB Agreement: - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. - FFS: Signaling details for indication in SIB

In this section, we share our views on signalling of polarization.

Reporting UE polarization capability

In RAN1#104_e, the following conclusion on reporting UE polarization capability has been achieved [2].

Conclusion: Discuss the necessity of reporting UE polarization capability considering at least following aspects, - Deployment scenarios. - UE implementation aspects with respect to polarization. - Satellite implementation aspects for switching between polarization states. - Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

In NTN networks, neighbouring cells may use different polarization modes (RHCP and LHCP) to mitigate inter-cell interference. Furthermore, there may be UEs with different antenna types. Some UEs may be equipped with linearly polarized antennas, while some other UEs may be equipped with circularly polarized antennas. If multiple polarization types are used, UE can adjust its polarization type based on network polarization indication. Regarding the target usage of polarization, our view is that inter-beam interference mitigation should be considered as first priority. The necessity of using polarization for multiplexing is unclear.

If polarization is used for inter beam interference mitigation, the polarization state for each beam in one cell will not change dynamically. Based on this, there seem meaningless to report the polarization capability to the gNB since the satellite cannot use the reported information to adjust the polarization state for one UE.

Proposal 7: UE reporting polarization capability is not needed.

Polarization indication

In RAN1#105_e, the following agreements on polarization indication has been achieved [1].

Agreement: For explicit indication of polarization information for DL by the network, support indication in SIB - FFS: Signaling details for indication in SIB Agreement: - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. - FFS: Signaling details for indication in SIB

Since polarization is used for inter-beam interference mitigation by satellite, it is beneficial to signal the polarization of each beam in one cell, including gNB Tx polarization and Rx polarization of each beam. Therefore, beam specific indication of polarization information for DL by the network should be supported.

Proposal 8: Beam specific indication of polarization information for DL by the network should be supported.

Conclusion

Based on the analyses and discussions, we have the following proposals:

Proposal 1: Additional association to map SSB index and BWP index is not needed.

Proposal 2: BWP switching of UL and DL simultaneously in NTN FRF>1 FDD scenario should be supported.

Proposal 3: BWP switching without data scheduling should be supported in NTN FRF>1 FDD scenario.

Proposal 4: Both Alt-1 and Alt-2 should be supported for predication based beam switch.

Proposal 5: The same signalling framework of beam/BWP switching as NR can be reused in NTN.

Proposal 6: Beam measurement on multiple RS associated with different beams within a same active BWP and within across BWPs should be supported.

Proposal 7: UE reporting polarization capability is not needed.

Proposal 8: Beam specific indication of polarization information for DL by the network should be supported.

Reference

- 3GPP RAN1#105_e Chairman’s Notes, May 10th – 27th, 2021.

- 3GPP RAN1#104_e Chairman’s Notes, 25th January - 5th February 2021.

- R1-2106336, “Final Summary of 8.4.4 Other Aspects of NR-NTN”, Moderator (OPPO).