3GPP TSG RAN WG1#106-e		       R1-2106712

e-Meeting, August 16th – 27th, 2021

Agenda Item:  8.8.2

Source:	Spreadtrum Communications

Title:	Discussion on PUCCH enhancements

Document for:	Discussion and decision

Introduction

In this contribution, we discuss coverage enhancement concerning PUCCH.

Dynamic PUCCH repetition factor indication

In RAN1#105e meeting, the following agreements were reached [1].

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details

The benefit of adding repetition factor per PUCCH resource is overhead saving of DCI, though UE may be configured more PUCCH resources. From our view, the advantage outweigh its disadvantage. Therefore, we can confirm this working assumption.

Proposal 1：Confirm the following working assumption:

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

FFS: RRC signaling enhancement details

DMRS bundling across PUCCH repetitions

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16. - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible Agreements: Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration. - FFS: the configuration is per UE or per PUCCH resource. - FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling - FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size Conclusion: In Rel-17, deprioritize the study of DMRS pattern/location/granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of DMRS pattern/location/granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3. Conclusion: For the study of enhancing inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, at least the following aspects can be considered: - Performance tradeoff between maximizing # consecutive UL slots in one frequency hop (to achieve more DMRS bundling gain) and maximizing # hops (to achieve more diversity gain) - Note: the maximum # frequency hopping positions is still 2 as in Rel-15/16., which is signaled by startingPRB and secondHopPRB - Interaction between hopping boundary determination and TDD configuration Conclusion: For the simulations to study the enhancement of inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, simulation assumptions in 38.830 are reused as a starting point. Note: Additional simulation scenarios/assumptions are not precluded.

Inter-slot frequency hopping pattern considering TDD configurations should be studied. For example, to facilitate joint channel estimation, the repetitions of PUCCH in consecutive UL slots can be mapped to the same hop as many as possible.

Proposal 2. Inter-slot frequency hopping pattern considering TDD configurations should be studied.

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements. - Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

As agreed for joint channel estimation of PUSCH, a RRC parameter can be used to enable/disable the functionality of joint channel estimation. We think coverage enhancement for PUSCH/PUCCH can be related tightly and they can share the same RRC parameter for enabling/disable.

Regarding the time domain window for PUCCH, we prefer using dynamic signaling. Similar to PUSCH’s discussion, one or more time domain window is needed.

Proposal 3. Same enabling/disabling parameter can be used for PUSCH/PUCCH.

Proposal 4.Use dynamic signaling for indicating the time domain window for PUCCH.

Conclusions

In this contribution, we have the following proposals:

Proposal 1：Confirm the following working assumption:

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

FFS: RRC signaling enhancement details

Proposal 2. Inter-slot frequency hopping pattern considering TDD configurations should be studied.

Proposal 3. Same enabling/disabling parameter can be used for PUSCH/PUCCH.

Proposal 4.Use dynamic signaling for indicating the time domain window for PUCCH.

References

RAN1#105e Chairman’s notes, May 2021