3GPP TSG RAN WG1 Meeting #106-e	R1-2106738

e-Meeting, August 16th – 27th, 2021

Source: 	ZTE

Title: 	Discussion on propagation delay compensation enhancements

Agenda item:	8.3.4

Document for:	Discussion and Decision

Introduction

In RAN1#104bis-e, the following agreements and working assumption were reached [1]. In this contribution, further discussion on the propagation delay compensation are presented.

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation Agreements: Take the following equation for evaluation of the DL propagation delay estimation error for TA based propagation delay compensation: - Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245. - FFS whether errorBS,DL,TX in the above equation should be included or not. Agreements: - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837. Working assumption: Agreement: Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC. - The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure. - errorUE,DL,RX is based on other signals (e.g. CSI-RS) instead of SSB. - errorBS, UL,RX iss based on other uplink signals instead of contention based PRACH, e.g. SRS. - Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method. Agreement: Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - FFS whether PRS can be used for UE Rx – Tx time difference estimation or not - FFS which DL reference signal(s) to be used if/when PRS is not used Conclusion: - Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

Discussion

Discussion on TA-based solution

According to the agreements reached in the previous meeting, the error components for the propagation delay especially for the TA-based solution include:

- BS transmitting time error(): 65ns

- UE downlink detection error(): 100ns

- UE transmitting time error(): Te including UE downlink detection error

- BS uplink detection error(): 100ns

- TA indication error(): 8*Ts for 15kHz SCS, 4*Ts for 30kHz SCS

For the UE downlink detection error, there was a discussion on whether it is included in the UE transmission error (Te) defined by RAN4. According to the reply LS from RAN4, RAN4 confirms that downlink frame timing detection error has already been included in the UE transmission error (Te) defined in section 7.1.2 in TS38.133 [2]. In this case, it is proposed to use  for the propagation delay estimation error evaluation.

Proposal 1: RAN1 to use  for the propagation delay estimation error evaluation.

In last meeting, the following steps were used for determining the overall time synchronization error for TA-based solution [3].

Step 1: gNB sends the reference time clock TBS (i.e. referenceTimeInfo-r16) to UE, and the actual time clock at the UE side should be TUE=TBS+errorBS, DL, TX+PDL+errorUE, DL, RX - errorBS, DL, TX: BS transmit timing error for transmitting the RRC signaling containing the reference time clock - errorUE, DL, RX: Downlink frame timing detection error for receiving the RRC signaling contacting the reference time clock Step 2: When the UE receives referenceTimeInfo-r16, UE obtains TBS indicated by referenceTimeInfo-r16. After UE does the propagation delay compensation, the estimated time clock at the UE side is TUE=TBS+PDL+errorPDL - errorPDL: DL propagation delay estimation error, e.g. errorPDL=TA2-PDL for TA-based PDC. Note that details for errorPDL is defined in step 4 below. Step 3: The overall time synchronization error (i.e. the difference between the actual time clock in step 1 and the estimated time clock in step 2) is errortotal, TAbased≤errorBS, DL, TX +errorUE, DL, RX +errorPDL

For the DL propagation delay estimation error, whether  should be included in the equation for TA based propagation delay compensation is FFS. Some companies believed that it should not be included since it reflects the error between the air interface time and the timestamp provided by the RRC signaling and the error should be accounted for in the RRC related error sources (i.e. step 1). In our understanding, the timestamp is from the time clock of the network. The UE transmits the UL signal based on the downlink timing of the DL signal transmitted by the network. When the network receives the UL signal from the UE, the TA is determined based on the time clock of the network but not the air interface time of the DL signal. Therefore, the errorBS,DL,TX should be included in the DL propagation delay estimation error.

Proposal 2:   should be included in the DL propagation delay estimation error

In last meeting, there were two alternatives for evaluation of the overall time synchronization error. Considering they are based on different implementations, we think both of them can be further studied. However, there is no need to meet the requirement for both of the two alternatives.

The difference between them is whether the consistent BS transmitting time error and UE downlink detection error are used in the step 1 and step 2. If the inconsistent BS transmitting time error and UE downlink detection error are used in the step 1 and step 2, then the Alt. 1 is achieved. For example, The UE receives the reference time and corresponding system frame (i.e., step 1) at a time and the UE gets the TA (i.e., step 2) at anther time. It means the step 1 and step 2 are performed at the different time. It is difficult to say that the BS transmitting time error or UE downlink detection error in the two steps is consistent in the two steps.

However, if the step 1 and step 2 are performed at the same time, the consistent value can be obtained in the step 1 and step 2. In this case, Alt. 2 is achieved. Take the following procedures as an example.

- The network sends the reference time providing the transmitting time of the ending boundary of the frame N.

- Then the UE receives the downlink signal in the frame N and performs the downlink synchronization to obtain the downlink frame timing based on the received signal. In this procedure, the BS transmitting time error and UE downlink detection error in step 1 are involved.

- The UE further transmits the UL signal based on the obtained downlink frame timing. It means the BS transmitting time error and UE downlink detection error in step 1 affect the UL signal transmission.

- Then the network sends a new TA after receiving the UL signal so that the UE obtains the new TA value.

- The UE use the new TA value to determine the time when it receives the frame N.

- We can see the both of the BS transmitting time error and UE downlink detection error are consistent in the step 1 and step 2 since the new TA is obtained based on the procedure in step 1. Therefore, we think Alt. 2 can also be achieved and slightly prefer to use it for evaluation since it can achieve a higher accuracy.

Proposal 3: Both Alt. 1 and Alt. 2 can be used for further study and Alt. 2 is preferred.

Discussion on RTT-based solution

In RAN1#104bis-e, almost all the companies agreed to use the same steps for evaluating the overall time synchronization error for RTT-based solution. Therefore, the overall time synchronization error is errortotal, TAbased≤errorBS, DL, TX +errorUE, DL, RX +errorPDL, where errorPDL is the DL propagation delay estimation error for RTT-based solution.

Similar as the TA-based solution, the DL propagation delay estimation error for RTT-based solution is , where is the Rx-Tx time difference indication error. The details are shown in the Figure 1. It should be noted the network and the UE should measure the same DL signal and the uplink signal to determine the Rx-Tx time difference.

Figure 1 RTT-based Propagation delay estimation

Observation 1: DL propagation delay estimation error for RTT-based solution is

For RTT-based solution, it has the same question with TA-based solution that whether the consistent BS transmitting time error and UE downlink detection error are used in the step 1 and step 2. Therefore, we also have two alternatives for the overall time synchronization error for RTT-based solutions.

- Alt. 1:

- Alt. 2:

Proposal 4: For RTT-based solution, the two alternatives are used for evaluation similar as that for TA-based solution.

- Alt. 1:

- Alt. 2:

Analysis results

According to the reply LS from RAN2, the budget of single Uu interface is (145ns, 275ns) for the control to control scenario [4]. RAN2 further point out that the error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is already included in the network part budget, and RAN1 should not include this error in Uu interface again. Based on the Alt. 1 and Alt. 2 above, the evaluation results for TA-based solution and RTT-based solution are shown in Table 1 and Table 2, respectively. It should be noted the Rx-Tx time difference indication error is Tc for RTT-based solution with the assumption of k=0[5].

Table 1 Evaluation results for the accuracy of the Uu interface based on TA-based solution

|  | Alt. 1 |  | Alt. 2 |  |
| --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 15kHz | 30kHz |
| BS transmission timing error | 65ns | 65ns | 65ns | 65ns |
| UE detection error | 100ns | 100ns | 100ns | 100ns |
| Initial transmission error | 391ns(12*64*Tc) | 260ns(8*64*Tc) | 391ns(12*64*Tc) | 260ns(8*64*Tc) |
| TA indication error | 260ns(8*64*Tc) | 130ns(4*64*Tc) | 260ns(8*64*Tc) | 130ns(4*64*Tc) |
| BS detection error | 100ns | 100ns | 100ns | 100ns |
| Total error | 573 | 442.5 | 408 | 277.5 |

Table 2 Evaluation results for the accuracy of the Uu interface based on RTT-based solution

|  | Alt. 1 |  | Alt. 2 |  |
| --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 15kHz | 30kHz |
| BS transmission timing error | 65ns | 65ns | 65ns | 65ns |
| UE detection error | 100ns | 100ns | 100ns | 100ns |
| Initial transmission error | 391ns(12*64*Tc) | 260ns(8*64*Tc) | 391ns(12*64*Tc) | 260ns(8*64*Tc) |
| Rx-Tx indication error | 0.5ns | 0.5ns | 0.5ns | 0.5ns |
| BS detection error | 100ns | 100ns | 100ns | 100ns |
| Total error | 443.25 | 377.75 | 278.25 | 212.75 |

It can be seen that, both of the TA-based solution and RTT-based solution cannot meet the requirement if Alt.1 is used. If Alt.2 is used, the TA-based solution still cannot meet the requirement and the evaluation results is very close to the target at 30kHz SCS. For RTT-based solution, the requirement can be met at 30kHz SCS and the evaluation results is very close to the target at 15kHz SCS.

Observation 2: Both of the TA-based solution and RTT-based solution cannot meet the requirement if Alt.1 is used

Observation 3: RTT-based solution can achieve the higher accuracy due to the smaller reporting resolution.

For TA-based solution, both of the enhanced TA indication granularity and initial transmission error Te can be considered for the enhancement. Table 2 is an example of the achieved the accuracy of Uu interface when the current values are reduced by half for the initial transmitting timing error (Te) and the TA indication granularity. It can be seen that the requirement can be satisfied after enhancement if Alt.2 is adopted.

For RTT-based, the granularity of the Rx-Tx time difference cannot be improved any more. So the other error components should be reduced anyway. For example, if the initial transmission error (Te) is reduced a bit, the requirement can be met. However, some other issues should be resolved as discussed in RAN1#104bis-e. For example, whether the PRS should be introduced or not.

Therefore, we propose to reduce the TA indication granularity or initial transmission error for the propagation delay compensation enhancements. Since the the value for TA indication granularity or the initial transmission error are defined by RAN4, an LS is needed to ask RAN4 if it is feasible to define the new requirements for the Te and TA indication granularity for propagation delay compensation enhancements and to provide the new requirement if feasible.

Table 3 An example of the achieved accuracy of Uu interface

|  | 15kHz | 30kHz |
| --- | --- | --- |
| Enhanced initial transmission error(Te) | 195ns | - |
| Enhanced TA indication error | 130ns | 260ns |
| Achieved accuracy of Uu interface | 245ns | 275.5ns |

Proposal 5: TA-based solution should be adopted for the propagation delay compensation enhancements, e.g., reduce the TA indication granularity or initial transmission error.

Proposal 6: Send an LS to RAN4 to ask if it is feasible to define the new requirements for the Te and TA indication granularity for propagation delay compensation enhancements and to provide the new requirement if feasible.

Conclusion

According to the discussion above, we have the following observations and proposals.

Observation 1: DL propagation delay estimation error for RTT-based solution is

Observation 2: Both of the TA-based solution and RTT-based solution cannot meet the requirement if Alt.1 is used

Observation 3: RTT-based solution can achieve the higher accuracy due to the smaller reporting resolution.

Proposal 1: RAN1 to use  for the propagation delay estimation error evaluation.

Proposal 2:   should be included in the DL propagation delay estimation error

Proposal 3: Both Alt. 1 and Alt. 2 can be used for further study and Alt. 2 is preferred.

Proposal 4: For RTT-based solution, the two alternatives are used for evaluation similar as that for TA-based solution.

- Alt. 1:

- Alt. 2:

Proposal 5: TA-based solution should be adopted for the propagation delay compensation enhancements, e.g., reduce the TA indication granularity or initial transmission error.

Proposal 6: Send an LS to RAN4 to ask if it is feasible to define the new requirements for the Te and TA indication granularity for propagation delay compensation enhancements and to provide the new requirement if feasible.

Reference

3GPP RAN1#104bis-e, Chairman notes

3GPP RAN1#105-e, R1-2104171, Reply LS on UE transmit timing error

3GPP RAN1#104bis-e, R1-2104136, Final feature lead summary on propagation delay compensation enhancements

3GPP RAN1#104-e, R1-2100024, Reply LS on propagation delay compensation enhancements

3GPP TS38.133-g50, Requirements for support of radio resource management