3GPP TSG RAN WG1 #106-e                                                  R1-2106740

e-Meeting, Aug 16th – Aug 27th, 2021

Source: 	ZTE Corporation

Title: 	Discussion on TB processing over multi-slot PUSCH

Agenda item:	8.8.1.2

Document for:	Discussion/Decision

Introduction

In the RAN1#105-e meeting, the enhancement methods for TBoMS were discussed, and the following agreements were achieved[1][2]:

Agreement:

For TBS determination of TBoMS:

- NohPRB is configured by xOverhead and represents the overhead per slot.

- NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated.

Note: xOverhead configuration is as per Rel-15/16.

Working assumption

A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission

- FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc.

- FFS: whether such concept will be specified or not.

Agreement:

- The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e)

  - Option 3, if a design based on single RV is adopted.

  - Option 4, if a design based on different RVs is adopted.

- FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc.

- The single RV is not constrained to have only the same coded bits in each slot or in each TOT

- The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on.

Agreement:

Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- FFS: details

- FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case)

Working assumption

Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Agreement:

The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected:

- Option a: Rate-matching is performed per slot;

- Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT;

- Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS

Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X.

Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

Agreement:

Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.

- FFS: details.

Agreement:

The following approach is used to calculate NInfo for TBoMS:

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K.

L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

In this contribution, we further analyze the potential enhancements and provide our views on TBoMS.

TB processing over multi-slot PUSCH

Time domain resource determination for TBoMS

- Use of special slot

In RAN1#105-e, it has been agreed that the time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA. For PUSCH Type A repetition in Rel-15/16, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, a repetition transmission should be omitted if at least one of the symbols allocated is not available, the similar way could be reused for TBoMS transmission. However, some companies argued that Type A like TDRA could not use the symbols in special slots in TDD and a method to optimize the using of special slots for TBoMS should be studied. Firstly, the symbols in special slot can be used for TBoMS as long as there are sufficient symbols compared to the TDRA indication. In addition, such optimization seems unnecessary because the same coverage performance can be achieved by configuring a relatively larger number of uplink slots/symbols or relying on more HARQ re-transmissions. It may result in a larger latency, but low latency is not the target in coverage limited scenario. Needless to say, the symbols in special slot can be reserved for transmission of other channels (e.g. PUCCH or SRS) or for transmission of cell center UEs. As a result, we have the following proposal.

Proposal 1: Confirming the working assumption of allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

- No optimization specific for the use of special slot in TDD is pursued.

- The number of slots for TB processing over multiple slots

In Rel-16, dynamic repetition indication is supported for both repetition Type A and repetition Type B. An additional column is added in TDRA table to indicate the number of repetitions (including repetition K=1), and the time resource allocation field in DCI (for DG or type 2 CG) or by RRC (for type 1 CG) is used to indicate one row of the TDRA table. Similar indication could be used for indicating of the number of slots for TB processing over multiple slots. In addition, the same set of candidate values as Rel-16 repetition type A could be reused for the number of slots for TBoMS, i.e., {1, 2, 3, 4, 7, 8, 12, 16}.

As a result, we have the following proposal.

Proposal 2: For TBoMS, add an new column in TDRA table to indicate the number of slots.

- Support {1, 2, 3, 4, 7, 8, 12, 16} as the candidate values.

- Collision handling due to DL/UL interaction

For PUSCH type A repetition, the Rel-15 collision handling rules are summarized as follows according to [3]. In Rel-16, a repetition could be dropped due to UL CI or high priority transmission. From our perspective, the legacy Rel-15/16 rules for PUSCH repetition type A could be reused by replacing a repetition to a slot of the multiple slots for TB processing. New collision handling rules can be considered only if strong motivation is identified.

Summary of Rel-15 behavior - For DG PUSCH and the first PUSCH after Type 2 CG activation DCI - Regardless of whether dynamic SFI is configured or not, - The case without repetition: no conflict with semi-static DL symbol is expected. - The case with repetitions: if a repetition conflicts with a semi-static DL symbol, the repetition is not transmitted. - If dynamic SFI is configured, - DG - The case with and without repetition: follow the dynamic grant. No conflict with dynamic DL symbols is expected. - The first PUSCH after Type 2 CG activation DCI - The case with and without repetition: follow the activation DCI. No conflict with dynamic DL/flexible symbols is expected. - For CG PUSCH (other than the first PUSCH after Type 2 CG activation DCI) - If dynamic SFI is not configured, - The case with and without repetition: if a repetition conflicts with a semi-static DL symbol, the repetition is not transmitted. - If dynamic SFI is configured and received, - The case with and without repetition: if a repetition conflicts with a semi-static DL symbol or a dynamic DL/flexible symbol, the repetition is not transmitted. - If dynamic SFI is configured and not received, - The case with and without repetition: if a repetition conflicts with a semi-static DL/flexible symbol, the repetition is not transmitted. (there is some timeline defined.)

Proposal 3: For collision handling of TBoMS, legacy Rel-15/16 collision handling rules for PUSCH repetition type A could be reused by replacing a repetition to a slot of the multiple slots for TB processing.

FDRA for TBoMS

The main motivation for TBoMS is to improve the coverage capability for cell-edge UE by obtaining low code rate with less number of RBs in the frequency domain, which could boost the PSD for cell-edge UE for better coverage. When TBoMS is enabled, there is no need to occupy a lager frequency domain resources to achieve even lower code rate thanks to the increased time domain resources from multiple slots. Thus, the maximum number of PRB in the frequency domain can be limited. As a result, some bits of FDRA field in DCI can be saved. As how to define the maximum number of PRBs, it could be further studied, e.g., whether it is a fixed value or it depends on the number of slots for TBoMS.

Proposal 4: The maximum number of PRBs can be limited when TBoMS is enabled.

- FFS how to determine the maximum number of PRBs.

RV determination,TOT and RM for TBoMS

The structure of TBoMS will be according to only one of these two options that have been agreed in RAN1#105e:

  - Option 3, if a design based on single RV is adopted.

  - Option 4, if a design based on different RVs is adopted.

The main difference between Option 3 and Option 4 is whether a single RV or different RVs is used for the TOTs/slots within TBoMS. To compare the performance of the two schemes, simulation evaluations are performed assuming a TBoMS with 8 slots. In the evaluation, TDD configuration with ‘DDDSUDDSUU’ is assumed and the special slots are not be used for TBoMS transmission. Thus, a TOT constitutes 1 or 2 consecutive slots under such assumptions and totally 6 TOTs are needed in the simulations. Simulation results for UMa and RMa scenario are provided in Figure 1 and Figure 2 respectively. For single RV case, RV#0 is mapped continuously to the 6 TOTs which include 8 available UL slots. For different RVs, RV sequence [0, 2, 3, 1] is used for RV cycling per TOT and per slot. More detailed simulation assumptions are given in the Appendix.

Figure 1. Single RV vs RV cycling for TBoMS transmission with 6 TOTs in UMa scenario

Figure 2. Single RV vs RV cycling for TBoMS transmission with 6 TOTs in RMa scenario

As can be observed in Figure 1, in Uma eMBB scenario, where the TBS is relative large, TBoMS with different RVs cycling among slots cannot be successfully decoded since the number of systematic bits of LDPC coding is too large to be fully mapped. In addition, using a single RV#0 for TBoMS can achieve better performance than different RVs cycling per TOT and per slot. As can be observed in Figure 2, in Rma eMBB scenario, about 0.85 and 1.25dB gain can be achieved by using a single RV than RV cycling for TBoMS transmission per TOT and per slot respectively.

Observation 1: For TBoMS without repetition, compared to use different RVs cycling among TOTs , using a single RV can achieve better performance under different scenarios.

Observation 2: For a given TBS, it is more likely that systematic bits of LDPC coding cannot be fully mapped within one slot in case of using different RVs cycling for TBoMS without repetition, which may cause decoding failure.

Proposal 5: Option 3 should be supported for TBoMS.

Proposal 6: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS.

In RAN1#105-e, a working assumption of the concept of transmission occasion for TBoMS (TOT) which includes one or more consecutive slots has been achieved, where a TOT is constituted of time domain resources which may or may not span multiple slots. Whether the concept of TOT should be specified and could be used for designing aspects related to signal generation or not are FFS. In our views, the mainly motivation of defining TOT is to facilitate the discussion progress and there is not obvious other motivation to specify this concept. Some companies proposed that the TOT could be used for designing aspects related to signal generation, such as RM and power control. However, as shown in Figure 1 and Figure 2, worse performance is observed if the RM for TBoMS is based on TOT due to the number of systematic bits of LDPC coding will be not fully mapped within a TOT. More specification efforts will be needed if transmission power of TBoMS is determined based on TOT. For instance, when the number of CBs within a TOT is not integer, a non-integer scaling factor k for transmission power determination need to be introduced. As a result, we have the following proposal.

Proposal 7: Confirming the WA of a transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission.

- No need to specify the concept of TOT.

- The concept of TOT will be not used for designing aspects related to signal generation.

TBS determination

- NInfo determination for TBoMS

It has been agreed that the NInfo for TBoMS calculation is based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated and scaled by K≥1. As discussed in section 2.1, no specific optimization for the use of special slot in TDD should be considered. In such case, the number of REs is the same in a normal UL slot and in a special slot in TDD within TBoMS if the special slot(s) in TDD is(are) used for TBoMS transmission. In addition, the number of slots for TBoMS is indicated by a column of TDRA. Naturally, the number of slots for TBoMS could be reused as scaling factor K for NInfo calculation.

Proposal 8: The number of slots for TBoMS could be reused as scaling factor K for NInfo calculation.

- TBS limitation for TBoMS

- For TBoMS, TBS is determined based on multiple slots and transmitted over multiple slots. It has been agreed that the maximum supported TBS for TBoMS should not exceed legacy maximum supported TBS in Rel-15/6 for the same number of layers. A following-up question is how to limit the maximum TBS considering the number of REs could be used is K times of the legacy.

- One simple way is to directly limit the maximum number of PRBs as discussed in section 2.2. However, it would need some cautious discussion to determine a specific maximum value. Another approach is to limit the maximum TBS by the conditions of date rate limitations DataRate and DataRateCC.

- Proposal 9: The maximum TBS can be limited by the conditions of date rate limitations DataRate and DataRateCC.

UCI multiplexing on PUSCH for TB processing

When TBoMS is enabled, and UE also transmits UCI information over the one or more slots which overlaps with the PUSCH transmission, the UCI multiplexing rules should be studied.

In Rel-15/16, when a UE would multiplex UCI on PUSCH, the first symbol of the earliest PUCCH or PUSCH among a group of overlapping PUCCHs and PUSCHs in the slot should satisfy some timeline conditions. In case of PUSCH repetition, UCI multiplexing is performed per repetition basis. In case of PUSCH overlapping with PUCCH repetition, the overlapping PUSCH should be dropped. Similar for TBoMS, at least the following aspects need to be further discussed.

- Whether the timeline condition for UCI multiplexing on PUSCH is based on the first slot or the overlapped slot(s) for TBoMS PUSCH

- Whether or not the UCI should be multiplexed only on the overlapping slots for TBoMS PUSCH.

- How to determine the number of resources for UCI multiplexing on TBoMS PUSCH.

- To minimize the specification impacts, RAN1 should aim for reusing the existing UCI multiplexing rules for PUSCH repetition type A as much as possible.

- Proposal 10: Further discuss UCI multiplexing rules for TBoMS with aiming for reusing existing UCI multiplexing rules for PUSCH repetition type A as much as possible.

Power control determination

In Rel-15/16, the PUSCH transmission power for a PUSCH transmission occasion depends on the total number of REs for the PUSCH with excluding DMRS and PTRS REs .

When TBoMS is enabled, the TBS is determination based on multiple slots for TB processing. Similarly, the PUSCH transmission power determination should also be based on the total number of REs within all slots for TB processing excluding the overhead of reference signals.

- Proposal 11: For TBoMS, the transmission power determination should be based on the total number of REs within all slots for TB processing with excluding the overhead of reference signals.

Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: Confirming the working assumption of allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

- No optimization specific for the use of special slot in TDD is pursued.

Proposal 2: For TBoMS, add an new column in TDRA table to indicate the number of slots.

- Support {1, 2, 3, 4, 7, 8, 12, 16} as the candidate values.

Proposal 3: For collision handling of TBoMS, legacy Rel-15/16 collision handling rules for PUSCH repetition type A could be reused by replacing a repetition to a slot of the multiple slots for TB processing.

Proposal 4: The maximum number of PRBs can be limited when TBoMS is enabled.

- FFS how to determine the maximum number of PRBs.

Observation 1: For TBoMS without repetition, compared to use different RVs cycling among TOTs , using a single RV can achieve better performance under different scenarios.

Observation 2: For a given TBS, it is more likely that systematic bits of LDPC coding cannot be fully mapped within one slot in case of using different RVs cycling for TBoMS without repetition, which may cause decoding failure.

Proposal 5: Option 3 should be supported for TBoMS.

Proposal 6: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS.

Proposal 7: Confirming the WA of a transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission.

- No need to specify the concept of TOT.

- The concept of TOT will be not used for designing aspects related to signal generation.

Proposal 8: The number of slots for TBoMS could be reused as scaling factor K for NInfo calculation.

- Proposal 9: The maximum TBS can be limited by the conditions of date rate limitations DataRate and DataRateCC.

- Proposal 10: Further discuss UCI multiplexing rules for TBoMS with aiming for reusing existing UCI multiplexing rules for PUSCH repetition type A as much as possible.

- Proposal 11: For TBoMS, the transmission power determination should be based on the total number of REs within all slots for TB processing with excluding the overhead of reference signals.

Reference

3GPP, RAN1#105-e Chairman’s notes.

3GPP, RAN1#105-e, R1-2106251, FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2), Moderator (Nokia, Nokia Shanghai Bell).

3GPP RAN1#99, R1-1913519, “Summary #4 of PUSCH enhancements for NR eURLLC (AI 7.2.6.3)	”, Nokia, Nokia Shanghai Bell.

3GPP TS 38.213 V16.3.0, “Physical layer procedures for control”, Release 16, 2020-09.

- Appendix

Table 1 Link level simulation assumption for TBoMS

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Frame structure | DDDSUDDSUU |
| Channel model | TDL-C (delay spread: 300ns) |
| UE speed | 3 km/h |
| BS antenna configuration | 4 Rx |
| UE antenna configuration | 1 Tx |
| System bandwidth | 100 MHz |
| Sub-carrier spacing | 30kHz |
| Occupied RBs | 4/30 |
| MCS | 2 for RMa eMBB, 4 for UMa eMBB |
| Waveform | DFT-s-OFDM |
| Number of OS per slot | 14 |
| DMRS overhead | 2 DMRS symbols |
| Number of slots for TBoMS | 8 |
| Frequency hopping | w/o |
| Channel estimation | Practical |
| Receiver type | MMSE |