3GPP TSG RAN WG1 #106-e			R1-2106763

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.16.1

Source: 	Qualcomm Incorporated, SWR, EBU

Title: 	PMCH allocation of 6/7/8MHz

Document for:	Discussion / Decision

# Introduction

In RAN#92-e, a new work item on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast was approved [1] with the following objective:

- For MBMS-dedicated cells: - Specify a PMCH allocation of 6/7/8 MHz and corresponding MBSFN reference signals [RAN1]. - Specify corresponding signaling [RAN2, RAN3]

In this contribution, we present our initial views on the above objective. The presented framework is based on a previous submission to RAN plenary [2].

# Proposed framework

The proposed framework for supporting 6/7/8MHz bandwidth relies on modifying the bandwidth allocation for PMCH to allocate a larger number of PRBs than that indicated by the system bandwidth. This allows to keep a commonality in the initial acquisition of the system (PSS/SSS/PBCH/PDCCH/SIBs are acquired assuming 5MHz bandwidth).

Designing a new system bandwidth would also be possible, but it would require introducing new entries in MIB, a different PDCCH interleaving, different DCI formats for resource allocation, etc.

In current TS 36.213, the resource allocation for PMCH is described as follows:

## 11.1	UE procedure for receiving the PMCH

[…]

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to.

We propose to add a configuration parameter to determine the number of resource blocks to be received for PMCH. We propose the following procedure:

- The UE operates in a cell that indicates a system bandwidth of 5 MHz (i.e, NRBDL=25 )

- The UE is configured with a PMCH bandwidth that is larger than this system bandwidth. We propose to support the following values:

  - 8MHz: NPRB=40

  - 7MHz: NPRB=35

  - 6MHz: NPRB=30

- The PMCH is received assuming the bandwidth is NPRB.

Proposal 1: For supporting 6/7/8MHz PMCH bandwidth, the initial acquisition and system information acquisition is performed over a 5MHz bandwidth.

Proposal 2: Allow configuring PMCH bandwidth larger than the system bandwidth indicated by MIB. The following PMCH bandwidth values are supported for NRBDL=25 (5MHz system bandwidth):

- 8MHz: NPRB=40

- 7MHz: NPRB=35

- 6MHz: NPRB=30

The PMCH bandwidth and 5MHz system bandwidth are centered around the same frequency.

The details of this method are shown in Figure 1:

- Step 1: MIB is unchanged, it indicates 25PRB (5MHz).

- Step 2: PDCCH is monitored in CAS assuming 25 PRB bandwidth.

- Step 3: SIB is received assuming 25 PRB bandwidth. Within SIB, there is an indication (e.g. per MBSFN area) of the bandwidth of PMCH)

- Step 4: The UE receives PMCH based on the larger system bandwidth.

Figure 1 MIB vs PMCH bandwidth.

# Other impacts

## TBS determination

The current TBS determination for PMCH relies on a configured MCS entry + a configured system bandwidth to determine the TBS:

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to.

For the case of 6/7/8MHz, the TBS determination should be based on the configured bandwidth (30/35/40PRBs) larger than the system bandwidth.

Proposal 3: The UE uses the configured bandwidth (30/35/40 PRBs) to determine the TBS as per TS 36.213, Subclause 11.1

## MBSFN-RS

The current equations for MBSFN-RS include the system bandwidth for determining both the start and length of the MBSFN-RS depends on the system bandwidth NRBDL:

##### 6.10.2.2.2	Mapping to resource elements for 1.25 kHz

The reference-signal sequence  in OFDM symbol  shall be mapped to complex-valued modulation symbols  with  according to

where

k=6mif nsfmod2=06m+3if nsfmod2=1
l=0
m=0,1,...,24NRBDL-1
m'=m+3NRBmax,DL-NRBDL

When 6/7/8MHz are configured for PMCH, this value should be replaced by the corresponding configured bandwidth.

Proposal 4: In the equations for determining the MBSFN-RS mapping to resource elements, NRBDL is replaced by the configured bandwidth for PMCH.

## RAN4 requirements

We would like to highlight that the RAN4 requirements for the new bandwidth of 6/7/8MHz will be developed in Rel-18 timeframe in a release independent manner (from Rel-17). The approved work item, which is scheduled to start after Q2’22, is approved in [3].

# Conclusions

In this contribution we presented our views on how to specify the bandwidth allocation of 6/7/8MHz for PMCH. We made the following proposals:

Proposal 1: For supporting 6/7/8MHz PMCH bandwidth, the initial acquisition and system information acquisition is performed over a 5MHz bandwidth.

Proposal 2: Allow configuring PMCH bandwidth larger than the system bandwidth indicated by MIB. The following PMCH bandwidth values are supported for NRBDL=25 (5MHz system bandwidth):

- 8MHz: NPRB=40

- 7MHz: NPRB=35

- 6MHz: NPRB=30

The PMCH bandwidth and 5MHz system bandwidth are centered around the same frequency.

Proposal 3: The UE uses the configured bandwidth (30/35/40 PRBs) to determine the TBS as per TS 36.213, Subclause 11.1

Proposal 4: In the equations for determining the MBSFN-RS mapping to resource elements, NRBDL is replaced by the configured bandwidth for PMCH.

# References

[1] RP-211144, “Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 1”, EBU, Qualcomm

[2] RP-202210, “Support of flexible bandwidth for MBMS”, EBU et al.

[3] RP-211145, “New WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm