3GPP TSG RAN WG1 #106-e	R1-2106856

# e-Meeting, August 16th – 27th, 2021

Agenda item:	7.2

Source:	Samsung

Title:	Discussion on counting PDCCH candidates in overbooking procedure

Document for:	Discussion and decision

# Introduction

This contribution discusses possible interpretations of the specifications [1] for the search space set overbooking/dropping procedure when a search space set is associated with multiple DCI format sizes. A draft CR for Rel-16 according to our understanding is provided in [2].

# PDCCH monitoring

In Rel-15, a CSS set is associated with DCI formats 0_0/1_0 and with a number of DCI formats 2_x. A USS set is associated either with DCI formats 0_0/1_0 or with DCI formats 0_1/1_1. In Rel-16, a USS set can also be associated either with DCI formats 0_2/1_2/0_1/1_1 or with DCI formats 0_2/1_2. There is also the constraint of having at most 4 sizes of DCI formats decoded, and at most 3 sizes of DCI formats with CRC scrambled by C-RNTI, decoded across all applicable search space sets for the active DL BWP of each serving cell.

Despite the DCI format size matching procedure, it is likely that a search space set will contain more than one sizes for respective DCI formats. For example, DCI formats 0_1 and 1_1 are practically never size matched. That probability became larger in Rel-16 with the introduction of DCI formats 0_2 and 1_2. Even DCI formats associated with a CSS set can have different sizes.

The following is captured in clause 10.1 of [1]. However, although monitoring of a PDCCH candidate implies decoding according to corresponding DCI formats in the search space set, that is not captured in the counting of PDCCH candidates.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats.

A UE capability for PDCCH monitoring relates to a number of decoding operations and channel estimates the UE can perform within a time period such as a slot or a span. However, a PDCCH candidate can be related with different number of decoding operations depending on the search space set associated with the PDCCH candidate. While the definition of MPDCCHmax,slot,μ and MPDCCHtotal,slot,μ can remain, as currently specified, as those PDCCH candidates are totals across all search space sets in a slot (or span), the definition of LMScss(i)(L) as counting PDCCH candidates for CSS set Scss(i), or LMSuss(j)(L) for USS set Suss(j), in the UE procedure for determining/resolving search space set overbooking needs to be clarified.

A first interpretation is that the above summations are for PDCCH candidates per search space set, as is currently defined in [1]. A second interpretation is that the above summations are for the number of decoding operations per PDCCH candidate per search space set. In our understanding, the second interpretation is the correct one.

Then, the number of sizes of DCI formats for a search space set needs to also be considered in the overbooking procedure. Basically, if for USS set Suss(j1) there is one DCI format size and for USS set Suss(j2) there are two DCI format sizes, using LMSuss(j1)(L) is correct but LMSuss(j2)(L) needs to be replaced by 2∙LMSuss(j2)(L) as LMSuss(j2)(L) is the total number of PDCCH candidates for Suss(j2), not the total number of decoding operations. Consequently, the search space set overbooking procedure in clause 10.1 of [1] needs to be amended as follows.

## 10.1 UE procedure for determining physical downlink control channel assignment < unchanged text is omitted> Denote by , , the number of counted PDCCH candidates for monitoring for CSS set and by , , the number of counted PDCCH candidates for monitoring for USS set , for CCE aggregation level L. For the CSS sets, a UE monitors MPDCCHcss=i=0Icss-1d(i)∙LMScss(i)(L)PDCCH candidates requiring a total of CPDCCHcss non-overlapping CCEs in a slot or in a span, where d(i) is a number of sizes for DCI formats for CSS set i after alignment of DCI format sizes [5, 38.212]. < unchanged text is omitted> Set Set Set d(j) to a number of sizes for DCI formats for USS set j after alignment of DCI format sizes Set while d(j)∙ AND allocate d(j)∙ PDCCH candidates for monitoring to USS set MPDCCHuss=MPDCCHuss-d(j)∙LMSuss(j)(L); ; ; end while

# Conclusions

Based on above discussion, we request RAN1 to provide an understanding for the counted variable in determining search space set overbooking. Possible interpretations are: (a) the PDCCH candidates for each search space set are counted, or (b) decoding operations for DCI formats per PDCCH candidate are counted. If RAN1 concludes that decoding operations for DCI formats per PDCCH candidate are counted, we request RAN1 to consider the CR in [2].

# References:

- TS 38.213, v16.6.0 “NR; Physical layer procedures for control”

- R1-2106857, “Correction on counting PDCCH candidates in overbooking procedure”, Samsung.