3GPP TSG RAN WG1 #106-e	                                                                 R1- 2106861

e-Meeting, August 16th – 27th, 2021

Agenda Item:		7.2.5

Source:		Samsung

Title:		Discussion on PUSCH skipping for URLLC

Document for:		Discussion

Introduction

This contribution discusses remaining issues about PUSCH skipping considering PHY and MAC priority.

Discussion

Overlapped data and SR are of equal L1 priority

In RAN1#104-e, there was a discussion on which scenario should be discussed with RAN2 and following cases are agreed to send RAN2 in [1]. And, the reply LS is sent to RAN1 in RAN1#106-e as shown in [2].

Case 2-1: other UCI(s) i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority and the SR overlaps with the PUSCH of an equal L1 priority Case 2-1: the final PUCCH resource after UCI multiplexing does not overlap with PUSCH For case 2-1, if there are other UCI(s) i.e., HARQ-ACK/CSI of the equal L1 priority overlapping with SR, and the final PUCCH resource after UCI multiplexing among different PUCCHs does not overlap with the PUSCH and does not overlap with any other PUSCH if any, RAN1 has the following two understandings: - Understanding 1: MAC is not aware of the UCI multiplexing in PHY, MAC does not know whether the final PUCCH overlaps with the PUSCH or not, MAC only knows configured PUCCH resource for SR. Therefore, MAC can decide to deliver SR or PUSCH. - Understanding 2: MAC is aware of the UCI multiplexing in PHY based on UL skipping agreement (as in LS R1-2009772). If MAC is aware that the final PUCCH resource does not overlap with the PUSCH, and does not overlap with any other PUSCH, then for case 2-1, MAC can send both SR and PUSCH to PHY.

For case 2-1, since understanding 1 is confirmed by RAN2, RAN1 would have different UE behaviors depending on which one is delivered between SR or PUSCH. If SR is generated from MAC, a UE would multiplex SR and AN/CSI on other PUCCH resources. If PDU is generated for PUSCH from MAC, the UE would multiplex AN/CSI on PUSCH for case 2-1(a) or not multiplex AN/CSI on PUSCH for case 2-1(b). Even though UCI multiplexing behavior and SR/PUSCH transmission behaviors would be different according to MAC decision, UE would check processing timeline requirement internally, and then if not satisfied, UE would assume that it is error case. Therefore, there is no need to do relaxation of processing timeline requirement for this case. Since various traffic priority level become one of key components for XR services, this key feature should be kept even if considering UL skipping.

Observation 1: There is no processing time impact for case 2-1 given that understanding 1 is confirmed by RAN2.

Case 2-2: the final PUCCH resource after UCI multiplexing among different PUCCHs carrying HARQ-ACK/CSI and SR overlaps with the PUSCH Case 2-2: the final PUCCH resource after UCI multiplexing overlaps with PUSCH For case 2-2, RAN1 has the following two different understandings: - Understanding 1: the UL skipping-related check is prioritized over the LCH based prioritization check in MAC. Therefore, if the PUSCH in the LS is expected to have UCI multiplexing, MAC does not prioritize SR over PUSCH, and send a MAC PDU to PUSCH instead. - Understanding 2: the LCH based prioritization check is prioritized over the UL skipping-related check in MAC. Therefore, the SR in the LS is prioritized in MAC and is delivered and MAC shall not deliver the MAC PDU for the PUSCH.

Case 3: other UCI(s) i.e., HARQ-ACK/CSI overlaps with a PUSCH of an equal L1 priority, SR overlaps with the PUSCH of equal L1 priority, but other UCI(s) do not overlap with the SR Case 3: other UCI(s) overlaps with a PUSCH, SR overlaps with the PUSCH, SR does not overlap with other UCI(s) For case 3, RAN1 has the following two different understandings: - Understanding 1: the UL skipping-related check is prioritized over the LCH based prioritization check in MAC. Therefore, if the PUSCH in the LS is expected to have UCI multiplexing, MAC does not prioritize SR over PUSCH, and send a MAC PDU to PUSCH instead. - Understanding 2: the LCH based prioritization check is prioritized over the UL skipping-related check in MAC. Therefore, the SR in the LS is prioritized in MAC and is delivered and MAC shall not deliver the MAC PDU for the PUSCH.

For case 2-2 and 3, understanding 2 is conditionally confirmed by RAN2. Therefore, for case 2-2, MAC can choose to send either SR or PUSCH according to LCH prioritization. So, if SR is prioritized from MAC, a UE would multiplex AN/CSI with SR on PUCCH resource in both case 2-2(a) and case 2-2(b). For case 3, MAC can choose to send either SR or PUSCH according to LCH prioritization. So, if SR is prioritized from MAC, a UE would transmit SR PUCCH and AN/CSI PUCCH, respectively. If PUSCH is prioritized from MAC, a UE would multiplex UCI on PUSCH and then transmit PUSCH. However, it is noted that if understanding 1 is assumed, it is likely that a UE cannot send SR in order to request PUSCH resource for more urgent traffic (like URLLC traffic) even if SR is associated with high MAC priority while PUSCH is associated with low MAC priority because MAC cannot prioritize SR over PUSCH regardless of MAC priority.

Observation 2:  For case 2-2 and case 3, if understanding 1 is considered that is not aligned with RAN2’s working assumption, SR is always deprioritized than PUSCH even if SR is higher MAC priority than PUSCH.

Case 4: other UCI(s), i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority Case 4: other UCI(s) overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority For case 4, if there is no resource overlapping between SR and PUSCH of an equal L1 priority, and the final PUCCH resource after UCI multiplexing among different PUCCHs overlap with the PUSCH, RAN1 has the following two understandings: - Understanding 1: MAC is not aware of the UCI multiplexing in PHY, MAC does not know whether the final PUCCH overlaps with the PUSCH or not, MAC only knows configured PUCCH resource for SR. Therefore, MAC can send both SR and PUSCH to PHY, based on current RAN1 specification TS 38.213, PHY will multiplex other UCI(s) i.e., HARQ-ACK/CSI in the PUSCH and does not transmit SR. - Understanding 2: MAC is aware of the UCI multiplexing in PHY, If MAC is aware that the final PUCCH resource overlaps with the PUSCH, then MAC can decide to deliver SR or PUSCH.

For case 4, since understanding 1 is confirmed by RAN2, it is likely that MAC can send both SR and PUSCH to PHY. In this case, current specification specified that a UE doesn’t transmit PUSCH if the UE would transmit a PUSCH without UL-SCH that overlaps with a PUCCH transmission that includes positive SR information. Otherwise, the UE would transmit PUSCH and drop SR transmission.

Observation 3:  For case 4, with understanding 1, a UE would transmit either SR or PUSCH according to whether PUSCH includes UL-SCH or not.

DG vs. CG in case where MAC priority is not configured and PHY priority is configured

There was a discussion on how to handle RAN2’s working assumption (DG always overrides CG) when lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities. Main issue is when CG has higher priority than DG in PHY perspective. Otherwise, PHY and MAC have same understanding. One argument confirming RAN2’s working assumption is that MAC cannot differentiate between DG and CG since PHY priority cannot be visible in MAC layer. However, one counter argument is that it is likely that HP CG cannot be transmitted on time since LP DG overrides HP CG since MAC would generate MAC PDU only for LP DG that might be against the purpose of introducing PHY priority. Therefore, it is reasonable to preclude the case by making RAN1 conclusion in order to move forward since both are valid arguments. For example, RAN1 assumes that it is not possible that HP DG and LP CG are overlapped in time When lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities.

Proposal 1: When lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities, RAN1 confirms RAN2’s working assumption that DG always overrides CG with the RAN1 except for the case where DG has high priority and CG has low priority.

Proposal 2: Conclude that that overlapping between HP DG and LP CG is not allowed when lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities.

LCH based priority has higher priority than UL skipping

As explained before, RAN2 made working assumption that LCH based priority has higher priority than UL skipping and also doesn’t consider UCI multiplexing results to generate MAC PDU or SR information. The main argument not confirming the working assumption is that it is likely to increase gNB blind detection probability. However, it is noted that there are existing RRC parameters (enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured) that can enable or disable UL skipping for both configured grant and dynamic grant. Therefore, gNB can control blind detection complexity using those RRC parameters. Beside, without considering UL skipping, if there are multiplexing overlapping CG PUSCHs in same serving cell or different serving cell, blind detection issues still exist. Therefore, blind detection issue should be manageable by gNB resource allocation. That is, gNB should schedule or configure uplink resources being overlapped as much as it can do blind detection.

Proposal 3: Confirm the working assumption that LCH based priority has higher priority than UL skipping.

Remaining issues for all possible cases considering UL skipping, PHY priority, MAC priority and repetition

It was discussed that a PUSCH#0 overlap with a PUCCH#0 including only HARQ-ACK and/or CSI with the same L1 priority on a same or different serving cell, PUSCH#1 overlaps with the PUSCH#0 on a serving cell with the same or different L1 priorities and the PUSCH#1 does not overlap with the PUCCH#0, if the PUSCH#0 is NOT delivered by MAC, as shown in New case (B) in Figure 1. For the same L1 priority and no MAC priority, this new case (B) is quite similar to case 1-6 (A) as shown in Figure 1.

Figure 1. Comparison between Case 1-6 and new case

For case 1-6 (A), it was discussed in 7.1 CR assuming Rel-16 LCH based prioritization is not configured and there is a single PHY priority and it was agreed that UCI is transmitted on PUCCH if there is no other CG PUSCH in different serving cell. Otherwise, UCI can be transmitted on other CG PUSCH in different serving cell when timeline is satisfied. Even though additional conditions such as PHY priority or MAC priority are considered, same principle can be applicable to new case (B) since main assumption is the same: MAC does not deliver MAC PDU for CG PUSCH in case 1-6 (A) and MAC does not deliver MAC PDU for PUSCH#0 in new case (B). That is, UCI is transmitted on PUCCH#0 if there is no other overlapping PUSCH resource in different serving cell. Otherwise, UCI can be transmitted on other PUSCH in different serving cell when processing timeline is met. It is noted that there is no UE behaviour if processing condition is not met. Even in current specification, UE behaviour is not defined if processing tineline condition is not met. Therefore, for new case, there is no need to define a certain UE behaviour if processing timeline condition is not met.

Proposal 4: When a PUSCH#0 overlap with a PUCCH#0 including only HARQ-ACK and/or CSI with the same L1 priority on a same or different serving cell, PUSCH#1 overlaps with the PUSCH#0 on a serving cell with the same or different L1 priorities and the PUSCH#1 does not overlap with the PUCCH#0, if the PUSCH#0 is NOT delivered by MAC, UCI is transmitted on PUCCH#0 if there is no other PUSCHs in different serving cell. Otherwise, UCI can be transmitted on other PUSCH if there is a PUSCH overlapping with PUCCH#0 in different serving cell.

For interaction between PUSCH skipping and PUSCH repetition (including both type A and type B), RAN1 concluded that UE does not expect to have both configurations are enabled together in RAN1#105-e in which Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions. There is no technical reason to have different behaviours in which at least one of configurations is enabled. So, RAN1 needs to conclude that UE is not expected that Rel-16 PUSCH skipping (including both enhancedSkipUplinkTxDynamic-r16 and enhancedSkipUplinkTxConfigured-r16) and PUSCH repetitions (including both type A and type B) are enabled together when Rel-16 LCH based prioritization is configured and/or there are two PHY priorities for UL transmissions.

Proposal 5: Concludes that UE is not expected that Rel-16 PUSCH skipping (including both enhancedSkipUplinkTxDynamic-r16 and enhancedSkipUplinkTxConfigured-r16) and PUSCH repetitions (including both type A and type B) are enabled together when Rel-16 LCH based prioritization is configured and/or there are two PHY priorities for UL transmissions.

Conclusions

This contribution discussed remaining issues about PUSCH skipping considering PHY and MAC priority. The followings are proposals in this contribution.

Proposal 1: When lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities, RAN1 confirms RAN2’s working assumption that DG always overrides CG with the RAN1 except for the case where DG has high priority and CG has low priority.

Proposal 2: Conclude that overlapping between HP DG and LP CG is not allowed when lch-BasedPrioritization is not configured and PHY is configured with two L1 priorities.

Proposal 3: Confirm the working assumption that LCH based priority has higher priority than UL skipping.

Proposal 4: When a PUSCH#0 overlap with a PUCCH#0 including only HARQ-ACK and/or CSI with the same L1 priority on a same or different serving cell, PUSCH#1 overlaps with the PUSCH#0 on a serving cell with the same or different L1 priorities and the PUSCH#1 does not overlap with the PUCCH#0, if the PUSCH#0 is NOT delivered by MAC, UCI is transmitted on PUCCH#0 if there is no other PUSCHs in different serving cell. Otherwise, UCI can be transmitted on other PUSCH if there is a PUSCH overlapping with PUCCH#0 in different serving cell.

Proposal 5: Concludes that UE is not expected that Rel-16 PUSCH skipping (including both enhancedSkipUplinkTxDynamic-r16 and enhancedSkipUplinkTxConfigured-r16) and PUSCH repetitions (including both type A and type B) are enabled together when Rel-16 LCH based prioritization is configured and/or there are two PHY priorities for UL transmissions.

The followings are observations in this contribution.

Observation 1: There is no processing time impact for case 2-1 given that understanding 1 is confirmed by RAN2.

Observation 2:  For case 2-2 and case 3, if understanding 1 is considered that is not aligned with RAN2’s working assumption, SR is always deprioritized than PUSCH even if SR is higher MAC priority than PUSCH.

Observation 3:  For case 4, with understanding 1, a UE would transmit either SR or PUSCH according to whether PUSCH includes UL-SCH or not.

Previous Agreements (7.1 CR)

- RAN1 #102-e

Agreement

- For UL skipping of dynamic UL grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH.

Agreement

The following text proposal for TS38.214 is endorsed. Final CR is agreed in R1-2007337 (TS 38.214, Rel-16, CR#0123, Cat F).

6.1 UE procedure for transmitting the physical uplink shared channel <unchanged part omitted> A UE shall upon detection of a DCI format scheduling a PUSCH transmit the corresponding PUSCH unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH, as described in clause 6.1.2.1, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo 16 operation applied. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process. <unchanged part omitted>

Agreement

Send an LS to RAN2 to inform them of the latest RAN1 agreement on uplink skipping.

In Rel-15, for dynamic UL skipping, RAN1 discussed the LS R1-2000015 from RAN2 and provided replies in R1-2001376 for Case 1 of dynamic PUSCH skipping without overlapping CSI/HARQ-ACK on PUCCH. Case 2 of dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH was further discussed in RAN1. In RAN1#101-e meeting, it was concluded that in Rel-15, the UE behavior is undefined for case 2 and case 2 can be addressed for Rel-16. Endorsed CR R1-2005044 (TS38.214, Rel-15, CR#0105, Cat. F) for Case 1 and Case 2 can be found in the attachment. In Rel-16, RAN1 continued the discussion for Case 2 and made following agreements in RAN1#102-e meeting: Agreement For UL skipping of dynamic UL grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH.. Based on above agreements, RAN1 in principle agreed the corrections for Rel-16 TS 38.214 (R1-200xxxx), assuming that RAN2 will update the Rel-16 sepcification TS 38.321 corresponding to the above agreement so that UE generates the MAC PDU for the PUSCH with UCI multiplexing. In addition, RAN1 noticed that in Rel-15, dynamic UL skipping is an optional feature with capability signaling (skipUplinkTxDynamic). It is RAN1’s understanding the dynamic UL skipping cannot be implemented based on the Rel-15 specification. For Rel-16 with the defined UE behavior for dynamic UL skipping, RAN1 has discussed following two options for the capability signaling handling. However, the final decision on the capability design for Rel-16 dynamic UL skipping should be decided by RAN2. - Option 1: introduce a new UE capability for Rel-16 dynamic UL skipping - Option 2: Reuse Rel-15 UE capability with the understanding that Rel-15 dynamic UL skipping is not implementable therefore UEs indicating this capability should implement Rel-16 behavior.

LS is approved in:

R1-2007338	LS on PUSCH with UL skipping	RAN1, vivo

- RAN1 #103-e

Agreement

The text proposal in R1-2008655 is endorsed for TS38.214 as revision of R1-2007337. Endorsed in R1-2009687 (TS38.214, Rel-16, CR#0123, Cat. F). Add the following in the CR cover sheet.

- This CR is expected to submit to RAN plenary for approval together with the corresponding endorsed RAN2 CR.

- Other specs affected: TS 38.321

Agreement:

For the case (Case 1-2) where only one or more CG PUSCHs overlapping with PUCCH

- In Rel.16, for CA and non-CA case, when Rel-16 LCH based prioritization is not configured and there is a single PHY priority for  UL transmissions, and when PUSCH repetition is not applied, in case of one or more CG PUSCHs overlapping with UCI and there is no DG PUSCH overlapping with the UCI and there is no DG PUSCH overlapping with the one or more CG PUSCHs, the CG PUSCH with UCI multiplexing from the one or more CG PUSCHs cannot be skipped.  MAC generates MAC PDU for the CG PUSCH and delivers the MAC PDU to PHY and the UCI is multiplexed on the CG PUSCH.

Conclusion

For the following cases, for CA and non-CA, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, MAC generates MAC PDU for the DG PUSCH and the UCI is multiplexed on the DG PUSCH. For the case 1-3 and 1-4, MAC does not generate a TB for the CG PUSCH(s) overlapping with the DG PUSCH on the same serving cell.  The GG PUSCH(s) is discarded and does not participate in subsequent physical layer procedure.

- (Case 1-3) DG PUSCH and CG PUSCH are overlapping and both DG/CG PUSCH are overlapping with PUCCH

- (Case 1-4) DG PUSCH and CG PUSCH are overlapping and DG PUSCH is overlapping with PUCCH, and CG PUSCH is non-overlapping with the PUCCH

- (Case 1-5) DG PUSCH and CG PUSCH are non-overlapping and both DG/CG PUSCH are overlapping with PUCCH

Working Assumption:

For the case (Case 1-6) when DG PUSCH and CG PUSCH are overlapping on a serving cell and CG PUSCH is overlapping with PUCCH, and DG PUSCH is non-overlapping with the PUCCH

- In Rel.16, for non-CA case, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied, in case of one or more CG PUSCHs overlapping with UCI and there is DG PUSCH overlapping with the CG PUSCHs on a serving cell and not overlapping with the UCI

  - Opt-3:

    - If there is data for DG, MAC generates PDU for DG PUSCH

      - UCI is transmitted on PUCCH.

    - If there is no data for DG, MAC does not generate PDU for DG or CG PUSCH

      - UCI is transmitted on PUCCH.

  - Opt-4:

    - If there is data for DG, MAC generates PDU for DG PUSCH

      - UCI is dropped together with CG PUSCH.

    - If there is no data for DG, MAC does not generate PDU for DG or CG PUSCH.

      - UCI is dropped together with CG PUSCH.

Note: In RAN1#104-e, aim to resolve case 1-6 using above options as a starting point, other options are not precluded.

Agreement

Send an LS to RAN2 to convey the above RAN1 agreement, conclusion, and working assumption on PUSCH skipping (Rel-16). The LS is endorsed in R1-2009772.

- RAN1 #104-e

Agreement

Send an LS to RAN2 to convey the latest RAN1 agreement on PUSCH skipping (Rel-16). LS is endorsed in R1-2102249.

Agreement

For the case (Case 1-6) when DG PUSCH and CG PUSCH are overlapping on a serving cell and CG PUSCH is overlapping with PUCCH, and DG PUSCH is non-overlapping with the PUCCH

- In Rel-16, when timeline condition is met, for Case 1-6 in non-CA and CA cases, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied,

  - When one or more CG PUSCH(s) overlap with a PUCCH on a same or different serving cell, a DG PUSCH overlaps with the one or more CG PUSCH(s) on one serving cell and the DG PUSCH does not overlap with the PUCCH, and there is no remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH, the UCI is transmitted on the PUCCH.

    - This is for case 1-6a and 1-6b in Figure 1.

    - MAC does not generate PDU for the one or more CG PUSCH(s)

    - If there is data for the DG PUSCH, MAC generates PDU for the DG PUSCH. If there is no data for the DG PUSCH, MAC does not generate PDU for the DG PUSCH

  - When one or more CG PUSCH(s) overlap with a PUCCH on a same or different serving cell, a DG PUSCH overlaps with the one or more CG PUSCH(s) on one serving cell and the DG PUSCH does not overlap with the PUCCH, and there is remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH, the PUSCH from the remaining PUSCH(s) for UCI multiplexing is determined following the existing UCI multiplexing rules, MAC generates MAC PDU for the PUSCH and delivers the MAC PDU to PHY and the UCI is multiplexed on the PUSCH.

    - Note the remaining CG PUSCH(s) are not overlapping with any DG PUSCH on the same serving cell

    - This is for case 1-6c in Figure 1.

    - MAC does not generate PDU for the one or more CG PUSCH(s)

    - If there is data for the DG PUSCH, MAC generates PDU for the DG PUSCH. If there is no data for the DG PUSCH, MAC does not generate PDU for the DG PUSCH

| Case 1-6a | Case 1-6b |
| --- | --- |
| Case 1-6c |  |

Figure 1: Case 1-6a/1-6b/1-6c for non-CA and CA case

Conclusion

For Case 1-6 when DG PUSCH and CG PUSCH are overlapping on a serving cell and CG PUSCH is overlapping with PUCCH, and DG PUSCH is non-overlapping with the PUCCH,

- The time condition is ensured by gNB, i.e. the ending symbol of UL grant for the DG PUSCH should be at least  symbols before the first symbol of the earliest PUCCH or PUSCH among the overlapping group of PUCCH/PUSCH channels.

- RAN1 understands that for Case 1-6 the PUCCH, the CG PUSCH and the DG PUSCH are considered as an overlapping group of PUCCH/PUSCH channels for which the multiplexing timeline needs to be satisfied.

  - The overlapping group of PUCCH/PUSCH channels for Case 1-6 is defined in the way such that a PUCCH/PUSCH would be included in a group if it overlaps with any channel in that group, regardless of whether multiplexing between these channels occurs or not.

- FFS whether or not additional spec change is needed

Conclusion

For Case 1-5, i.e. when DG PUSCH and CG PUSCH are non-overlapping and both DG/CG PUSCH are overlapping with PUCCH, PUCCH, CG PUSCH and DG PUSCH are considered as an overlapping group of PUCCH/PUSCH channels.

- No spec change is needed

- RAN1 #104b-e

Conclusion

For Rel-16, In case of UCI overlapping with multiple CG with the same starting time on a serving cell, it is up to UE implementation to determine the CG resource for UCI multiplexing from multiple CG configurations.

- No spec change is needed

- RAN1 #105-e

Conclusion

UE is not expected that Rel-16 PUSCH skipping (including both enhancedSkipUplinkTxDynamic-r16 and enhancedSkipUplinkTxConfigured-r16) and PUSCH repetitions (including both type A and type B) are enabled together when Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions.

Agreement

LS to RAN2 on UL skipping for PUSCH in Rel-16 is endorsed in R1-2106370

Previous Agreements (7.2.5 eURLLC)

- RAN1 #103-e

Agreement

Send an LS to RAN2 to convey the following:

- For the collision scenario between CG and DG with same/different PHY-priority index, if there is no collision between PUCCH and the CG  and there is no collision between PUCCH and the DG , the behavior mentioned in the LS is consistent with RAN1’s understanding if taking into account the TP to Rel-16 TS 38.214, i.e., revision CR in R1-2008655.

- When the MAC entity is configured with lch-basedPrioritization, for the collision scenario between CG and DG with same/different PHY-priority index, and when there is collision between PUCCH and the CG with the same priority and/or there is collision between PUCCH and the DG with the same priority, RAN1 is still discussing the related PHY layer behavior.

LS is endorsed in R1-2009680.

- RAN1 #104-e

Agreement

Reply LS to R1-2100026 on overlapped data and SR with equal L1 priority for Rel-16 URLLC is endorsed in R1-2102244.

- RAN1 #105-e

Conclusion

When LCH based prioritization is configured, for the case that only SR overlaps with PUSCH of equal L1 priority i.e., case 1 in Reply LS R1-2102244, RAN1 conclude that there is no impact on the PHY processing timeline to support the intended UE behavior as described in the LS R2-2011124 (i.e., if SR is prioritized in MAC, MAC shall not deliver the MAC PDU for the PUSCH and shall instruct PHY for SR transmission).

Conclusion

When LCH based prioritization is configured, the timeline (i.e., N2) defined in TS 38.214 section 6.1 is used as processing timeline for handling the collision between DG and CG without considering the UCI overlapping with the CG/DG of the same L1 priority so that MAC will only deliver one MAC PDU to PHY. There is no need to discuss further about relaxing the processing timeline for DG vs. CG collision case.

LS from RAN2 (R2-2106746)

For case 2-1 and case 4, RAN2 has made the following agreement in RAN2#114-e:

We go with Understanding 1: MAC does not use knowledge of UCI multiplexing when MAC executes LCH based prioritization and deciding when to transmit SR

For case 2-2 and case 3, RAN2 has made the following working assumption in RAN2#113-e:

Working assumption: The MAC entity does not generate a MAC PDU for a deprioritized uplink grant even when its associated PUSCH is overlapping with PUCCH. This working assumption is not agreed until confirmed by RAN1.

It was further confirmed in RAN2#113bis-e:

Confirm the WA that LCH based prio has higher priority than UL skipping still applies, and we expect that if there are issues, RAN1 will come-back.

The itended MAC layer behaviour of the working assumption is Understanding 2.

Appendix

[1] R1-2102244, Reply LS on overlapped data and SR are of equal L1 priority

[2] R2-2106746, Reply LS on overlapped data and SR with equal L1 priority