3GPP TSG RAN WG1 #106-e	                                                         R1-2106888

e-Meeting, Aug 16th – 27th, 2021

Agenda Item:		8.5.1

Source:				Samsung

Title:					Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Document for:	Discussion

# Introduction

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

In the approved new WI for positioning enhancement [1], one important direction is to refine the timing based estimation.

This contribution discusses the accuracy improvements on timing based positioning solutions especially for the timing error and timing error group.

# Handling of Timing error

At RAN1#104 meeting, UE Rx/Tx errors were discussed. RAN1 agreed on a definition of timing errors:

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

- Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

In addition, RAN1 introduced the concept of Timing Error Group (TEG):

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

- UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

- TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

In this section, we will discuss TEG for DL TDOA, UL TDOA and Multi-RTT with corresponding impacts on the measurement or the signaling for positioning.

## UE Rx TEG for DL-TDOA

In RAN1#104b meeting, the following agreements related to TEG for DL-TDOA were reached [1]:

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signaling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

For DL-TDOA, the UE can provide the Rx TEG associated with RSTD measurements to LMF for timing error mitigation. In Rel-16, the IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server. Similarly, the reported Rx TEG from UE to LMF can be included in IE NR-DL-TDOA-SignalMeasurementInformation as well.

In addition, the condition to have the association information is when the UE or TRP has multiple TEGs. In terms of saving signaling overhead, it is beneficial to indicate reference TEG value or index, and if there are multiple TEGs, the indication of the differences could be enough. In addition, the absent of the difference indication could imply that UE/TRP only has one TEG or it happens to have same TEG as the reference one.  The final format of the indication will also depend the feedback from RAN4.

Proposal 1: For DL-TDOA, the UE can report the association information of Rx TEGs associated with RSTD measurements to LMF (e.g., in the IE NR-DL-TDOA-SignalMeasurementInformation).

Proposal 2: For the indication of TEG in DL-TDOA method, one single reference TEG plus the TEG differences (in case of multiple different TEGs) can be considered.

## UE Tx TEG for UL-TDOA

In RAN1#105 meeting, the following agreements related to TEG for UL-TDOA were reached:

Agreement: - For mitigating UE Tx timing errors for UL TDOA, support one of the following options: - Option 1: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs. - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs - Option 2: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs. - Support the serving gNB to forward the association information provided by the UE to the LMF - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs - FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

For the issue whether having the indication of the association information known by gNB (e.g., first provided to gNB and forwarded to LMF), it is clearly dependent on whether the calculation of UL-TDOA is performed in gNB or LMF. If it is done at gNB (or TRP) side, it seems natural for gNB to know the information whereas it is not necessary when done at LMF side. Considering that the calculation done at LMF side is more general and beneficial, the option 1 is preferred.

Proposal 3: The association information of UL SRS resources for positioning with Tx TEGs is sent directly from UE to LMF.

## UE RxTx TEG for Multi-RTT

In RAN1#105e meeting, the following agreements related to TEG for Multi-RTT were reached:

Agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

For how to indicate the association in multi-RTT method, from our understanding, both two options can work. However,  they might be suitable for different use cases or UE capability, e.g, if the UE is capable of generating the overall TEG, known as the Rx-Tx TEG, such information could be provided; but if UE is unable to do it, it can generate the Tx TEG and Rx TEG separately.

Proposal 4: Both options for reporting the TEG (i.e., reporting the RxTx TEG ID or reporting both Rx TEG ID and Tx TEG ID) are supported for DL+UL positioning subject to UE capability.

Regarding the issue that which information the Tx TEG ID is associated with, we think Alt.1 is more suitable, because it provides the association between the TEG and a SRS resource for a Tx timing for one measurement. In our understanding, the TEG could vary due to some hardware specificitiesas well as the transmission configuration/settings, so that even with same resource or same timing, the TEG might be different. In order to accurately associate the TEG ID, both factors need to be considered.

Proposal 5: For the reporting of UE Tx TEG in DL+UL positioning, a Tx TEG ID is associated with  an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

# Conclusion

This contribution discusses the accuracy improvements on timing based positioning solutions. Observations and proposals are summarized as follows:

Proposal 1: For DL-TDOA, the UE can report the association information of Rx TEGs associated with RSTD measurements to LMF (e.g., in the IE NR-DL-TDOA-SignalMeasurementInformation).

Proposal 2: For the indication of TEG in DL-TDOA method, one single reference TEG plus the TEG differences (in case of multiple different TEGs) can be considered.

Proposal 3: The association information of UL SRS resources for positioning with Tx TEGs is sent directly from UE to LMF.

Proposal 4: Both options for reporting the TEG (i.e., reporting the RxTx TEG ID or reporting both Rx TEG ID and Tx TEG ID) are supported for DL+UL positioning subject to UE capability.

Proposal 5: For the reporting of UE Tx TEG in DL+UL positioning, a Tx TEG ID is associated with  an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

# References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.