3GPP TSG RAN WG1 #106-e             	                                       R1-2106898

e-Meeting, August 16th – 27th, 2021

Agenda Item:		8.7.1.1

Source:			Samsung

Title:			Discussion on paging enhancements

Document for:		Discussion and decision

# Introduction

In RAN1#105 e-meeting [1], the following were agreed:

Agreement:

For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

Conclusion:

To down-select one solution for PEI physical-layer channel/signal in RAN1 #106-e, using below as a starting point:

- PDCCH-based PEI

- SSS-based PEI

- TRS/CSI-RS-based PEI

Note: Additional details for each of the above 3 solutions are encouraged for more informed down-selection

Note: further refinement of the above list is possible, e.g., by merging/further splitting, depending on significance of the commonality and/or differences

Agreement:

Observation:

Dynamically sharing PDCCH resources of Rel-15 UEs (whether or not this is an important aspect to consider for PEI is FFS)

  - For PDCCH-based PEI,

    - PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates

      - Exact number of multiplexed/impacted Rel-15 PDCCH candidates depends on AL used for PDCCH-based PEI and relative size of PDCCH CORESET, etc.

  - For SSS-based PEI and for the case of partial overlap of CORESET and PEI

    - For interleaved CORESET (such as CORESET#0), SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity

    - For non-interleaved CORESET, SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates

      - Exact number of impacted Rel-15 PDCCH candidates depends on relative size and location of PDCCH CORESET, etc.

  - For TRS/CSI-RS-based PEI and for the case of partial overlap of CORESET and PEI

    - For interleaved CORESET (such as CORESET#0), TRS/CSI-RS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity

    - For non-interleaved CORESET, TRS/CSI-RS-based can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at candidate level granularity

      - Exact number of impacted Rel-15 PDCCH candidates depends on CSI-RS mapping pattern, relative size and location of PDCCH CORESET, etc.)

Agreement:

For paging indication to the subgroups in a PO,

- For PDCCH-based PEI, subgroups in a PO are indicated by one PEI

  - One bit in the DCI payload indicating one UE subgroup is supported

    - FFS: Whether code-point based mapping is utilized, and, if so, how to map to the subgroups in a PO

- For SSS-based PEI, subgroups in a PO are indicated by a set of sequence realizations

  - FFS: Sequence mapping design for supporting up to 8 subgroups per PO

  - Physical-layer configuration(s) and sequence generation design are subject to no impact to initial access and RRM measurements of legacy UEs

- For TRS/CSI-RS-based PEI, subgroups in a PO can be indicated by the following alternatives

  - Alt 1: One TRS sequence with orthogonal cover as PEI transmitted in the PEI monitoring occasion where one orthogonal cover of the PEI indicates one subgroup or combination of subgroups

    - FFS: Design details for the orthogonal cover

  - Alt S2: A set of TRS sequences indicating the subgroups with one selected sequence transmitting in one TRS resource

    - FFS: Sequence mapping design for supporting up to 8 subgroups per PO and combination of subgroups

  - Alt 3: Multiple TRS/CSI-RS resources FDMed/TDMed /CDMed in the same monitoring occasion where one TRS/CSI-RS resource indicates one subgroup

    - Reuse Rel-15/16 CSI-RS FDM/TDM/CDM patterns for supporting up to 8 subgroups per PO

- Note: It is RAN1 understanding that Physical-layer configuration(s) for paging early indication to the subgroups is subject to the same idle-mode reception bandwidth as CORESET-0 frequency span

Agreement

Observation:

For the comparison of PEI candidate designs, the following table summarizes average power saving gains based on companies’ contributions:

(Table skipped due to space limitation)

This contribution discusses two paging enhancement techniques and corresponding design aspects.

# 2. Paging Enhancement Technique #1: Paging Early Indication (PEI)

It was agreed in RAN1#103-e to support paging early indication (PEI) for UE power saving in idle/inactive mode. A PEI included in a L1 signal/channel can indicate UE whether or not to receive paging PDCCH in a PO. In this section, we discuss design aspects of PEI, including

- Physical layer signal/channel

- Configurations, and

- UE monitoring behaviors

## Physical layer signal/channel

Physical layer signal/channel is the most important design aspect for PEI. The following three candidates were proposed for down-selection in this meeting.

- PDCCH based solution

- SSS based solution

- CSI-RS/TRS based solution

For PDCCH based solution, PEI can be carried in the payload of a DCI format. A binary bit in DCI can indicate whether or not to monitor a PO. A DCI payload can be used to provide a large size of PEI to one or more UE sub-groups. However, channel resources will be wasted if the PEI is less than 12 bits since the minimum DCI payload size should be at least 12 bits to due to the requirement of Polar Coding for PDCCH decoding.

Both SSS based and CSI-RS/TRS based PEI rely on detection of a RS. A RS of PEI can be transmitted by NW on demand to indicate the existence or absence of paging PDCCH in a PO. If the RS based PEI indicates the existence of PO, a UE wakes up for paging PDCCH reception in the PO when the UE detects corresponding RS.

For SSS based solution, a RS can be generated from two M-sequences. The RS has a fixed length and is mapped into 127 continuous REs in the frequency domain. To avoid impact to legacy synchronization performance, SSS based PEI should consider some modification on sequence generation, for example consider a cycle shift of the M-sequences.

For CSI-RS/TRS based solution, a RS is generated from a ZC sequence, where the RS has a flexible length according to a number of occupied RBs configured by higher layer. Any existing CSI-RS/TRS, including new CSI-RS/TRS introduced in Rel-17 can be considered as potential L1 signal design for PEI. A CSI-RS/TRS can be mapped into non-continuous REs across the allocated RBs in the frequency domain and 1/2/4 OFDM symbols in the time domain.  The RE mapping for CSI-RS/TRS based PEI can be determined based on a density in terms of a number of D REs per RB, and a number of N occupied OFDM symbols.

In previous three meetings, RAN1 spent great efforts to evaluate performance of the three candidates, including power saving gain, resource overhead, and coexistence with legacy UEs. The down-selection of PEI physical layer signal/channel should be determined based on performance in order to select the best solution for highest power saving gain, low resource overhead, and no impact to legacy UEs.

Proposal 1: To down-select PEI physical layer signal/channel for best performance, including

- highest power saving gain,

- low resource overhead, and

- no impact to legacy Rel-15/16 UEs.

Considering limited time left in Rel-17, specification effort is another concern for down-selection of PEI candidates by many companies. However, it’s not clear which candidate is best in terms of low specification efforts. It depends on many details, which haven’t been discussed much yet. For PDCCH-based solution, specification efforts are expected at least for a new DCI format, determination of CSS set/CORESET/PDCCH candidates for PDCCH monitoring, which are non-trivial in idle mode. For SSS based PEI, the main efforts would be i) sequence generation to avoid impact to initial access and RRM measurement, and ii) sequence mapping for UE subgroup indication if to support. For CSI-RS/TRS based PEI, NR already supports flexible configuration and sequence generation of CSI-RS resources. The main specification efforts could be UE subgroup indication if to support. Three alternatives for UE subgroup indication were identified in RAN1-105-e meeting for CSI-RS/TRS based PEI. Some of them are simple extension of existing method supported in Rel-15. For example, to reuse Rel-15/16 CSI-RS FDM/TDM/CDM patterns for supporting up to 8 subgroups per PO.

Observation 1: Potential specification efforts needed for the three PEI candidates are:

- PDCCH based PEI: a new DCI format, new CSS set, and CORESET/PDCCH candidate determination

- SSS-based PEI: sequence generation to avoid impact to SSB detection, and sequence mapping for UE subgroup indication

- CSI-RS/TRS based solution: method for UE subgroup indication if not reusing Rel-15/16 CSI-RS FDM/TDM/CDM patterns.

To achieve the basic feature of PEI, such as PEI indication without UE subgroup indication, CSI-RS/TRS based PEI has the lowest specification effort.

Observation 2: CSI-RS/TRS based PEI requires lowest specification efforts to achieve basic functionality of PEI.

## Performance comparison of PEI candidates

### 2.2.1 Power saving gain (PSG)

PSG should be the most important target for down-selecting the best PEI candidate. Although same UE power model for PEI reception is assumed regardless of the physical layer signal/channel design, PSG of PEI varies, depending on UE power consumption in many other aspects. According to the simulation results collected in RAN1#105e meeting [1], the three PEI candidates result in different UE power consumption in the following aspects

- A1: a number of SS burst(s) before PEI

- A1: additional number of SS burst(s) between PEI and PO when UE is paged

- A3: UE subgroup indication

|  | PDCCH based PEI | RS based (SSS, TRS/CSI-RS) PEI |
| --- | --- | --- |
| A1 | one or more SS burst(s) for - serving cell RRM measurement, and - coarse/relaxed synchronization for PDCCH reception | one SS burst(s) for - serving cell RRM measurement only |
| A2 | additional SS burst(s) for fine synchronization in order to receive paging PDSCH | No need |
| A3 | No impact to UE power consumption on PEI reception | No impact to UE power consumption on PEI reception based on UE subgroup indication alternatives agreed in last meeting |

Table 1: Summary of UE power consumption difference for the three PEI candidates

Regarding the first aspect, A1, idle mode UE needs to process a number of SS bursts for serving cell RRM measurement and synchronization after waking up from long DRX OFF period. Since RS based PEI is not always-on, but transmitted on-demand by gNB. UE power consumption regarding serving cell RRM measurement is assumed to be same for all PEI candidates regardless of the physical layer signal/channel design. However, UE power consumption in terms of SS burst based synchronization for PEI reception varies depending on the physical layer signal/channel of PEI. For PDCCH based PEI, the number of SS burst(s) needed depending on real-time channel condition. UE may be able to relax synchronization for PDCCH reception compared with PDSCH reception, but UE still needs process at least one SS burst(s) before PEI to be prepared for PDCCH decoding. For RS based PEI, UE can perform PEI detection and synchronization in terms of time/frequency offset estimation simultaneously within the monitoring occasion of PEI since both synchronization and PEI detection are based on correlation between received data and pre-known sequence. To perform synchronization together with PEI detection, UE just needs to conduct multiple trails of correction assuming different time/frequency offsets. When the maximum correction exceeds a predetermined threshold, the UE can claim detection of PEI and the corresponding time/frequency offsets are synchronization results, which can be used for paging PDCCH or PDSCH reception in next PO. In our old contribution [3], we evaluate the synchronization performance for CSI-RS/TRS based PEI. We observed that TRS can achieve residual error to be less than 0.1ppm for SNR >-6dB, while more than 2 SS bursts are needed to achieve 0.1 ppm residual error at SNR of -4dB. CSI-RS resources based synchronization can achieve better performance than SSB because there are more symbols of RS resources in the time domain. So, UE can do correlation in both time domain and frequency domain for time/frequency offset estimation.

Observation 3: RS based PEI can be used for synchronization in idle/inactive mode.

For the second aspect, A2, UE may need to process additional number of SS bursts between PEI and PO for fine synchronization in order to receive paging PDSCH when the UE is paged. Since PDCCH has lower synchronization requirement than PDSCH, the majority assume UE can relax synchronization processing before PDCCH based PEI reception, and the UE needs to process additional number of SS burst between PEI and PO when the UE is paged. For RS based PEI, there is no need to assume a big gap between PEI monitoring occasions and PO. The sequence of PEI can be used directly for coarse and fine synchronization when UE is paged. So, there is no need for additional SS bursts between PEI and PO for synchronization when support RS based PEI.

Observation 4: RS based PEI achieves higher power saving gain than PDCCH based PEI, due to reduced synchronization overhead on SS bursts reception before and after PEI monitoring occasions.

For the third aspect, A3, some companies reported PSG change with respect to the size of UE subgroup indication before we agreed on the potential methods for UE subgroup indication. For PDCCH-based PEI, it’s straightforward that there is no impact on UE power consumption as the UE subgroup indication is indicated by DCI with payload no less than 12 bits. For RS based PEI, whether or not UE power consumption increases depends on detailed method of UE subgroup indication. For SSS-based PEI, only one method was agreed in last meeting [1], such that sequence mapping design for supporting up to 8 subgroups per PO. For TRS/CSI-RS based PEI, there are three alternatives agreed as potential methods for UE subgroup indication. For each of the three alternative, a UE only needs to monitor the predetermined sequence or resource allocation pattern. The UE power consumption on PEI reception remains same regardless of UE subgroup indication size for any RS based PEI, too.

Observation 5: UE subgroup indication has no impact to UE power consumption on PEI reception regardless of physical layer signal/channel design of the PEI.

### 2.2.1 Resource overhead

In RAN1-104bis-e meeting, companies’ evaluations on resource overhead to achieve target reliability for different PEI designs were captured. According to the observation [2], the resource overhead for different PEI designs can be summarized in Table 2 and 3 below.

Table 2: Average resource overhead per PO (REs) for different PEI designs for Behav-A

| Alternatives | Dynamic rate-matching | Semi-static rate matching |
| --- | --- | --- |
| PDCCH-based | 28.8 -57.6 | 288.0 – 576.0 |
| SSS-based | 25.4 - 34.0 | 254.0 - 437.0 |
| TRS/CSI-RS based | 14.4 – 30.0 | 168.0 - 300.0 |

Table 3: Average resource overhead per PO (REs) for different PEI designs for Behav-B

| Alternatives | Dynamic rate-matching | Semi-static rate matching |
| --- | --- | --- |
| PDCCH-based | 518.4 (always transmitted) |  |
| SSS-based | 228.6 | 254.0 - 561.0 |
| TRS/CSI-RS based | 259.2 | 168.0 - 279.0 |

According to Table 2 and Table 3, we can conclude that the performance regarding the resource overhead, such that the TRS/CSI-RS based PEI requires lowest resource overhead, PDCCH-based PEI requires highest resource overhead, and SSS-based PEI is in the middle.

Observation 6: PDCCH based PEI requires much larger (~2X larger) resource overhead than RS-based PEI, especially TRS/CSI-RS based PEI.

In addition to required number of REs per PEI monitoring occasion, resource overhead of PEI is also determined by number of PEI reception occasions for multi-beam operation in idle mode.

For PDCCH-based PEI, gNB has to configure N >1 reception occasions, where each reception occasion is QCLed with a SSB. Similar as paging PDCCH, the number of PEI reception occasions should equal to the number of M transmitted SSBs per burst, such that N = M. For RS-based PEI, gNB has a high flexibility in terms of multi-beam transmission, given the fact that the RS based PEI can be received based on non-coherent detection method without using SSB as QCL RS. Therefore, it’s not necessary to configure 1:1 mapping between RS based PEI reception occasions and SSB reception occasions. gNB has the flexibility to configure a smaller number of reception occasions for RS based PEI to reduce resource overhead by using a wider beam for RS-based PEI.

Observation 7: For multi-beam operation in idle mode, gNB has flexibility to configure less number of reception occasions for RS based PEI to reduce resource overhead, due to non-coherent detection characteristics of RS based PEI.

### 2.2.3 Coexistence with existing signal/channel

The last aspect evaluated in RAN1 to compare performance for the three PEI candidates is coexistence with existing signal/channels. In the last two meetings, observations regarding coexistence with legacy PDCCH and PDSCH were captured. The observation can be summarized in Table 4.

Table 4: Summary of coexistence with legacy PDCCH/PDSCH

| Existing channels |  | PDCCH-based PEI | SSS-based PEI | TRS/CSI-RS based PEI |
| --- | --- | --- | --- | --- |
| PDCCH | Non-interleaved | at granularity of one or more candidates | at granularity of one or more candidates |  |
|  | Interleaved |  | at CORESET-level granularity |  |
| PDSCH | Semi-static RM | RB-symbol level | RB-symbol level | RE-level rate-matching |
|  | Dynamic RM indicated by DCI format 1_1 | CORESET-level, mandatory capability | CORESET-level, mandatory capability | RE-level, mandatory capability |

According to Table 4, there is no issue for any of the three PEI candidates to coexist with legacy PDCCH or PDSCH. For coexistence with PDCCH, all PEI candidates support dynamic resource sharing thanks to the nature of UE blind decoding of PDCCH. PDCCH based PEI is slightly better than RS based solution in terms of finer granularity in PDCCH candidate level. For coexistence with PDSCH, rate matching (RM) methods in Rel-15/16 can be reused. Since PDSCH is anyways scheduled with dynamic resources, gNB can avoid symbols/RBs occupied by PEI for PDSCH and support the multiplexing even without RM on PDSCH.

To sum up, no new gNB handling is needed to support coexistence of PEI with legacy PDCCH or PDSCH. However, coexistence with PDCCH will have impact to PDCCH blocking rate for Rel-15/16 UEs, especially in CORESET#0 for IDLE/INACTIVE UEs. Therefore, coexistence of PEI with PDSCH should be prioritized over coexistence with PDCCH to avoid impact on Rel-15/16 UEs.

Observation 8: No new gNB handling is expected to support coexistence of PEI with legacy PDCCH or PDSCH.

Observation 9: Coexistence with PDSCH is better than coexistence with PDCCH to avoid PDCCH blocking for Rel-15/16 UEs.

According to the above analysis and performance comparison, it’s clear that RS based solution outperforms PDCCH based solution to achieve the basic functionality of paging early indication. PDCCH-based solution may have the benefit of carrying larger size of information, which can be used for other functionality, such as availability indication of TRS/CSI-RS occasions for idle/inactive UEs. However, the design should be focus on supporting the basic functionality of paging early indication.

Proposal 2: Support RS based PEI for better performance than PDCCH based PEI, including

- higher power saving gain,

- lower resource overhead, and

- no impact to Rel-15/16 UEs, especially on PDCCH blocking rate.

## Additional Details

### 2.3.1 Configuration

Configuration of PEI should be provided to UEs based on higher layer signaling, for example SIB. The configuration can include information to indicate PEI monitoring occasions relative to associated PO and time/frequency domain resources allocation for each PEI monitoring occasion.

As illustrated in Figure 1, start of PEI monitoring occasions (MOs) can be determined based on a, a time offset, O, relative to the start of an associated PO. The time offset, O, can be small as illustrated in Figure 1 (a), where PEI MOs are allocated right before an associated PO. In this case, the UE only needs to wake up one time to process PEI and PO sequentially. UE can finish synchronization before PEI or during PEI MOs since there is no additional RS resources available before PO or paging PDSCH. Alternatively, a large time offset, O, can also be considered to allow additional SS bursts between PEI and PO as illustrated in Figure 1 (b). A UE may relax synchronization based on SS bursts before PEI reception, and process additional SS bursts for synchronization after PEI reception if the UE is indicated to wake up for paging reception. However, large time offset can increase UE power consumption as the UE has to wake up multiple times for PEI reception and paging reception. So, a small time offset is better and sufficient for RS based PEI.

- PEI allocated right before PO w/ small time offset, O

- PEI w/ large time offset, O, and additional SSBs between PEI and associated PO

Figure 1: Illustration of PEI MOs and UE processing timeline in idle/inactive mode

.

Same as legacy signal/channels transmitted in the idle/inactive mode, gNB can configure a number of PEI monitoring occasions for multi-beam operation, where each PEI monitoring occasion can have a predetermined QCL information. Figure 2 illustrates an example of multi-beam transmissions of SS burst, PEI and PO in idle/inactive mode. The number of PEI reception occasions can be either predetermined or configured by higher layer. For RS based PEI, since the RS resource can be detected without knowledge of QCL RS, gNB can configure the number of PEI reception occasions as needed to balance resource overhead and UE power saving gain.

Figure 2: Illustration of multi-beam transmission of PEI

Proposal 3: Support SIB based configuration of PEI, including

- a time offset, O, relative to start of an associated PO, to indicate start of PEI monitoring, and

- a number of PEI monitoring occasions for multi-beam operation.

For the time/frequency resource allocation of PEI, Rel-15 configuration of physical layer signal/channel can be reused to reduce specification efforts. For RS based PEI, for example CSI-RS/TRS based PEI, gNB can reuse Rel-15 configuration of NZP-CSI-RS-Resource. The configuration of PEI can include a number of RS resources, where each of the RS resources has a QCL information. For PDCCH based PEI, a number of PDCCH MOs can be configured based on a configuration of a search space set for PEI reception.

Proposal 4: Further study reusing Rel-15 configuration of physical layer signal/channel for determining time/frequency resource allocation of PEI.

According to our analysis in Section 2.2, we support RS-based PEI due to the better performance in all three evaluation aspects. However, PDCCH based PEI is preferred by many companies mainly for the capability of carrying large size of power saving information. If we can reuse Rel-15 configuration of physical layer signal/channel without new physical layer signal/channel design, it’s possible to merge multiple PEI candidates to a common solution without increasing much specification efforts. For a merged solution, the configuration regarding PEI MOs relative to associated PO can be common to all PEI candidates. gNB has the flexibility to select a PEI type according to the requirement of the use case in reality. In most cases, a RS based PEI without UE subgroup indication is sufficient to achieve large power saving gain with low resource overhead. A PDCCH based PEI can be considered for a few use cases, for example, to provide UE subgroup indication when UE subgroup paging rate is very high. In this sense, it’s worth striving for a common design to get benefits of all PEI candidates. Due to the limited timeline, the specification efforts can be restricted by avoiding new physical layer signal/channel design. For example, to deprioritize new sequence design for UE subgroup indication for RS based PEI.

Proposal 5: Merge RS-based PEI and PDCCH based PEI to a unified solution if necessary, considering

- common configuration of PEI monitoring occasions. and

- no new physical layer signal/channel design.

### 2.3.2 UE behavior for PEI reception

The following two types of UE behaviors are considered during the evaluations of PEI performance.

- Behav-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behav-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

According to the observation captured in RAN1#104bis-e meeting [2], Behav-B requires much larger (approximately 9x larger) resource overhead than Behav-A for baseline (w/ group paging rate =10%). Therefore, Behav-A should be supported for the benefit of low resource overhead. For Behav-B, it may provide some flexibility for gNB to balance power saving gain and resource overhead when group paging rate is high. For example, gNB can skip PEI transmission, and UE wakes up for PO by default. However, the group paging rate is expected to be low, e.g. 10%, in idle/inactive mode for most of the use cases. When the group paging rate is high, it’s likely gNB won’t configure PEI as there is no much room to achieve UE power saving.

Observation 10: Behav-B requires much larger (9x larger @ group paging rate = 10%) resource overhead than Behav-A.

Proposal 6: Support only Behav-A for PEI for the benefit of low resource overhead.

# 3. Paging Enhancement Technique #2: UE-subgrouping for paging

In RAN1-105e meeting [1], it was agreed to support maximum of 8 subgroups per PO. According to the agreement in RAN1#104-e, UE subgroup indication can be further studied over PEI and/or paging PDCCH.

For PEI based UE subgroup indication, PEI is likely to be configured for power saving when the group paging rate is small, for example 10%. According to evaluation results collected in RAN1-105e meeting [1], we observe that the PSG from UE subgroup indication in addition to PEI is very limited, i.e. <2% for baseline with paging group rate of 10%.

Observation 11: UE subgroup indication achieves limited PSG (e.g. < 2%) for most use cases when paging group rate is not high, such as the baseline of 10%.

Proposal 7: Deprioritize UE subgroup indication in PEI due to the limited power saving gain and increased resource overhead.

For a few use cases when group paging rate is high, for example RedCap use case with large connectivity, the resource overhead for transmitting PEI increases accordingly. gNB may not want to configure or transmit PEI due to increased resource overhead in practice. In this case, the UE subgrouping indication based on paging PDCCH only is beneficial to achieve power saving gain without increasing resource overhead. Currently, there are 6 reserved bits in short message and additional 8 reserved bits DCI format 0_1 with CRC scrambled by P-RNTI. So, it’s straightforward to consider legacy paging PDCCH for UE subgroup indication without increase of resource overhead and impact to legacy signal/channel.

Observation 12: There are 14 reserved bits in legacy paging PDCCH that can be used for UE subgroups per PO without additional resource overhead.

Proposal 8: Support paging PDCCH for UE subgrouping indication for the benefit of no additional resource overhead.

# 5. Conclusion

This contribution considered techniques of paging enhancement for power saving in idle/inactive mode. Following observations and proposals are made:

Observation 1: Potential specification efforts needed for the three PEI candidates are:

- PDCCH based PEI: a new DCI format, new CSS set, and CORESET/PDCCH candidate determination

- SSS-based PEI: sequence generation to avoid impact to SSB detection, and sequence mapping for UE subgroup indication

- CSI-RS/TRS based solution: method for UE subgroup indication if not reusing Rel-15/16 CSI-RS FDM/TDM/CDM patterns.

Observation 2: CSI-RS/TRS based PEI requires lowest specification efforts to achieve basic functionality of PEI.

Observation 3: RS based PEI can be used for synchronization in idle/inactive mode.

Observation 4: RS based PEI achieves higher power saving gain than PDCCH based PEI, due to reduced synchronization overhead on SS bursts reception before and after PEI monitoring occasions.

Observation 5: UE subgroup indication has no impact to UE power consumption on PEI reception regardless of physical layer signal/channel design of the PEI.

Observation 6: PDCCH based PEI requires much larger (~2X larger) resource overhead than RS-based PEI, especially TRS/CSI-RS based PEI.

Observation 7: For multi-beam operation in idle mode, gNB has flexibility to configure less number of reception occasions for RS based PEI to reduce resource overhead, due to non-coherent detection characteristics of RS based PEI.

Observation 8: No new gNB handling is expected to support coexistence of PEI with legacy PDCCH or PDSCH.

Observation 9: Coexistence with PDSCH is better than coexistence with PDCCH to avoid PDCCH blocking for Rel-15/16 UEs.

Observation 10: Behav-B requires much larger (9x larger @ group paging rate = 10%) resource overhead than Behav-A.

Observation 11: UE subgroup indication achieves limited PSG (e.g. < 2%) for most use cases when paging group rate is not high, such as the baseline of 10%.

Observation 12: There are 14 reserved bits in legacy paging PDCCH that can be used for UE subgroups per PO without additional resource overhead.

Proposal 1: To down-select PEI physical layer signal/channel for best performance, including

- highest power saving gain,

- low resource overhead, and

- no impact to legacy Rel-15/16 UEs.

Proposal 2: Support RS based PEI for better performance than PDCCH based PEI, including

- higher power saving gain,

- lower resource overhead, and

- no impact to Rel-15/16 UEs, especially on PDCCH blocking rate.

Proposal 3: Support SIB based configuration of PEI, including

- a time offset, O, relative to start of an associated PO, to indicate start of PEI monitoring, and

- a number of PEI monitoring occasions for multi-beam operation.

Proposal 4: Further study reusing Rel-15 configuration of physical layer signal/channel for determining time/frequency resource allocation of PEI.

Proposal 5: Merge RS-based PEI and PDCCH based PEI to a unified solution if necessary, considering

- common configuration of PEI monitoring occasions. and

- no new physical layer signal/channel design.

Proposal 6: Support only Behav-A for PEI for the benefit of low resource overhead.

Proposal 7: Deprioritize UE subgroup indication in PEI due to the limited power saving gain and increased resource overhead.

Proposal 8: Support paging PDCCH for UE subgrouping indication for the benefit of no additional resource overhead.

# 6. Reference

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #105-e

[2] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #104bis-e

[3] R1-2008175, “Discussion on paging enhancements”, Samsung