3GPP TSG RAN WG1#106-e								R1-2106917

e-Meeting, August 16th– 27th, 2021

Agenda item:	8.14.1

Source: 	Samsung

Title: 					Traffic Models for XR

Document for:	Discussion and Decision

# Introduction

Substantial progress was made in RAN1#105-e towards finalizing the XR traffic modeling assumptions. This contribution considers a few remaining open issues.

# XR Traffic Models

In RAN1#104b-e, single stream modeling per UE was agreed as baseline for DL evaluations. 3 options for dual-stream models were agreed as optional: (1) I-frame and P-frame, (2) video and audio/data, (3) FOV+ omni.

In RAN1#105-e, more details of modeling assumptions for I-frame and P-frame were agreed. The following agreement was made with all parameter values for , A-H left as FFS.

Agreement

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of , A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream |  |  |  |  |  |  |  |
|  | - R: average data rate of a single stream video - : average size ratio between one I-frame/slice and one P-frame/slice, e.g. = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = | Mean = |  |  | Mean = | Mean = |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

In our view, simplicity of modelling assumptions is the most important consideration. We do not think there is much usefulness associated with too many small refinements of XR traffic models, when considering that several factors impacting the XR QoE for the user cannot be modelled with the RAN1 statistical traffic models. For example, XR applications will implement rate adaptation and feedback at application layer. Code rates and modes will change and be adapted over the lifetime of an XR connection. Therefore, there is no “one-size-fits-all” statistical distribution. A certain correlation may be expected when considering packet sizes and their distributions.

Therefore, A-H should simply re-use the same values that are also used for DL video for the case of no I- and P-frame differentiation. For the ratio between I-frames and P-frames, we prefer a conservative value of , e.g. 2. In practice, it can be expected that the ratio has a large range of variation, because it greatly depends on the encoded content and scene variation.

Proposal 1

For AR/VR and CG: A=B=E=F=1%

For AR/VR: C=D=G=H=10ms and for CG: C=D=G=H=15ms

= 2

# Conclusions

This contribution considered remaining issues on XR traffic models and proposes the following.

Proposal 1

For AR/VR and CG: A=B=E=F=1%

For AR/VR: C=D=G=H=10ms and for CG: C=D=G=H=15ms

= 2

# References

- R1-2105342 “Traffic Models for XR”; Samsung

- R1-2105343 “Evaluation methodology and KPIs for XR”; Samsung

- R1-2105344 “Potential enhancements for better XR support in NR”; Samsung