3GPP TSG RAN WG1 Meeting #106-e		                                                      R1-2106928

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on cancellation of semi-static transmission due to dynamic transmission

Agenda Item:	7.1

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the cancellation of a semi-static transmission due to overlapping with a dynamic transmission on at least one flexible symbol.

Discussion

In TS 38.213 [1], a semi-static transmission cancelled by dynamic transmissions is defined in clause 11.1 as highlighted in yellow, while dropping of a dynamic PDSCH/PUSCH repetition due to collision with semi-static UL/DL configurations is defined in clause 11.1 as highlighted in green or due to collision with SSB/valid PRACH occasion is defined in clause 11.1 as highlighted in blue.

11.1 Slot configuration […] For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; - otherwise, the UE cancels the PUCCH, or PUSCH, or PRACH transmission in the set of symbols. - the UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0. For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. For a set of symbols of a slot corresponding to a valid PRACH occasion and symbols before the valid PRACH occasion, as described in Sublcause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. If a UE is scheduled by a DCI format 1_1 to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot. If a UE is scheduled by a DCI format 0_1 to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

For an overlapping case of a DL/UL semi-static transmission and an UL/DL dynamic transmission which collides with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion, it is not clear enough that before performing the behaviour as defined in the yellow highlight part to cancel the semi-static transmission, whether the dropping of a dynamic transmission due to collision with semi-static DL/UL symbol(s) as defined in the green highlight part or SSB/valid PRACH occasion as defined in the blue highlight part would be performed first. Different operation orders may result in different transmission results. Then there may be the following two understandings:

- Understanding 1: both the dynamic transmission colliding with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion and the semi-static transmission overlapping with the dynamic transmission are not transmitted / received.

- Understanding 2: semi-static transmission could be transmitted / received while dynamic transmission colliding with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion is dropped.

Taking a collision of semi-static UL transmission and a dynamic DL transmission shown in Figure 1 as an example, CG PUSCH overlaps with PDSCH repetition 2 scheduled by a DL grant. With understanding 1, UE does not transmit CG PUSCH and not receive the 2nd repetition of the dynamic PDSCH. With understanding 2, UE can transmit CG PUSCH since the 2nd repetition of the dynamic PDSCH is dropped.

Figure 1: collision of CG PUSCH and dynamic PDSCH

For a collision of semi-static DL transmission and a dynamic UL transmission as shown in Figure 2, there is similar issue.

Figure 2: collision of SPS PDSCH and dynamic PUSCH

There may be misunderstanding between UE and gNB when UE and gNB have different understandings. If it is up to UE implementation to follow understanding 1 or understanding 2, gNB needs to always detect semi-static UL transmission even when UE does not transmit anything based on understanding 1 and needs to always send semi-static DL transmission even when UE does not expect to receive anything based on understanding 1. To avoid the misunderstanding between UE and gNB and to simplify the gNB implementation, it is desirable to clarify the intended UE behaviour.

Based on the above examples, understanding 1 is not good for system efficiency with at least the following reasons:

- Semi-static resource is wasted even though there is no real collision with dynamic transmissions

- Dropping probability of UCI is increased especially for the case of SPS HARQ-ACK on PUCCH and dynamic HARQ-ACK on CG PUSCH if any

- Dropping probability of a CG PDU already delivered to PHY is increased

- PDCCH is blocked due to cancellation by dynamic UL transmission colliding with semi-static DL symbol(s) or SSB symbol(s).

Based on understanding 2, both gNB and UE have the same understanding on whether the dynamic transmission is dropped or not before performing the cancellation, hence behaviour to allow semi-static transmission is clear and aligned between both gNB and UE. Hence, we prefer to adopt understanding 2 with a corresponding CR as shown in the Appendix.

Proposal: For an overlapping case of a DL/UL semi-static transmission and an UL/DL dynamic transmission which collides with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion, it is clarified that the semi-static transmission is not cancelled by the dynamic transmission.

Conclusion

In this contribution, we discussed the cancellation of a semi-static transmission due to overlapping with a dynamic transmission on at least one flexible symbol with the following proposal.

Proposal: For an overlapping case of a DL/UL semi-static transmission and an UL/DL dynamic transmission which collides with semi-static DL/UL symbol(s) or SSB/valid PRACH occasion, it is clarified that the semi-static transmission is not cancelled by the dynamic transmission.

References

- TS 38.213 V15.14.0

Appendix

The following TP could be considered to capture the above proposal.

11.1 Slot configuration < Unchanged part is omitted > For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot or the PUSCH is scheduled to be transmitted in at least one symbol, indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or indicated for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon in the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols except that the PDSCH is scheduled to be received in at least one symbol, indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion as described in clause 8.1 in the slot, then - the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; - otherwise, the UE cancels the PUCCH, or PUSCH, or PRACH transmission in the set of symbols. - the UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0. < Unchanged part is omitted >