3GPP TSG RAN WG1 #106-e		                                                          R1-2106946

e-Meeting, Aug.16th – 27th, 2021

Source:	CATT,CBN

Title:	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #105-e meeting [1], the main issues on MBS transmission reliability improvement were discussed and the following agreements were reached:

Agreement: - The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook - FFS: The case for the same priority. - FFS: The case of Type-1 HARQ codebook - FFS: Whether this applies to separate PUCCH transmissions only - Agreement: - Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast. Agreement: Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling. - FFS for SPS activation/deactivation. Agreement: The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback. Agreement: NR supports at least the following cases for UE supporting multicast: - UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability. - UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability. Agreement: For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support - Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP). - FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4). - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed. Conclusion: PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback. Agreement: - For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI. Agreement: For support of ACK/NACK based HARQ-ACK feedback for SPS multicast, - the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast. - UE determines a priority index from the HARQ-ACK codebook index - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations. Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows: - Alt 1: - for slot timing values in the intersection of set for unicast (termed set A) and set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values in the union of set for unicast and set for multicast, based on the union of the PDSCH TDRA sets. - Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting. Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH.

In this document, the main issues on improving reliability of  Multicast/Broadcast/ service (MBS) are discussed, such as  NACK-only based HARQ-ACK feedback scheme, Type-1/Type-2 HARQ-ACK codebook, enabling/disabling HARQ-ACK feedback.

- Discussion

  - HARQ-ACK feedback in MBS

    - Enabling/disabling HARQ-ACK feedback

In RAN1#105-e meeting, a working assumption for enabling/disabling HARQ-ACK feedback was reached with some FFSs.

For DCI indication of whether ACK/NACK based HARQ-ACK feedback is enabled or disabled, the following alternatives can be considered:

- Alt 1: A new DCI field

For Alt 1, 1 extra bit can be added to indicate the HARQ-ACK feedback enabling/disabling. It is straightforward and simple, but it needs 1 extra bit in DCI which increases the DCI payload size. Since the RRC signaling is UE-specific, different UEs in a MBS group may have different RRC states of HARQ-ACK enabling/disabling. Since the group-common DCI targets a group UEs, whether this new field is needed in DCI for UEs which are configured by State 1 or State 2 should be further studied. This will bring extra work and high complexity on protocol design. Therefore, Alt 1 is not supported.

- Alt 2: NRU-like mechanism

In current NRU operation, the special k1 value (k1= -1) is used to indicate to the UE that the HARQ-ACK feedback for the corresponding PDSCH is postponed.

There are some limitations for k1 configuration if this mechanism is reused for disabling HARQ-ACK feedback. Firstly, instead of using the default k1 value, gNB will configure a new k1 value for each UE. Secondly, when the enabling/disabling HARQ-ACK feedback state changes (e.g. from enabling to disabling state), Alt 2 needs to re-configured the k1 set. Thirdly, if the NRU-like is further considered for MBS, with this extra HARQ-ACK disabling function introducing, UEs in NRU cannot know what the k1 (i.e. -1) exactly mean is. In other words, the k1=1 may indicate both postponing PDSCH function and the enabling/disabling function for UEs in NRU. Thus, this method is not suitable for disabling HARQ-ACK feedback.

- Alt 3: A special state of an existing DCI field

gNB can pre-configure or pre-define a special state of an existing field to indicate the HARQ-ACK feedback enabling/disabling. Since the existing field e.g. PDSCH-to-HARQ feedback timing (k1) field and PUCCH resource indicator (PRI) field will be used to indicate the valid timing for PUCCH and the PUCCH resource when implementing the HARQ-ACK feedback, so a special state of the k1 field or PRI field can be used to indicate the HARQ-ACK feedback enabling/disabling. For example, in DCI format 1_0, k1 uses 3bit which corresponds 8 states to indicate the PDSCH-to-HARQ feedback timing. gNB can pre-configure or pre-define a special k1 index (e.g. ‘000’) as the disabling indication. When receiving ‘000’ of k1 index, UE will not feedback the HARQ-ACK information; otherwise, UE will report the HARQ-ACK feedback information based on the rules in R15. The shortage of this method is that once a special k1 index is used to indicate the HARQ-ACK feedback disabling, it can’t be used to indicate the valid timing for PUCCH anymore. Thus, the flexibility of the PDSCH-to-HARQ feedback timing indication is reduced. So is PRI field.

Comparing with Alt 1, for different UEs in a MBS group, Alt 3 will not increase the DCI payload size and guarantee different UEs in a MBS group have same DCI payload size. Moreover, comparing with Alt 2, when the HARQ-ACK feedback state changes (e.g. from enabling to disabling state), Alt 3 do not need to re-configured the k1 set. Thus, Alt 3 is supported.

- Alt 4：Implicit indication (e.g. indicating PUCCH downlink symbols)

When UE receives a group-common scheduling, k1 field can indicate the slot number of PUCCH which carrying the HARQ-ACK information. For unpaired spectrum, slots may be configured as uplink or downlink slots/symbols. If the PUCCH resource indicated by k1 includes one or more downlink slots/symbols, UE can regard as the HARQ-ACK feedback is disabling.

Proposal 1: For DCI indicating the HARQ ACK enabling/disabling, Alt 3 (A special state of an existing DCI field) and Alt 4(Implicit indication) can be supported.

It has been agreed that the NACK-only based HARQ-ACK feedback is supported for MBS transmission. The NACK-only based HARQ-ACK feedback can also be disabled for power saving and improving HARQ-ACK feedback efficiency. For example, when gNB cannot allocate the orthogonal PUCCH resources for carrying NAK-only information, the NACK-only based HARQ-ACK feedback can be disabled. The enabling/disabling mechanism of ACK/NACK-based HARQ-ACK feedback as discussed above can be reused for NACK-only based HARQ-ACK feedback.

Proposal 2: The enabling/disabling ACK/NACK-based HARQ-ACK feedback mechanism can be used for NACK-only based HARQ-ACK feedback.

It has been agreed that the PTM scheme 1 and PTP transmission can be the retransmission scheme for MBS. When the PTM scheme 1 is used for retransmission, two options as discussed above can also be applied for retransmission scheme. In other words, the indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling as well. For example, when the State 2 (ACK/NACK disabling) is configured for retransmission, the HARQ-ACK feedback for retransmission is disabled. When PTP transmission is used for retransmission, it implies that the initial transmission is enabling. In our understanding, the RRC configuration may be changed for retransmission. Even when the initial transmission is enabling, it is not reasonable to always configure the PTP retransmission as enabling, which is same as the unicast services. Moreover, UE can identify whether the PTP transmission is for unicast or PTP retransmission for multicast, as discussed in agenda 8.12.1. Thus, for the PTP retransmission, two options as discussed above can also be applied. The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling as well.

Proposal 3: The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling same as initial transmission.

According to the working assumption, three states of the RRC configurations are supported for enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH.

- State 1: ACK/NACK enabling

If enabling ACK/NACK based HARQ-ACK feedback is configured by RRC, UE will always feedback HARQ-ACK information for multicast via dynamic group-common PDSCH.

- State 2: ACK/NACK disabling

If disabling ACK/NACK based HARQ-ACK feedback is configured by RRC, UE will not feedback any HARQ-ACK information for multicast via dynamic group-common PDSCH.

- State 3: enabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback ACK/NACK

After receiving this configuration, ACK/NACK based HARQ-ACK feedback is enabled/disabled as indicated in group-common DCI.

As discussed above, different UEs may be configured different states due to the RRC signaling is UE-specific. For example, in a MBS group, some UE may be configured with State 1, some UE may be configured with State 2 and the rest of the UEs are configured with State 3. However, the State 1 and State 3 cannot be configured simultaneously when Type-2 HARQ-ACK codebook is configured. This is because when the Type-2 HARQ-ACK codebook is configured, the UE with State 1 and the UE with State 3 need different DAI value setting. The UE which is configured State 1expects C-DAI/T-DAI increase by 1 based on the last DCI. But UE which is configured with State 3 does not expect increase C-DAI/T-DAI when disabling is indicated by DCI signaling. Thus, the procedure of Type-2 HARQ-ACK generation does not work well when State 1 and State 3 are configured simultaneously. For other combinations (e.g. State 1+State 2, State 2 and State 3) , the Type-2 HARQ-ACK construction is workable. For example, when only State 2 and State 3 are configured simultaneously, UEs which is configured State 2 will not feedback any HARQ-ACK information, and they does not need to read the C-DAI/T-DAI value. Thus, the Type-2 HARQ-ACK construction is workable when State 2 and State 3 are configured simultaneously.

Moreover, it is noted that when the State 3 is configured, the DCI signaling targets a group UEs. Thus, whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback is considered during last meeting. In our understanding, it is possible that the DCI signaling is invalid when the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling for a certain UE.

Proposal 4： State 1(ACK/NACK enabling) and State 3 (enabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback ACK/NACK) cannot be configured simultaneously.

Proposal 5: When only State 2 (RRC disabling) and State 3 (The function of group-common DCI based indication) are configured simultaneously, UE is allowed not to react to the DCI signaling but instead follow UE-specific RRC configuration for HARQ feedback.

When Type-1 HARQ-ACK codebook is configured, State 3 cannot be supported. The reason is that the construction of Type-1 HARQ-ACK codebook is based on each k1 value. In each slot that k1 configured, the feedback information is needed according to current specification. If the disabling is indicated by DCI, the feedback information will be not needed, which is conflict with the current construction of Type-1 HARQ-ACK codebook. Thus, the State 3 cannot work when constructing the Type-1 HARQ-ACK codebook.

Proposal 6: When Type-1 HARQ-ACK feedback is configured, State 3 (The function of group-common DCI based indication) is not supported.

For multicast transmission, the SPS scheduling mechanism is supported when the multicast traffic is periodic. Comparing to dynamic scheduling, it can save scheduling signaling. As shown in Table 2.1-1, SPS operation for a multicast transmission has four cases for HARQ-ACK enabling/disabling, the possible applying HARQ-ACK enabling/disabling to SPS group-common PDSCH is listed. For Case1, since the scheduling information (e.g. FDRA/TDRA information) will be also used for the subsequent scheduling, so it is very important for each UE in the multicast group. Thus, the HARQ-ACK feedback for the first active SPS PDSCH should always be enabled regardless of the HARQ-ACK enabling/disabling configuration. For the PDSCH transmission without DCI (Case 3), it is natural that the HARQ-ACK feedback can be based on the DCI indication of the active command when State 3 is configured.

Table 2.1-1: Applying HARQ-ACK enabling/disabling to SPS group-common PDSCH

| Use case | State-1 (enabling by RRC) | State-2 (disabling by RRC) | State-3 (determining by DCI) |
| --- | --- | --- | --- |
| Case1：Active by G-CS-RNTI scrambling DCI, | Enabling | Enabling | Enabling |
| Case2：Re-transmission by G-CS-RNTI scrambling DCI | Enabling | Disabling | By DCI |
| Case3：PDSCH transmission without DCI | Enabling | Disabling | By active DCI |
| Case4：De-active by G-CS-RNTI scrambling DCI. | Enabling | Disabling | By DCI |

Proposal 7: For SPS group-common PDSCH, HARQ-ACK feedback for the first active SPS PDSCH is always enabled regardless of HARQ-ACK enabling/disabling configuration.

Proposal 8: If HARQ-ACK enabling/disabling is determined by DCI, the HARQ-ACK feedback of the PDSCH transmission without DCI (Case3) can be based on the DCI indication of the activation command.

    - NACK-only based HARQ-ACK feedback

To indicate the PUCCH resource for NACK-only feedback, the ACK/NACK based feedback framework can be reused. Specifically, the PDSCH-to-HARQ feedback timing indicator (k1) can be used to indicate the PUCCH feedback slot and the PUCCH resource indicator (PRI) and DCI CCE index can be applied to indicate the PUCCH resource ID.

Proposal 9: NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

In Rel-15 NR, up to 4 PUCCH resource sets can be configured, and resource set 0 can be configured with up to 32 PUCCH resources for 1-2 bit(s) UCI transmissions. The PUCCH resource set can be used by all MBS services using NACK-only feedback. Thus, up to 32 PUCCH for 1-2bit UCI transmissions can also be configured.

Proposal 10: Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

Proposal 11: The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

    - HARQ-ACK codebook

It was agreed that Type-1 HARQ-ACK codebook and Type-2 HARQ-ACK codebook are supported for UEs to receive MBS. And multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast is supported. Some issues on HARQ-ACK codebook are discussed in this section.

- Construct separate HARQ-ACK codebook for multicast

In R16, two HARQ-ACK codebooks can be constructed, when the UE is indicated by pdsch-HARQ-ACK-CodebookList for unicast to generate two HARQ-ACK codebooks with different priorities. When the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0

Since two priority indexes (i.e. 0 and 1) are also introduced for multicast, two PUCCH-configs for multicast which correspond to two HARQ-ACK codebooks will also be needed for multicast. Thus, the simplest way is to configure a separate pdsch-HARQ-ACK-CodebookList for multicast to indicate the number of HARQ-ACK codebooks for multicast. Thus, when gNB configures the separate pdsch-HARQ-ACK-CodebookList for multicast, UE will assume that separate codebooks between unicast and multicast are needed.

Proposal 12: When PUCCH-ConfigurationList for multicast and PUCCH-ConfigurationList for unicast are separately configured, UE is expected to construct separate codebooks for multicast and unicast.

- Type-1 HARQ-ACK codebook

FDM-ed between unicast and multicast

For supporting FDM-ed unicast and multicast with same priority for Type-1 HARQ-ACK codebook, Opt 4 was agreed which is similar to joint Type-1 codebook for mTRP. The assumption is that UE reports the capability that two PDSCHS can be FDM-ed  in a slot, and gNB configures one multicast service (e.g multicast -1) and one unicast for a UE. The mTRP like scheme can be descried as following procedure

Step 1:

-	a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a unicast search space configuration .

-	a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a group common search space configuration for multicast-1 transmission.

-	serving cells are placed in a set according to an ascending order of a serving cell index

Step 2:

- the UE generates a Type-1 HARQ-ACK codebook for the set  S0 and the set S1 of serving cells separately

- The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1.

However, if gNB configures two or more multicast services on a serving cell, for example, multicast-1 PDSCH is scheduled by DCI scrambled by G-RNTI-1, multicast-2 PDSCH is scheduled by DCI scrambled by G-RNTI-2, there are three FDM-ed cases in a same slot as shown in Figure 2.1-1.

Figure 2.1-1: Three FDM-ed cases for unicast+multicast

For Type-1 HARQ-ACK codebook construction, the simplest way is to add another severing cell set S2, and UE will generate three sub-codebook where the first sub-codebook is for unicast, the second sub-codebook is for multicast-1, and the third sub-codebook is for multicast-2. It increases the HARQ-ACK codebook size unnecessarily since only up to two FDM-ed PDSCHs are supported in a same slot. So when the number of services configured including unicast and multicast is greater than the number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

Proposal 13: when the number of services configured including unicast and multicast is greater than number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

TDM-ed between unicast and multicast

For TDM-ed unicast and multicast, when Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, two alternatives of determining PDSCH reception candidate occasions were discussed during RAN1#105-e meeting.

For Alt 1, when UE determines the PDSCH reception candidate occasion, only the slots in the intersection for k1 set for multicast and k1 set for unicast can use the union of PDSCH TDRA set. This alternative can reduce redundant reception occasions of PDSCH, and increase the efficiency of Type-1 HARQ-ACK feedback.  For Alt 2, when UE determines the PDSCH reception candidate occasion, the union of PDSCH TDRA set is used for each slot. Although Alt 2 brings some redundant reception occasions of PDSCH, it is simple and has low complexity for UEs.  Further, the number of redundant bits depends on two factors: one factor is the number of  in the intersection of  set; another factor is the difference of the number of set between union of PDSCH TDRA sets and the PDSCH TDRA set for unicast/multicast set. Thus, in our opinion, the Alt 2 can be used as the baseline scheme, and the Alt 1 is the used as the enhancedscheme.

Proposal 14：Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction, and Alt 2 can be used as baseline scheme.

- Type-2 HARQ-ACK codebook

It has been agreed that concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported. The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast. The HARQ-ACK codebooks for MBS and unicast are determined independently by reusing Rel-15 HARQ-ACK feedback codebook determination mechanism since the DAI for unicast and DAI for multicast are separately counted.

In R15, up to two Type-2 HARQ-ACK sub-codebooks can be concatenated in a PUCCH/PUSCH, one is TB-based HARQ-ACK sub-codebook and the other is CBG-based HARQ-ACK sub-codebook. For multicast service transmission, one G-RNTI maps to one multicast service. When N G-RNTIs (N≥1) is configured in a PUCCH cell group, N Type-2 HARQ-ACK sub-codebooks for multicast are generated and will be concatenated in a PUCCH/PUSCH.  However, as N is increasing, the following issues are arisen:

- Issue#1: “last DCI” problem becomes serious

Generally, for each sub-codebook, when the “last DCI” is missing, the HARQ-ACK codebook size will mismatch between the UE and gNB, which is unacceptable. Thus, with the increase of N, the probability of last DCI missing issue increases as shown in Table 2.1-2, where the probability of PDCCH detecting correctly is assumed as 95%.

Table 2.1-2：Probability of last DCI missing

| N (total number of sub-codebooks) | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Probability of last DCI issue | 9.75% | 14.26% | 18.55% | 22.62% | 26.49% | 30.17% | 33.66% |

- Issue #2: UL DAI overhead issue

In R15 operation, the total UL DAI size is 4 bits and up to 2 DAI fields are included in format 0_1/0_2. The first DAI filed is used for TB-based HARQ-ACK sub-codebook, and the second DAI filed is used for CBG-based HARQ-ACK sub-codebook. However, the UL DAI overhead will increase with the increasing of N for multicast, as shown in Table 2.1-3. The number of Type-2 HARQ-ACK sub-codebooks for multicast can be larger than one, but the number shall be limited within no more than 3 so that the overhead of the UL DAI is acceptable.

Table 2.1-3：UL DAI overhead

| N (total number of sub-codebooks) | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Overhead of UL DAI (bit) | 4 | 6 | 8 | 10 | 12 | 14 | 16 |

Proposal 15: Support concatenating no more than 3 Type-2 HARQ-ACK codebooks for multicast scheduling.

If multiplexing of more than one Type-2 HARQ-ACK codebooks for multicast is supported, the concatenating rule in R15/R16 is applied. Before concatenating the different multicast sub-codebook into one sub-codebook, the gNB can pre-define the concatenating order based on the G-RNTI increasing order.

Figure 2.1-2 Concatenating for more than one Type-2 HARQ-ACK codebooks

As shown in Figure 2.1-2, there are two sub-codebook of unicast (i.e. Unicast sub-codebook1 and Unicast sub-codebook2); UE will concatenate these two sub-codebooks into a union unicast sub-codebook (i.e. Unicast sub-codebook) according to the rules in R15/R16. Similarly, three sub-codebooks of multicast (i.e. Multicast sub-codebook1, Multicast sub-codebook2, and Multicast sub-codebook3) can be concatenated in to a Multicast sub-codebook according to R15/R16 rules. The order of concatenating multicast sub-codebooks can be based on the increasing order of G-RNTI. Then, UE appends the Multicast sub-codebook after the Unicast sub-codebook.

Proposal 16: For concatenating more than one Type-2 HARQ-ACK codebooks for multicast, the order between sub-codebooks can be based on the increasing order of G-RNTI.

    - PUCCH resource determination

In R15/R16, when PUCCHs with the same priority are scheduled on same slot (no matter overlapping or not), multiple sub-codebooks can be concatenated into one HARQ ACK codebook and UE will feedback the HARQ-ACK codebook on a PUCCH that the last DCI indicates. The PRI in the last DCI indicates the PUCCH resource. For multiplexing the HARQ-ACKs of multicast and unicast, there are two possible alternatives to interpret the “last DCI”.

- Alt.1: the last DCI for unicast

The benefit of using last DCI for unicast is that both PUCCH resource configuration and the PRI indication for unicast are UE specific. This enables gNB to schedule the orthogonal PUCCH resources among UEs. But Alt.1 requires a unicast PDSCH always to be scheduled. Moreover, the number of unicast DCIs shall be more or equal to four to support the scenario where at most three consecutive DCI are missing. Thus, Alt.1 may cause high limitations and bring negative impact on downlink scheduling flexibility.

- Alt.2: the last DCI across unicast and multicast

This alternative can keep same understanding as R15/R16 and no more specification modification are needed. When the last DCI is unicast, the benefit of Alt.1 can be obtained; when the last DCI is multicast, gNB will guarantee the PUCCH resource are orthogonal among  different UEs in a same group, which was already supported. If it is difficult for gNB to assign the orthogonal PUCCH resource, based on gNB’s implementation, gNB can make unicast DCI as last DCI.

Proposal 17: For multiplexing the HARQ-ACKs of multicast and unicast, the last DCI refers to the last DCI across unicast and multicast.

  - Multiple transmission

- Multiple SSB beams sweeping

In Rel-15, Paging and SIBx message will be broadcasted for all UE in a cell. In single-beam operation, only one SSB is configured. The messages are transmitted one time within a configured window. In multi-beam operation, more than one SSB are configured in SIB1, and UE assumes that the same paging message and the same SIBx message are repeated in all transmitted SSB-beams.

For a MBS traffic transmission, the interested UEs may be anywhere in a cell. In order to provide good coverage for all UEs, gNB should repeat the same MBS traffic data in all transmitted SSB-beams, especially for the interested UEs in IDLE/IN_ACTIVE state.

Proposal 18: To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

For UE in the center of SSB-beam area, MBS receiving performance may be good enough. However, for the UE in the edge of SSB-beam area, the downlink channel state may be worse no matter UE selects which one SSB-beam to receive MBS data. Since same MBS traffic data may be repeatedly transmitted in neighbor SSB-beams, soft combination of the data from current SSB-beam and neighbor SSB-beams can be applied to improve the reliability of the MBS reception. For implementation, gNB should configure the adjacent SSB-beams in MCCH or through RRC signaling, and the soft-combination indication is transmitted in MBS scheduling signaling.

Proposal 19: UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

- Retransmission schemes

It has been agreed that the PTP and PTM scheme can be the retransmission schemes when initial transmission for multicast is based on PTM transmission scheme 1.Currently, CBG based retransmission is indicated by DCI format 1_1/1_2 for unicast transmission.  If CBG based transmission of unicast is configured for a UE, CBG based PTP retransmission can be used for delivering MBS services. However, supporting CBG based PTM scheme -1 will introduce complexity of DCI format design, and there is not obvious benefit because different UEs may have different decoding results on CBGs. Thus, only CBG based PTP retransmission can be supported in multicast.

Proposal 20: CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

  - CSI report in MBS

To improve the reliability of unicast transmission, apart from HARQ-ACK feedback, CSI report should also be supported. CSI report is used for link adaptation with various modulation schemes and channel coding rates, which can improve the radio resource efficiency. For channel state measurement and report, the UE can be configured with CSI-MeasConfig which includes the reference signal configuration and measurement report configuration. Similar with unicast CSI feedback, UE is configured by RRC signalling to estimate channel state and reports it to gNB, and it is not necessary for the UE to know the CSI report is used for unicast or MBS transmission. Transparent principle for UE of reporting CSI minimizes standardization work.

It was agreed to further study on the CSI feedback enhancement for MBS with some details listed above. All of these enhancements need specification changes but the potential enhancement or benefit is not quite clear or worth.

On the other hand, for MBS transmission, link adaptation is based on multi-UE’s CSI report. Since there are a lot of UEs to receive the MBS PDSCH at the same time and the location of UEs are scattered in a cell, the benefit of AMC according to multi-UE CSI reports may not be obvious for the MBS transmission compared with unicast.

Above all, the benefit of CSI enhancement for MBS can be further studied. The discussion priority can be low by considering potential specification work.

Proposal 21: CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

- Conclusion

In this document, the issue on improving reliability of Broadcast/Multicast service is discussed. The proposals are given as following:

Proposal 1: For DCI indicating the HARQ ACK enabling/disabling, Alt 3 (A special state of an existing DCI field) and Alt 4(Implicit indication) can be supported.

Proposal 2: The enabling/disabling ACK/NACK-based HARQ-ACK feedback mechanism can be used for NACK-only based HARQ-ACK feedback.

Proposal 3: The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling same as initial transmission.

Proposal 4： State 1(ACK/NACK enabling) and State 3 (enabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback ACK/NACK) cannot be configured simultaneously.

Proposal 5: When only State 2 (RRC disabling) and State 3 (The function of group-common DCI based indication) are configured simultaneously, UE is allowed not to react to the DCI signaling but instead follow UE-specific RRC configuration for HARQ feedback.

Proposal 6: When Type-1 HARQ-ACK feedback is configured, State 3 (The function of group-common DCI based indication) is not supported.

Proposal 7: For SPS group-common PDSCH, HARQ-ACK feedback for activation command is always enabled regardless of HARQ-ACK enabling/disabling configuration.

Proposal 8: If HARQ-ACK enabling/disabling is determined by DCI, the HARQ-ACK feedback of the PDSCH transmission without DCI (Case3) can be based on the DCI indication of the activation command.

Proposal 9: NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

Proposal 10: Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

Proposal 11: The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

Proposal 12: When PUCCH-ConfigurationList for multicast and PUCCH-ConfigurationList for unicast are separately configured, UE is expected to construct separate codebooks for multicast and unicast.

Proposal 13: when the number of services configured including unicast and multicast is greater than number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

Proposal 14：Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction, and Alt 2 can be used as baseline scheme.

Proposal 15: Support concatenating no more than 3 Type-2 HARQ-ACK codebooks for multicast scheduling.

Proposal 16: For concatenating more than one Type-2 HARQ-ACK codebooks for multicast, the order between sub-codebooks can be based on the increasing order of G-RNTI.

Proposal 17: For multiplexing the HARQ-ACKs of multicast and unicast, the last DCI refers to the last DCI across unicast and multicast.

Proposal 18: To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

Proposal 19: UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

Proposal 20: CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

Proposal 21: CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

- References

3GPP TSG RAN WG1 Meeting RAN1 #105-e chairman notes, May 19th – 27th, 2021.